3GPP TSG RAN WG1 Meeting #99                                                           	       R1-1913291

Reno, USA, November 18th – 22nd, 2019

Agenda Item:	7.2.2.2.2

Source: 	Intel Corporation

Title:	Enhancements to initial access and mobility for NR-unlicensed

Document for: 	Discussion/Decision

# Introduction

In this contribution, the following aspects related to initial access and mobility procedures are addressed:

- PRB offset values and indication of Q

- Remaining issues on PDCCH monitoring occasions

- ssb-PositionsInBurst functions

- RLM

- Enhancements to the 4-step random access procedure

# PRB offset values and indication of Q

The relevant RAN4 agreement is shown below:

- Definition of synchronization raster for band n46 in RAN4 should take at least the following principles into account: - Minimum frequency separation from sub-band edge to guarantee compliance with inter-carrier and intra-carrier minimum guard-bands for all possible channel bandwidths - 925kHz for inter-carrier guardband (80MHz channel bandwidth) - 3 full RBs (30kHz SCS) for intra-carrier guardbands - Maximize spectrum utilization for the case in which SSB is frequency multiplexed with RMSI or CSI-RS - The GSCN point closest to the edge and compliant with inter and intra-carrier guardbands shall be selected - Considering a frequency offset of minimum 3 full RBs at 30kHz SCS to account for intra-carrier guardbands for each sub-band, define the following GSCN points: - GSCN = {[8996], 9010, 9024, [9037/9038], 9051, 9065, 9079, 9093, 9107, [9121], [9218], 9232, 9246, 9260, 9274, 9287, 9301, 9315, 9329, 9343, 9357, [9371], 9385, 9402, 9416, 9430, 9444, 9458, [9471], [9485], [9499], [9513]} - Proposed values are conditioned on NR minimum channel guard bands - If it is later found that larger guard bands are needed, it is common understanding that some of the SSB points may need to be changed

RAN4 in the last meeting has mostly agreed on NR-U channel raster entries for 20, 40, 60 and 80 MHz channel bandwidths as well as SS raster entries for band n46 (although some of the SS raster entries are not finalized yet). In the case of 20 MHz channels, the minimum (inter-carrier) guard band requirements are available. It is therefore, straightforward to check the number of PRB offsets necessary to position CORESET#0 within the maximum initial BWP that can be defined for each of these 20 MHz channels. In our preliminary analysis (assuming 30 kHz SCS) a PRB offset of 0 and 1 is sufficient for this purpose.

In the case of wideband channels of 40, 60 and 80 MHz channel bandwidths, multiple SS raster entries reside in each of the channels. Then minimum inter-carrier and intra-carrier guard bands corresponding to 80 MHz can be safely used to check the number of PRB offsets necessary to position CORESET#0 within the (multiple) maximum initial BWPs that can be defined within each of the wideband channels.

In the Appendix we provide the analysis on the required SSB to CORESET#0 offset for 20 MHz CA and WB (40 MHz, 60 MHz, 80 MHz) system operation. Note that:

- An inter-carrier guard band of 925 kHz is used as per RAN4 agreement

- An intra-carrier guard band of minimum 3 PRBs is used as per RAN4 agreement. This results in 7 PRBs of guard band across two LBT sub-bands given that the 20 MHz channel boundaries are not PRB aligned

- We observe that CORESET#0 to SSB offset values of 0, 1 is sufficient for 20 MHz. In the case of 40 MHz, 60 MHz, 80 MHz operation, CORESET#0 to SSB offset values of 0, 1, 2 enables placing multiple PCells in wideband operation to a certain extent. Note that not all GSCN values can be used as a PCell as shown by the orange shaded GSCN values.

- The number of usable PRBs in a LBT sub-band is observed to be at least 50 for 20 MHz, at least 49 for 40 MHz, at least 49 for 60 MHz and at least 48 for 80 MHz operation. Therefore the flexibility of placing a 48 PRB CORESET gets more limited for larger BW operation and consequently more offsets are needed for WB operation.

Proposal-1: Support CORESET#0 to SSB offset values of 0, 1 and 2.

# Remaining issues on PDCCH monitoring occasions

Agreement RAN1#98: If Q is known, candidate monitoring slots for Type0 PDCCH search space are the PDCCH monitoring slots associated with SS/PBCH blocks that are QCL with the SS/PBCH block from which the UE determines that a CORESET for Type0-PDCCH CSS set is present - Note: Q may be always known depending on where Q is signalled. This aspect is to be discussed further.

In Rel-15, a UE is able to receive DCI formats with CRC scrambled with C-RNTI on a search space set associated with CORESET#0 (assume active BWP is same as initial BWP). In such case a UE monitors corresponding PDCCH candidates only at monitoring occasions associated with a SS/PBCH block index which is either indicated by MAC-CE or known through a random access procedure. A natural extension of the above agreement is to require the UE to monitor corresponding PDCCH candidates at monitoring occasions associated with a set of SS/PBCH block indices that are QCL.

Proposal-2: Extend PDCCH monitoring of a search space associated with CORESET#0 to a set of SS/PBCH block indices that are QCL

# ssb-PositionsInBurst functions

The actual transmitted SSBs are indicated using ssb-PositionsInBurst bitmap. A UE should expect (or assume) that the bit positions beyond Q is set to zero as shown below:

Figure 1: ssb-PositionsInBurst interpretation with Q=4

PDSCH rate matching

In Rel-15, each bit in ssb-PositionsInBurst represents an SS/PBCH block index and a bit set to 1 is used for PDSCH rate-matching with respect to the corresponding SS/PBCH block index (except for SIB1). In NR-U, depending on the value of Q, multiple SS/PBCH block indices are QCL and it is natural to consider a bit in ssb-PositionsInBurst to represent a SS/PBCH block set that is QCL. This allows a UE to perform PDSCH rate-matching with respect to a corresponding SS/PBCH block set. As an example if Q=8, the first bit in ssb-PositionsInBurst represents SSB index = 0; if Q=4 the first bit in ssb-PositionsInBurst represents SSB indices = 0, 4; if Q=2 the first bit in ssb-PositionsInBurst represents SSB indices = 0, 2, 4, 6; if Q=1 the first bit in ssb-PositionsInBurst represents SSB indices = 0, 1, 2, 3, 4, 5, 6, 7.

SSB to PRACH mapping

In Rel-15, SS/PBCH block indexes provided by ssb-PositionsInBurst are mapped to valid PRACH occasions based on first preamble indices, then frequency indices of PRACH occasions, then time resource indices within a slot and then increasing order of slots. Following the same principles, one or more SS/PBCH block sets should be provided by ssb-PositionsInBurst and a SS/PBCH block set can be mapped to valid PRACH occasions based on Rel-15 rules.

In Rel-15, as part of SSB to RACH mapping, we have the following:

- a number N of SS/PBCH blocks is associated with a PRACH occasion. If N < 1, one SS/PBCH block is mapped to 1/N consecutive valid PRACH occasions.

- a number R of contention based preambles per SS/PBCH block per valid PRACH occasion

- a mapping of preamble indices to SS/PBCH block indices

In NR-U, based on the value of Q, a set of SS/PBCH block indices are QCL. As a consequence, a QCL-ed SS/PBCH block set should be considered for SSB to RACH mapping. Specifically

- a number N of SS/PBCH block sets should be associated with a PRACH occasion. If N < 1, one SS/PBCH block set should be mapped to 1/N consecutive valid PRACH occasions.

- a number R of contention based preambles per SS/PBCH block set per valid PRACH occasion

- a mapping of preamble indices to SS/PBCH block sets

where a SS/PBCH block set comprises of a set of SS/PBCH block indices that are QCL (based on Q value).

Another related aspect of mapping SS/PBCH block indices to PRACH occasions based on ssb-PositionsInBurst is addressed in section 6.

Just for readability purposes in 38.213/38.214, it may be reasonable to define a SS/PBCH block set index given by mod(A,Q) associated with SS/PBCH block index A or PBCH DMRS sequence index A. This way, the changes related to the above sections could be implemented by simply replacing SS/PBCH block or SS/PBCH block index in L1 specifications with SS/PBCH block set index.

Proposal-3: A bit position in ssb-PositionsInBurst should indicate a set of SS/PBCH block indices that are QCL. This new interpretation affects PDSCH rate-matching and mapping of SSB indices to PRACH occasions.

# RLM

Agreement RAN1#97: For SSB-based RLM, UE may assume the RLM measurement window to be the same as the DRS transmission window. - Note: This implies that the SSB-based RLM-RS cannot fall outside the measurement window - FFS: Whether and how DRS transmission window is configured to the UE

OOS probability is significantly increased due to medium congestion

As an example, let us assume that for a particular UE, the probability that link quality is poor is 20%. Let us assume 3 different medium congestion scenarios, with LBT failure probability = 0%, 20% and 80%. Then we have the following situation based on Rel-15 specifications:

Table 1: Example of probability of detection for various OOS indicators and IS indicator

| Case | L1 to higher layer indicator | LBT failure = 0% | LBT failure = 20% | LBT failure = 80% |
| --- | --- | --- | --- | --- |
| 1 | IS (link good and LBT success) | 80% | 64% | 16% |
| 2 | OOS (link poor & LBT success) | 20% | 16% | 4% |
| 3 | OOS (link good & LBT failure) | 0% | 16% | 64% |
| 4 | OOS (link poor & LBT failure) | 0% | 4% | 16% |
|  | Total OOS % | 20% | 36% | 84% |

From the above, we observe that ideally (for example in licensed band), OOS probability = 20%. Depending on the medium congestion, this OOS probability does up to 36% (when LBT failure is 20%) and up to 84% (when LBT failure is 80%).

OOS suppression mechanism

Figure 2: Case 2 in Table 1 includes the case where RLM-RS is successfully detected and SINR is below the SINR threshold for OOS detection as shown in the shaded region of the figure

From Table 1 and Figure 2 we observe that LBT success may be detected at the UE based on SSB detection or other side information (like remaining COT time). We denote this indicator as failure to detect (FTD) = False. Based on such information, Case 2 OOS indications could weighted with respect to Case 3/Case 4 OOS indications in 80:20 ratio as shown below.

Table 2: Modified probability of detection of OOS after weighting using a third indicator (FTD)

| Case | L1 to higher layer indicator | LBT failure = 0% | LBT failure = 20% | LBT failure = 80% |
| --- | --- | --- | --- | --- |
| 1 | IS & FTD=False (link good and LBT success) | 80% | 64% | 16% |
| 2 | OOS & FTD=False (link poor & LBT success) | 20% x 0.8=16% | 16% x 0.8=12.8% | 4% x 0.8=3.2% |
| 3 | OOS & FTD=True (link good & LBT failure) | 0% | 16% x 0.2=3.2% | 64% x 0.2=12.8% |
| 4 | OOS & FTD=True (link poor & LBT failure) | 0% | 4% x 0.2=0.8% | 16% x 0.2=3.2% |
|  | Total OOS % | 16% | 16.8% | 19.2% |

From Table 2 above, we observe that suppression or weighting of some OOS indicators can help to reduce the false OOS indicators due to LBT failure (to a certain extent) although such mechanism is non-trivial and may need to adjust to different congestion levels. Then we have the following observations:

- OOS probability depends heavily on the medium congestion and may not reflect true link quality due to congestion

- In order to reduce false RLF a mechanism is needed to suppress some OOS indications. If it is left to UE implementation then there will be no such requirement (from RAN4 perspective) and the NW will have no control over the responsiveness of the RLM procedure because the UE autonomously will suppress certain OOS indicators.

Proposal-4: Specify a third indicator type (e.g. failed to detect – FTD indication) to higher layers in addition to IS and OOS indications.

# Enhancements to the 4-step random access procedure

In previous RAN1 meetings during the study on NR-based access to unlicensed spectrum the following are some of the agreements that were made related to initial access and mobility:

Agreement (RAN1#94bis):

Following options have been identified for potential RACH resource enhancements in NR-U beyond the flexibility already available in Rel-15:

- Frequency-domain enhancement

  - Multiple PRACH resources across multiple LBT sub-bands/carriers for both contention-free and contention-based RA

- Time-domain enhancements

  - For connected mode UE, scheduling of PRACH resources via DCI.

    - Triggered PRACH within TXOP can use a new resource

  - For idle mode UE, scheduling of PRACH resources via paging

    - Note: potential inefficiency in network resource due to paging across multiple cells

  - Additional, new RACH resources are used immediately following detection of DRS transmission

  - Multiple PRACH transmissions before Msg2 reception in RAR window for initial access

    - Number of allowed transmissions is pre-defined or indicated, e.g., in RMSI

    - FFS: How to handle potential multiple RARs to same UE

  - Group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain

Release 15 NR uses 4-step random access procedure like LTE. The 4-step random access procedure consists of Msg-1 (PRACH), Msg-2 (RAR), Msg-3 (RRC connection request) and Msg-4 (contention resolution) transmissions. As per TR38.889, 4 –step RACH procedure can be considered as the baseline for NR-unlicensed and mechanisms to handle LBT related aspects (e.g. reduced transmission opportunities for Msg. 1/2/3/4) can be further studied. Moreover, several frequency and time domain enhancements were identified in TR38.889 as potential options to RACH resource enhancements for unlicensed operation, as mentioned in the above agreement. In this regard, the following enhancements are envisioned for 4-step random access procedure.

## Msg1 enhancement

The following agreement was made in RAN1 in the past regarding channel access mechanism during initial access:

Agreement:

- Initial active DL/UL BWP is approximately 20MHz for 5GHz band

  - The final value will be quantized to number of PRBs

- Initial active DL/UL BWP is approximately 20MHz for 6GHz band if similar channelization as 5GHz band is used for 6GHz band

As per the above agreement, the initial active UL BWP for 5 GHz band is subject to one sub-band LBT (20 MHz). Consider that a single PRACH resource is configured within one configured initial UL BWP to an idle mode UE for initial access. If 20 MHz spectrum of the UL BWP that includes the PRACH resource for CBRA is occupied by other neighbouring network, the preamble transmission has to be delayed to the next RACH occasion in time domain and, thereby, the overall initial access delay will increase.

In order to overcome the abovementioned issue, it can be considered that gNB can configure multiple initial BWPs, each with a PRACH resource configuration, as illustrated in the figure below. The configured BWPs may be contiguous or non-contiguous across frequency. Out of these multiple configured BWPs, one single UL BWP is activated based on LBT outcome (each BWP being subject to independent 20 MHz sub-band LBT) at the UE side and UE transmits PRACH on this single activated BWP. In case CCA is successful on multiple configured BWPs, UE selects one BWP out of these multiple candidates as the initial active UL BWP and use the corresponding PRACH resource to transmit the preamble.

Figure 3: Multiple PRACH resource configurations in frequency domain on multiple configured initial UL BWPs

Proposal-5: NR-unlicensed supports multiple PRACH resource configurations in the frequency domain on multiple configured UL BWPs.

## Msg2 enhancement

In the last RAN1 meeting [4], the following agreement was made related to Msg2 enhancement during initial access:

Agreement:

LBT category for msg 3 initial transmission is provided to the UE in RAR

The main motivation behind the above agreement is to facilitate COT sharing between Msg2 an Msg3 transmissions, such that the LBT congestion for Msg3 is potentially alleviated. In Rel-15 NR, the MAC RAR format is octet aligned and from specification impact perspective, it would be desirable to keep MAC RAR format unchanged while providing LBT category to the UE in RAR.

Figure 4: MAC RAR (octet format) in Rel-15 NR

One way to indicate LBT category via RAR without changing the existing MAC RAR octet alignment would be to embed the COT sharing information within RAR UL grant in such a way, that the total number of UL grant field bits (27 bits in Rel-15 NR) remains the same.

Table 3 RAR UL grant content field in Rel-15 NR

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| PUSCH frequency resource allocation | 14 |
| PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for PUSCH | 3 |
| CSI request | 1 |

If separate field is introduced for LBT scheme indication (including LBT category and Msg3 PUSCH starting position) in the UL grant field, up to 3 bits from the existing bit fields can be extracted/repurposed and allocated to the newly introduced field, so that 27 bits in total is maintained within RAR UL grant [5]. Examples of few existing grant field(s)/bit(s) that can be repurposed and assigned to LBT scheme indication field are: CSI request (1 bit is “reserved”), Frequency hopping flag (1 bit- can be repurposed since frequency hopping may not be necessary for interlace based Msg3 PUSCH transmission in NR-unlicensed), PUSCH frequency resource allocation (14 bits are available, out of which maximum 10 bits will be required for interlace based PUSCH resource allocation within initial active UL BWP of 20 MHz) etc.

The bit(s) corresponding to the LBT scheme indication field can at least indicate CAT2 or CAT4 LBT (depending on whether COT sharing is enabled or disabled).

Proposal-6: LBT scheme and Msg3 PUSCH starting position are indicated as a separate field of UL grant in RAR.

- LBT scheme can at least indicate whether the LBT is CAT2/4.

- FFS: indication of CAT1.

- FFS: details of the UL grant fields.

To take into account the possible delay in RAR transmission due to LBT congestion in unlicensed operation, it has been agreed in RAN2 to extend the RAR window size (from 10 ms to [20] ms) for NR-unlicensed operation. In NR, The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in subclause 5.3.2 in 3GPP TS 38.211 v15.5.0, f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

RA-RNTI calculation doesn’t take into account System Frame Number (SFN) in NR, since max configurable RAR window size in NR is 10 ms (1 frame) and the minimum periodicity with which same RO (i.e. same RA-RNTI) can occur is 10 ms or 1 frame as well (nSFN mod x=y; x=1, y=0), which implies that within RAR window, RA-RNTIs are unique.

But with enhancement of RAR window size beyond 10 ms, there’s a possibility with the current NR RA-RNTI calculation framework that multiple ROs corresponding to the same RA-RNTI may occur within the extended RAR window. Hence, if more than one UEs happen to use these ROs for preamble transmission, then due to overlap in their RAR window, RAR with the same RA-RNTI may be received by these UEs. There would be no way to distinguish between these RARs unless these RA-RNTIs are made unique or SFN is conveyed in the DCI/RAR.

Since there are already 16 reserved bits in DCI format 1_0 (with CRC scrambled with RA-RNTI), the most straightforward way to include SFN information is to use 1-bit out of these 16 reserved bits to indicate 1-bit LSB of SFN, which is sufficient for 20 ms RAR window.

Proposal-7: Include SFN information in DCI format 1_0 (CRC scrambled with RA-RNTI) using the reserved bits to distinguish between RA-RNTIs associated with RARs for different UEs received within overlapping RAR window (enhanced beyond 10 ms) in NR-unlicensed.

## Msg3 enhancement

RAN2 had sent an LS to RAN1 [3] requesting for RAN1’s feedback on support of multiple msg3 transmission opportunities for 4-step RACH in NR-unlicensed. RAN2 had discussed multiple msg3 transmission opportunities to alleviate the LBT impact and inferred that multiple grants can be provided to UE via a single RAR or having multiple RARs. In addition, RAN2 also discussed the possibility of msg2 sharing COT with msg3, which can be an alternative way to reduce LBT congestion for msg3 transmission.

In replying to an LS from RAN2, RAN1 made the following agreement in the last meeting [4]:

Agreement:

Reply to the RAN2 LS informing them of the following:

- RAN1 has made the following agreement which facilitates COT sharing between Msg2 and Msg3:

  - LBT category for msg 3 initial transmission is provided to the UE in RAR

- Multiple msg3 tx opportunities with a single or multiple RARs in the time domain is feasible from a RAN1 perspective but there is no consensus at this time in RAN1 to support this. RAN1 will continue discussions on the support of multiple msg3 tx opportunities.

From RAN1’s perspective, there haven’t been any discussion yet regarding the need of multiple msg3 transmission opportunities from physical layer perspective. However, RAN1 has already agreed to facilitate COT sharing between Msg2 and Msg3, as mentioned in the agreement quoted above. In this case, depending on the UL grant, the gap between msg3 and the end of DL burst within the gNB’s shared COT can be adjusted such that msg3 can either be transmitted without performing any LBT (for gap <16 µs) or using single-shot (or CAT 2) LBT. With high priority channel access provision for msg3 transmission within gNB’s shared COT, any further enhancement in RACH procedure (e.g. multiple msg3 transmission opportunities) may not bring any significant improvement to the performance of RACH procedure. Furthermore, HARQ is already supported for msg3 transmission. Therefore, even though the UE misses the chance for transmitting initial transmission of msg3 due to LBT failure, additional transmission of msg3 can be performed by the form of HARQ retransmission. Support of HARQ for msg3 also decreases the motivation to introduce multiple opportunities for msg3. Considering the above aspects, in our view, support of multiple Msg3 transmission opportunities is not necessary.

Proposal-8: Do not support scheduling of multiple transmission opportunities for Msg3 transmission.

# Conclusions

In this contribution, we made the following proposals/observations:

Proposal-1: Support CORESET#0 to SSB offset values of 0, 1 and 2.

Proposal-2: Extend PDCCH monitoring of a search space associated with CORESET#0 to a set of SS/PBCH block indices that are QCL

Proposal-3: A bit position in ssb-PositionsInBurst should indicate a set of SS/PBCH block indices that are QCL. This new interpretation affects PDSCH rate-matching and mapping of SSB indices to PRACH occasions.

Proposal-4: Specify indicating a third indicator type (e.g. failed to detect – FTD indication) to higher layers in addition to IS and OOS indications.

Proposal-5: NR-unlicensed supports multiple PRACH resource configurations in the frequency domain on multiple configured UL BWPs.

Proposal-6: LBT scheme and Msg3 PUSCH starting position are indicated as a separate field of UL grant in RAR.

- LBT scheme can at least indicate whether the LBT is CAT2/4.

- FFS: indication of CAT1.

- FFS: details of the UL grant fields.

Proposal-7: Include SFN information in DCI format 1_0 (CRC scrambled with RA-RNTI) using the reserved bits to distinguish between RA-RNTIs associated with RARs for different UEs received within overlapping RAR window (enhanced beyond 10 ms) in NR-unlicensed.

Proposal-8: Do not support scheduling of multiple transmission opportunities for Msg3 transmission.

# References

[1] RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm Inc, 3GPP TSG RAN Meeting #82, Dec. 2018.

[2] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February, 2019.

[3] R2-1905444, “LS in RACH agreements for NR-U”, 3GPP TSG RAN WSG2 Meeting #105bis, Xi’an, China, April 2019.

[4] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #97, Reno, NV, USA, May, 2019.

[5] R1-1910640, “Channel access mechanism for NR-unlicensed”, Intel Corporation, 3GPP TSG RAN WG1 Meeting #99, Nov. 2019.

# Appendix

## 20 MHz

## 40 MHz

## 60 MHz

## 80 MHz