3GPP TSG RAN WG1 #99			R1-1913348

Reno, USA, 18th – 22nd, Nov. 2019

Source:	NTT DOCOMO, INC.

Title:	Summary#2 on UE feature related discussion

Agenda Item:	7.1.6

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the UE feature related discussions and proposals in AI 7.1.6.

- Discussion on remaining issues on Rel-15 UE features

2.1	Issue on XDD-FRX capability signaling [1]

At the RAN1#98bis meeting, an LS from RAN2 was received in [2] on FDD-TDD and FR1-FR2 capability differentiation. After the discussion in the meeting and on the email reflector, a RAN1 response was agreed in [3]. In [1], Qualcomm raised following issue and corresponding proposals to remove any ambiguity regarding XDD-FRX capability signaling.

When a particular capability has both FDD-TDD and FR1-FR2 differentiation, there are 4 capability bits enabling 16 ‘codepoints’, but there are effectively only 8 capability cases, which are the combination of support/no-support for the following three independent capabilities: - FR1 FDD support Yes/No - FR1 TDD support Yes/No - FR2 support Yes/No The first question that may be raised is that there are ‘erroneous’ settings possible. For example, assume the UE sets the available bits for a particular capability as follows: Example 1: - FDD support: Yes, TDD support: No - FR1 support: No, FR2 support: Yes Obviously, this UE does not seem to support the feature in any band, so it could have or should have set all bits to No. However, it does not seem absolutely necessary to clarify whether signaling such capability combination is allowed or not, it can be assumed that the UE would not use such signaling. There are other more important cases though. Consider for example, how the UE should set the capability bits in the following case: Example 2: - FR1 FDD support: Yes - FR1 TDD support: No - FR2 support: Yes The following setting would be possible for Example 2: - FDD support: Yes, TDD support: No - FR1 support: Yes, FR2 support: Yes However, it is somewhat unclear if all networks would interpret this setting as one corresponding to Example 2. Some networks could interpret this an erroneous setting. To remove any ambiguity, we propose to agree on the interpretation given in Table 1 for the capability bits: FDD-TDD Diff; FR1-FR2 Diff; Capability per UE; Capability setting in FDD; Capability setting in TDD; Capability setting in FR1; Capability setting in FR2; Implied capability in FR1-FDD; Implied capability in FR1-TDD; Implied capability in FR2. N; N; N; N/A; N/A; N/A; N/A; N; N; N. Y; Y; Y; Y. N; Y; N/A; N/A; N/A; N; N; N; N; N. N; Y; N; N; Y. Y; N; Y; Y; N. Y; Y; Y; Y; Y. Y; N; N; N; N/A; N/A; N; N; N. N; Y; N; Y; Y. Y; N; Y; N; N. Y; Y; Y; Y; Y. Y; Y; N; N; N; N; N; N; N. N; Y; N; N; Y. Y; N; N/A; N/A; N/A. Y; Y; N/A; N/A; N/A. N; Y; N; N; N/A; N/A; N/A. N; Y; N/A; N/A; N/A. Y; N; N; Y; N. Y; Y; N; Y; Y. Y; N; N; N; N/A; N/A; N/A. N; Y; N/A; N/A; N/A. Y; N; Y; N; N. Y; Y; Y; N; Y. Y; Y; N; N; N/A; N/A; N/A. N; Y; N/A; N/A; N/A. Y; N; Y; Y; N. Y; Y; Y; Y; Y. Table 1 Interpretation of capability bits with FDD-TDD and/or FR1-FR2 differentiation Note that Table 1 includes cases without both FDD-TDD and FR1-FR2 differentiation for completeness; however, only the cases with both FDD-TDD and FR1-FR2 differentiation can have any ambiguity.

The proposal seems quite straightforward clarification of XDD-FRX capability signaling. We should also discuss where/how to capture the proposed clarification with considering possibility to send a LS to RAN2.

Proposal:

- When for a given capability there is both FDD-TDD and FR1-FR2 differentiation, adopt the following convention:

  - Capability setting for FDD is set to ‘supported’ if the UE supports the feature in FR1-FDD, otherwise it is set to ‘not supported’

  - Capability setting for TDD is set to ‘supported’ if the UE supports the feature in FR1-TDD, irrespective of whether the UE supports the feature in FR2, otherwise it is set to ‘not supported’

  - Capability setting for FR1 is set to ‘supported’ if the UE supports the feature in either FR1-FDD or FR1-TDD, or both, otherwise it is set to ‘not supported’

  - Capability setting for FR2 is set to ‘supported’ if the UE supports the feature in FR2, otherwise it is set to ‘not supported’

- When for a given capability there is both FDD-TDD and FR1-FR2 differentiation, those settings are not used (they are reserved) for which either of the following holds:

  - The capability for FR1 is set to ‘not supported’ while either the capability for FDD or that for TDD is set to ‘supported’

  - The capability for FR1 is set to ‘supported’ while both the capability for FDD and that for TDD are set to ‘not supported’

Companies are encouraged to provide their views on the above proposal.

| Company | View |
| --- | --- |
| Qualcomm | If the proposal is agreed, an LS should be sent to RAN2, which could include a 38.306 text proposal. |
| ZTE | We have some concern on this proposal. From our perspective, for UE feature with both XDD and FRX differentiation, it was RAN1 common understanding that UE needs to indicate both TDD (Yes) and FR2 (Yes) in order to support this feature in FR2. However, this proposal tends to change this, which may cause a NBC issue. In such a late stage, we prefer not to change this fundamental UE-capability-bit interpretation. |
| Intel | The example 1 is mis-signaling by UE. One other way to address the concern from example 2 may be to clarify that, when there is capability signaling for FR1/FR2 differentiation, the capability setting for FDD and TDD is for FR1 *only* while the supportability of FR2 can be covered by the capability setting for FR1 and FR2, at least from Rel-16. It is because we don’t need FDD/TDD differentiation in FR2 since there is only TDD in FR2. |

According to the above feedbacks from companies, followings are the updated proposals.

Proposal:

Alt.1:

- When for a given capability there is both FDD-TDD and FR1-FR2 differentiation, adopt the following convention:

  - Capability setting for FDD is set to ‘supported’ if the UE supports the feature in FR1-FDD, otherwise it is set to ‘not supported’

  - Capability setting for TDD is set to ‘supported’ if the UE supports the feature in FR1-TDD, irrespective of whether the UE supports the feature in FR2, otherwise it is set to ‘not supported’

  - Capability setting for FR1 is set to ‘supported’ if the UE supports the feature in either FR1-FDD or FR1-TDD, or both, otherwise it is set to ‘not supported’

  - Capability setting for FR2 is set to ‘supported’ if the UE supports the feature in FR2, otherwise it is set to ‘not supported’

- When for a given capability there is both FDD-TDD and FR1-FR2 differentiation, those settings are not used (they are reserved) for which either of the following holds:

  - The capability for FR1 is set to ‘not supported’ while either the capability for FDD or that for TDD is set to ‘supported’

  - The capability for FR1 is set to ‘supported’ while both the capability for FDD and that for TDD are set to ‘not supported’

Alt.1’:

- When for a given capability there is both FDD-TDD and FR1-FR2 differentiation, the capability setting for FDD and TDD is for FR1 only and the capability setting for FR1 is consistent with the capability setting for FDD and TDD.

Alt.2:

- When for a given capability there is both FDD-TDD and FR1-FR2 differentiation, adopt the following convention:

  - Capability setting for FDD is set to ‘supported’ if the UE supports the feature in FR1-FDD, otherwise it is set to ‘not supported’

  - Capability setting for TDD is set to ‘supported’ if the UE supports the feature in either FR1-TDD or FR2, or both, otherwise it is set to ‘not supported’

  - Capability setting for FR1 is set to ‘supported’ if the UE supports the feature in either FR1-FDD or FR1-TDD, or both, otherwise it is set to ‘not supported’

  - Capability setting for FR2 is set to ‘supported’ if the UE supports the feature in FR2, otherwise it is set to ‘not supported’

- When for a given capability there is both FDD-TDD and FR1-FR2 differentiation, those settings are not used (they are reserved) for which either of the following holds:

  - The capability for TDD is set to ‘not supported’ while the capability for FR2 is set to ‘supported’

  - The capability for TDD is set to ‘supported’ while both the capability for FR1 and that for FR2 are set to ‘not supported’

  - The capability for FR1 is set to ‘supported’ while both the capability for FDD and that for TDD are set to ‘not supported’

Proposal:

Send LS to ask RAN2 to capture the above clarification in either one of followings.

- Alt.A: TS38.306 for Rel-15

- Alt.B: TR38.822 (as a part of the update for Rel-16)

Companies are encouraged to provide their views on the above proposal.

| Company | View |
| --- | --- |
| Qualcomm | We don’t agree with Alt.2. We can explain our concern with an example: Take two UEs: - UE1: - FR1 FDD: Yes - FR1 TDD: No - FR2 TDD: Yes - UE2: - FR1 FDD: Yes - FR1 TDD: Yes - FR2 TDD: Yes Our understanding is that the resulting setting is the following: - UE1: - FDD: Yes - TDD: Yes - FR1: Yes - FR2: Yes - UE2: - FDD: Yes - TDD: Yes - FR1: Yes - FR2: Yes So it appears that Alt.2 is not able to differentiate between UE1 and UE2, and we do not think it should be adopted. |
| Huawei | As I’m trying to understand how the entire existing UE capability signalling framework works, it appears to me that the potential ambiguity occurs from the signaling overhead reduction design that RAN2 has intentionally designated for. For example, according to 306 (copied below), the UE could firstly/reasonably report a support or not for FDD/TDD at per UE level, such that there is no need to indicate a support or not (N N or Y Y) for XDD for each UE capability. In this sense, indeed there is already an approach that UE does not need to suffer from the potential ambiguity by not reporting entire 4 available bits from itself. The case of a different value with Y&N report for XDD or FRX exists, but as I read the same text below there also seems to be a logic for interpretation by the order of the UE first setting the XDD bits then FRX bits. So the meaning of a certain combination of {FDD, TDD, FR1, FR2} may already be there, commonly understood by RAN2. 1> set all fields of UE-NR/MRDC-Capability except fdd-Add-UE-NR/MRDC-Capabilities, tdd-Add-UE-NR/MRDC-Capabilities, fr1-Add-UE-NR/MRDC-Capabilities and fr2-Add-UE-NR/MRDC-Capabilities, to include the values applicable for all duplex mode(s) and frequency range(s) that the UE supports; 1> if UE supports both FDD and TDD and if (some of) the UE capability fields have a different value for FDD and TDD 2> if for FDD, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability: 3> include field fdd-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FDD; 2> if for TDD, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability: 3> include field tdd-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for TDD; 1> if UE supports both FR1 and FR2 and if (some of) the UE capability fields have a different value for FR1 and FR2: …. Overall I think the issue is more about RAN2 expertise and responsibility at this moment, while operators may share your strong preference on some special deployment scenario that requires a fix for the incapability of support by current UE capability framework - which seems also can be addressed by RAN2 if needed. The potential conclusion from RAN1 may cause significant RAN2 spec impact on the entire reporting framework, so we’d like to propose to leave it for RAN2 to clarify. |

2.2	Issue on potential ambiguity on the current FG 5-11/5-12/5-13 in handling dropped/cancelled/skip decoded PDSCH and PUSCH [4]

In [4], Samsung raised following issue and corresponding proposals to confirm RAN1 common understanding.

FG 5-11/5-11a/5-11b describe the maximum number of unicast PDSCH’s per slot per CC a UE supports for UE processing time capability 1. FG 5-12/5-12a/5-12b describe the maximum number of unicast PUSCH’s per slot per CC a UE supports for UE processing time capability 1. FG 5-13/5-13a/5-13b/5-13c/5-13d/5-13e/5-13f describe the maximum number of unicast PDSCH’s and PUSCH’s per slot per CC a UE supports for UE processing time capability 2. On the other hand, dropping/cancellation/skip decoding behaviors are described as below in 38.213 and 38.214. 38.213 11.1.1 If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot. If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time for the corresponding PUSCH processing capability [6, TS 38.214] assuming and corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0, DCI format 1_0, DCI format 1_1 or DCI format 0_1 and the SCS configuration of the SRS, PUCCH, PUSCH or r, where r corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise r=0 - the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols.. 38.214 5.1.3 The UE may skip decoding a transport block in an initial transmission if the effective channel code rate is higher than 0.95, where the effective channel code rate is defined as the number of downlink information bits (including CRC bits) divided by the number of physical channel bits on PDSCH. 38.214 5.3 The UE may skip decoding a number of PDSCHs with last symbol within 10 symbols before the start of a PDSCH that is scheduled to follow Capability 2, if any of those PDSCHs are scheduled with more than 136 RBs with 30kHz SCS and following Capability 1 processing time.. In the current description of 5-11/5-12/5-13, it is not very clear if those dropped/cancelled/skip decoded PDSCH and PUSCH are counted toward the number a UE reported. In our view, all these cases involve a UE at least potentially partially processing PDSCH and PUSCH, and hence, those PDSCH and PUSCH should be counted. Some optimization is technically possible in a way that such counting is selectively applied depending on when the corresponding dynamic explicit or implicit indication is provided. However, such a thing would likely cause an NBC issue for a rel-15 UE. Hence, we would like to propose to confirm RAN1’s common understanding in the following way. Proposal 1: Confirm RAN1 common understanding that all PDSCHs/PUSCHs which a UE is scheduled or configured to receive/transmit are counted for the purpose of FG 5-11/5-12/5-13 regardless of dynamically occurring dropping/cancellation/skip decoding of them.

The proposal is to count all PDSCHs/PUSCHs regardless of dropping/cancellation/skip decoding for the purpose of FG 5-11/5-12/5-13, and it is a safer way of clarification considering potential NBC issue. We should also discuss where/how to capture the proposed clarification with considering possibility to send a LS to RAN2.

Proposal:

Confirm RAN1 common understanding that all PDSCHs/PUSCHs which a UE is scheduled or configured to receive/transmit are counted for the purpose of FG 5-11/5-12/5-13 regardless of dynamically occurring dropping/cancellation/skip decoding of them.

Companies are encouraged to provide their views on the above proposal.

| Company | View |
| --- | --- |
| Qualcomm | We agree with the proposal. |
| ZTE | We agree with this proposal. |

During the Tuesday online session, Ericsson raised an issue for the case in which DG PUSCH/PDSCH is prioritized over CG PUSCH/PDSCH with appropriate timeline. In that case, Samsung can consider counting only DG PUSCH/PDSCH. Following is the updated proposal from Samsung.

Proposal:

Confirm RAN1 common understanding that all PDSCHs/PUSCHs which a UE is scheduled or configured to receive/transmit are counted for the purpose of FG 5-11/5-12/5-13 regardless of dynamically occurring dropping/cancellation/skip decoding of them except for the cases in which dynamic grant PDSCH/PUSCH is prioritized over configured grant PDSCH/PUSCH with appropriate timeline satisfied. In those cases of dynamic grant prioritization which are described below, configured grant PDSCH/PUSCH is not counted.

    - A case in which a UE receives PDCCH ending in symbol i scheduling PUSCH overlapping in time with a transmission occasion where the UE is allowed to transmit a PUSCH with configured grant starting in a symbol j on the same serving cell such that the end of symbol i is at least N2 symbols before the beginning of symbol j.

    - A case in which a UE receives PDCCH ending in symbol i scheduling PUSCH on a given serving cell for a given HARQ process while the UE is allowed to transmit a PUSCH with configured grant with the same HARQ process starting in a symbol j after symbol i on the same serving cell such that the end of symbol i is at least N2 symbols before the beginning of symbol j.

    - A case in which a UE receives PDCCH ending in symbol i scheduling PDSCH with C-RNTI or MCS-C-RNTI overlapping in time with PDSCH scheduled with CS-RNTI starting in a symbol j in the same serving cell such that the end of symbol i is at least 14 symbols before the beginning of symbol j.

Companies are encouraged to provide their views on the above proposal.

| Company | View |
| --- | --- |

2.3	Issue on capability signalling related to UE peak data rate [6]

In [6], Ericsson raised following issue and corresponding proposals.

The motivations for scalingFactor signaling from the UE (taking values from 0.4, 0.75, 0.8 and 1) was described in RAN1 LS to RAN2 (R1-1807651) and were mainly for cases with CA/DC cases where UE supporting several carriers can indicate that it supports fewer layers or lower data rate. The supportedModulationOrderDL signaling from the UE was introduced allowing a UE that supports e.g. 256QAM modulation, but only supporting a peak data rate corresponding to a lower modulation order (indicated by supportedModulationOrderDL) such as anyone from {pi/2-BPSK,QPSK,16QAM, 64QAM, 256QAM}corresponding to (e.g. using Q = 1/2/4/6/8). Combining above two capabilities signaled together, the effective data rate scaling factor (SCF) becomes (1, 0.8, 0.75, 0.4) x (8/8, 6/8, 4/8, 2/8, 1/8) leading to following possible values for SCF : 1, 0.8, 0.75, 0.6, 0.5625, 0.5, 0.4, 0.375, 0.3, 0.25, 0.2, 0.1875, 0.125, 0.1, 0.0938, 0.05. Thus, an unintended consequence of the capability signaling framework is that a UE can potentially indicate extremely low peak data rate by signaling most pessimistic values for scalingFactor and supportedModulationOrderDL as shown in following observations. - Assuming a configuration with 256QAM, a UE can indicate that it supports a maximum bitrate or a peak data rate that is only ~0.4*1/8 = 0.05 of the theoretical bitrate of the carrier (scaling factor 0.4 & pi/2-modulation) which is extremely low peak data rate. - Assuming a configuration with 64QAM, a UE can indicate that it supports a maximum bitrate or a peak data rate that is only ~0.4*1/6 = 0.067 of the theoretical bitrate of the carrier (scaling factor 0.4 & pi/2-modulation) which is extremely low peak data rate. - In an example, for a 20 MHz, 30kHz SCS, with single layer MIMO layer, the UE can indicate an extremely low peak data rate as small as 5 Mbps via pessimistic setting of scaling factor and SupportedModulationOrderDL. Issues/problems with extremely low peak rate indication 5 Mbps with 0.5 ms slot implies a TBS of 2500 bits, extremely small even considering the size of RRC messages. Having all networks to adapt to such a worst case and potentially slowdown the initial access for all NR UEs is not reasonable. Such extremely low peak rates also do not sound relevant even for a super-low-end UE. While the example uses 1-layer, the same arguments hold for UEs support 2 or 4 layers indicating extremely pessimistic data rates. Some more detailed analysis is shown below. In our understanding, UEs shall support the following - the maximum bandwidth defined for the respective band or support at least the bandwidth of the initial BWP - at least 64QAM and at least 256QAM for FR1 - at least 2 layers or 4 layers (in some bands) in FR1 and 2 layers in FR2 Since the MIMO layer (2 or 4 layers) and higher order modulation (256-QAM) are supported as mandatory with capability signalling, the NW would have to assume and support UEs that may support single-layer MIMO and 64-QAM (e.g. due to lack of IODT). It would of course be extremely preferable if NW can assume UE always support >= 2 layers, but is not possible given yellow text below. maxNumberMIMO-LayersPDSCH Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier.; FSPC; CY; No; No. ~ The two parameters (scaling factor and supportedModulationOrderDL) should not lead to peak rate limitations, in single carrier operation with lower modulation order (64QAM) and MIMO layers (1 MIMO layer) with very pessimistic peak rate. Therefore, we propose that at least for Rel-15 single CC standalone NR operation, the UE capability signaling is such that the indicated UE peak rate is not lower than the carrier bit rate assuming single-layer MIMO and 64QAM. It would be desirable to also ensure the same applies also to NSA single carrier operation since the network resource cost of supporting transfer of data volume to a UE increases as the UE becomes less capable. - For Rel-15 single CC standalone or single CC non-standalone NR in the downlink, the UE reported capability signaling is such that the product of indicated number of MIMO layers, scalingFactor and supportedModulationOrderDL is no smaller than 6. - Note: 6 corresponds e.g. to 1 MIMO layer, scaling factor = 1 and supportedModulationOrderDL = 64-QAM - For Rel-15 single CC standalone or single CC non-standalone NR in the uplink, the UE reported capability signaling is such that the product of indicated number of MIMO layers, scalingFactor and supportedModulationOrderUL is no smaller than 6 - Note: 6 corresponds e.g. to 1 MIMO layer, scaling factor = 1 and supportedModulationOrderUL = 64-QAM. It is further preferable if the lower limit is larger (e.g. 12 for 2-layer and 24 for 4-layer indication) for the single carrier standalone case.

The proposal is to avoid some possible capability reporting combinations of scalingFactor and supportedModulationOrderDL(UL) which leads to extremely low data rate at least for single CC standalone operation in Rel-15. During the online discussion in AI 7.1.3, there is a company who considers it is difficult to find the appropriate value (threshold) for the limitation, and there is another company who wonders if it is a corner case. So, further discussion on the proposal would be necessary. In addition, how to clarify the proposed limitation e.g., in TS38.306 should also be discussed.

Proposal:

- For Rel-15 single CC standalone or single CC non-standalone NR in the downlink, the UE reported capability signaling is such that the product of indicated number of MIMO layers, scalingFactor and supportedModulationOrderDL is no smaller than 6.

  - Note: 6 corresponds e.g. to 1 MIMO layer, scaling factor = 1 and supportedModulationOrderDL = 64-QAM

- For Rel-15 single CC standalone or single CC non-standalone NR in the uplink, the UE reported capability signaling is such that the product of indicated number of MIMO layers, scalingFactor and supportedModulationOrderUL is no smaller than 6

  - Note: 6 corresponds e.g. to 1 MIMO layer, scaling factor = 1 and supportedModulationOrderUL = 64-QAM.

Companies are encouraged to provide their views on the above proposal.

| Company | View |
| --- | --- |

References

[1]	R1-1912931	XDD-FRX capability signaling	Qualcomm Incorporated

[2]	R1-1909953	LS on ambiguity of UE L1 FDD/TDD FR1/FR2 capabilities	RAN2, ZTE

[3]	R1-1911741	Reply LS on ambiguity of UE L1 FDD/TDD FR1/FR2 capabilities	RAN1, ZTE

[4]	R1-1912991	On FG 5-11/5-12/5-13	Samsung

[5]	3GPP, TR38.822 v1.1.0, June 2019.

[6]	R1-1912779	Rel-15 NR capability signaling related to UE peak data rate	Ericsson

Appendix: Layer-1 UE feature list in TR38.822 v1.1.0 [5]

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL | 1) CP-OFDM for DL 2) CP -OFDM for UL |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-2 | DFT-S-OFDM waveform for UL | Transform precoding for single-layer PUSCH |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-3 | DL modulation scheme | 1) QPSK modulation 2) 16QAM modulation 3) 64QAM modulation for FR1 |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-4 | UL modulation scheme | 1) QPSK modulation 2) 16QAM modulation |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-5 | Extended CP | Extended CP | 1-1 in Table 4.3-1 | extendedCP | BandNR | n/a | n/a |  | Optional with capability signalling |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures | 1) RACH preamble format 2) SS block based RRM measurement 3) Broadcast SIB reception including RMSI/OSI and paging |  | n/a | n/a | No | No | Broadcast SIB reception including RMSI/OSI and paging are components of basic initial access channels and procedures for NR standalone and NE-DC | Mandatory without capability signalling |
|  | 1-2 | SS block based SINR measurement (SS-SINR) | SS-SINR measurement | 1-1 | ss-SINR-Meas | MeasAndMobParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 1-3 | SS block based RLM | SS block based RLM | 1-1 | ssb-RLM | MeasAndMobParametersCommon | No | No |  | Mandatory with capability signalling which shall be set to ‘1’ |
|  | 1-4 | CSI-RS based RRM measurement with associated SS-block | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement | 1-1, CSI-RS | csi-RSRP-AndRSRQ-MeasWithSSB | MeasAndMobParametersFRX-Diff | No | Yes | This does not discourage RAN4 to complete their work. There is expectation that RAN4 will complete the corresponding RRM measurement | Optional with capability signalling |
|  | 1-5 | CSI-RS based RRM measurement without associated SS-block | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement 3) There is SS-block in the target frequency on which the RRM measurement is performed | 1-1, CSI-RS | csi-RSRP-AndRSRQ-MeasWithoutSSB | MeasAndMobParametersFRX-Diff | No | Yes | This does not discourage RAN4 to complete their work. There is expectation that RAN4 will complete the corresponding RRM measurement | Optional with capability signalling |
|  | 1-6 | CSI-RS based RS-SINR measurement | CSI-SINR measurements | 1-1, 1-4 | csi-SINR-Meas | MeasAndMobParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 1-7 | CSI-RS based RLM | CSI-RS based RLM | 1-1, CSI-RS | csi-RS-RLM | MeasAndMobParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 1-8 | RLM based on a mix of SS block and CSI-RS signals within active BWP | RLM based on a mix of SS block and CSI-RS signals within active BWP | 1-3 and 1-7 | ssb-AndCSI-RS-RLM | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 1-9 | CSI-RS based contention free RA for HO | CSI-RS based contention free RA for HO | 1-1, CSI-RS, 1-4 or 1-5 | csi-RS-CFRA-ForHO | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 1-10 | Support of SCell without SS/PBCH block | Support SCell without SS/PBCH block | 1-1 | scellWithoutSSB | FeatureSetDownlink | n/a | n/a | Component 1) Whether or not UE is able to use SS/PBCH block from other Cells for time/frequency synchronization of SCell without SS/PBCH block | Mandatory with capability signalling for intra-band CA This feature is not supported for inter band CA |
|  | 1-11 | Support of CSI-RS RRM measurement for SCell without SS/PBCH block |  | 1-10 | csi-RS-MeasSCellWithoutSSB | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 1-12 | E-UTRA RS-SINR measurement |  |  | rs-SINR-MeasEUTRA | EUTRA-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 1-13 | Maximal number of CSI-RS resources for RRM and RS-SINR measurement across all measurement frequencies per slot |  | 1-4 or 1-5 or 1-6 | maxNumberCSI-RS-RRM-RS-SINR | MeasAndMobParametersCommon | No | No | If UE supports any of 1-5, 1-5a, and 1-6, UE shall report this capability 1-13 | Candidate value set: {4,8,16,32,64, 96} |
|  | 1-14 | Maximal number of CSI-RS resources within a slot per PCell/PSCell for CSI-RS based RLM |  | 1-7 or 1-8 | maxNumberResource-CSI-RS-RLM | MeasAndMobParametersFRX-Diff | No | Yes | If UE supports any of 1-7 and 1-8, UE shall report this capability 1-14 | Candidate value set: {2,4, 6, 8} |
| 2. MIMO | 2-1 | Basic PDSCH reception | 1) Data RE mapping 2) Single layer transmission 3) Support one TCI state |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 2-2 | PDSCH beam switching | 1) Time duration (definition follows section 5.1.5 in TS 38.214), Xi, to determine and apply spatial QCL information for corresponding PDSCH reception. Time duration is defined counting from end of last symbol of PDCCH to beginning of the first symbol of PDSCH. Xi is the number of OFDM symbols, i is the index of SCS, l=1,2, corresponding to 60,120 kHz SCS. | 2-1 | timeDurationForQCL | FeatureSetDownlink | No | Applicable only to FR2 |  | Mandatory with capability signalling for FR2 Candidate value set for X1 is {7, 14, 28}, Candidate value set for X2, {14, 28} |
|  | 2-3 | PDSCH MIMO layers | Supported maximal number of MIMO layers | 2-1 | maxNumberMIMO-LayersPDSCH | FeatureSetDownlinkPerCC | n/a | n/a |  | For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. Some relaxations to this requirement may be applicable in the future (including in Rel-15). Mandatory in all cases means mandatory with capability signalling. It is not expected that there is a signalling change (i.e. signalling remains to be defined as {1, 2, 4, 8} in every band and every band combination, including FR1 and FR2 in all cases. |
|  | 2-4 | TCI states for PDSCH | 1) Support number of active TCI states per BWP per CC, including control and data 2) Maximum number of configured TCI states per CC for PDSCH | 2-1 | tci-StatePDSCH { 1. maxNumberActiveTCI-PerBWP 2. maxNumberConfiguredTCIstatesPerCC } | MIMO-ParametersPerBand | n/a | n/a | UE is required to track only the active TCI states For component 1 of FG2-4, if a UE reports X active TCI state(s), it is not expected that more than X active QCL type D assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell become active for the UE. | Mandatory with capability signalling Component-1: Candidate value set: {1, 2, 4, 8} Component-2: candidate value set: {4, 8, 16, 32, 64, 128} UE is mandated to signal 64 for FR2. For FR1, UE is mandated to report at least the max number of allowed SSB in the band. |
|  | 2-4a | Additional active TCI state for PDCCH | Support one additional active TCI state for control in addition to the supported number of active TCI states for PDSCH | 2-1 | additionalActiveTCI-StatePDCCH | MIMO-ParametersPerBand | n/a | n/a | Only applicable if Component-1 of 2-4 is set to 1 | Mandatory with capability signalling |
|  | 2-5 | Basic downlink DMRS for scheduling type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port. | 2-1 | n/a | n/a | n/a | n/a | conditioned to whether PDSCH scheduling type A is supported | Mandatory without capability signalling (condition to scheduling capability) |
|  | 2-6 | Basic downlink DMRS for scheduling type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  | n/a | n/a | n/a | n/a | conditioned to whether PDSCH scheduling type B is supported | Mandatory without capability signalling (condition to scheduling capability) |
|  | 2-6a | Support 1+2 DMRS (downlink) | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port | 2-5 | oneFL-DMRS-TwoAdditionalDMRS-DL | FeatureSetDownlink-v1540 | No | Yes |  | Mandatory with capability signalling |
|  | 2-6b | Support alternative additional DMRS location | Support alternative additional DMRS position for co-existence with LTE CRS | 2-5 and 5-28 | additionalDMRS-DL-Alt | FeatureSetDownlink-v1540 | No | n/a | This FG applies to FR1 only and 15kHz SCS. This applies to one additional DMRS case only | Optional with capability signalling |
|  | 2-7 | Supported 2 symbols front-loaded DMRS (downlink) | Support 2 symbols FL-DMRS | 2-5 | twoFL-DMRS (MSB) | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-8 | Supported 2 symbols front-loaded +2 symbols additional DMRS (downlink) | Support 2-symbol FL DMRS + one additional 2-symbols DMRS | 2-5 | twoFL-DMRS-TwoAdditionalDMRS-DL | FeatureSetDownlink-v1540 | No | Yes |  | Optional with capability signalling |
|  | 2-9 | Support 1+3 DMRS symbols(downlink) | Support 1 symbol FL DMRS and 3 additional DMRS symbols | 2-5 | oneFL-DMRS-ThreeAdditionalDMRS-DL | FeatureSetDownlink-v1540 | No | Yes |  | Optional with capability signalling |
|  | 2-10 | Support DMRS type (downlink) | Support DMRS {type 1, both type 1 and type 2} |  | supportedDMRS-TypeDL | Phy-ParametersFRX-Diff | No | Yes |  | Type 1 is mandatory with capability signalling. Type 2 is optional with capability signalling |
|  | 2-11 | Downlink dynamic PRB bundling (downlink) | Support dynamic PRB bundling indication via DCI | 2-1 | dynamicPRB-BundlingDL | Phy-ParametersCommon | No | No | Support of semi-static PRB bundling is mandatory | Optional with capability signalling |
|  | 2-12 | Basic PUSCH transmission | Data RE mapping Single layer (single Tx) transmission Single port, single resource SRS transmission (SRS set use is configured as for codebook) |  | n/a | n/a | n/a | n/a | Support of SRS set usage configured as for codebook does not imply UE support of codebook based PUSCH MIMO transmission. | Mandatory without capability signalling |
|  | 2-13 | PUSCH codebook coherency subset | Supported codebook coherency subset type | 2-12 | pusch-TransCoherence | MIMO-ParametersPerBand | n/a | n/a |  | Optional with UE capability Candidate value set: {non-coherent, partial/non-coherent, full/partial/non-coherent} |
|  | 2-14 | Codebook based PUSCH MIMO transmission | 1) Supported codebook based PUSCH MIMO with maximal number of supported layers 2) Supported max number of SRS resource per set (SRS set use is configured as for codebook). | 2-13 | mimo-CB-PUSCH { 1. maxNumberMIMO-LayersCB-PUSCH 2. maxNumberSRS-ResourcePerSet } | FeatureSetUplinkPerCC | n/a | n/a | For SUL, uplink MIMO is not supported. | Optional with UE capability Component-1: Candidate value: {no-codebook based MIMO, 1, 2, 4} Component-2 Candidate value: {1, 2} |
|  | 2-15 | non-codebook based PUSCH transmission | 1) Maximal number of supported layers (non-codebook transmission scheme) 2) Supported max number of SRS resource per set (SRS set use is configured as for non-codebook transmission). 3) Maximum number of simultaneous transmitted SRS resources at one symbol | 2-12 | 1. maxNumberMIMO-LayersNonCB-PUSCH mimo-NonCB-PUSCH { 2. maxNumberSRS-ResourcePerSet 3. maxNumberSimultaneousSRS-ResourceTx } | 1. FeatureSetUplinkPerCC 2, 3. FeatureSetUplinkPerCC-v1540 | n/a | n/a | For SUL, uplink MIMO is not supported | Optional with UE capability Component-1 candidate values: {1, 2, 4} Component-2 Candidate value: {1,2,3,4} Component-3 Candidate value: {1,2,3,4} |
|  | 2-15a | Association between CSI-RS and SRS | 1) Support association between NZP-CSI-RS and SRS resource set via RRC parameter “SRSresoureset” 2) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. | 2-15 | srs-AssocCSI-RS SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 2.1. maxNumberTxPortsPerResource 2.2. maxNumberResourcesPerBand 2.3. totalNumberTxPortsPerBand } | MIMO-ParametersPerBand | n/a | n/a |  | Optional with capability signalling Component-2: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 2.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 2.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-15b | CSI-RS processing framework for SRS | 1) Maximum number of periodic SRS resources associated with CSI-RS per BWP 2) Maximum number of aperiodic SRS resources associated with CSI-RS per BWP 3) Maximum number of semi-persistent SRS resources associated with CSI-RS per BWP 4) UE can process Y SRS resources associated with CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. 5) UE can process X SRS resources associated with CSI-RS resources simultaneously across all CCs. Includes P/SP/A SRS. | 2-15a | csi-RS-ProcFrameworkForSRS { 1. maxNumberPeriodicSRS-AssocCSI-RS-PerBWP 2. maxNumberAperiodicSRS-AssocCSI-RS-PerBWP 3. maxNumberSP-SRS-AssocCSI-RS-PerBWP 4. simultaneousSRS-AssocCSI-RS-PerCC } | MIMO-ParametersPerBand Phy-ParametersFRX-Diff (for FR1 + FR2 band combination) | n/a | n/a | Other MIMO capabilities than component 5 may further restrict (reduce) the number of SRS associated with CSI-RS that the UE has to simultaneously derive. | Optional with capability signalling Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |
|  |  |  |  |  | 5. simultaneousSRS-AssocCSI-RS-AllCC | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols |  | n/a | n/a | n/a | n/a | Conditioned to whether PUSCH scheduling type A is supported | Mandatory without capability signalling |
|  | 2-16a | Basic uplink DMRS for scheduling type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  | n/a | n/a | n/a | n/a | conditioned to whether PUSCH scheduling type B is supported | Mandatory without capability signalling |
|  | 2-16b | Support 1+2 DMRS (uplink) | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port | 2-16a and 2-16 | oneFL-DMRS-TwoAdditionalDMRS-UL | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-17 | Support DMRS type (uplink) | Support DMRS {type 1, both type 1 and type 2} | 2-16 | supportedDMRS-TypeUL | Phy-ParametersFRX-Diff | No | Yes |  | Support both type 1 and type 2 are mandatory with capability signalling |
|  | 2-18 | Supported 2 symbols front-loaded DMRS (uplink) | Support 2 symbols FL-DMRS | 2-16 | twoFL-DMRS (LSB) | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-18a | Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink) | Support 2-symbol FL DMRS + one additional 2-symbols DMRS | 2-16 | twoFL-DMRS-TwoAdditionalDMRS-UL | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-19 | Support 1+3 uplink DMRS symbols(uplink) | Support 1 symbol FL DMRS and 3 additional DMRS symbols | 2-16 | oneFL-DMRS-ThreeAdditionalDMRS-UL | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-20 | Beam correspondence | Support Beam correspondence |  | beamCorrespondenceWithoutUL-BeamSweeping | MIMO-ParametersPerBand | No | Applicable only to FR2 | Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports. | Mandatory with capability signalling - UE that fulfils the beam correspondence requirement without the uplink beam sweeping shall set the bit to 1 - UE that fulfils the beam correspondence requirement with the uplink beam sweeping shall set the bit to 0 |
|  | 2-21 | Periodic beam report | 1) Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2) Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot |  | periodicBeamReport | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling for both FR1 and FR2 |
|  | 2-22 | Aperiodic beam report | Support aperiodic report on PUSCH |  | aperiodicBeamReport | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling for both FR1 and FR2 |
|  | 2-23 | Semi-persistent beam report on PUCCH | 1) Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) 2) Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) |  | sp-BeamReportPUCCH | MIMO-ParametersPerBand | n/a | Yes |  | Optional with capability signalling |
|  | 2-23a | Semi-persistent beam report on PUSCH | Support semi-persistent report on PUSCH |  | sp-BeamReportPUSCH | MIMO-ParametersPerBand | n/a | Yes |  | Optional with capability signalling |
|  | 2-24 | SSB/CSI-RS for beam measurement | 1) The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 2) The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 3) The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 4) Supported density of CSI-RS 5) The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a | beamManagementSSB-CSI-RS { 1. maxNumberSSB-CSI-RS-ResourceOneTx 2. maxNumberCSI-RS-Resource 3. maxNumberCSI-RS-ResourceTwoTx 4. supportedCSI-RS-Density 5. maxNumberAperiodicCSI-RS-Resource } | MIMO-ParametersPerBand | No | Yes |  | Mandatory with capability signalling Component-1, candidate value set for MB_1 is {0, 8, 16, 32, 64} On FR2, UE is mandated to signal MB_1 >=8 On FR1, MB_1 >=8 is supported mandatory with capability signalling. Component-2, candidate value set for MC_1 is {0, 4, 8, 16, 32, 64} For FR1, UE is mandated to report at least 8. Component-3, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-4: candidate value set: {“not supported”, “1 only”, “3 only”, “both 1 and 3”} On FR2, UE is mandated to signal either “3 only” or “both 1 and 3” On FR1, either “3 only” or “both 1 and 3” is mandatory with UE capability signalling. Component-5, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} For both FR1 and FR2, UE is mandated to report at least 4 |
|  | 2-25 | Beam reporting timing | The number of symbols, Xi, between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report is at least RBi, where i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-24 | beamReportTiming | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability Candidate value sets: X1 is {2, 4, 8} X2 is {4, 8, 14, 28} X3 is {8,14, 28} X4 is{14,28, 56} |
|  | 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" | 1. Support Rx beam switching procedure using CSI-RS resource repetition "ON" 2. Recommended CSI-RS resource repetition number per resource set, |  | maxNumberRxBeam | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with UE capability at least for FR2 Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} |
|  | 2-27 | Beam switching | Maximum number of Tx + Rx beam changes a UE can conduct during a slot across the whole band CC B_(B_Total,). This number is defined as per SCS | 2-24 | maxNumberRxTxBeamSwitchDL | MIMO-ParametersPerBand | n/a | Applicable only to FR2 | It is assumed that spec enable the possibility to restrict the same beam across intra-band CCs | Optional with capability signalling Candidate value set: {4, 7, 14} |
|  | 2-28 | A-CSI-RS beam switching timing | Minimum time between the DCI triggering of AP-CSI-RS and aperiodic CSI-RS transmission shall be at least KBi symbols. (Symbols measured from last symbol containing the indication to first symbol of CSI-RS), where i is the index of SCS, l=1,2 corresponding to 60,120 kHz SCS. |  | beamSwitchTiming | MIMO-ParametersPerBand | n/a | Applicable only to FR2 |  | Optional with capability signalling Candidate values: {14, 28, 48, 224, 336} |
|  | 2-29 | Non-group based beam reporting | Support of non-group based RSRP reporting with N_max RSRP values reported |  | maxNumberNonGroupBeamReporting | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability for both FR1 and FR2 candidate value set is {1, 2, 4} |
|  | 2-29a | Group based beam reporting | Support of beam group RSRP reporting for group of 2 beams |  | groupBeamReporting | MIMO-ParametersPerBand | n/a | n/a |  | Optional with capability signalling |
|  | 2-30 | Uplink beam management | 1) Support of SRS based beam management 2) Supported max number of SRS resource per set (SRS set use is configured as for beam management). 3) Supported max number of SRS resource sets (SRS set use is configured as for beam management). |  | uplinkBeamManagement { 2. maxNumberSRS-ResourcePerSet-BM 3. maxNumberSRS-ResourceSet } | MIMO-ParametersPerBand | n/a | n/a | Component-3 also impose additional constraint on the maximum number of SRS resource sets per supported time domain behaviour (periodic/semi-persistent/aperiodic) as {1,1,1,2,2,2,4,4} corresponding to reported values {from 1 to 8} | Optional with capability signalling - Capability signalling shall be set to 1 if 2-20 is set to 0 - For the UE meeting the minimum peak EIRP and spherical coverage requirements without the uplink beam sweeping, this feature is optional Component-2, candidate value set is {2, 4, 8, 16} Component-3, candidate value set is {from 1 to 8} |
|  | 2-31 | Beam failure recovery | 1) Maximal number of CSI-RS resources across all CCs for UE to monitor PDCCH quality 2) Maximal number of different SSBs across all CCs for UE to monitor PDCCH quality 3) Maximal number of different CSI-RS and/or SSB resources across all CCs for new beam identifications. | 1-7 for CSI-RS based BFD/BFR | 1. maxNumberCSI-RS-BFD 2. maxNumberSSB-BFD 3. maxNumberCSI-RS-SSB-CBD | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 Component-1 candidate value set: {from 1 to 16} Component-2 candidate: {from 1 to 16} Component-3: Candidate value set is: {from 1 to 128} UE is mandated to support at least 32 for FR2 |
|  | 2-32 | Basic CSI feedback | 1) Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory) 2) 2Tx codebook for FR1 and FR2 3) 4Tx codebook for FR1 4) 8Tx codebook for FR1 when configured as wideband CSI report 5) p-CSI on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) 6) p-CSI report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) 7) a-CSI on PUSCH (at least Z value >= 14 symbols, detail processing time to be discussed separately) further check a-CSI on p-CSI-RS and/or SP-CSI-RS from component-7 |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 2-32a | Semi-persistent CSI report on PUCCH | 1) Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) s 2) Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) |  | sp-CSI-ReportPUCCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-32b | Semi-persistent CSI report on PUSCH | Support semi-persistent CSI report on PUSCH |  | sp-CSI-ReportPUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-32c | New CQI table | CQI table with target BLER of 10^-5 |  | cqi-TableAlt | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-33 | CSI-RS and CSI-IM reception for CSI feedback | 1) Supported max # of configured NZP-CSI-RS resources per CC, 2) Supported max # of ports across all configured NZP-CSI-RS resources per CC 3) Supported max # of configured CSI-IM resources per CC 4) Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5) Supported max # simultaneous NZP-CSI-RS resources per CC 6) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources per CC | 2-32 | csi-RS-IM-ReceptionForFeedback { 1. maxConfigNumberNZP-CSI-RS-PerCC 2. maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC 3. maxConfigNumberCSI-IM-PerCC 5. maxNumberSimultaneousNZP-CSI-RS-PerCC 7. totalNumberPortsSimultaneousNZP-CSI-RS-PerCC } | MIMO-ParametersPerBand Phy-ParametersFRX-Diff (for FR1 + FR2 band combination) | n/a | n/a | All the candidate values are the range of capability signalling which doesn’t determine whether UE is mandatory to support all the signalling values. | Mandatory with capability signalling Component-1 candidate values: {from 1 to 32} Component-2 candidate values: {2, 4, 8, 12, 16, 24, 32, 40, 48 … ,256} Component-3: candidate values: {1,2,4,8,16,32} Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-5: candidate values {1, 2, 3 … 32} Component-6: candidate values {8, 16, 24, …, 248, 256} Component-7: candidate values {8, 16, 24, … 128 } |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 4. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 6. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-33a | Supported PDSCH RE-mapping patterns | 1) Supported max # of RE mapping patterns, each pattern can be described as a resource (including NZP/ZP CSI-RS and CRS, CORESET and SSB and bitmap configured in 5-26/27) Note: patterns are counted as per symbol per CC 2) Supported max # of RE mapping patterns, each pattern can be described as a resource (including NZP/ZP CSI-RS and CRS, CORESET and SSB and bitmap configured in 5-26/27/27a) Note: patterns are counted as per slot per CC |  | 1. pdsch-RE-MappingFR1-PerSymbol 2. pdsch-RE-MappingFR1-PerSlot | Phy-ParametersFR1 | No | Yes |  | Mandatory with capability signalling candidate values: {10, 20} for FR1 {6, 20} for FR2 Compponent-2 candidate values: {from 16: 16: 256} for FR1 {16: 16: 256} for FR2 |
|  |  |  |  |  | 1. pdsch-RE-MappingFR2-PerSymbol 2. pdsch-RE-MappingFR2-PerSlot | Phy-ParametersFR2 |  |  |  |  |
|  | 2-33b | SP CSI-RS | Support SP CSI-RS | 2-1 | sp-CSI-RS | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-33c | SP CSI-IM | Support SP CSI-IM | 2-1 | sp-CSI-IM | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-34 | NZP-CSI-RS based interference measurement | Support NZP-CSI-RS based interference measurement | 2-33 | nzp-CSI-RS-IntefMgmt | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-35 | CSI report framework | 1) Maximum number of periodic CSI report setting per BWP for CSI report 2) Maximum number of periodic CSI report setting per BWP for beam report 3) Maximum number of aperiodic CSI report setting per BWP for CSI report 4) Maximum number of aperiodic CSI report setting per BWP for beam report 5) Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC, 6) Maximum number of semi-persistent CSI report setting per BWP for CSI report 7) Maximum number of semi-persistent CSI report setting per BWP for beam report 8) UE can process Y CSI report(s) simultaneously in a CC. CSI reports can be P/SP/A CSI and any latency class and codebook type. 9) UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. | 2-32 | csi-ReportFramework { 1. maxNumberPeriodicCSI-PerBWP-ForCSI-Report 2. maxNumberAperiodicCSI-PerBWP-ForCSI-Report 3. maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report 4. maxNumberPeriodicCSI-PerBWP-ForBeamReport 5. maxNumberAperiodicCSI-PerBWP-ForBeamReport 6. maxNumberAperiodicCSI-triggeringStatePerCC 7. maxNumberSemiPersistentCSI-PerBWP-ForBeamReport 8. simultaneousCSI-ReportsPerCC } | MIMO-ParametersPerBand Phy-ParametersFRX-Diff (for FR1 + FR2 band combination) | n/a | n/a | Other MIMO capabilities than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update The CSI report in component 4 and 5 includes the beam report and CSI report Each component is independent CSI report setting are counted in the CC indicated by the parameter carrier in CSI-ResourceConfig. | Mandatory with capability signaling Component-1 candidate values: {1, 2, 3, 4} Component-1a candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-2a candidate values {1, 2, 3, 4} Component-2b candidate values {3, 7, 15, 31, 63, 128} Component-3 candidate values: {0, 1, 2, 3, 4} Component-3a candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |
|  |  |  |  |  | 9. simultaneousCSI-ReportsAllCC | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-36 | Type I single panel codebook | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Supported Codebook Mode(s) 3) Max # of CSI-RS resource in a resource set | 2-35 | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. modes 3. maxNumberCSI-RS-PerResourceSet | CodebookParameters | No | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Mandatory with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: {“Mode-1 only”, “Mode-1 and Mode-2”}. Component-3 Candidate values set: {1:8} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-37 | Support Semi-open loop CSI | Support Semi-open loop CSI report | 2-35 | semiOpenLoopCSI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-38 | CSI report without PMI | Support CSI report without PMI | 2-35 | csi-ReportWithoutPMI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-39a | CSI report without CQI | Support CSI report without CQI | 2-35 | csi-ReportWithoutCQI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-40 | Type I multi-panel codebook | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Supported Codebook Mode(s): 3) Supported number of panels, Ng 4) Max # of CSI-RS resource in a resource set | 2-35 | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. modes 3. nrofPanels 4. maxNumberCSI-RS-PerResourceSet | CodebookParameters | n/a | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Optional with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {8, 16, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: {Mode-1, Mode-2, both} Component-3: Candidate value: {2,4} Component-4: candidate value set is {1:8} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-41 | Type II codebook | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Parameter “Lx” (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3) Support amplitude scaling type 4) Support amplitude subset restriction level | 2-35 | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. parameterLx 3. amplitudeScalingType 4. amplitudeSubsetRestriction | CodebookParameters | n/a | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Optional with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4, candidate value set: {“no amplitude subset restriction”, “support amplitude subset restriction”} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-42 | Support Type II SP-CSI feedback on long PUCCH | Support type II SP-CSI feedback part-1 on PUCCH formats over 4 – 14 OFDM symbols once per slot | 2-41 | type2-SP-CSI-Feedback-LongPUCCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-43 | Type II codebook with port selection | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Parameter “Lx” (number of selected ports) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3) Support amplitude scaling type |  | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. parameterLx 3. amplitudeScalingType | CodebookParameters | n/a | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Optional with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values set for “Lx” is {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4: candidate value set is {1:8} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-44 | Basic DL PTRS | Support 1 port of DL PTRS |  | onePortsPTRS (MSB) | Phy-ParametersFRX-Diff | n/a | Yes |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 |
|  | 2-46 | Downlink PTRS density recommendation | Preferred threshold sets, TSi for determine PTRS density, candidate value range is the same as that of downlink PTRS RRC configuration. i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-44 | ptrs-DensityRecommendationSetDL { 1. frequencyDensity1 2. frequencyDensity2 3. timeDensity1 4. timeDensity2 5. timeDensity3 } | MIMO-ParametersPerBand | n/a | n/a | For each TSi, it composes of two values each selected from {1..276} for frequency density, and three values each selected from {0..29} for time density | Optional with capability signalling |
|  | 2-47 | Basic UL PTRS | Support 1 port of UL PTRS |  | onePortsPTRS (LSB) | Phy-ParametersFRX-Diff | n/a | Yes |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 |
|  | 2-48 | Uplink PTRS | Supported 2 ports of PTRS | 2-47 | twoPortsPTRS-UL | MIMO-ParametersPerBand | n/a | n/a |  | Optional with capability signalling |
|  | 2-49 | Uplink PTRS density recommendation | Preferred threshold sets, TSi, for determine PTRS density, candidate value range is the same as that of uplink PTRS RRC configuration. i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-47 | ptrs-DensityRecommendationSetUL { 1. frequencyDensity1 2. frequencyDensity2 3. timeDensity1 4. timeDensity2 5. timeDensity3 6. sampleDensity1 7. sampleDensity2 8. sampleDensity3 9. sampleDensity4 10. sampleDensity5 } | MIMO-ParametersPerBand | n/a | n/a | For each TSi, it composes of two values each selected from {1..276} for frequency density, and three values each selected from {0..29} for time density, and five values each selected from {1..276} for sample density | Optional with capability signalling |
|  | 2-50 | Basic TRS | 1) Support of TRS (mandatory) 2) All the periodicity are supported. 3) Support TRS bandwidth configuration as both “BWP” and “min(52, BWP)” |  | n/a | n/a | n/a | n/a | TRS bandwidth configuration does not imply UE processing bandwidth | Mandatory without capability signalling |
|  | 2-51 | TRS (CSI-RS for tracking) | 1) TRS burst length (X), 2) Max # of TRS resource sets (per CC) UE is able to track simultaneously 3) Max # of TRS resource sets configured to UE per CC 4) Max # of TRS resource sets configured to UE across CCs | 2-50 | csi-RS-ForTracking { 1. maxBurstLength 2. maxSimultaneousResourceSetsPerCC 3. maxConfiguredResourceSetsPerCC 4. maxConfiguredResourceSetsAllCC } | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling Component-1: candidate values {1, ”both 1 and 2”}. UE is mandated to report “both 1 and 2” Component-2: Candidate value set: {1 to 8} Component-3: Candidate value set: {1 to 64} UE is mandated to report at least 8 for FR1 and 16 for FR2. Component-4: Candidate value set: {1 to 256} UE is mandated to report at least 16 for FR1 and 32 for FR2. |
|  | 2-51a | Aperiodic TRS | DCI triggering Aperiodic TRS associated with periodic TRS | 2-50 | aperiodicTRS | MIMO-ParametersPerBand | n/a | Yes |  | Optional with capability signalling |
|  | 2-52 | Basic SRS | 1) Support 1 port SRS transmission 2) Support periodic/aperiodic SRS transmission 3) Support SRS Frequency intra/inter-slot hopping within BWP 4) At least one SRS resource per CC for aperiodic and periodic separately |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 2-53 | SRS resources | 1) Maximum number of aperiodic SRS resources (configured to UE) per BWP 2) Maximum number of aperiodic SRS resources (configured to UE) per BWP per slot 3) Maximum number of periodic SRS resources (configured to UE) per BWP 4) Maximum number of periodic SRS resources (configured to UE) per BWP per slot 5) Maximum number of semi-persistent SRS resources (configured to UE) per BWP 6) Maximum number of semi-persistent SRS resources (configured to UE) per BWP per slot 7) Maximum number of SRS port per resource | 2-52 | supportedSRS-Resources { 1. maxNumberAperiodicSRS-PerBWP 2. maxNumberAperiodicSRS-PerBWP-PerSlot 3. maxNumberPeriodicSRS-PerBWP 4. maxNumberPeriodicSRS-PerBWP-PerSlot 5. maxNumberSemiPersitentSRS-PerBWP 6. maxNumberSP-SRS-PerBWP-PerSlot 7. maxNumberSRS-Ports-PerResource } | FeatureSetUplink | n/a | n/a |  | Mandatory with capability signalling Component-1: candidate value: {from 1, 2, 4, 8, 16} Component-2 candidate value: {1,2,3,4,5,6} Component-3: candidate value: {from 1, 2, 4, 8, 16} Component-4 candidate value: {1,2,3,4,5, 6} Component-5: candidate value: {from 1, 2, 4, 8, 16} } Component-6 candidate value: {1, 2,3,4,5, 6} Component-7 candidate values: {1, 2, 4} Support SP-SRS is mandatory with capability |
|  | 2-55 | SRS Tx switch | 1) Support SRS Tx port switch, 2) Report whether the uplink TX switching impact to downlink receiving in a band, 3) Report whether the UL Tx is switched together with UL Tx in another band | 2-53 | srs-TxSwitch { 1. supportedSRS-TxPortSwitch 2. txSwitchImpactToRx 3. txSwitchWithAnotherBand } | BandParameters-v1540 | n/a | n/a | Component-2 is per band pair per band combination Component-3 is per band pair per band combination The band pair in Component-2 and Component-3 can be an LTE band and an NR band 2T4R is 2 pairs of antennas “R” refers to a subset/set of receive antennas for PDSCH; “T” refers to the SRS antennas used for DL CSI acquisition | Mandatory with capability signalling Component-1 is a list of TRx pairs, candidates are {“Not supported”, “1T2R”, “1T4R”, “2T4R”, “1T4R/2T4R”, “1T=1R”, “2T=2R”, “4T=4R”} Component-2: Candidate value set: {yes, no} Component-3: Candidate value set: {yes, no} |
|  | 2-56 | SRS carrier switch | Report inter-cell switching time capability | 2-53 | srs-CarrierSwitch { srs-SwitchingTimesListNR, or srs-SwitchingTimesListEUTRA } | BandParameters-v1540 | No | n/a | RAN4 reply LS, R1-1805817, includes candidate value sets | Optional with capability signalling |
|  | 2-58 | For SRS for CB PUSCH and antenna switching on FR1, zero slot offset for aperiodic SRS transmission | For SRS for CB PUSCH and antenna switching on FR1, support of zero slot offset between aperiodic SRS triggering and transmission | 2-53 | zeroSlotOffsetAperiodicSRS | FeatureSetUplink-v1540 | n/a | n/a |  | Optional with capability signalling |
|  | 2-59 | Configured spatial relations | Maximum number of configured spatial relations per CC for PUCCH and SRS |  | spatialRelations { 2-59. maxNumberConfiguredSpatialRelations 2-60. maxNumberActiveSpatialRelations 2-61. additionalActiveSpatialRelationPUCCH 2-62. maxNumberDL-RS-QCL-TypeD } | MIMO-ParametersPerBand | n/a | Only applicable to FR2 |  | Candidate value set: {4, 8, 16, 32, 64, 96} UE is mandated to report 16 or higher values. |
|  | 2-60 | Active spatial relations | Maximum total number of {unique DL RS (except for aperiodic NZP CSI-RS) and SRS without spatial relation configured, and, TCI states available for DCI triggering of aperiodic NZP CSI-RS}, for indicating spatial domain transmit filter for PUCCH and SRS for PUSCH, per BWP per CC | 2-59 |  |  | n/a | Only applicable to FR2 | “Unique” means RS identity. An SSB and a CSI-RS are always counted as different. Two CSI-RSs are different if they have different CSI-RS resource IDs. | Mandatory with capability signalling Candidate value set: {1, 2, 4, 8, 14} |
|  | 2-61 | Additional active spatial relation for PUCCH | Support one additional active spatial relation for PUCCH | 2-60 |  |  | n/a | n/a | Only applicable if 2-60 is set to 1. | Mandatory with capability signalling |
|  | 2-62 | Max number of downlink RS resources used for QCL type-D in the active TCI states and active spatial relation info | Max number of downlink RS resources in the active TCI states and active spatial relation info per CC | 2-4, 2-4a and 2-60 |  |  | n/a | n/a | Reference relationship follows 2-4/2-60 | Optional with capability signalling Candidate value set: {1,2,4,8, 14} |
| 3. DL control channel and procedure | 3-1 | Basic DL control channel | 1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and UE-SS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - UP to 3 search space sets in a slot for a scheduled SCell per BWP This search space limit is before applying all dropping rules. - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 3-1’ | For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2 | For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2 |  | type1-3-CSS | FeatureSetDownlink | n/a | n/a |  | Mandatory with capability signalling |
|  | 3-2 | PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot |  | pdcchMonitoringSingleOccasion | Phy-ParametersFR1 | No | Applicable only to FR1 |  | Optional with capability signalling |
|  | 3-3 | More than one CORESET configurations per BWP in addition to CORESET0 | More than one CORESET configurations per BWP in addition to CORESET0 |  | multipleCORESET | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 3-4 | More than one TCI state configurations per CORESET | More than one TCI state configurations per CORESET |  | multipleTCI | BandNR | n/a | n/a | UE is only required to track one active TCI state per CORESET UE is required to support minimum between 64 and number of configured TCI states in 2-4, component 2). | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 3-5 | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 |  | pdcch-MonitoringAnyOccasions { 3-5. withoutDCI-Gap 3-5a. withDCI-Gap } | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 3-5a | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a DCI gap | For type 1 CSS with dedicated RRC configuration, type 3 CSS and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2, with minimum time separation (including the cross-slot boundary case) between two DL unicast DCIs, between two UL unicast DCIs, or between a DL and an UL unicast DCI in different monitoring occasions where at least one of them is not the monitoring occasions of FG-3-1, for a same UE as - 2OFDM symbols for 15kHz - 4OFDM symbols for 30kHz - 7OFDM symbols for 60kHz with NCP - 11OFDM symbols for 120kHz Up to one unicast DL DCI and up to one unicast UL DCI in a monitoring occasion except for the monitoring occasions of FG 3-1. In addition for TDD the minimum separation between the first two UL unicast DCIs within the first 3 OFDM symbols of a slot can be zero OFDM symbols. |  |  |  | n/a | n/a |  | Optional with capability signalling |
|  | 3-5b | All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap | PDCCH monitoring occasions of FG-3-1, plus additional PDCCH monitoring occasion(s) can be any OFDM symbol(s) of a slot for Case 2, and for any two PDCCH monitoring occasions belonging to different spans, where at least one of them is not the monitoring occasions of FG-3-1, in same or different search spaces, there is a minimum time separation of X OFDM symbols (including the cross-slot boundary case) between the start of two spans, where each span is of length up to Y consecutive OFDM symbols of a slot. Spans do not overlap. Every span is contained in a single slot. The same span pattern repeats in every slot. The separation between consecutive spans within and across slots may be unequal but the same (X, Y) limit must be satisfied by all spans. Every monitoring occasion is fully contained in one span. In order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. The span duration is max{maximum value of all CORESET durations, minimum value of Y in the UE reported candidate value} except possibly the last span in a slot which can be of shorter duration. A particular PDCCH monitoring configuration meets the UE capability limitation if the span arrangement satisfies the gap separation for at least one (X, Y) in the UE reported candidate value set in every slot, including cross slot boundary. For the set of monitoring occasions which are within the same span: • Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD • Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD • Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD The number of different start symbol indices of spans for all PDCCH monitoring occasions per slot, including PDCCH monitoring occasions of FG-3-1, is no more than floor(14/X) (X is minimum among values reported by UE). The number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions of FG-3-1, is no more than 7. The number of different start symbol indices of PDCCH monitoring occasions per half-slot including PDCCH monitoring occasions of FG-3-1 is no more than 4 in SCell. |  | pdcch-MonitoringAnyOccasionsWithSpanGap (X, Y): set1 = (7, 3); set2 = (4, 3) and (7, 3); set3 = (2, 2) and (4, 3) and (7, 3). | FeatureSetDownlink-v1540 | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3) and (7, 3), (2, 2) and (4, 3) and (7, 3)} |
|  | 3-6 | Dynamic SFI monitoring | Adjust periodic and semi-persistent signal reception and transmission in response to detected dynamic UL/DL configuration |  | dynamicSFI | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Optional with capability signalling |
|  | 3-7 | Precoder-granularity of CORESET size | Precoder-granularity of CORESET size |  | precoderGranularityCORESET | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 3-8 | Up to 10 search spaces in a SCell | Up to 10 search spaces in a slot in an SCell per BWP |  | maxNumberSearchSpaces | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
| 4. UL control channel and procedure | 4-1 | Basic UL control channel | 1) PUCCH format 0 over 1 OFDM symbols once per slot 2) PUCCH format 0 over 2 OFDM symbols once per slot with frequency hopping as “enabled” 3) PUCCH format 1 over 4 – 14 OFDM symbols once per slot with intra-slot frequency hopping as “enabled” 5) One SR configuration per PUCCH group 6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI 7) SR/HARQ multiplexing once per slot using a PUCCH when SR/HARQ-ACK are supposed to be sent by overlapping PUCCH resources with the same starting symbols in a slot 8) HARQ-ACK piggyback on PUSCH with/without aperiodic CSI once per slot when the starting OFDM symbol of the PUSCH is the same as the starting OFDM symbols of the PUCCH resource that HARQ-ACK would have been transmitted on 9) Semi-static beta-offset configuration for HARQ-ACK 10) Single group of overlapping PUCCH/PUCCH and overlapping PUCCH/PUSCH s per slot per PUCCH cell group for control multiplexing |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 4-2 | 2 PUCCH of format 0 or 2 in consecutive symbols | 1) 2 PUCCH format 0/2 in different symbols and once per slot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per slot for SR 3) 2 PUCCH format 2 in different symbols and once per slot for CSI over two consecutive OFDM symbols |  | twoPUCCH-F0-2-ConsecSymbols | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Optional with capability signalling |
|  | 4-3 | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as “enabled” | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as “enabled” |  | pucch-F2-WithFH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling which shall be set to ‘1’ |
|  | 4-4 | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” |  | pucch-F3-WithFH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling which shall be set to ‘1’ |
|  | 4-5 | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” |  | pucch-F4-WithFH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 4-6 | Non-frequency hopping for PUCCH formats 0 and 2 with frequency hopping as “disabled” | Non-frequency hopping for PUCCH formats 0 and 2 with frequency hopping as “disabled” |  | freqHoppingPUCCH-F0-2 | Phy-ParametersFRX-Diff | No | Yes | The value indicated by this field is “notSupported”. | Mandatory with capability signalling |
|  | 4-7 | Non-frequency hopping for PUCCH format 1, 3, and 4 with frequency hopping as “disabled” | Non-frequency hopping for PUCCH format 1, 3, and 4 with frequency hopping as “disabled” |  | freqHoppingPUCCH-F1-3-4 | Phy-ParametersFRX-Diff | No | Yes | The value indicated by this field is “notSupported”. | Mandatory with capability signalling |
|  | 4-10 | Dynamic HARQ-ACK codebook | Dynamic HARQ-ACK codebook |  | dynamicHARQ-ACK-Codebook | Phy-ParametersCommon | No | No |  | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 4-11 | Semi-static HARQ-ACK codebook | Semi-static HARQ-ACK codebook |  | semiStaticHARQ-ACK-Codebook | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 4-12 | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group |  | spatialBundlingHARQ-ACK | Phy-ParametersCommon | No | No | Applicable to UE supporting more than 4 layers | Mandatory with capability signalling |
|  | 4-13 | More than one SR configurations per PUCCH group | More than one SR configurations per PUCCH group |  | multipleSR-Configurations | MAC-ParametersXDD-Diff | Yes | No |  | Optional with capability signalling |
|  | 4-19 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same starting symbol on the PUCCH resources in a slot | SR/HARQ-ACK/CSI multiplexing once per slot, where overlapping PUCCH resources have the same starting symbols on the PUCCH resources in a slot while precluding the case of SR/HARQ-ACK by overlapping PUCCH resources with the same starting symbols on the PUCCH resources in a slot |  | sameSymbol in mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot | Phy-ParametersFRX-Diff | No | Yes | If FG4-28 is not included or not supported, HARQ-ACK/CSI piggyback on PUSCH once per slot when the starting OFDM symbol of the PUSCH is the same as the starting OFDM symbols of the PUCCH resource(s) that would have been transmitted on If FG4-28 is included and supported, HARQ-ACK/CSI piggyback on PUSCH once per slot for which case the starting OFDM symbol of the PUSCH is the different from the starting OFDM symbols of the PUCCH resource(s) that would have been transmitted on | Mandatory with capability signalling |
|  | 4-19a | SR/HARQ-ACK multiplexing once per slot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK are supposed to be sent with different starting symbols in a slot | Overlapping PUCCH resources have different starting symbols in a slot | 4-19 | mux-SR-HARQ-ACK-PUCCH | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-19b | SR/HARQ-ACK/CSI multiplexing more than once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same or different starting symbol in a slot | Overlapping PUCCH resources have same or different starting symbols in a slot | 4-19c | mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-19c | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with different starting symbols in a slot | Overlapping PUCCH resources have different starting symbols in a slot | 4-19a | diffSymbol in mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-20 | UCI code-block segmentation | UCI code-block segmentation |  | uci-CodeBlockSegmentation | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 4-21 | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI |  | dynamicBetaOffsetInd-HARQ-ACK-CSI | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 4-22 | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3, or 4 in the same slot | 1 PUCCH format 0 or 2and 1 PUCCH format 1, 3, and 4 in the same slot |  | onePUCCH-LongAndShortFormat | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-22a | 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2 | 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2 |  | twoPUCCH-AnyOthersInSlot | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-23 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 |  | pucch-Repetition-F1-3-4 | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 4-24 | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource |  | pucch-SpatialRelInfoMAC-CE | BandNR | n/a | n/a |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 |
|  | 4-25 | Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA | Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA | 2-52, 4-1, 2-12, 6-6 | parallelTxSRS-PUCCH-PUSCH | CA-ParametersNR | n/a | n/a | This feature is supported only in inter-band CA. | Optional with capability signalling |
|  | 4-26 | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | 1-1, 2-52, 4-1, 2-12, 6-6 | parallelTxPRACH-SRS-PUCCH-PUSCH | CA-ParametersNR | n/a | n/a | This feature is supported only in inter-band CA. | Optional with capability signalling |
|  | 4-27 | More than one group of overlapping channels for control multiplexing | More than one group of overlapping PUCCHs and PUSCHs per slot per PUCCH cell group for control multiplexing |  | mux-MultipleGroupCtrlCH-Overlap | Phy-ParametersFRX-Diff | No | Yes | If a UE does not indicate supporting any of 4-2, 4-22, and 4-22a, the UE is not expected to be scheduled with more than one group of overlapping PUCCHs without PUSCH in each of the groups | Optional with capability signalling |
|  | 4-28 | HARQ-ACK multiplexing on PUSCH with different PUCCH/PUSCH starting OFDM symbols | HARQ-ACK piggyback on a PUSCH with/without aperiodic CSI once per slot when the starting OFDM symbol of the PUSCH is different from the starting OFDM symbols of the PUCCH resource that HARQ-ACK would have been transmitted on | 4-1 | mux-HARQ-ACK-PUSCH-DiffSymbol | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | 1) Frequency-domain resource allocation - RA Type 0 only and Type 1 only for PDSCH without interleaving - RA Type 1 for PUSCH without interleaving 2) Time-domain resource allocation - 1-14 OFDM symbols for PUSCH once per slot - One unicast PDSCH per slot - Starting symbol, and duration are determined by using the DCI - PDSCH mapping type A with 7-14 OFDM symbols - PUSCH mapping type A and type B - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, PDSCH mapping type A with {4-14} OFDM symbols and type B with {2, 4, 7} OFDM symbols 3) TBS determination 4) Nominal UE processing time for N1 and N2 (Capability #1) 5) HARQ process operation with configurable number of DL HARQ processes of up to 16 6) Cell specific RRC configured UL/DL assignment for TDD 7) Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment 8) Intra-slot frequency-hopping for PUSCH scheduled by Type 1 CSS before RRC connection 9) In TDD support at most one switch point per slot for actual DL/UL transmission(s) 10) DL scheduling slot offset K0=0 11) DL scheduling slot offset K0=1 for type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS 12) UL scheduling slot offset K2<=12 For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, interleaving for VRB-to-PRB mapping for PDSCH |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 5-1a | UE specific RRC configure UL/DL assignment | Dynamic UL/DL determination based on L1 scheduling DCI with cell-specific and UE specific RRC configured UL/DL assignment |  | ue-SpecificUL-DL-Assignment | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 5-1b | More than one DL/UL switch point in a slot | In TDD support more than one switch points in a slot for actual DL/UL transmission(s) |  | tdd-MultiDL-UL-SwitchPerSlot | Phy-ParametersFRX-Diff | TDD only | Yes |  | Optional with capability signalling |
|  | 5-2 | RA Type 0 for PUSCH | RA Type 0 for PUSCH |  | ra-Type0-PUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-3 | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH |  | dynamicSwitchRA-Type0-1-PDSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-4 | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH | 5-2 | dynamicSwitchRA-Type0-1-PUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-5a | UE PDSCH processing capability #2 | UE can report values ‘X’ and ‘Fallback’, and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 1 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled, otherwise - If Fallback = ‘SC’, UE supports Capability #2 processing time on lowest cell index among the configured carriers in the band where the value is reported - If Fallback = ‘Cap1-only’, UE supports only Capability #1, in the band where the value is reported 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz |  | pdsch-ProcessingType2 | FeatureSetDownlink-v1540 | n/a | Applicable to FR1 only | This capability is necessary for each SCS (15kHz, 30kHz, 60kHz) More than one set of per SCS per band reports can be signaled for a given band combination | ptional with capability signaling Candidate values for Component 1: X in {1, ..., 16}, Fallback {‘SC’,’Cap1-only’} |
|  | 5-5b | UE PDSCH processing capability #2 with scheduling limitation for 30kHz-SCS | Capability #2 supported only if 1 carrier configured in the band (independent of #carriers configured in other bands) 2) Max PDSCH BW of 136 PRBs on the configured serving cell which processingType2Enabled is configured and set to enabled 3) N1 based on Table 5.3-2 of TS 38.214 for 30 kHz SCS 4) UE reports the number of unicast PDSCH per slot for different TBs |  | pdsch-ProcessingType2-Limited | FeatureSetDownlink-v1540 | n/a | Applicable to FR1 only | This capability is applicable to 30kHz-SCS only | Optional with capability signaling Component 4) the value ranges {1, 2, 4, 7} |
|  | 5-5c | UE PUSCH processing capability #2 | UE can report values ‘X’ and ‘Fallback’, and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 1 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled, otherwise - If Fallback = ‘SC’, UE supports Capability #2 processing time on lowest cell index among the configured carriers in the band where the value is reported - If Fallback = ‘Cap1-only’, UE supports only Capability #1, in the band where the value is reported 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz |  | pusch-ProcessingType2 | FeatureSetUplink-v1540 | n/a | Applicable to FR1 only | This capability is necessary for each SCS (15kHz, 30kHz, 60kHz) More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signaling Candidate values for Component 1: X in {1, …, 16}, Fallback {‘SC’,’Cap1-only’} |
|  | 5-6 | PDSCH mapping type A with less than 7 OFDM symbols | or type 1 CSS with dedicated RRC configuration, for type 3 CSS and UE-SS, PDSCH mapping type A with less than 7 OFDM symbols |  | pdsch-MappingTypeA | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling which shall be set to “1” |
|  | 5-6a | PDSCH mapping type B | PDSCH mapping type B |  | pdsch-MappingTypeB | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-7 | Interleaving for VRB-to-PRB mapping for PDSCH | Interleaving for VRB-to-PRB mapping for PDSCH |  | interleavingVRB-ToPRB-PDSCH | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-9 | Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 CSS before RRC connection | Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 CSS before RRC connection |  | intraSlotFreqHopping-PUSCH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 5-10 | Inter-slot frequency hopping for PUSCH | Inter-slot frequency hopping for PUSCH |  | interSlotFreqHopping-PUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-11 | Up to 2 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 2 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  | pdsch-ProcessingType1-DifferentTB-PerSlot | FeatureSetDownlink | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-11a | Up to 7 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 7 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-11b | Up to 4 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 4 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-12 | Up to 2 PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 2 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  | pusch-ProcessingType1-DifferentTB-PerSlot | FeatureSetUplink | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-12a | Up to 7 PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 7 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-12b | Up to 4 PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 4 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-13 | Up to 2 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 2 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 2 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a | pdsch-ProcessingType2 | FeatureSetDownlink | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13a | Up to 7 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 7 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 7 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a |  |  | n/a | n//a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13c | Up to 4 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 4 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 4 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a |  |  | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13d | Up to 2 PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 2 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 2 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c | pusch-ProcessingType2 | FeatureSetUplink | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13e | Up to 7 PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 7 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 7 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c |  |  | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13f | Up to 4 PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 4 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 4 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c |  |  | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-14 | Type 1 configured PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions with RV sequences |  | type1-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-16 | Type 2 configured PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions with RV sequences |  | type2-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-17 | PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions |  | pusch-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-17a | PDSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions |  | pdsch-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-18 | DL SPS | DL SPS |  | downlinkSPS | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-19 | Type 1 Configured UL grant | K = 1 |  | configuredUL-GrantType1 | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-20 | Type 2 Configured UL grant | K = 1 |  | configuredUL-GrantType2 | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-21 | Pre-emption indication for DL | Pre-emption indication for DL |  | pre-EmptIndication-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-22 | CBG-based re-transmission for DL using CBGTI | CBG-based re-transmission for DL using CBGTI |  | cbg-TransIndication-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-23 | CBGFI for CBG-based re-transmission for DL | CBGFI for CBG-based re-transmission for DL | 5-22 | cbg-FlushIndication-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-24 | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL |  | dynamicHARQ-ACK-CodeB-CBG-Retx-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-25 | CBG-based re-transmission for UL using CBGTI | CBG-based re-transmission for UL using CBGTI |  | cbg-TransIndication-UL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-26 | Semi-static rate-matching resource set configuration for DL | 1) Bitmap 1/2/3 2) controlResourceSet |  | rateMatchingResrcSetSemi-Static | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-27 | Dynamic rate-matching resource set configuration for DL | Bitmap 1/2/3 |  | rateMatchingResrcSetDynamic | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-27a | Dynamic rate-matching control resource set for DL | Dynamic rate-matching control resource set for DL |  | rateMatchingCtrlResrcSetDynamic | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-28 | Rate-matching around LTE CRS | Rate-matching around LTE CRS |  | rateMatchingLTE-CRS | BandNR | n/a | n/a |  | Mandatory with capability signalling |
|  | 5-29 | LBRM for PUSCH | Limited buffer rate matching in UL |  | pusch-LBRM | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 5-30 | DL scheduling slot offset greater than zero for PDSCH mapping type A | Support of DL scheduling slot offset (K0) greater than zero for PDSCH mapping type A |  | dl-SchedulingOffset-PDSCH-TypeA | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 5-30a | DL scheduling slot offset greater than zero for PDSCH mapping type B | Support of DL scheduling slot offset (K0) greater than zero for PDSCH mapping type B |  | dl-SchedulingOffset-PDSCH-TypeB | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 5-31 | UL scheduling slot offset greater than 12 | Support of UL scheduling slot offset (K2) greater than 12 |  | ul-SchedulingOffset | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 5-32 | Separation of two unicast PDSCHs with a gap | For any two consecutive slots n and n+1, if there are more than 1 unicast PDSCH in either slot, the minimum time separation between starting time of any two unicast PDSCHs within the duration of these slots is 4 OFDM symbol for 30kHz and 7 OFDM symbol for 60kHz | 5-11, 5-11b, 5-13, or 5-13c | pdsch-SeparationWithGap | FeatureSetDownlink-v1540 | No | No | This feature only applies to SCS 30kHz and 60kHz | Optional with capability signalling |
|  | 5-33 | Separation of two unicast PUSCHs with a gap | For any two consecutive slots n and n+1, if there are more than 1 unicast PUSCH in either slot, the minimum time separation between starting time of any two unicast PUSCHs within the duration of these slots is 2OFDM symbols for 15kHz, 4 OFDM symbols for 30kHz and 7 OFDM symbols for 60kHz | 5-12, 5-12b, 5-13d, or 5-13f | pusch-SeparationWithGap | FeatureSetUplink-v1540 | No | No | This feature only applies to SCS 15kHz, 30kHz and 60kHz | Optional with capability signalling |
|  | 5-34 | New 64QAM MCS table for PDSCH | New 64QAM MCS table for PDSCH |  | dl-64QAM-MCS-TableAlt | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 5-34a | New 64QAM MCS table for PUSCH | New 64QAM MCS tables for PUSCH with and without transform precoding respectively |  | ul-64QAM-MCS-TableAlt | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 5-34b | Dynamic indication of MCS table with MCS-C-RNTI for PDSCH | Dynamic indication of MCS table using MCS-C-RNTI for PDSCH | 5-34 | dl-MCS-TableAlt-DynamicIndication | FeatureSetDownlink-v1540 | n/a | n/a |  | Optional with capability signalling |
|  | 5-34c | Dynamic indication of MCS tables with MCS-C-RNTI for PUSCH | Dynamic indication of MCS tables using MCS-C-RNTI for PUSCH | 5-34a | ul-MCS-TableAlt-DynamicIndication | FeatureSetUplink-v1540 | n/a | n/a |  | Optional with capability signalling |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |  | n/a | n/a | n/a | n/a | This feature should be mandatory without capability signalling for at least BWPs which is the same as the set of specified channel BW UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP | Mandatory without capability signalling |
|  | 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell | 6-1, 6-2, 6-3, or 6-4 | bwp-WithoutRestriction | BandNR | n/a | n/a | 6-1a is applicable to 6-1, 6-2, 6-3, or 6-4. | Optional with capability signalling |
|  | 6-2 | Type A BWP adaptation with same numerology | 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto2 in bwp-SameNumerology | BandNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-3 | Type B BWP adaptation with same numerology | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto4 in bwp-SameNumerology | BandNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-4 | BWP adaptation with different numerologies | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto4 in bwp-DiffNumerology | BandNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-5 | Basic DL NR-NR CA operation | 1) Up to16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |  | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of DL CA band combination(s). | Optional with capability signalling |
|  | 6-5a | PDCCH blind detection capability for CA | 1) More than 4 DL CCs 2) Reporting value is one of integer from 4 to 16 | 6-5 | pdcch-BlindDetectionCA | Phy-ParametersFRX-Diff | No | Yes | If UE supports CA with more than 4 DL CCs, UE should report this capability | {4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16} |
|  | 6-6 | Basic UL NR-NR CA operation | 1) Up to16 UL carriers 2) Same numerology across carrier for data/control channel at a given time 3) One PUCCH group 4) Single TAG | 6-5 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of UL CA band combination(s). The terminology ‘carrier’ in the components in this FG does not refer to ‘SUL’. | Optional with capability signalling |
|  | 6-7 | Two NR PUCCH group with same numerology | 1) For NR CA UE, same numerology across NR carriers for data/control channel at a given time 2) For EN-DC UE, same numerology across NR carriers for data/control channel at a given time, wherein an NR PUCCH group is configured in FR1 and another NR PUCCH group is configured in FR2 | 6-5, 6-6 | twoPUCCH-Group | FeatureSetUplink | n/a | n/a |  | Optional with capability signalling |
|  | 6-8 | Different numerology across NR PUCCH groups | For both NR CA UE and EN-DC UE, different numerology between two NR PUCCH groups for data/control channel at a given time | 6-5, 6-7 | diffNumerologyAcrossPUCCH-Group | CA-ParametersNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-9 | Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of smaller SCS | 1) For both NR CA UE, EN-DC/NE-DC UE and NR-DC UEs, same numerology between DL and UL per carrier for data/control channel at a given time 2) For both NR CA UE and EN-DC/NE-DC UE with one NR PUCCH group, different numerologies across NR carriers within the same NR PUCCH groups up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with smaller SCS for data/control channel at a given time 3-1) For NR CA UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with smaller SCS for data/control channel at a given time 3-2) For EN-DC/NE-DC UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within an NR PUCCH group in FR1 wherein NR PUCCH is sent on the carrier with smaller SCS, and same numerology across NR carriers within another NR PUCCH group in FR2 for data/control channel at a given time 4) For NR DC UE, different numerologies across NR carriers within the same NR PUCCH group in MCG (in FR1) and up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with smaller SCS for data/control channel at a given time; and same numerology across NR carriers in SCG (in FR2). | 6-5 | diffNumerologyWithinPUCCH-GroupSmallerSCS | CA-ParametersNR | n/a | n/a | The terminologies ‘UL’ and ‘carrier’ in this FG do not refer to ‘SUL’. NR PUCCH is sent on a carrier with SCS not larger than SCS of any DL carriers corresponding to the NR PUCCH group. The case with PUCCH on UL carrier with different numerologies within SCG is not supported for NR-DC. | Optional with capability signalling |
|  | 6-9a | Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of larger SCS | 1) For both NR CA UE, EN-DC/NE-DC UE and NR DC UEs, same numerology between DL and UL per carrier for data/control channel at a given time 2) For both NR CA UE and EN-DC/NE-DC UE with one NR PUCCH group, different numerologies across NR carriers within the same NR PUCCH groups up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is on the carrier with larger SCS for data/control channel at a given time 3-1) For NR CA UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with larger SCS for data/control channel at a given time 3-2) For EN-DC/NE-DC UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within an NR PUCCH group in FR1 wherein NR PUCCH is sent on the carrier with larger SCS, and same numerology across NR carriers within another NR PUCCH group in FR2 for data/control channel at a given time 4) For NR DC UE, different numerologies across NR carriers within the same NR PUCCH group in MCG (in FR1) and up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with larger SCS for data/control channel at a given time; and same numerology across NR carriers in SCG (in FR2). | 6-5 | diffNumerologyWithinPUCCH-GroupLargerSCS | CA-ParametersNR-v1560 | n/a | n/a | The terminologies ‘UL’ and ‘carrier’ in this FG do not refer to ‘SUL’. NR PUCCH is sent on a carrier with SCS not smaller than SCS of any DL carriers corresponding to the NR PUCCH group. The case with PUCCH on UL carrier with different numerologies within SCG is not supported for NR-DC. | Optional with capability signalling |
|  | 6-10 | Cross carrier scheduling for the same numerology | Cross carrier scheduling for the same numerology with CIF where numerologies for scheduling cell and scheduled cell are same | 6-5, 6-6 | crossCarrierScheduling-SameSCS | BandNR | n/a | n/a |  |  |
|  | 6-10a | Cross carrier scheduling for different numerologies | Cross carrier scheduling for the different numerologies with CIF where numerologies for scheduling cell and scheduled cell are different | 6-10 | crossCarrierScheduling-OtherSCS | FeatureSetDownlink FeatureSetUplink | n/a | n/a | This is not supported in Rel-15 |  |
|  | 6-11 | Number of supported TAGs | Need of multiple capability question about the resolution here |  | supportedNumberTAG | CA-ParametersNR | n/a | n/a | This feature group is applied to NR-NR CA and EN-DC. For EN-DC, the feature group indicates number of TAGs only for NR CG. The number of TAGs for the LTE MCG is signalled by existing LTE TAG capability signalling | {1, 2, 3, 4} |
|  | 6-12 | Support 2 simultaneous UL transmissions for problematic cases | Support 2 simultaneous UL transmissions for problematic cases |  | singleUL-Transmission | MRDC-Parameters | n/a | n/a | This is a UE feature for LTE for a LTE/NR dual connectivity UE | Optional with capability signalling |
|  | 6-13 | Case 1 Single Tx UL LTE-NR DC | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) HARQ subframe offset |  | tdm-Pattern NOTE: This capability bit also indicates support of the feature 8-2, i.e. Operation A with single UL Tx case 1. | MRDC-Parameters | Yes | Yes | This is a UE feature for LTE for a LTE/NR dual connectivity UE | Mandatory with capability signalling conditional on the UE not supporting simultaneous dual-Tx operation in the band combination; optional if the UE supports simultaneous dual-Tx operation in the band combination |
|  | 6-16 | Supplemental uplink | 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL 2) Supplemental uplink with same numerology between SUL and non SUL carriers | 6-15 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of SUL band combination(s). | Optional with capability signalling |
|  | 6-17 | Supplemental uplink with different numerologies between SUL and non SUL carriers | Different numerologies between SUL and non SUL | 6-16 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of SUL band combination(s). | Mandatory with capability signalling |
|  | 6-18 | Supplemental uplink with dynamic switch | DCI based selection of PUSCH carrier | 6-16 | dynamicSwitchSUL | FeatureSetUplink | n/a | n/a | his is conditioned on the support of SUL band combination(s). | Optional with capability signalling |
|  | 6-19 | Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS on the other UL carrier in the same cell | Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS on the other UL carrier in the same cell | 6-16 | simultaneousTxSUL-NonSUL | FeatureSetUplink | n/a | n/a |  | Optional with capability signalling |
|  | 6-21 | DL search space sharing for CA | DL search space sharing for CA | 6-10 or 6-10a | searchSpaceSharingCA-DL | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 6-22 | UL search space sharing for CA | UL search space sharing for CA | 6-10 or 6-10a | searchSpaceSharingCA-UL | FeatureSetUplink | n/a | n/a |  | Optional with capability signalling |
|  | 6-23 | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP |  | pa-PhaseDiscontinuityImpacts | FeatureSetUplink-v1540 | n/a | n/a | See LS (R1-1809992) | Optional with capability signalling |
|  | 6-24 | Applying the same UL timing between NR and LTE | Applying the same UL timing between NR and LTE for dynamic power sharing capable UE operating in intra-band contiguous synchronous EN-DC | 8-1 | ul-TimingAlignmentEUTRA-NR | MRDC-Parameters | n/a | n/a | UEs that set this bit to 0 should be able to operate with a timing difference up to applicable MTTD requirements when operating in a synchronous intra-band contiguous EN-DC network. | Optional with capability signalling |
|  | 6-25 | Support of synchronous NR-NR DC operation only wherein MCG is only in FR1 and SCG is only in FR2 | Support of synchronous NR-NR DC operation only wherein MCG is only in FR1 and SCG is only in FR2 |  | ca-ParametersNRDC | BandCombination-v1560 | n/a | n/a | This is conditioned on the support of DC band combination(s). UE reports a set of supported band partitionings corresponding to MCG in FR1 and to SCG in FR2. | Optional with capability signalling |
|  | 6-25a | PDCCH blind detection capability for MCG and for SCG in synchronous NR-NR DC | RRC parameters pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE for optional new UE capability signalling that informs the maximum values for pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively | 6-5, 6-25 | pdcch-BlindDetectionMCG-UE pdcch-BlindDetectionSCG-UE | Phy-ParametersFRX-Diff | No | Yes | pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE are per UE capability signalling. The value range of pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE is - [1, …, pdcch-BlindDetectionCA-1] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and - [1, 2, 3] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= the maximum number of DL serving cells over CGs that UE can support if the UE does not report pdcch-BlindDetectionCA. If the UE does not report pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, - pdcch-BlindDetectionCA for NR-CA is re-used as the UE capability signalling for NR-DC to determine BD/CCE limit across serving cells over CGs if the UE reports pdcch-BlindDetectionCA, and - the number of configured DL serving cells over CGs is used to determine BD/CCE limit across serving cells over CGs if the UE does not report pdcch-BlindDetectionCA. If the UE reports pdcch-BlindDetectionMCG-UE or pdcch-BlindDetectionSCG-UE, both of them are reported (i.e., not either of them). | Optional with capability signalling |
| 7. Channel coding | 7-1 | Channel coding | 1) LDPC encoding and associated functions for data on DL and UL 2) Polar encoding and associated functions for PBCH, DCI, and UCI 3) Coding for very small blocks |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
| 8. UL TPC | 8-1 | Dynamic power sharing for LTE-NR DC | When total transmission power exceeds Pcmax, UE scales NR transmission power. | EN-DC | dynamicPowerSharing | MRDC-Parameters | n/a | n/a | RP-172833 | Mandatory with capability signalling |
|  | 8-2 | Operation A with single UL Tx case 1 | Operation A with single UL Tx case 1 | EN-DC | tdm-Pattern NOTE: This capability bit also indicates support of the feature 6-13, i.e. Case 1 Single Tx UL LTE-NR DC. | MRDC-Parameters | Yes | Yes | RP-172833 | Mandatory with capability signalling conditioned that UE does not support dynamic power sharing, i.e., UE indicate “0” as non-support for 8-1, optional for UEs supporting dynamic power sharing |
|  | 8-3 | Basic power control operation | 1) Accumulated power control mode for closed loop 2) 1 TPC command loop for PUSCH, PUCCH respectively 3) One or multiple DL RS configured for pathloss estimation 4) One or multiple p0-alpha values configured for open loop PC 5) PUSCH power control 6) PUCCH power control 7) PRACH power control 8) SRS power control 9) PHR |  | n/a | n/a | No | No |  | Mandatory without capability signalling |
|  | 8-4 | TPC-PUSCH-RNTI | Specific group DCI message for TPC commands for PUSCH |  | tpc-PUSCH-RNTI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-5 | TPC-PUCCH-RNTI | Specific group DCI message for TPC commands for PUCCH |  | tpc-PUCCH-RNTI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-6 | TPC-SRS-RNTI | Specific group DCI message for TPC commands for SRS |  | tpc-SRS-RNTI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-7 | Absolute TPC command mode | Absolute TPC command mode |  | absoluteTPC-Command | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-8 | UL power control with 2 PUSCH closed loops | Two different TPC loops |  | twoDifferentTPC-Loop-PUSCH | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 8-9 | UL power control with 2 PUCCH closed loops | Two different TPC loops |  | twoDifferentTPC-Loop-PUCCH | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |