3GPP TSG RAN WG1#99                                      R1-1913397

Reno, USA, November 18th – 22th, 2019

Source:	vivo

Title:	Summary of offline discussion on ULFPTx

Agenda Item:	7.2.8.4

Document for:	Discussion and Decision

Offline agreement

- RRC parameters ULFPTx, ULFPTxModes are configured per UL BWP

Possible Proposal

- For Mode 2, full power TPMI indication is optional, if full power TPMI is not reported then full power Tx for rank 1 is only supported with 1 port SRS.

Possible Agreement

For 2 ports, max number of bits to indicate TPMI(s) which can deliver UL full power:

  - Alt 1: 1 bit

  - Alt 2: 2 bits (this does not imply that the same principle will be used to determined the bitwidth for the 4 port case)

Offline Proposal:

For Mode 2, for 4 ports, max number of bits to indicate TPMI(s) which can deliver UL full power:

  - Alt 1: bitmap based scheme, max 10 or 11 bits

Support: Futurewei, Huawei/HiSi, Apple, ZTE/Sanechips. OPPO, DOCOMO, LGE

Concern: Samsung, Intel, CATT, IDC, QC, CMCC Nokia/NSB, Ericsson (9)

  - Alt 2: based on size of SRS resource, bitmap based , max 4 bit

Support: IDC, Nokia/NSB

Concern: ZTE/Sanechips, Futurewei, DOCOMO, Apple, LGE, Huawei/HiSi (8)

  - Alt2a: based on number non-zero elements of TPMI precoders, 4 bits bitmap

Support: QC, Nokia/NSB, CMCC, CATT, IDC

Concern: Apple, ZTE/Sanechips, Futurewei, DOCOMO, Huawei/HiSi (7)

  - Alt3: Only for rank1 TPMI, bitmap based scheme

    - For higher rank can be implicitly derive full power TPMI

    - 4 bits for non-coherent, in addition rank 2 full power Tx TPMIs are also considered

    - 6 bits for partial-coherent, based only on rank 1 full power Tx TPMI

Support: Samsung, CATT, LGE,

Concern: Apple, ZTE/Sanechips, Futurewei, DOCOMO, Huawei/HiSi (7)

  - Alt4: based on TPMI combination per rank,

    - 2 or 3 or 6 bits for non-coherent

    - 4 bits for partial-coherent

Support: Intel, CMCC, Ericsson, Nokia/NSB, IDC

Concern: Apple, ZTE/Sanechips, CATT, DOCOMO, LGE, QC, Huawei/HiSi (9)

Possible Agreement

For Mode-2, for the codebook-based UL PUSCH of a UE configured with a 2-port SRS resource and a 4-port SRS resource for full Tx power transmission, if codebookSubset is configured as partialAndNonCoherent,

- The DCI indicates a TPMI from the Coherent or nonCoherent subset if the SRI indicates a 2-port SRS resource

  - Support of Coherent or noncoherent subset depends on UE capability

    - This capability is supported in UE granularity