3GPP TSG-RAN WG1 #99		R1-1913407

Reno, USA, November 18th – 22nd, 2019

Source: 	Apple Inc.

Title:                     Feature lead summary #2 for uplink power control for NR-NR DC

Agenda item:	7.2.13.1

Document for:	Discussion and Decision

# 1 Introduction

This document summarizes contributions submitted to RAN1#99 on AI 7.2.13.1 to facilitate progress on uplink power control for NN-DC.

Table of Contents

1 Introduction	1

2. On Semi-Static Power Sharing Operation	2

2.1 On Semi-Static Power Sharing	2

Issue 1: Synchronous and asynchronous DC scenarios for Alt. 2	2

Issue 2: Synchronous and asynchronous DC scenarios for Alt. 1-2	3

Issue 3: Slot-level vs. symbol-level overlapping determination	4

2.2 On Details of Dynamic Power Sharing	5

Issue 5: With ‘Look-ahead’ behavior	5

Issue 6: Without ‘Look-ahead’ behavior	8

3 Others	8

Issue 7: On support for FR1+FR2 CA in one or both CGs	8

Issue 8: Power scaling vs. dropping for deprioritized Uplink Transmissions	9

# 2. On Semi-Static Power Sharing Operation

## 2.1 On Semi-Static Power Sharing

The following was agreed in RAN1 #98 meeting for semi-static power sharing operation for NR-NR DC [1]:

Agreements: - Considering the following two alternatives for semi-static power sharing with PMCG+ PSCG≤PTotalNN- DC - Alt.1: For the uplink transmission in MCG, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of SCG, and vice versa. - If such overlapping with UL transmission on the SCG is possible (i.e. collides with semi-static ‘UL’ and ‘flexible’ symbols on some CCs of SCG), UE limits its actual transmission power pMCGin MCG such that pMCG≤PMCG; - Otherwise (i.e. collides with only semi-static ‘DL’ symbols on all CCs of SCG), pMCG can be up to PMCG' and pSCG can be up to PSCG' . - Alt.1-1: PMCG' and PSCG' are configured by RRC signaling. - Alt.1-2: PMCG' and PSCG' are determined by RAN4 requirement. - Alt.2: For the uplink transmission in MCG and in SCG, UE limits its actual transmission power pMCG to be up to PMCGand pSCG to be up to PSCG

In RAN1 #98bis meeting, it was further agreed for semi-static power sharing operation [2]:

Agreements: - Adopt Alt.1-2 and Alt.2 for semi-static power sharing for NR-NR DC. - Alt.1-2 is only subject to configured maximum transmission power defined by RAN4 - Configuration between Alt.1-2 and Alt.2 is supported. - FFS: add more clarification - FFS: applied for synchronous DC only or applied for both synchronous and asynchronous DC (which may be the same or different for Alt.1-2 and Alt. 2) Agreements: For semi-static power sharing for NR-NR DC - Virtual PHR for active CCs of another CG

### Issue 1: Synchronous and asynchronous DC scenarios for Alt. 2

One of remaining issues is applicable deployment scenario for semi-static power sharing operations. The proposals from different companies for Alt.2 can be summarized in the following table.

Table 1: Summary of companies views on applicable DC deployment scenarios of Alt.2

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: Synchronous DC only |  |  |
| Alt.2: Both synchronous and asynchronous DC | 6 | OPPO, CATT, Intel, Ericsson, Apple, Samsung |

It is apparent that applying semi-static power sharing Alt.2 to both synchronous and asynchronous DC scenarios is supported for majority companies and the following was therefore proposed:

[Agreement]:

- Alt.2 of semi-static power sharing can be configured for both synchronous and asynchronous DC scenarios.

### Issue 2: Synchronous and asynchronous DC scenarios for Alt. 1-2

Table 2: Summary of companies views on applicable DC deployment scenarios of Alt.1-2

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: Synchronous DC only | 6 | OPPO, Intel, Ericsson, MTK, Huawei, HiSilicon |
| Alt.2: Both synchronous and asynchronous DC | 4 | vivo, CATT, Samsung, ZTE (if UE supports determining the time offset between MCG and SCG e.g. based on UE capability sftd-MeasPSCell) |

Another relevant FFS aspect for Alt.1-2 is the need to add the following clarification:

It is up to UE to determine whether the overlapping with UL transmission on the SCG is possible, if/when factors other than the TDD UL-DL configurations of the serving cells in the SCG (e.g., timing difference, drift) need to be taken into account.

Only 4 companies show their positions on this issue, which were summarized in Table 3 below. The justification provided in contribution, e.g. [14] to add this clarification is that, for both synchronous and asynchronous cases, the actual overlapping transmission directions across CG cannot purely rely on semi-static UL/DL configurations, instead also accounting for the TA differences and drift. For Alt.1-2, UE should not be required to take into account these dynamic factors to determine the overlapping transmission(s).

Table 3: Summary of companies views on adding clarification sentence for Alt.1-2

| Category | No. companies | Companies |
| --- | --- | --- |
| Yes | 4 | Qualcomm, vivo, Apple, Samsung, MTK, |
| No |  |  |

Companies are encouraged to provide more inputs/comments on applicable DC scenario for Alt.1-2 and the need to add clarification sentence for it:

| Companies | Views |
| --- | --- |

[Offline consensus #1]:

- Alt.1-2 of semi-static power sharing can be configured for synchronous DC scenario only.

  - It is up to UE to determine whether the overlapping with UL transmission on the SCG is possible, if/when factors other than the TDD UL-DL configurations of the serving cells in the SCG (e.g., timing difference, drift) need to be taken into account.

### Issue 3: Slot-level vs. symbol-level overlapping determination

One discussion point that was raised in last meeting in [3] and remains open is the checking duration for Alt.1-2. Two alternatives were identified in the feature leader summary of RAN1 #98bis meeting [4]:

- Alt.1: On a symbol-level.

- Alt.2: On a per uplink transmission level.

Referring to Alt.1 above, “for the uplink transmission in MCG, the UE checks the semi-statically configured direction of the overlapped symbols of all serving cells of SCG”. In accordance to the text highlighted, UE applies PMCG' and PSCG' only if all overlapping symbols with the uplink transmission are DL on the other CG. In other words, the overlapping is determined on a per uplink transmission occasion level. This was further illustrated in FIG.1 and its description in [17]. As a consequence, the transmission power is constant across symbols within any transmission occasion.

Table 4 summarized companies views shown in RAN1 #98bis and 99 meetings on this aspect:

Table 4: Summary of companies views on checking duration for Alt.1-2 operation

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: On a symbol-level. |  |  |
| Alt.2: On a per uplink transmission level. | 8 | OPPO, ZTE, Apple, Qualcomm, vivo, Motorola Mobility, Lenovo, Samsung |

Alt.2 is clearly majority companies view and hence feature leader tends to add one conclusion to clarify this aspect:

One relevant discussion point was brought up in [7] and needs to at least reach a common understanding in RAN1 such that this feature can be correctly implemented into specification. FIG.2 is the interesting scenario copied from [7]:

Figure1. Overlapping determination for NR-NR DC.

As depicted in FIG.1 above, there are two overlapping transmissions in MCG and another in SCG. More specifically, UL transmission 1 in MCG is overlapping with D and X symbol in SCG, while UL transmission 2 is overlapping with the D symbols only. Following the agreed Alt.1-2 rule and assuming maximum transmission power is determined on a per transmission basis, different maximum transmission powers may be determined as follows:

- The maximum transmission power for transmission 1 is PMCG since it overlaps with X symbol;

- The maximum transmission power for transmission 2 is PTotal since it only overlaps with D symbol.

Different options can be considered to move forward on this issue. One option (Opt.1) was brought up in RAN1 #98 meeting, in which Alt.1-2 is clarified that the overlapping transmissions are treated as a whole to determine whether overlapping with downlink transmission of the other CG. With Opt.1, the maximum transmission power in the first two symbols of transmission 1 and 2 of MCG would be PMCG. On potential drawback is that it requires sort of ‘look-ahead’ operation when the starting symbol of overlapping transmission are not aligned but two transmissions are still partially overlapped.

On the other hand, Alt.1-2 works well even without any new agreement. It can be discussed regarding whether needs to capture a common understanding on Alt.1-2 that different maximum transmission powers may be determined for different symbols of a single UL transmission (e.g. first 2 symbols vs. last 2 symbols of UL transmission 1 in FIG.1) and potentially causes phase discontinuity problem.

Companies views/comments on the UE behaviour in case of FIG.1, if any, can be provided in the following Table:

| Companies | Views |
| --- | --- |
| Samsung | Consideration of transmissions per CG (not per cell) is assumed when determining direction non/overlapping with the other CG. |

[Offline consensus #2]: Make the following modification to the previous agreement:

- Alt.1: For the uplink transmission occasion in MCG, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of SCG, and vice versa. - If such overlapping for any ongoing uplink transmission(s) and UL transmission on the SCG is possible (i.e. collides with semi-static ‘UL’ and ‘flexible’ symbols on some CCs of SCG), UE limits its actual transmission power pMCGin MCG such that pMCG≤PMCG; - Otherwise (i.e. collides with only semi-static ‘DL’ symbols on all CCs of SCG), pMCG can be up to PMCG' and pSCG can be up to PSCG' . - Alt.1-1: PMCG' and PSCG' are configured by RRC signaling. - Alt.1-2: PMCG' and PSCG' are determined by RAN4 requirement.

## 2.2 On Details of Dynamic Power Sharing

In the RAN1 #98bis meeting, the following was agreed for dynamic power sharing operation[2]:

Agreements: For dynamic power sharing for NR-NR DC (if supported), - Virtual PHR for active CCs of another CG Agreements: - Support dynamic power sharing - If there is no overlapping transmission, maximum power on CG i is determined by RAN4 spec without considering P_CG_i. - If there is overlapping transmission, maximum power on CG i is limited to P_CG_i. - Note: “look-ahead” operation is included as a UE capability below - In case of power limitation, MCG is prioritized over SCG and reuse CA rule within each CG - Optional UE capability to indicate the support of dynamic power sharing operation - Separate optional UE capability to indicate the support of ’look-ahead’ operation on condition that UE indicates support of dynamic power sharing operation.

However, the controversial issue regarding the details of dynamic power sharing operation is still FFS, e.g. the value of ‘look-ahead’ behavior.

### Issue 4: With ‘Look-ahead’ behavior

Table 5 summarize the proposals of ‘look-ahead’ operations based on the company’s contributions. Companies are invited to provide more inputs/proposals on Table 3 to capture proposals and position.

Table 5: Summary of proposals on ‘look-ahead’ operation

| Category | Description | No. companies | Companies |
| --- | --- | --- | --- |
| Alt.1 | - To compute transmit power of each transmission, a UE considers other transmissions in another CG which are overlapping with it and jointly determines power of all overlapping transmissions through the following procedures. - Find the leading transmission among the overlapping transmissions. - For each dynamic transmission i overlapping with the leading transmission, check if its scheduling information is known T_offset,i ahead of the start of the leading transmission, where T_offset,i depends on numerology of transmission i to acknowledge processing time of its scheduling information. - Consider all overlapping dynamic transmissions satisfying the above condition as well as all overlapping semi-static transmissions together to jointly determine the power based on rel-15 priority rule. |  | Samsung |
| Alt.2 | - For NR-DC dynamic power sharing, to compute the transmit power for CG1 UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping CG2 UL transmission, and if such PDCCH(s) are detected, UE limits it’s transmit power in CG1 (pwr_CG1) such that pwr_CG1 <= P_limit_CG1; - UE also checks (based on RRC parameters) if any possible overlapping CG2 UL transmissions can be triggered by PDCCH(s) after T0-T_offset, and if such overlapping CG2 UL transmissions are possible, UE limits it’s transmit power in CG1 (pwr_CG1) such that pwr_CG1 <= P_limit_CG1; - Otherwise (i.e., if a, b above are not applicable), P_limit_CG1 is not applied for CG1 UL transmission. |  | Ericsson |
| Alt.3 | A two-step power allocation could be used in NR-NR DC - Step 1): Power allocation for all UL transmission which are known to both CGs. Then, a dynamic minimum guaranteed power is determined for each CG; - Step 2): Within each CG, power allocation follows NR Rel-15 CA rules. The dynamic minimum guaranteed power for both CGs are respected. |  | Intel |

In addition, the value proposed for Toffset by proponents of ‘look-ahead’ operation are listed in Table 5 below:

Table 6: Summary of values of Toffset proposed by companies

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: Toffset=max⁡{TMCG, proc,2, TSCG, proc,2 } | 1 | vivo |
| Alt.2: Toffset=max⁡{N2i}i=0k-1where N2i is the N2 value corresponding to CC i. | 1 | MediaTek |
| Alt.3: Toffset=max{Tproc.2i}i=0k-1⁡ where Tproc.2i is the Tproc,2 value corresponding to CC i. | 1 | Intel, |
| Alt.4: up to RAN4 decision | 1 | Nokia |

Generally speaking, Alt.1, Alt.2 and Alt.3 are similar in terms of reusing N2 or Tproc,2 for ‘look-ahead’ definition. In Rel-15, there are at least two PUSCH preparation procedure times defined in different specifications for different cases, i.e.  Tproc,2 in section 6.4 of TS 38.214 and Tproc,2muxin section 9.2.5 of TS 38.213 for UCI multiplexing, as following:

Tproc,2=maxN2+d2,12048+144∙Κ2-u∙Tc,d2,2                                               (1)

Tproc,2mux=maxN2+d2,1+12048+144∙Κ2-u∙Tc,d2,2                                    (2)

For both these two parameters, the value is not fixed and instead determined by different factors e.g. UE processing time capability N2, numerology u, d2,1, PUSCH multiplexing with DMRS etc. More specifically, different values of Tproc,2 or Tproc,2mux may be determined for different UL CCs in case of CA based on equation (1) and (2). Correspondingly, it is another discussion point regarding how to determine the exact offset value taking into account this aspect. Although 5 more companies [5][6][7][12][14] proposed to either reuse Rel-15 timelines for UCI multiplexing or Tproc,2, potential different values across CCs were not explicitly discussed in contributions. Companies are therefore invited to provide more details or preference on the alternatives listed Table 5 and Table 6, if any:

| Companies | Views |
| --- | --- |
| Samsung | For Table 5, we continue to support Alt1. Alt3 may also be OK after discussion for details. For Table 6, we think that Tproc,2 is already too loose a bound and taking the maximum over all CCs will be inefficient. A timeline per CC using Tproc,2 is preferred. |

[Offline proposal #6]: Continue discussion

For uplink transmission without DCI format, (e.g. CG-PUSCH, semiPersistentOnPUSCH and SP/P-SRS transmission), determination of cut-off time needs to be discussed. 2 companies discussed these aspects in their contributions [6][7] with a little bit different proposal as follows:

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: reusing the cut-off time Tproc,2' defined in section 7.7 in TS 38.213 with assuming d2,1=1 and d2,2=0 and with uDL corresponding to the SCS of the active BWP of the scheduling cell | 1 | Huawei |
| Alt.2: Tproc,2 without factoring in the numerology of PDCCH uDL is defined as the look-ahead offset | 1 | ZTE (only for CG-PUSCH) |

Companies are therefore invited to provide more details or preference on the alternatives listed Table 4 and Table 5, if any:

| Companies | Views |
| --- | --- |
| Samsung | UCI multiplexing in Rel-15 considers a comprehensive list of cases. The above are isolated cases from that list. We will need to discuss all combinations applicable as for UCI multiplexing. |

[Offline proposal #7]: Continue discussion

### Issue 5: Without ‘Look-ahead’ behavior

In addition, how to support dynamic power sharing without ‘look-ahead’ operation also needs to be addressed. Table 6 summarized the companies’ proposals:

Table 7: Summary of proposals on dynamic power sharing without ‘look-ahead’ operation

| Category | Description | No. companies | Companies |
| --- | --- | --- | --- |
| Alt.1 | - For dynamic power-sharing without look-ahead, the procedure should be following: - Step 1: Per MCG transmission occasion, the sum power of MCG across all the serving cells is calculated and is passed to SCG. If needed, the sum power of MCG is scaled-down, taking into account the limitation configured by PMCG. - Step 2: For a given SCG transmission occasion, if there is an overlapping transmission in MCG, the max power of the SCG is min{PSCG, Ptotal – MCG tx power}. Otherwise, it is PCMAX. - Step 3: Per CG, CA power determination is performed similar to Rel.15 NR. | 1 | Qualcomm |

Companies are invited to provide views on the proposal above, if any:

| Companies | Views |
| --- | --- |
| Samsung | Can be treated in a joint framework with look-ahead – can result from a ‘long’ processing timeline for determining overlapping. No need to consider separately. |

[Offline proposal #8]: Continue discussion

# 3 Others

### Issue 6: On support for FR1+FR2 CA in one or both CGs

In [15], it was proposed to clarify that clarify the cases where one or both CGs have CCs over FR1 and FR2 are supported in Rel.16 NR-DC. One example was provided in [15] as copied below:

Figure 2: Example of NR-DC with FR1+FR2 CA per CG

In WID description, there is no any restriction on CA combinations in a CG. CA in one or both CGs are possible. Actually, this clarification was raised online before and more importantly, has been assumed when discussing the power sharing solutions in past. To avoid any misunderstanding on this regard, feature leader proposes to include this proposal into RAN1 chairman note as a conclusion:

[Agreement] Conclusion:

- At least from UE power sharing perspective, the cases where one or both CGs have CC[s] over FR1 and FR2 are supported in Rel.16 NR-DC.

One more FFS for this scenario is how to configure power control operation for different FRs. Note that UL power-control is independent between CCs in FR1 and CCs in FR2. Therefore, it is not necessary to configure the same PC mode for CCs in FR1 and for CCs in FR2 from network operational perspective. It was proposed in [15] to allow separate power-control mode (e.g. Alt.1-2 for FR1 and Alt.2 for FR2) configuration per FR.

Companies are invited to provide views on this proposal to allow separate power-control mode configuration for FRs, if any:

| Companies | Views |
| --- | --- |
| Samsung | Rel-16 NR DC is considered for cells in same FR. Above case is just a parallelization of having both FR1 NR DC and FR2 NR DC. Whether or not to have separate PC mode configuration can be further discussed. |

The following was agreed on Tuesday morning online session

Agreements: - Support per FR configuration of parameter NR-DC-PC-mode for NR-DC

[Offline consensus #3]:

- Whether semi-static power sharing or dynamic power sharing is explicitly configured by RRC signalling.

  - parameter NR-DC-PC-mode is extended to include dynamic power sharing configuration.

In addition, whether allows the configurability of dynamic power sharing with and without ‘look-ahead’ if UE indicates the support of ’look-ahead’ operation.

[Offline proposal #10]:

- Alt.1: ‘look-ahead’ operation is enabled by RRC signalling configuration.

- Alt.2: If UE indicates the support of ’look-ahead’ operation, it is always used for power control if dynamic power sharing operation is configured to the UE for NR-DC.

### Issue 8: Power scaling vs. dropping for deprioritized Uplink Transmissions

For EN-DC, deprioritized uplink transmission is dropped or scaled down depending on whether the reduced power exceeds the value provided by Xscale. However, this behaviour is not defined for NE-DC and it is up to UE implementation to determine drop or scaling down. Discussion is needed for NR-NR DC on this aspect.

Companies views/proposals are summarized in Table 8 below:

Table 8: Summary of proposals on handling deprioritized Uplink Transmissions

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt.1: left for UE implementation to determine scaling down or dropping | 3 | CATT, MediaTek, Apple, Qualcomm |
| Alt. 2: Re-use EN-DC rule (i.e. based on Xscale) | 1 | Samsung, ZTE, Huawei, HiSilicon, |

[Offline proposal 11]:  Make decision following majority views.

# References

- Chairman notes of RAN1 #98 meeting

- Chairman notes of RAN1 #98bis meeting

- R1-1910107	Discussion on uplink power control for NR DC	ZTE

- R1-1911661	Feature lead summary #3 for uplink power control for NR-NR DC		Apple Inc.

- R1-1911851	UL power control for NR-NR DC	OPPO

- R1-1911879	UL power control for NR-NR dual connectivity	Huawei, HiSilicon

- R1-1911970	Discussion on uplink power control for NR DC	ZTE

- R1-1912054	Remaining issues on uplink power control for NR-NR DC	vivo

- R1-1912099	On UL Power Control for NN-DC	MediaTek Inc.

- R1-1912149	UL Power control for NR-DC	CATT

- R1-1912233	Uplink power control for NN-DC	Intel Corporation

- R1-1912495	Uplink Power Control for NR DC	Samsung

- R1-1912784	Power Control for NR-NR DC	Ericsson

- R1-1912832	Uplink power control for NR-NR Dual Connectivity	Apple Inc.

- R1-1912978	Remaining issues on uplink power control for NR-DC	Qualcomm Incorporated

- R1-1912998	Uplink PC for NR-NR Dual Connectivity	Nokia, Nokia Shanghai Bell

- R1-1911137	Uplink Power Control Solutions for NR-NR Dual Connectivity	Qualcomm Incorporated

# Appendix: Contributions used as basis for the summary

## R1-1911851 (OPPO)

  - Observation 1: For the semi-static power sharing for NR-NR DC

    - Alt.1-2 is applicable for synchronous NR-NR DC

    - Alt.2 is applicable for both synchronous and asynchronous NR-NR DC

  - Proposal 1: For the semi-static power sharing for NR-NR DC

    - The overlapping of MCG and SCG is determined on a per uplink transmission level

    - The transmission power is constant across symbols within any transmission occasion

  - Proposal 2: For the dynamic power sharing for NR-NR DC with “look-ahead” operation

    - When calculating the transmission power for a given UL transmission occasion starting at time T in one cell group, UE will consider overlapping UL transmission in the other cell group which is scheduled or triggered by PDCCH(s) received before T- Tproc,2

## R1-1911879 (Huawei)

  - Observation: For configured grant and periodic/semi-persistent sounding reference signal transmissions, the power calculation remains the same regardless of look-ahead mode.

  - Proposal 1: Look-ahead operation should be defined as:

      - To compute the transmission power for the DCI-scheduling uplink transmission starting at time T0 on a CG, a UE takes into account all DCIs received for the other CG until the time T0-Toffset.

      - Toffset should be specified as the value Tproc,2.

  - Proposal 2: For transmission power calculation of configured grant and periodic/semi-persistent sounding reference signal transmissions, the cut-off time defined in TS 38.213 can be reused.

## R1-1911970 (ZTE)

- Observation 1: Alt 1-2 is applicable to asynchronous NR-DC network if the UE supports determining the timing offset between MCG and SCG. UE capability sftd-MeasPSCell can be used as an implicit indication to indicate whether UE supports determining the timing offset between MCG and SCG or not.

- Observation 2: If the determination is based on a per-transmission level, two overlapping uplink transmissions may be associated with different maximum transmission power even in the same symbol.

- Proposal 1: Introduce a one-bit RRC IE to differentiate configuration of Alt 1-2 and Alt 2.

- Proposal 2: For Alt 1-2, the overlapping determination is based on a per-transmission level.

- Proposal 3: The cut-off time of look-ahead operation is determined as an offset before the first symbol of one uplink transmission.

  - For dynamic scheduled PUSCH, Tpro,2 is defined as the look-ahead offset;

  - For configured grant PUSCH, Tpro,2 without factoring in the numerology of PDCCH uDL is defined as the look-ahead offset;

  - For PUCCH, PRACH and SRS, further discuss how to determine the look-ahead offset.

## R1-1912054 (vivo)

- Proposal 1:

  - For semi-static power sharing, Alt.1-2 and Alt.2 can be applied for both synchronous and asynchronous NN-DC cases respectively.

- Proposal 2:

  - It is up to UE implementation to determine whether the overlapping between CGs is possible or not.

- Proposal 3:

  - Supporting of semi-static power sharing is a mandatory UE feature if the UE reports capable of NN-DC.

- Proposal 4:

  - For NN-DC dynamic power sharing with power ‘look-ahead’,

      - The start of the power ‘look-ahead’ window is defined as the earliest receiving time of a UL grant, which schedules an UL channel/RS with undetermined power due to possible overlapping between MCG and SCG.

      - The end of the power ‘look-ahead’ window is defined as the earliest transmission time of the scheduled UL channel/RS between MCG and SCG minus Toffset.

      - Toffset = max{TMCG,proc,2, TSCG,proc,2}.

- Proposal 5:

  - Dynamic power sharing can be applied for both synchronous and asynchronous NN-DC cases

## R1-1912099 (MediaTek)

- Proposal 1: UE to report separate capabilities of dynamic power sharing support for synchronous and asynchronous NN-DC.

- Proposal 2: Minimum reserved power is configured for both MCG and SCG.

- Proposal 3: Power scaling or dropping for deprioritized uplink transmissions for NN-DC in dynamic power sharing is left for UE implementation.

- Proposal 4: For CGs of different SCS in the same FR, let “Toffset = maxi{N2i in unit of us}, where N2i means N2 of the i-th carrier”, and apply the same Toffset for all carriers, for the look-ahead operation of dynamic power sharing.

## R1-1912149 (CATT)

- Proposal 1: Semi-static power sharing between MCG and SCG in NR-DC applies to both synchronous and asynchronous NR DC.

- Proposal 2: If the UE output power at some OFDM symbols exceeds the maximum power of the UE power class, it would be UE implementation choice of scaling the UL Tx power on one or more UL channels based on the power scaling rule. The look-ahead window is an UE implementation and should not be specified in the specification.

## R1-1912233 (Intel)

- Proposal 1: For semi-static power sharing, Alt 2 can be used in both synchronized DC or asynchronized DC. Alt 1-2 is only used in synchronized DC.

- Proposal 2: No explicit configuration of minimum guaranteed power of SCG.

- Proposal 3: For a NR UE not capable of look-ahead, if there are potential overlapped UL transmissions from two CGs, the transmission power of MCG is limited by PMCG, on the other hand, the transmission power of SCG may be reduced later if more transmission power is required by MCG.

- Proposal 4: For a NR UE capable of look-ahead, if there are potential overlapped UL transmissions from two CGs, the only limitation for the power allocation of MCG is given by PMCG, meanwhile UE could avoid too aggressive power allocation to SCG by look-ahead.

- Proposal 5: Toffset could equal to the maximum of Tproc,2 considering all the factors impacting Tproc,2.

- Proposal 6: A two-step power allocation could be used in NR-NR DC

  - Step 1): Power allocation for all UL transmission which are known to both CGs. Then, a dynamic minimum guaranteed power is determined for each CG;

  - Step 2): Within each CG, power allocation follows NR Rel-15 CA rules. The dynamic minimum guaranteed power for both CGs are respected.

## R1-1912495 (Samsung)

- Proposal 1: Re-use the Rel-15 timelines for UCI multiplexing for determining a transmission power of overlapping channels between CGs in NR-DC.

- Proposal 2: For determining a power at a transmission occasion on a CG, a UE can consider a total power for later scheduled overlapping transmissions on another CG subject to Rel-15 UE processing timelines.

- Proposal 3: RAN1 to provide a justification for the need to introduce NR-DC-PC-mode. Absence of a justification, NR-DC-PC-mode should be removed.

## R1-1912784 (Ericsson)

- Proposal 1

  - Semi-static power sharing Alt 1-2 is supported for sync DC case.

  - Semi-static power sharing Alt 2 is supported for both sync and async DC cases

- Proposal 2

  - For NR-DC dynamic power sharing, to compute the transmit power for CG1 UL transmission starting at time T0,

    - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping CG2 UL transmission, and if such PDCCH(s) are detected, UE limits it’s transmit power in CG1 (pwr_CG1) such that pwr_CG1 <= P_limit_CG1;

    - UE also checks (based on RRC parameters) if any possible overlapping CG2 UL transmissions can be triggered by PDCCH(s) after T0-T_offset, and if such overlapping CG2 UL transmissions are possible, UE limits it’s transmit power in CG1 (pwr_CG1) such that pwr_CG1 <= P_limit_CG1;

    - Otherwise (i.e., if a, b above are not applicable), P_limit_CG1 is not applied for CG1 UL transmission.

## R1-1912832 (Apple)

- Proposal 1:

  - Support to add the clarification sentence for Alt.1-2.

- Proposal 2:

  - Reuse Rel-15 timeline defined for UCI multiplexing on overlapped UL transmissions as staring point to define ‘look-ahead’ behavior for NR-NR DC power control in Rel-16.

- Proposal 3:

  - Semi-static power sharing schemes, including both Alt.2 and Alt.1-2, are supported for both synchronous and asynchronous DC scenarios.

- Proposal 4:

  - Dynamical power sharing is configurable for both synchronous and asynchronous DC scenarios subject to UE reports to support this capability.

## R1-1912978 (Qualcomm)

- Proposal 1: RAN1 to clarify the cases where one or both CGs have CCs over FR1 and FR2 are supported in Rel.16 NR-DC.

- Proposal 2: Enable configuration of power-control mode per FR.

- Proposal 3: For Alt.1-2, the check of semi-static direction of overlapping symbol(s) should not require taking into account TAs and drift. For symbols where TAs and/or drift may impact the direction of overlapping symbol(s), it is up to UE whether to really check the directions, or to limit UL power of the symbols of the CG up to PCG.

- Proposal 4: For dynamic power-sharing without look-ahead, the procedure should be following:

  - Step 1: Per MCG transmission occasion, the sum power of MCG across all the serving cells is calculated and is passed to SCG. If needed, the sum power of MCG is scaled-down, taking into account the limitation configured by PMCG.

  - Step 2: For a given SCG transmission occasion, if there is an overlapping transmission in MCG, the max power of the SCG is min{PSCG, Ptotal – MCG tx power}. Otherwise, it is PCMAX.

  - Step 3: Per CG, CA power determination is performed similar to Rel.15 NR.

## R1-1912998 (Nokia)

- Proposal 1: It is up to gNb’s configuration whether to configure alt 1-2 or alt 2 for semi-static power sharing. It is up to implementation whether/how to consideration the misalignment of slot boundaries in UL

- Proposal 2: When UE is configured with alt 1-2 in asynchronous DC, for power control at UL symbol/slots where partial overlapping is possible due to slot boundary misalignment, UE can perform either option 1 or option 2 below.

  - Option 1: UE considers the offline offset between slot boundaries to extract non-overlapped symbol/slot for UL transmission

  - Option 2: UE calculates transmission power with P’MCG/P’SCG, then performs power scaling, if needed

- Proposal 3: When look-ahead is configured, gNb assmes UE will detect DCI in the other cell received at least k slots earlier than UL transmission in each cell, where k is the default vale of PUSCH scheduling offset or SRS triggering offset.

  - Up to RAN4 discussion for the exact value of look-ahead window size