3GPP TSG RAN WG1 #99			R1-1913412 Reno, USA, November 18th – 22nd, 2019

Agenda item:	6.2.4

Source: 	Qualcomm Incorporated

Title: 	Proposals for terrestrial broadcast (v2)

Document for:	Discussion and Decision

# CAS

Proposal 3.1: PBCH repetition is specified as following,

- For downlink bandwidth configuration NRBDL≥25, PBCH is repeated in subframe #0 of SFN satisfying SFN mod 4 = 0 (already agreed)

- For downlink bandwidth configuration NRBDL<25, PBCH is repeated in subframe #0 of SFN satisfying SFN mod 8 = 4.

- For downlink bandwidth configuration NRBDL=6, PBCH is not repeated.

The copied PBCH symbols are multiplied with a θl,k at OFDM symbol l and subcarrier k, where the length-144 Gold sequence at each OFDM symbol l used to determine θl,k is initialized by cinit,l=213(NID+1)×l+24NID+l  , and θl,k is given by:

θl,k= 1,            cl2k=0, cl2k+1=0-1,            cl2k=0, cl2k+1=1j,           cl2k=1, cl2k+1=0-j,           cl2k=1, cl2k+1=1

Proposal 3.2: For the PDCCH enhancement in CAS:

- Alt1: Al16 is introduced, where the lowest CCE of AL16 is aligned with the lowest CCE of a legacy AL8 candidate.

- Alt2: Repetition of AL8 is introduced, where the second repetition is scrambled by a different pseudo-random sequence.

- Alt3: Repetition of AL8 is introduced, where the rate-matched output of the second AL8 candidate is obtained by starting within the circular buffer from the point where the first AL8 candidate ended.

# Interleaving

The following proposal is supported by: EBU, BBC, IRT, NHK, SJTU, ABS, Qualcomm, [Huawei?]

Proposal 4.1: For 2.5kHz numerology, introduce “HARQ-based time interleaving” as follows:

- Reuse the modulation and TBS index table in TS 36.213 and according to the signaled IMCS, obtain ITBS

- Reuse the transport block size table in TS 36.213 and according to the obtained ITBS, obtain the corresponding TBS

- Calculate the used TBS’ as TBS'=round{TBS ×n}, where the round operation maps TBS ×n to the closest TBS in the legacy transport block size tables for TBs mapped to 1 and 2 layers.

  - In case two TBSs are at the same distance of TBS ×n, the larger TBS is chosen

- For each of the n RVIDs, k0=2RsubblockTC+rvidxEmin, where k0 is the pointer to the circular buffer defined in TS 36.212, and Emin=NLQmG'/C.

- Across a set of m×n consecutive subframes not including MSI and/or MCCH, the TB mapped to subframe j∈0,1,…,m×n-1 is given by ITBj=j mod m. The values of m,n are configurable within PMCH-Config.

  - m∈8,16 (Number of parallel HARQ processes)

  - n∈{1,4,8,16} (Number of subframes/RVs)

    - NOTE: n=1 means interleaving is disabled

- The RV sequence rvidx is as follows:

  - For n=4, 0,2,3,1 in sequence

  - For n=8,{0,4,6,2,7,3,5,1 }

  - For n=16,{0,8,4,12,6,14,2,10,7,15,3,11,5,13,1,9}

- Interleaving is not applied for PMCH carrying MSI and/or MCCH.

For 2.5kHz and 0.370kHz numerology, introduce “frequency interleaving” as follows:

- Group the modulation symbols in one OFDM symbol y=y0,y1,…,yNscDL-1 into sub-blocks ot M tones each to obtain a vector z=z0,z1,…zNscDLM-1,  zp=<yMp+0,yMp+1,…yMp+M-1> and yleftover denotes the last elements of y not part of z.

  - M=9 for 2.5kHz  and

  - M=10 for 0.370kHz with T_d=2

  - M=11 for 0.370kHz with T_d=4

- Apply the CRBI interleaver in R1-1913290 (where the number of columns equals the number of codeblocks) over the vector z with the following modification:

  - Cyclically shift by sj the elements of each row j∈0,1,…,RsubblockTC-1, where sj is the decimal equivalent of the binary string 1+c8j2,…,1+c8j+72, and cn is a length 8RsubblockTC pseudorandom number, generated according to subclause 7.2 of TS 36.211 with cinit=NIDMBSFN.

- Map the output of the CRBI interleaver consecutively to the data tones frequency first.

- Map yleftover consecutively to the remaining tones.

Alt proposal (High level):

Adopt time and frequency interleaving as follows:

- For time interleaving, adopt the solution in R1-1912692

- For frequency interleaving, adopt the solution in R1-1912331 with an additional cyclic shift per row.

- FFS: Details, taking as starting point proposal 4.1