3GPP TSG RAN WG1 #99	R1-1913428

Reno, USA, November 18th – 22nd, 2019

Agenda item:	7.2.2.1.1

Source:	Samsung

Title:	Further Discussion on CORESET #0 Offset Configuration

Document for:	Discussion and Decision

# CORESET #0 Offsets for 0.805 MHz Intra-band Guard-band

The CORESET#0 offset values are calculated based on the RAN4 agreements [1] [2], assuming intra-band guard-band as 0.805 MHz. A summary is provided in Table 1-1 to Table 1-4 for 20 MHz, 40 MHz, 60 MHz, and 80 MHz, correspondingly. A detailed calculation of the offsets is provided by the accompany spreadsheet document to this file.

Observation 1-1: k_SSB only takes values from {0, 4, 8, 12, 16, 20}.

Observation 1-2: At least 50 RBs can be available within each LBT bandwidth.

Observation 1-3: Two offsets are sufficient for guaranteeing at least one CORESET #0 with 48 RBs can fit into the available RBs within each LBT bandwidth, and the two offsets can be selected as (0, 2) or (0, 3).

Table 1-1 CORESET #0 offsets for 20 MHz channels

| nominal channel center MHz | channel raster | Number of RB | min RB offset | max RB offset | k_SSB |
| --- | --- | --- | --- | --- | --- |
| 5160 | 5160.000 | 51 | 0 | 3 | 12 |
| 5180 | 5179.980 | 50 | 1 | 3 | 12 |
| 5200 | 5200.020 | 51 | 1 | 4 | 8 |
| 5220 | 5220.000 | 51 | 0 | 0 | 20 |
| 5240 | 5239.980 | 50 | 0 | 0 | 20 |
| 5260 | 5260.020 | 51 | 0 | 1 | 16 |
| 5280 | 5280.000 | 51 | 0 | 2 | 4 |
| 5300 | 5299.980 | 50 | 0 | 2 | 4 |
| 5320 | 5320.020 | 51 | 0 | 3 | 0 |
| 5340 | 5340.000 | 51 | 0 | 3 | 12 |
| 5480 | 5479.980 | 50 | 0 | 2 | 4 |
| 5500 | 5500.020 | 51 | 0 | 3 | 0 |
| 5520 | 5520.000 | 51 | 0 | 3 | 12 |
| 5540 | 5539.980 | 50 | 1 | 3 | 12 |
| 5560 | 5560.020 | 51 | 1 | 4 | 8 |
| 5580 | 5580.000 | 51 | 0 | 0 | 20 |
| 5600 | 5599.980 | 50 | 0 | 0 | 20 |
| 5620 | 5620.020 | 51 | 0 | 1 | 16 |
| 5640 | 5640.000 | 51 | 0 | 2 | 4 |
| 5660 | 5659.980 | 50 | 0 | 2 | 4 |
| 5680 | 5680.020 | 51 | 0 | 3 | 0 |
| 5700 | 5700.000 | 51 | 0 | 3 | 12 |
| 5720 | 5719.980 | 50 | 1 | 3 | 12 |
| 5745 | 5745.000 | 51 | 0 | 2 | 12 |
| 5765 | 5764.980 | 50 | 0 | 2 | 12 |
| 5785 | 5785.020 | 51 | 0 | 3 | 8 |
| 5805 | 5805.000 | 51 | 0 | 3 | 20 |
| 5825 | 5824.980 | 50 | 1 | 3 | 20 |
| 5845 | 5845.020 | 51 | 0 | 0 | 16 |
| 5865 | 5865.000 | 51 | 0 | 1 | 4 |
| 5885 | 5884.980 | 50 | 0 | 1 | 4 |
| 5905 | 5905.020 | 51 | 0 | 2 | 0 |

Table 1-2 CORESET #0 offsets for 40 MHz channels

| nominal carrier center MHz | channel raster MHz | Number of RB | k_SSB | sub-band 1 |  |  | sub-band 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset |
| 5170 | 5170.020 | 106 | 4 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5190 | 5190.000 | 106 | 16 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5230 | 5230.020 | 106 | 12 | 50 | 0 | 0 | 50 | 0 | 0 |
| 5270 | 5269.980 | 105 | 0 | 50 | 0 | 1 | 50 | 0 | 2 |
| 5310 | 5310.000 | 106 | 8 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5330 | 5329.980 | 105 | 8 | 50 | 0 | 2 | 50 | 1 | 3 |
| 5490 | 5490.000 | 106 | 8 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5510 | 5509.980 | 105 | 8 | 50 | 0 | 2 | 50 | 1 | 3 |
| 5550 | 5550.000 | 106 | 16 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5590 | 5590.020 | 106 | 12 | 50 | 0 | 0 | 50 | 0 | 0 |
| 5630 | 5629.980 | 105 | 0 | 50 | 0 | 1 | 50 | 0 | 2 |
| 5670 | 5670.000 | 106 | 8 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5710 | 5710.020 | 106 | 4 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5755 | 5755.020 | 106 | 4 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5795 | 5794.980 | 105 | 16 | 50 | 0 | 2 | 50 | 1 | 3 |
| 5815 | 5815.020 | 106 | 12 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5835 | 5835.000 | 106 | 0 | 50 | 2 | 4 | 50 | 0 | 0 |
| 5875 | 5875.020 | 106 | 20 | 50 | 0 | 0 | 50 | 0 | 0 |

Table 1-3 CORESET #0 offsets for 60 MHz channels

| nominal carrier center MHz | channel raster MHz | Number of RB | k_SSB | sub-band 1 |  |  | sub-band 2 |  |  | sub-band 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset |
| 5180 | 5179.980 | 161 | 0 | 50 | 1 | 3 | 50 | 1 | 3 | 50 | 2 | 4 |
| 5200 | 5200.020 | 162 | 20 | 50 | 1 | 3 | 50 | 1 | 3 | 51 | 0 | 0 |
| 5220 | 5220.000 | 162 | 8 | 51 | 1 | 4 | 50 | 0 | 0 | 50 | 0 | 0 |
| 5280 | 5280.000 | 162 | 16 | 51 | 0 | 1 | 50 | 0 | 1 | 50 | 0 | 1 |
| 5300 | 5299.980 | 161 | 16 | 50 | 0 | 1 | 50 | 0 | 1 | 50 | 0 | 2 |
| 5320 | 5320.020 | 162 | 12 | 50 | 0 | 2 | 50 | 0 | 2 | 51 | 0 | 3 |
| 5500 | 5500.020 | 162 | 12 | 50 | 0 | 2 | 50 | 0 | 2 | 51 | 0 | 3 |
| 5520 | 5520.000 | 162 | 0 | 51 | 0 | 3 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5540 | 5539.980 | 161 | 0 | 50 | 1 | 3 | 50 | 1 | 3 | 50 | 2 | 4 |
| 5580 | 5580.000 | 162 | 8 | 51 | 1 | 4 | 50 | 0 | 0 | 50 | 0 | 0 |
| 5600 | 5599.980 | 161 | 8 | 50 | 0 | 0 | 50 | 0 | 0 | 50 | 0 | 1 |
| 5620 | 5620.020 | 162 | 4 | 50 | 0 | 1 | 50 | 0 | 1 | 51 | 0 | 2 |
| 5680 | 5680.020 | 162 | 12 | 50 | 0 | 2 | 50 | 0 | 2 | 51 | 0 | 3 |
| 5700 | 5700.000 | 162 | 0 | 51 | 0 | 3 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5765 | 5764.980 | 161 | 0 | 50 | 0 | 2 | 50 | 0 | 2 | 50 | 1 | 3 |
| 5785 | 5785.020 | 162 | 20 | 50 | 0 | 2 | 50 | 0 | 2 | 51 | 0 | 3 |
| 5805 | 5805.000 | 162 | 8 | 51 | 0 | 3 | 50 | 1 | 3 | 50 | 1 | 3 |

Table 1-4 CORESET #0 offsets for 80 MHz channels

| nominal carrier center MHz | channel raster MHz | Number of RB | k_SSB | sub-band 1 |  |  | sub-band 2 |  |  | sub-band 3 |  |  | sub-band 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset |
| 5190 | 5190.000 | 217 | 4 | 50 | 1 | 3 | 50 | 1 | 3 | 50 | 2 | 4 | 50 | 0 | 0 |
| 5210 | 5209.980 | 216 | 4 | 50 | 1 | 3 | 50 | 2 | 4 | 50 | 0 | 0 | 50 | 0 | 1 |
| 5290 | 5290.020 | 217 | 8 | 50 | 0 | 1 | 50 | 0 | 1 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5530 | 5530.020 | 217 | 16 | 50 | 0 | 2 | 50 | 0 | 2 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5610 | 5610.000 | 217 | 12 | 50 | 0 | 0 | 50 | 0 | 0 | 50 | 0 | 1 | 50 | 0 | 1 |
| 5690 | 5689.980 | 216 | 20 | 50 | 0 | 1 | 50 | 0 | 2 | 50 | 0 | 2 | 50 | 1 | 3 |
| 5775 | 5775.000 | 217 | 4 | 50 | 0 | 2 | 50 | 0 | 2 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5795 | 5794.980 | 216 | 4 | 50 | 0 | 2 | 50 | 1 | 3 | 50 | 1 | 3 | 50 | 2 | 4 |
| 5855 | 5854.980 | 216 | 12 | 50 | 1 | 3 | 50 | 0 | 0 | 50 | 0 | 0 | 50 | 0 | 1 |

# CORESET #0 Offsets for Larger Intra-band Guard-band

There is an on-going discussion on increasing the size of intra-band guard-band from 0.805 MHz, and the maximum value for the intra-band guard-band proposed across all companies is 0.925 MHz.

Similar study has been performed for intra-band guard-band as 0.925 MHz, for all the supported channels in NR-U, and a summary is provided in Table 2-1 to Table 2-4 for 20 MHz, 40 MHz, 60 MHz, and 80 MHz, correspondingly. A detailed calculation of the offsets is provided by the accompany spreadsheet document to this file. Based on the results, we have the following observations, with the assumption that RAN4 needs to shift two synchronization raster entries (SS/PBCH block according to current GSCN is conflicting with guard-band). In this set of results, the following changes to synchronization raster entries are applied: GSCN 9037 changed to 9038, and GSCN 9471 changed to 9472.

Observation 2-1: k_SSB only takes values from {0, 4, 8, 12, 16, 20}.

Observation 2-2: At least 49 RBs can be available within each LBT bandwidth.

Observation 2-3: Two offsets are sufficient for guaranteeing at least one CORESET #0 with 48 RBs can fit into the available RBs within each LBT bandwidth, and the two offsets can be (0, 2).

Table 2-1 CORESET #0 offsets for 20 MHz channels

| nominal channel center MHz | channel raster | Number of RB | min RB offset | max RB offset | k_SSB |
| --- | --- | --- | --- | --- | --- |
| 5160 | 5160.000 | 51 | 0 | 3 | 12 |
| 5180 | 5179.980 | 50 | 1 | 3 | 12 |
| 5200 | 5200.020 | 51 | 1 | 4 | 8 |
| 5220 | 5220.000 | 51 | 1 | 4 | 20 |
| 5240 | 5239.980 | 50 | 0 | 0 | 20 |
| 5260 | 5260.020 | 51 | 0 | 1 | 16 |
| 5280 | 5280.000 | 51 | 0 | 2 | 4 |
| 5300 | 5299.980 | 50 | 0 | 2 | 4 |
| 5320 | 5320.020 | 51 | 0 | 3 | 0 |
| 5340 | 5340.000 | 51 | 0 | 3 | 12 |
| 5480 | 5479.980 | 50 | 0 | 2 | 4 |
| 5500 | 5500.020 | 51 | 0 | 3 | 0 |
| 5520 | 5520.000 | 51 | 0 | 3 | 12 |
| 5540 | 5539.980 | 50 | 1 | 3 | 12 |
| 5560 | 5560.020 | 51 | 1 | 4 | 8 |
| 5580 | 5580.000 | 51 | 0 | 0 | 20 |
| 5600 | 5599.980 | 50 | 0 | 0 | 20 |
| 5620 | 5620.020 | 51 | 0 | 1 | 16 |
| 5640 | 5640.000 | 51 | 0 | 2 | 4 |
| 5660 | 5659.980 | 50 | 0 | 2 | 4 |
| 5680 | 5680.020 | 51 | 0 | 3 | 0 |
| 5700 | 5700.000 | 51 | 0 | 3 | 12 |
| 5720 | 5719.980 | 50 | 1 | 3 | 12 |
| 5745 | 5745.000 | 51 | 0 | 2 | 12 |
| 5765 | 5764.980 | 50 | 0 | 2 | 12 |
| 5785 | 5785.020 | 51 | 0 | 3 | 8 |
| 5805 | 5805.000 | 51 | 0 | 3 | 20 |
| 5825 | 5824.980 | 50 | 1 | 3 | 20 |
| 5845 | 5845.020 | 51 | 1 | 4 | 16 |
| 5865 | 5865.000 | 51 | 0 | 1 | 4 |
| 5885 | 5884.980 | 50 | 0 | 1 | 4 |
| 5905 | 5905.020 | 51 | 0 | 2 | 0 |

Table 2-2 CORESET #0 offsets for 40 MHz channels

| nominal carrier center MHz | channel raster MHz | Number of RB | k_SSB | sub-band 1 |  |  | sub-band 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset |
| 5170 | 5170.020 | 106 | 4 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5190 | 5190.000 | 106 | 16 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5230 | 5230.020 | 106 | 12 | 50 | 2 | 4 | 50 | 0 | 0 |
| 5270 | 5269.980 | 105 | 0 | 50 | 0 | 1 | 49 | 0 | 1 |
| 5310 | 5310.000 | 106 | 8 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5330 | 5329.980 | 105 | 8 | 50 | 0 | 2 | 49 | 1 | 2 |
| 5490 | 5490.000 | 106 | 8 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5510 | 5509.980 | 105 | 8 | 50 | 0 | 2 | 49 | 1 | 2 |
| 5550 | 5550.000 | 106 | 16 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5590 | 5590.020 | 106 | 12 | 50 | 0 | 0 | 50 | 0 | 0 |
| 5630 | 5629.980 | 105 | 0 | 50 | 0 | 1 | 49 | 0 | 1 |
| 5670 | 5670.000 | 106 | 8 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5710 | 5710.020 | 106 | 4 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5755 | 5755.020 | 106 | 4 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5795 | 5794.980 | 105 | 16 | 50 | 0 | 2 | 49 | 1 | 2 |
| 5815 | 5815.020 | 106 | 12 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5835 | 5835.000 | 106 | 0 | 50 | 2 | 4 | 50 | 2 | 4 |
| 5875 | 5875.020 | 106 | 20 | 50 | 0 | 0 | 50 | 0 | 0 |

Table 2-3 CORESET #0 offsets for 60 MHz channels

| nominal carrier center MHz | channel raster MHz | Number of RB | k_SSB | sub-band 1 |  |  | sub-band 2 |  |  | sub-band 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset |
| 5180 | 5179.980 | 161 | 0 | 50 | 1 | 3 | 49 | 2 | 3 | 50 | 2 | 4 |
| 5200 | 5200.020 | 162 | 20 | 50 | 1 | 3 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5220 | 5220.000 | 162 | 8 | 50 | 2 | 4 | 50 | 2 | 4 | 50 | 0 | 0 |
| 5280 | 5280.000 | 162 | 16 | 50 | 0 | 1 | 50 | 0 | 1 | 50 | 0 | 1 |
| 5300 | 5299.980 | 161 | 16 | 50 | 0 | 1 | 49 | 0 | 1 | 50 | 0 | 2 |
| 5320 | 5320.020 | 162 | 12 | 50 | 0 | 2 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5500 | 5500.020 | 162 | 12 | 50 | 0 | 2 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5520 | 5520.000 | 162 | 0 | 50 | 1 | 3 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5540 | 5539.980 | 161 | 0 | 50 | 1 | 3 | 49 | 2 | 3 | 50 | 2 | 4 |
| 5580 | 5580.000 | 162 | 8 | 50 | 2 | 4 | 50 | 0 | 0 | 50 | 0 | 0 |
| 5600 | 5599.980 | 161 | 8 | 50 | 0 | 0 | 49 | 0 | 0 | 50 | 0 | 1 |
| 5620 | 5620.020 | 162 | 4 | 50 | 0 | 1 | 50 | 0 | 1 | 50 | 0 | 1 |
| 5680 | 5680.020 | 162 | 12 | 50 | 0 | 2 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5700 | 5700.000 | 162 | 0 | 50 | 1 | 3 | 50 | 1 | 3 | 50 | 1 | 3 |
| 5765 | 5764.980 | 161 | 0 | 50 | 0 | 2 | 49 | 1 | 2 | 50 | 1 | 3 |
| 5785 | 5785.020 | 162 | 20 | 50 | 0 | 2 | 50 | 0 | 2 | 50 | 0 | 2 |
| 5805 | 5805.000 | 162 | 8 | 50 | 1 | 3 | 50 | 1 | 3 | 50 | 1 | 3 |

Table 2-4 CORESET #0 offsets for 80 MHz channels

| nominal carrier center MHz | channel raster MHz | Number of RB | k_SSB | sub-band 1 |  |  | sub-band 2 |  |  | sub-band 3 |  |  | sub-band 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset | # RB | min RB offset | max RB offset |
| 5190 | 5190.000 | 217 | 4 | 50 | 1 | 3 | 49 | 2 | 3 | 49 | 2 | 3 | 50 | 2 | 4 |
| 5210 | 5209.980 | 216 | 4 | 49 | 2 | 3 | 49 | 2 | 3 | 50 | 2 | 4 | 49 | 0 | 0 |
| 5290 | 5290.020 | 217 | 8 | 50 | 0 | 1 | 49 | 0 | 1 | 49 | 0 | 1 | 50 | 0 | 2 |
| 5530 | 5530.020 | 217 | 16 | 50 | 0 | 2 | 49 | 1 | 2 | 49 | 1 | 2 | 50 | 1 | 3 |
| 5610 | 5610.000 | 217 | 12 | 50 | 0 | 0 | 49 | 0 | 0 | 49 | 0 | 0 | 50 | 0 | 1 |
| 5690 | 5689.980 | 216 | 20 | 49 | 0 | 1 | 49 | 0 | 1 | 50 | 0 | 2 | 49 | 1 | 2 |
| 5775 | 5775.000 | 217 | 4 | 50 | 0 | 2 | 49 | 1 | 2 | 49 | 1 | 2 | 50 | 1 | 3 |
| 5795 | 5794.980 | 216 | 4 | 49 | 1 | 2 | 49 | 1 | 2 | 50 | 1 | 3 | 49 | 2 | 3 |
| 5855 | 5854.980 | 216 | 12 | 49 | 2 | 3 | 49 | 2 | 3 | 50 | 0 | 0 | 49 | 0 | 0 |

# Future Compatibility

Since GSCN is defined as 4 RBs in term of 30 kHz, theoretically, 4 offsets should be sufficient by performing a proper synchronization rater design and channel raster design.

Observation 3: Four CORESET#0 offsets are sufficient for any band.

# Indication of Q

Based on the above discussion, the validation of offline proposals is clear.

Based on Observation 1-1 and 1-2, k_SSB is always a multiple of 4, for cell-defining SS/PBCH block, hence, using 2 LSB of ssb-SubcarrierOffset can be reused for other purpose.

Based on Observation 1-3 and 2-3, two CORESET #0 offsets are sufficient, and offsets as (0, 2) is more robust to RAN4’s decision on intra-band guard-band value.

Based on Observation 3, 4 CORESET #0 offsets are sufficient for future compatibility.

Offline proposal for Q indication in MIB:

- If supporting more than 4 CORESET #0 offsets, 2 LSBs of ssb-SubcarrierOffset can be used;

- If supporting no more than 2 CORESET #0 offsets, 2 bits of ControlResourceSetZero can be used;

- Else, 1 bit of ControlResourceSetZero combined with 1 bit of subCarrerSpacingCommon can be used.

Based on the above analysis, either offline proposal can work, and hence, we have the following proposal.

Proposal: Support CORESET #0 offsets as (0, 2) for 30 kHz SCS, and further down-select one from the following take into future compatibility into account:

- Alt 1: Use 2 bits of ControlResourceSetZero to indicate Q;

- Alt 2: Reserve 1 bit in ControlResourceSetZero, and use 1 bit of ControlResourceSetZero combined with 1 bit of subCarrerSpacingCommon to indicate Q;

- Alt 3: Reserve 2 bits in ControlResourceSetZero, and use 2 LSBs of ssb-SubcarrierOffset to indicate Q.

# Conclusion

Observations and proposal are summarized as follow.

Observation 1-1: k_SSB only takes values from {0, 4, 8, 12, 16, 20}.

Observation 1-2: At least 50 RBs can be available within each LBT bandwidth.

Observation 1-3: Two offsets are sufficient for guaranteeing at least one CORESET #0 with 48 RBs can fit into the available RBs within each LBT bandwidth, and the two offsets can be selected as (0, 2) or (0, 3).

Observation 2-1: k_SSB only takes values from {0, 4, 8, 12, 16, 20}.

Observation 2-2: At least 49 RBs can be available within each LBT bandwidth.

Observation 2-3: Two offsets are sufficient for guaranteeing at least one CORESET #0 with 48 RBs can fit into the available RBs within each LBT bandwidth, and the two offsets can be (0, 2).

Observation 3: Four CORESET#0 offsets are sufficient for any band.

Proposal: Support CORESET #0 offsets as (0, 2) for 30 kHz SCS, and further down-select one from the following take into future compatibility into account:

- Alt 1: Use 2 bits of ControlResourceSetZero to indicate Q;

- Alt 2: Reserve 1 bit in ControlResourceSetZero, and use 1 bit of ControlResourceSetZero combined with 1 bit of subCarrerSpacingCommon to indicate Q;

- Alt 3: Reserve 2 bits in ControlResourceSetZero, and use 2 LSBs of ssb-SubcarrierOffset to indicate Q.

# References

[1] R4-1912870, WF on the NR-U channel raster on 5GHz band, Ericsson.

[2] R4-1912982, WF for sync raster for NR-U, Qualcomm.