3GPP TSG-RAN WG1 Meeting #99 	R1-1913434

# Reno, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0498 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MWUS sequence generation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Sequans Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4 |  |  |  |  |  | Date: |  |  | 2019-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Rel-15 wake-up signal (WUS) in LTE-MTC is designed as a two time repetition of the NB-IoT WUS. This was also what was originally specified in Rel-15. Unfortunately, between versions 15.2.0 and 15.3.0 a negative sign has been lost in the MWUS design such that it no longer is similar to the NB-IoT WUS. Additionally, the signal list in 6.1.2 is incomplete since it is missing the MWUS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reinstall the negative sign in order to return MWUS to its original formulation and to regain its relation to NB-IoT. Additionally, the signal list in 6.1.2 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unfortunate that two different versions of MWUS exist but there is nothing to do about the past. Companies have developed MWUS in many cases based on agreements in the first versions of Rel-15. Furthermore, co-design between LTE-MTC and NB-IoT may be expected in which case the present formulation will result in erroneous sequence generation for MWUS. For those reasons, it is proposed that MWUS is changed back to its original formulation, in line with the NB-IoT expression. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2 and 6.11B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis This CR changes the LTE-MTC WUS base sequence design back to its original formulation. Due to the change in signal design, legacy WUS UEs not implementing this change will lose their WUS capability. However, not making the change yields the same result for UEs with Rel-15.0 to Rel-15.2. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ************************* Begin text proposal *************************

### 6.1.2	Physical signals

A downlink physical signal corresponds to a set of resource elements used by the physical layer but does not carry information originating from higher layers. The following downlink physical signals are defined:

-	Reference signal

-	Synchronization signal

-	Discovery signal

-	MTC wake-up signal, MWUS

## *********************** Unchanged parts omitted ***********************

## 6.11B	MTC wake-up signal (MWUS)

### 6.11B.1	Sequence generation

The MWUS sequence  in subframe x=0, 1, …, M-1 is defined by

wm=θnf,nsm'∙e-jπunn+1131

m=0, 1,…, 131

m'=m+132x

n=m mod 132

θnf,nsm'=1, if cnf,ns2m'=0 and  cnf,ns2m'+1=0-1, if cnf,ns2m'=0 and  cnf,ns2m'+1=1j, if cnf,ns2m'=1 and  cnf,ns2m'+1=0-j, if cnf,ns2m'=1 and  cnf,ns2m'+1=1

u=NIDcellmod 126+3

where  is the actual duration of MWUS as defined in [4].

The scrambling sequence cnf,nsi, i=0, 1, …, 2∙132M-1 is given by clause 7.2, and shall be initialized at the start of the MWUS with

cinit_WUS=(NIDcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDcell

where nf_start_PO is the first frame of the first PO to which the MWUS is associated, and ns_start_PO is the first slot of the first PO to which the MWUS is associated.

************************* End text proposal *************************