3GPP TSG RAN WG1 #99			R1-1913439 Reno, USA, November 18th – 22nd, 2019

Agenda item:	6.2.4

Source: 	EBU, BBC, IRT, NHK, ABS, DISH, Eutelsat, Shanghai Jiao Tong University, Fraunhofer HHI, Fraunhofer IIS, Enensys, Qualcomm, Rohde&Schwarz, Nokia, Nokia Shanghai Bell, ATEME, Nomor Research, Sony

Title: 	On time and frequency interleaving for terrestrial broadcast

Document for:	Discussion and Decision

# Background

During the work item on 5G terrestrial broadcast, it has been discussed whether time and/or frequency interleaving should be specified for the new numerologies. The following agreement was reached in RAN1#98:

Agreement:

Companies are encouraged to evaluate the performance of rooftop reception with tone interleaving.

In this contribution, we summarize our views on the essentiality of time-frequency interleaving for broadcast systems including both rooftop and car-mounted scenarios, and present a proposal for support of this feature in Rel-16.

# Benefits of time-frequency interleaving

A survey of the presence of time-interleaving in current broadcasting systems is presented in [1]. All modern broadcast standards (e.g. DVB-T2, DTMB, ISDB-T, DAB/DAB+) have time-interleaving as an essential feature that provides protection against rapid fading and impulsive noise.

Observation 1: Time interleaving is present in all modern broadcast standards as an essential feature to combat rapid fading and impulsive noise, as summarized in [1].

The benefits of frequency and time interleaving have been verified under 3GPP system in several contributions:

- Time interleaving: [2], [3], [4], [6]

- Frequency interleaving: [3], [5], [6]

As one example on the benefits on interleaving, the results on [4] on time-frequency interleaving are reproduced below, where a ~3dB gain is observed at 10-3 BLER and a ~2dB gain is observed at 10-2 BLER. This allows to achieve a gain in spectral efficiency of ~30% with respect to the no-interleaving case.

Figure 1 Evaluation of HARQ-based time interleaving (from [4])

Observation 2: There is significant gain from time and frequency interleaving, based on evaluations provided in [2-6]

# Proposed solution for Rel-16

In view of the input from [1-6], and the clear benefits from time-frequency interleaving shown in [2-6], we make the following proposal:

Proposal:

For 2.5kHz numerology, introduce “HARQ-based time interleaving” as follows:

- Reuse the modulation and TBS index table in TS 36.213 and according to the signaled IMCS, obtain ITBS

- Reuse the transport block size table in TS 36.213 and according to the obtained ITBS, obtain the corresponding TBS

- Calculate the used TBS’ as TBS'=round{TBS ×n}, where the round operation maps TBS ×n to the closest TBS in the legacy transport block size tables for TBs mapped to 1 and 2 layers.

  - In case two TBSs are at the same distance of TBS ×n, the larger TBS is chosen

- For each of the n RVIDs, k0=2RsubblockTC+rvidxEmin, where k0 is the pointer to the circular buffer defined in TS 36.212, and Emin=NLQmG'/C.

- Across a set of m×n consecutive subframes not including MSI and/or MCCH, the TB mapped to subframe j∈0,1,…,m×n-1 is given by ITBj=j mod m. The values of m,n are configurable within PMCH-Config.

  - m∈8,16 (Number of parallel HARQ processes)

  - n∈{1,4,8,16} (Number of subframes/RVs)

    - NOTE: n=1 means interleaving is disabled

- The RV sequence rvidx is as follows:

  - For n=4, 0,2,3,1 in sequence

  - For n=8,{0,4,6,2,7,3,5,1 }

  - For n=16,{0,8,4,12,6,14,2,10,7,15,3,11,5,13,1,9}

- Interleaving is not applied for PMCH carrying MSI and/or MCCH.

For 2.5kHz and 0.370kHz numerology, introduce “frequency interleaving” as follows:

- Group the modulation symbols in one OFDM symbol y=y0,y1,…,yNscDL-1 into sub-blocks ot M tones each to obtain a vector z=z0,z1,…zNscDLM-1,  zp=<yMp+0,yMp+1,…yMp+M-1> and yleftover denotes the last elements of y not part of z.

  - M=9 for 2.5kHz  and

  - M=10 for 0.370kHz with T_d=2

  - M=11 for 0.370kHz with T_d=4

- Apply the CRBI interleaver in R1-1913290 (where the number of columns equals the number of codeblocks) over the vector z [with the following modification:

  - Cyclically shift by sj the elements of each row j∈0,1,…,RsubblockTC-1, where sj is the decimal equivalent of the binary string 1+c8j2,…,1+c8j+72, and cn is a length 8RsubblockTC pseudorandom number, generated according to subclause 7.2 of TS 36.211 with cinit=NIDMBSFN.]

- Map the output of the CRBI interleaver consecutively to the data tones frequency first.

- Map yleftover consecutively to the remaining tones.

# References

[1] R1-1913095 Information on Time Interleaving in State-of-the-Art Terrestrial Broadcast Systems, EBU/BBC/IRT/ABS/NHK

[2] R1-1912692 Numerology with 100us CP, Qualcomm Incorporated

[3] R1-1912331 CRBI-based interleaver for 5G terrestrial broadcasting, ABS,SARI-CAS,Shanghai Jiao Tong University

[4] R1-1913343 On the performance evaluation of HARQ-based time-interleaving for LTE-based 5G terrestrial broadcast, EBU, BBC, IRT

[5] R1-1912690 Support of longer numerologies for rooftop reception, Qualcomm Incorporated

[6] R1-1910979 Time and Frequency Interleaver for LTE-based 5G Terrestrial Broadcast, EBU, BBC, IRT