3GPP TSG-RAN WG1 Meeting #99	R1-1913448

Reno, USA, 18th – 22nd November 2019

Agenda Item:	6.2.1.3

Source:	Ericsson

Title:	Feature lead summary #3 for Scheduling of multiple DL/UL transport blocks for LTE-MTC

Document for:	Discussion and Decision

# 1	Introduction

In the Rel-16 work item on “Additional MTC enhancements for LTE” [1], one of the objectives is to specify support for scheduling of multiple DL/UL transport blocks.

The objective is to specify the following set of improvements for machine-type communications for BL/CE UEs. […] Scheduling enhancement: - Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast [RAN1, RAN2] - Enhancement of SPS can be discussed.

RAN1 and RAN2 agreements made in earlier meetings for this topic are summarized in [2] and [3]. The most recent L1 parameter list and UE feature list can be found in [4] and [5]. This document provides a summary and recommendations based on the contributions in [6] – [16] and the revised contributions in [17] – [19]. This is an updated version of the earlier feature lead summaries provided in [20] and [21].

# 2	Interleaving

The following proposals concern interleaving for unicast multi-TB scheduling.

| [6] Ericsson | Proposal 9: For the unicast multi-TB feature, do not further consider RRC parameters or DCI fields related to the interleaving beyond the RRC parameters for enabling the feature in DL and UL, respectively. |
| --- | --- |
| [7] Huawei, HiSilicon | Proposal 1: Interleaving granularity should be introduced and its value is integral multiple of Ych. Proposal 2: When interleaving is enabled for multiple TBs scheduling, the PUSCH/PDSCH starts at the subframe n which satisfies that n mode Ych = 0. Proposal 3: When interleaving is enabled for multiple TBs scheduling, the frequency hopping pattern is determined based on the following equation: Proposal 4: For interleaving, the symbol counter is reset at the start of the first TB transmission and incremented for each symbol during the whole transmission of the two TBs. l=0,1,…,NNrepPUSCHMRUMslotsULMsymbUL-1, where N is the number of scheduled TBs. |
| [8] Nokia, Nokia Shanghai Bell | Proposal 5: For unicast interleaved transmission, the interleaving is done every N subframes. |
| [11] Beijing Xiaomi Software Tech | Proposal 2: The number of TBs involved in one cycle of interleaving transmission can be configured Proposal 3: When frequency hopping is NOT enabled, the granularity for the time interleaving is configured via higher layer signalling Proposal 4: When frequency hopping is enabled - The granularity for the time interleaving equals to the frequency hopping interval - The narrow bands used for the whole multi-TB transmission is switched every (M_tb*F_ch), where M_tb is the number of TBs involved in the interleaving transmission and F_ch is the frequency hopping interval configured in SIB |
| [12] Sierra Wireless | Proposal 12: For full PRB transmissions in CE Mode A when multiple TBs with repetitions and interleaving are scheduled by one DCI, the TB changes every SF - If frequency hopping (FH) is used and #TB scheduled=8 and # of FH narrowband=2 then the interleave order follows this pattern [1 2 3 4, 5 6 7 8, 5 6 7 8, 1 2 3 4] - else, the interleaving order is sequential Proposal 13: For full PRB transmissions in CE Mode B, when multiple TBs with repetitions and interleaving are scheduled by one DCI - If frequency hopping (FH) is used and mod(# of TBs, # of FH narrowbands)=0 then the TB changes when mod(SF,8)=0 - else, the TB changes when mod(SF,4)=0 Proposal 14: For sub-PRB transmissions, when multiple TBs with repetitions and interleaving are scheduled by one DCI – when to switch TBs should follow similar NB-IOT agreements |
| [14] ZTE | Proposal 3: - For CE mode A, - If hopping feature is configured and enabled by DCI, interleaving granularity is 4*Ych - If hopping feature is configured and disabled by DCI, interleaving granularity is 4 - If hopping feature is not configured, interleaving granularity is 4 - For CE mode B - If hopping feature is configured, interleaving granularity is 16*Ych - If hopping feature is not configured, interleaving granularity is 16 Proposal 4: When the repetition is less than the granularity, interleaving is not supported |
| [15] Qualcomm Incorporated | Proposal 8: For subframe index i∈{0,1,2,…} the TB index mapping ITB(i), which implicitly involves the design of the time-domain interleaver, should be designed such that all TBs have an equitable distribution of repetitions across all configured narrowbands Proposal 9: With frequency hopping enabled, for subframe index i∈{0,1,2,…} the TB index mapping ITB(i) should depend on the actual number of TBs scheduled, and the relationship may be determined by ITBi=iNIL+iLCMNsched×NIL, NNBch,DL×NNB,hopch,DL×1 mod GCFNsched, (NNBch,DL×NNB,hopch,DL)NIL×NNB,hopch,DLNILmod Nsched, where the parameters are as defined in this contribution. Proposal 10: The interleaving granularity is 1 subframe for CE Mode A and 4 subframes for CE Mode B with and without frequency hopping. Table 1: TB Interleaving pattern with Frequency Hopping for CE Mode A with Interleaving Granularity = 1NNB,hopch,DL=2). Green entries can use sequential interleaving. NNB,hopch,DL=2 (Number of narrowbands); Nsched (Number of TBs scheduled). 1; 2; 3; 4; 5; 6; 7; 8. NNBch=1 (Hopping Interval); {1,2,2,1}; {1,2,3,4, 2,3,4,1}; {1,2,3,4,5,6 ,2,3,4,5,6,1}; {1,2,3,4,5,6,7,8, 2,3,4,5,6,7,8,1}. NNBch=2 (Hopping Interval); {1,2,3,4, 3,4,1,2}; {1,2,3,4,5,6,7,8, 3,4,5,6,7,8,1,2}. NNBch=4 (Hopping Interval); {1,2,3,4,5,6,7,8, 5,6,7,8,1,2,3,4}. NNBch=8 (Hopping Interval). Table 2: TB Interleaving pattern with Frequency Hopping for CE Mode A with Interleaving Granularity = 1 (NNB,hopch,DL=4). Green entries can use sequential interleaving. NNB,hopch,DL=4 (Number of narrowbands); Nsched (Number of TBs scheduled). 1; 2; 3; 4; 5; 6; 7; 8. NNBch=1 (Hopping Interval); {1,2,2,1 2,1,1,2}; {1,2,3,4, 2,3,4,1, 3,4,1,2, 4,1,2,3}; {1,2,3,4,5,6, 1,2,3,4,5,6, 2,3,4,5,6,1, 2,3,4,5,6,1}; {1,2,3,4,5,6,7,8, 2,3,4,5,6,7,8,1, 3,4,5,6,7,8,1,2, 4,5,6,7,8,1,2,3}. NNBch=2 (Hopping Interval); {1,2,3,4, 1,2,3,4, 3,4,1,2, 3,4,1,2}; {1,2,3,4,5,6,7,8, 3,4,5,6,7,8,1,2, 5,6,7,8,1,2,3,4, 7,8,1,2,3,4,5,6}. NNBch=4 (Hopping Interval); {1,2,3,4,5,6,7,8, 1,2,3,4,5,6,7,8, 5,6,7,8,1,2,3,4 5,6,7,8,1,2,3,4}. NNBch=8 (Hopping Interval). Table 3: TB Interleaving pattern with Frequency Hopping for CE Mode B with Interleaving Granularity = 4, NNB,hopch,DL=2). Green entries can use sequential interleaving. NNB,hopch,DL=2 (Number of narrowbands); Nsched. 1; 2; 3; 4. NNBch=2 (Hopping Interval). NNBch=4 (Hopping Interval); {1,1,1,1,2,2,2,2, 2,2,2,2,1,1,1,1}; {1,1,1,1,2,2,2,2, 3,3,3,3,4,4,4,4, 2,2,2,2,3,3,3,3 4,4,4,4,1,1,1,1}. NNBch=8 (Hopping Interval); {1,1,1,1,2,2,2,2, 3,3,3,3,4,4,4,4, 3,3,3,3,4,4,4,4, 1,1,1,1,2,2,2,2}. NNBch=16. |

Based on the proposals listed above, the following can be considered.

- For unicast interleaved transmission, for the PUSCH sub-PRB 2-subcarrier case, the symbol counter  is reset at the start of the first TB transmission and incremented for each symbol during the whole transmission of the TBs

- l=0,1,…, Y NrepPUSCH MRUMslotsULMsymbUL-1

- where Y is the number of scheduled TBs.

- For unicast interleaved transmission without frequency hopping, the interleaving granularity is 1 repetition (RU) of a TB in CE mode A and 4 repetitions (RUs) of a TB in CE mode B in FDD [and TDD].

- For unicast interleaved transmission with frequency hopping, discuss and decide among the proposals for interleaving granularity.

# 3	DCI design aspects for CE mode B

The following proposals concern indication of HARQ ID, NDI and HARQ processes in CE mode B.

| [6] Ericsson | Proposal 15: For unicast in CE mode B, the same DCI design option is adopted for DL and UL. |
| --- | --- |
| [7] Huawei, HiSilicon | Proposal 9: For scheduling multiple TBs in CE modeB, using Table 3 to indicate the number of scheduled TBs, the HARQ process number and the NDI of each scheduled TB and the index of the MCS. |
| [8] Nokia, Nokia Shanghai Bell | Proposal 3: For DCI contents for scheduling multiple DL/UL transport blocks for CE Mode B, select Option 3 or Option 6 in R1-1911381. |
| [9] Intel Corporation | Proposal 5: For CE Mode B: - Jointly encode HARQ process ID, NDI, the number of scheduled TBs. - HARQ process ID, NDI, bit fields from DCI format 6-0/6-1 B are repurposed. |
| [12] Sierra Wireless | Proposal 3: For CE mode B MTBG DCI design, support coding HARQ ID, # of HARQs, and NDI together Proposal 4: For CE mode B MTBG DCI design, support scheduling of only 1,2, and 4 HARQ processes (i.e. do not support 3) |
| [13] LG Electronics | Proposal 6: For DCI design for CE mode B, 6 bits are used for HARQ process ID, NDI, and number of scheduled TBs - If the first bit is 1, - 2 TBs are scheduled - 3 bits are used for allocating HARQ process ID and 2 bits are used for NDI - Else if second bit is 1, - 4 TBs are scheduled - 4 bits are used for NDI - Else if third bit is 1, - 1 TB is scheduled - 2 bits are used for allocating HARQ process ID and 1 bit are used for NDI |
| [15] Qualcomm Incorporated | Proposal 1: For the DCI design for scheduling multiple transport blocks in CE Mode B: - Retain same scheduling flexibility as legacy DCI when the number of TBs scheduled ∈ {1,2} - Retain full scheduling flexibility across all combinations of (up to 4) HARQ processes for all values of number of TBs scheduled - Jointly encode HARQ processes scheduled and NDIs for all values of number of TBs scheduled - For number of scheduled TBs ∈ {3,4}, restrict the MCSs allowed when number of repetitions is large Proposal 2: Adopt the DCI design for scheduling multiple transport blocks in CE Mode B outlined in Section 2. |
| [18] ZTE | Proposal 11: If DCI size is increased by no more than 3 bits - MCS are reduced to 8 options with 3 bits - 1 bit for mixed scheduling and non-mixed scheduling - For mixed scheduling, all the HARQ process combinations are continuous - For non-mixed scheduling, bitmap method for HARQ process, 1 bit NDI and MSB 4 bits(0000) for early termination are adopted - If DCI size increased by 4 bits, the full scheduling flexibility should be provided, with joint-coding of 10 bits value X, indicating MCS, HARQ processes and NDI, X is calculated by - X=80*ITBS+M, where ITBS is for uplink and for downlink, M is and for uplink and for downlink - for 1 TB, for 2 TB, for 3 TB, and for 4TB. |

Based on the proposals listed above, the following can be considered.

- For unicast in CE mode B, select one of the following alternative encoding solutions for MCS, HARQ ID, NDI, and number of HARQ processes.

Option A:

6 bits indicate a value of V. 1) Imcs = floor(V/5). 2) The result of Vmod5 indicates N0 and TB number. For Vmod5=0 or 1, it implicitly indicates N0. Moreover, if H0=H1, it means one TB. Otherwise two TBs. For two TBs, the comparison of H0 and H1 can implicitly indicate NDI1 for the second TB. For example, N1=0 for 2nd TB if H0 < H1, and N1=1 for 2nd TB if H0 > H1. For two TBs case, N0 is the NDI for the TB with smaller HARQ process number, N1 is the NDI for the TB with larger HARQ process number. For Vmod5=2 or 3: it implicitly indicates N0. Moreover, two bits to indicate the one HARQ process ID combination from all four combinations (012, 013,023,123) for 3TBs. For Vmod5=4, N0, N1, N2, N3 are four NDIs related to 4TBs. TB number; V mod 5=; b0; b1; b2; b3; IMCS. 1,2; 0 {N0=0}; H0; H1; floor(V/5). 1 {N0=1}. 3; 2 {N0=0}; HARQ indication; N1; N2. 3 {N0=1}. 4; 4; N0; N1; N2; N3.

Option B:

Joint_coded_result=80*I_tbs+M For CE mode A, 4 HARQ processes are numbered as 0, 1, 2, 3 separately. Assume that when m HARQ processes are scheduled, h1~hm denotes the m HARQ processes number which belongs to {0,1,2,3} and the corresponding NDI is denoted as x1~xm which belongs to {0,1}. for example, if HARQ ID 0 and 2 are scheduled, we have h1 =0, h2=2. (Actually, we can give the constraint h1<h2<...<hm)

Option C:

For CE mode B MTBG DCI design, for DL grant and full PRB UL grant and for Sub-PRB UL grant, the following bit map to indicate HARQ ID, # of HARQ, NDI, MCS: Bits; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10. 1 HARQ; 0; 0; 0; MCS; HARQ1; N1. 2 HARQS; 0; MCS+3; HARQ2 Index; N1; N2. 3 HARQS; MCS+15; HARQ3; N2; N3; N4. 4 HARQS; MCS +52; N1; N2; N3; N4. where: - HARQ1 = HARQ ID scheduled - HARQ2 index = nChoosek(4- H1,2)-H2+H1 - HARQ3 = HARQ ID not scheduled MCS is 11 legacy values for DL grant and 10 legacy values UL grant for full PRB and 8 legacy valued for UL grant for sub-PRB

# 4	Feature combinations

RAN1#98bis made the following agreements regarding the configuration or use of the unicast multi-TB feature in combination with other features:

Agreement

Unicast multi-TB scheduling can be configured and used together with at least the following other features:

- Rel-14 feature for 2984 bits max UL TBS in 1.4 MHz in CE mode A

FFS till RAN1#99 whether unicast multi-TB scheduling can be configured and/or used together with the following other features:

- Rel-14 feature for new numbers of repetitions for PUSCH and modulation restrictions for PDSCH/PUSCH in CE mode A

- Rel-14 feature for modulation restrictions for PDSCH/PUSCH in CE mode A

- Rel-14 feature on HARQ-ACK bundling in HD-FDD in CE mode A

- Rel-14 features for 5 or 20 MHz max PDSCH/PUSCH channel bandwidths in CE mode A/B

- Rel-14 feature for 10 downlink HARQ processes in FDD in CE mode A

- Rel-14 feature for dynamic HARQ-ACK delay for HD-FDD in CE mode A

- Rel-15 feature for PUSCH sub-PRB allocation in CE mode A/B

- Rel-15 feature for 64QAM for non-repeated unicast PDSCH in CE mode A

- Rel-15 feature for uplink HARQ-ACK feedback in DCI in CE mode A/B

- Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B

- Any other feature that was supported since Rel-13 LTE eMTC

The following proposals concern the relation between the multi-TB feature and legacy Rel-14/15 features.

| [6] Ericsson | Proposal 19: For unicast, multi-TB scheduling can be configured and used in combination with: - Rel-14 feature for new numbers of repetitions for PUSCH - Rel-14 feature for modulation restrictions for PDSCH/PUSCH in CE mode A - Rel-15 feature for PUSCH sub-PRB allocation in CE mode A/B - Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B Proposal 20: For unicast, if a DCI design can support configuration and/or use of the multi-TB feature in combination with the following Rel-14/15 features, it is in counted as a benefit for that DCI design. - Rel-14 features for 5 or 20 MHz max PDSCH/PUSCH channel BW in CE mode A/B - Rel-14 feature for 10 downlink HARQ processes in FDD in CE mode A - Rel-14 feature for dynamic HARQ-ACK delay for HD-FDD in CE mode A - Rel-15 feature for 64QAM for non-repeated unicast PDSCH in CE mode A - Rel-15 feature for uplink HARQ-ACK feedback in DCI in CE mode A/B |
| --- | --- |
| [9] Intel Corporation | Proposal 6: Do not support Rel-14 feature for dynamic HARQ-ACK delay for HD-FDD in CE mode A together with scheduling of multiple TBs. |
| [14] ZTE | Proposal 16: Support: - Rel-14 feature for modulation restrictions for PDSCH/PUSCH in CE mode A - Rel-14 features for 5 or 20 MHz max PDSCH/PUSCH channel bandwidths in CE mode A/B - Rel-15 feature for PUSCH sub-PRB allocation in CE mode A/B - Rel-15 feature for 64QAM for non-repeated unicast PDSCH in CE mode A - Rel-15 feature for uplink HARQ-ACK feedback in DCI in CE mode A/B - Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B Proposal 16: Not support: - Rel-14 feature for new numbers of repetitions for PUSCH in CE mode A - Rel-14 feature on HARQ-ACK bundling in HD-FDD in CE mode A - Rel-14 feature for 10 downlink HARQ processes in FDD in CE mode A - Rel-14 feature for dynamic HARQ-ACK delay for HD-FDD in CE mode A |
| [17] Qualcomm Incorporated | Proposal 11: For optional features: - Supported: Modulation restrictions, 2984 bits, flexible PRB, HARQ-ACK feedback (except early termination) - Deprioritized/FFS: 10 HARQ processes, sub-PRB, 64-QAM, early PUSCH termination. - Not supported: New repetitions for PDSCH/PUSCH, larger BW, dynamic HARQ-Ack delay |

Based on the proposals listed above, the following can be considered, where the Rel-14 features ‘HARQ-ACK bundling in HD-FDD in CE mode A’ and ’10 downlink HARQ processes in FDD in CE mode A’ are excluded.

- The following features can be configured together with the unicast multi-TB feature and used at least when scheduling a single TB:

- Rel-14 feature for new numbers of repetitions for PUSCH

- Rel-14 feature for modulation restrictions for PDSCH/PUSCH in CE mode A

- Rel-14 features for 5 or 20 MHz max PDSCH/PUSCH channel bandwidths in CE mode A/B

- Rel-14 feature for dynamic HARQ-ACK delay for HD-FDD in CE mode A

- Rel-15 feature for PUSCH sub-PRB allocation in CE mode A/B

- Rel-15 feature for 64QAM for non-repeated unicast PDSCH in CE mode A

- Rel-15 feature for uplink HARQ-ACK feedback in DCI in CE mode A/B

- Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B

# 5	UE capabilities

RAN2#106 made the following agreements:

Agreement

UE capability for multiple TB is indicated separately for CE Mode A and CE mode B.

UE capability for multiple TB is indicated separately for uplink and downlink.

RAN1#98 made the following agreements:

Agreement

For unicast, scheduling gaps for multiple transport blocks is supported and a scheduling gap can be configured by [RRC and/or DCI].

- The support of scheduling gaps is UE optional feature regardless of the support of multiple TBs.

- FFS: Details on the scheduling gap such as duration, applicability, etc.

So, it has already been agreed that the multi-TB feature has separate UE capability indications for sub-features 1 through 5 in the following list.

- For DL in CEmodeA

- For DL in CEmodeB

- For UL in CEmodeA

- For UL in CEmodeB

- With scheduling gaps

- With TB interleaving

- With HARQ bundling

- With UL sub-PRB

The following proposal concern the indication UE capabilities.

| [6] Ericsson | Proposal 21: For unicast, discuss whether the following sub-features of the multi-TB feature should have separate UE capability indications: - TB interleaving - HARQ bundling - UL sub-PRB |
| --- | --- |

Based on the proposal listed above, the following can be considered.

- Discuss UE capability indications for the multi-TB feature.

# References

[1]	RP-191356, “Revised WID: Additional MTC enhancements for LTE”

[2]	R1-1911573, “RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE”

[3]	R2-1914102, “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”

[4]	R1-1911574, “Summary of RRC parameters for L1 configuration for Rel-16 LTE-MTC”

[5]	R1-1911750, “RAN1 UE features list for Rel-16 LTE”

[6]	R1-1911832, “Scheduling of multiple DL/UL transport blocks in LTE-MTC”, Ericsson

[7]	R1-1911919, “Scheduling of multiple transport blocks”, Huawei, HiSilicon

[8]	R1-1911982, “Scheduling of multiple DL/UL transport blocks”, Nokia, Nokia Shanghai Bell

[9]	R1-1912188, “Scheduling of multiple DL/UL TBs for eMTC”, Intel Corporation

[10]	R1-1912334, “On scheduling of multiple DL/UL transport blocks”, Sony

[11]	R1-1912365, “Discussion on the remaining issues of scheduling of multiple DL/UL TBs”, Beijing Xiaomi Software Tech

[12]	R1-1912369, “LTE-M Multiple Transport Block Grant Design Considerations”, Sierra Wireless, S.A.

[13]	R1-1912376, “Discussion on multiple transport blocks scheduling in MTC”, LG Electronics

[14]	R1-1912411, “Consideration on scheduling enhancement for MTC”, ZTE

[15]	R1-1912679, “Scheduling of multiple DL/UL transport blocks”, Qualcomm Incorporated

[16]	R1-1912701, “Scheduling of multiple DL/UL transport blocks”, Sequans Communications

[17]	R1-1913246, “Scheduling of multiple DL/UL transport blocks”, Qualcomm Incorporated

[18]	R1-1913318, “Consideration on scheduling enhancement for MTC”, ZTE

[19]	R1-1913403, “Scheduling of multiple transport blocks”, Huawei, HiSilicon

[20]	R1-1913229, “Feature lead summary for Scheduling of multiple DL/UL transport blocks for LTE-MTC”

[21]	R1-1913371, “Feature lead summary #2 for Scheduling of multiple DL/UL transport blocks for LTE-MTC”