3GPP TSG RAN WG1 Meeting #99	R1-1913449

Reno, USA, 18th – 22nd November 2019

Agenda Item:	6.2.1.4

Source:	Ericsson

Title:	Feature lead summary #2 for Coexistence of LTE-MTC with NR

Document for:	Discussion and Decision

# Introduction

In the Rel-16 work item on “Additional MTC enhancements for LTE” [1], one of the objectives is to specify performance improvements for LTE-MTC coexistence with NR.

The objective is to specify the following set of improvements for machine-type communications for BL/CE UEs. […] Coexistence with NR: - Specify the following performance improvements for LTE-MTC coexistence with NR [RAN1, RAN2, RAN4] - LTE-MTC resource reservation at least in the time domain with at least one of slot-level and symbol-level granularity at least in DL to avoid resource overlap between NR and LTE-MTC when LTE-MTC is deployed within an NR carrier - LTE-MTC subcarrier puncturing for a small number of LTE-MTC DL subcarriers (excluding CRS) to reduce the number of NR resource blocks that need to be reserved for LTE-MTC when LTE-MTC is deployed within an NR carrier - For LTE-MTC in-band operation co-existence with NR, RAN4 will investigate the following: - 15 kHz, 30 kHz, and 60 kHz numerologies for NR FR1 bands, with higher priority given first to 15 kHz and then to 30 kHz - Study feasible LTE-MTC placement allocation without RF backward compatibility impact and compatible with Rel-13 LTE-MTC and Rel-15 NR, to operate simultaneously within various NR channel bandwidths - Channel raster, PRB and subcarrier grid alignment between LTE-MTC and NR - Synchronization issue between LTE-MTC and NR, including timing advance - Frequency band support in LTE-MTC and NR - Testability applicability - Compatibility for Rel-15 NR and Rel-13/14/15 LTE-MTC - The case of NR configured with 15 kHz SS block SCS and the case of 30 kHz SS block SCS as specified in 38.101-1 are included in the study. Note: After RAN1 concludes the objective on R16 LTE-MTC coexistence aspects, evaluate coexistence between R15 NR and R16 LTE-MTC.

This document provides a summary and recommendations based on contributions in [2] – [8]. This is an updated version of the initial feature lead summary provided in .

# LTE-MTC resource reservation

In RAN1#98 and RAN1#98bis, the following agreements were made regarding “LTE-MTC resource reservation at least in the time domain with at least one of slot-level and symbol-level granularity at least in DL to avoid resource overlap between NR and LTE-MTC when LTE-MTC is deployed within an NR carrier”.

RAN1#98 agreements - LTE-MTC DL time-domain resource reservation is supported with a finer granularity than slot level. Transmissions in these reserved resources are dropped. - FFS: How to handle DMRS - Support LTE-MTC DL time-domain resources reservation with slot-level granularity. Transmissions in these reserved resources are dropped. - Whether to support LTE-MTC UL time-domain resource reservation and/or LTE-MTC DL frequency-domain resource reservation is FFS. - The LTE-MTC resource reservation is configured by higher layers via [SIB or UE-specific RRC signaling]. It is FFS whether DCI can override or modify the higher-layer configuration. - It is FFS whether/how the higher layer configuration relates to the existing Rel-13 parameters for LTE-MTC valid subframe configuration and MBSFN configuration. RAN1#98bis agreements - For resource reservation for LTE eMTC in Rel-16, the configuration is independent from legacy subframe level resource reservation. - For LTE-MTC resource reservation, for unicast, symbols of MPDCCH/PDSCH scrambled by C-RNTI or SPS-C-RNTI that would fall into the reserved resource are dropped for symbol-level and slot-level reserved resources. - Dropped means punctured - FFS: CSI-RS - For LTE-MTC resource reservation, UL resource reservation for LTE eMTC with slot-level and symbol(s)-level granularity in addition to subframe-level granularity is supported. - For LTE-MTC resource reservation, for unicast transmission, dynamic DCI signalling can be used to [indicate or override] which reserved resources are used for the scheduled LTE eMTC transmission - For LTE-MTC resource reservation, the higher-layer configuration is cell-specific - LTE-MTC DL frequency-domain resource reservation is supported - FFS: granularity of resource reservation

The table below summarizes proposals related to DL/UL time-domain resource reservation.

| [2] Ericsson | Proposal 8: A bitmap with length 20 bits or 80 bits is used for indicating reserved symbols and slots within 10 ms or 40 ms periodicity. Proposal 9: The periodicity of resource reservation (10 ms or 40 ms) and whether it applies to symbol-level or slot-level are configurable. Proposal 10: Four possible patterns of reserved symbols/slots within each subframe are supported: - Two patterns (out of four possible patterns) are used to indicate valid subframe and invalid subframe. - For slot-level, the other two patterns indicate whether the first slot or the second slot is reserved - For symbol-level, the other two patterns are configurable. |
| --- | --- |
| [3] Huawei, HiSilicon | Proposal 4: The reserved resource is configured by one periodicity, one start position and one duration. - In the configured duration, a bitmap is used to indicate whether a symbol/slot is configured as the reserved resource or not. |
| [4] Nokia, Nokia Shanghai Bell | Proposal 1: eMTC reserved resource configuration is given using a bitmap-based scheme. Proposal 2: In the time domain, reserved resource configuration is given using two-level bitmap – the first bitmap (10 or 40 bits) indicates the reserved subframes and the second bitmap indicates the reserved symbols or slot within the subframes. |
| [5] LG Electronics | Proposal 4: For resource reservation for LTE eMTC in Rel-16, UE-specific RRC configuration is supported based on UE capability. |
| [6] ZTE | Proposal 5: For DL LTE-MTC time-domain resources reservation with slot-level granularity, fixed length bitmap(s) can be configured for the first four DL LTE slots within a NR SSB periodicity. - The bitmap length is 4. - The NR SSB periodicity is signalled to the LTE-MTC UEs by RRC signalling. Proposal 6: For LTE-MTC DL time-domain resources reservation with symbol-level granularity, fixed length bitmap(s) can be configured for partial time region within a periodicity. - The periodicity and the time region within the periodicity are signalled to the LTE-MTC UEs by RRC signalling. Proposal 9: For LTE-MTC UL time-domain resources reservation with symbol-level granularity, fixed length bitmap(s) can be configured for partial time region within a periodicity. - The bitmap length is 12 bits for every subframe. |
| [7] Futurewei | Proposal 1: Bitmap based method is used for subframe-level, slot-level, or symbol(s)-level NB-IoT reserved resource configuration. - Value 0 in the bitmap indicates that the corresponding subframe/slot/symbol is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe/slot/symbol is valid for transmission. Proposal 2: For slot-level resource reservation, the configuration signaling is 20 bits bitmap where each bit corresponds to one slot in a frame. Proposal 3: For symbol-level resource reservation, the configuration signaling includes 10 bits to indicate the subframe and x bits to indicate a reservation pattern for symbols in a subframe, x is FFS. |
| [8] Qualcomm Incorporated | Proposal 6: The resource reservation for LTE eMTC in Rel-16 shall be compatible with NR PDSCH rate matching pattern configuration. Proposal 7: For resource reservation for LTE eMTC in Rel-16, the configuration includes: - A first bitmap of RB granularity (only for DL) with up to 100 bits - A second bitmap of symbol or slot level granularity spanning one subframe for which the reserved RBs apply with up to 10 bits - An optional third bitmap of time domain pattern with up to 40 bits where each bit corresponds to a subframe and a bit value equal to 1 indicates the pair of RB and symbol/slot level bitmap is present in the subframe |

Time-domain resource reservation was treated in an offline session in RAN1#99 jointly for LTE-MTC and NB-IoT. The outcome of the session is captured in [10].

- For time-domain resource reservation, discuss the options listed in [10].

The table below summarizes proposals related to DL frequency-domain resource reservation.

| [2] Ericsson | Proposal 11: Frequency-domain LTE-M resource reservation is configured for a contiguous set of PRBs. Proposal 12: LTE-M PRBs are reserved by indicating the starting PRB and the number of contiguous PRBs (i.e., length). Proposal 13: The starting PRB index and the length are determined based on the locations of the LTE-M carrier and NR channels/signals which need to be protected. Proposal 14: For N LTE-M PRBs (indexing from 0 to N-1), we have the following conditions: - 0 ≤ starting PRB index ≤ N-1 - 1 ≤ length ≤ N Proposal 15: Time and frequency domains resource reservations are jointly applied to indicate the reserved time-frequency LTE-M resources (two-dimensional chunk of resources as shown in Figure 9). That is, symbols are reserved only within PRBs indicated by frequency-domain resource reservation. |
| --- | --- |
| [4] Nokia, Nokia Shanghai Bell | Proposal 3: In the frequency domain, reserved resource configuration is given using PRB(s)-level bitmap – the granularity can be Resource Block Group. |
| [6] ZTE | Proposal 10: For frequency-domain resource reservation in LTE-MTC, reserved RBs are in lower edge or upper edge or both edges of LTE-MTC carrier. The reserved RBs in lower edge or upper edge are consecutive. |
| [7] Futurewei | Proposal 4: For frequency domain resource reservation, the granularity is at PRB(s)-level. A bitmap signaling is used for PRB(s)-level resource reservation. |
| [8] Qualcomm Incorporated | Proposal 4: At least the PRB level frequency domain resource reservation is supported, and LTE-MTC MPDCCH/PDSCH transmission including DMRS on these reserved PRBs is dropped or punctured Proposal 6: The resource reservation for LTE eMTC in Rel-16 shall be compatible with NR PDSCH rate matching pattern configuration. Proposal 7: For resource reservation for LTE eMTC in Rel-16, the configuration includes: - A first bitmap of RB granularity (only for DL) with up to 100 bits - A second bitmap of symbol or slot level granularity spanning one subframe for which the reserved RBs apply with up to 10 bits - An optional third bitmap of time domain pattern with up to 40 bits where each bit corresponds to a subframe and a bit value equal to 1 indicates the pair of RB and symbol/slot level bitmap is present in the subframe |

Given the proposals in the contributions and the discussion that has taken place in the RAN1#99 meeting, the following proposals can be considered:

- For DL frequency-domain resource reservation, select one of the following options:

- Option 1: Contiguous PRB allocation is indicated using 5-13 bits depending on the LTE system bandwidth.

- Option 2: Non-contiguous PRB allocation is indicated using a bitmap with [TBD] size and [TBD] granularity.

- Discuss and decide DL frequency-domain resource reservation and DL time-domain resource reservation are independently configured or not.

The table below summarizes proposals related to DCI for indicating or overriding higher-layer configuration.

| [2] Ericsson | Proposal 16: DCI can be used to make [some or all] of the Rel-13 invalid subframes valid for the duration of the scheduled PDSCH/PUSCH transmission. Proposal 17: A 1-bit DCI field indicates whether the Rel-13 valid subframe bitmap or the Rel-16 reserved resource configuration should apply. Proposal 18: If no Rel-16 reserved resource configuration is signaled in SIB, then the DCI bit can be used to select between the Rel-13 valid subframe bitmap and continuous transmission (which can be expressed as if the default value of the Rel-16 reserved resource configuration is ‘empty’ or ‘continuous transmission’). |
| --- | --- |
| [3] Huawei, HiSilicon | Proposal 5: The timing between the DCI and the subframe/slot that indicated resource reservation takes effects can be either fixed or configured by the eNB. Proposal 6: 2 bits in the DCI are used to indicate the reserved resource including the following patterns: - The same as the legacy higher layer indication, i.e. no dynamic reserve resource indication - No reservation resource - Preconfigured reserved resource pattern 1 - Preconfigured reserved resource pattern 2 |
| [5] LG Electronics | Proposal 5: For dynamic TD resource reservation, N candidate TD resource reservation pattern(s) is/are pre-configured by RRC signaling and the dynamic DCI signaling indicates one of the N candidates. - N is determined based on the number of bits in the DCI for dynamic TD resource reservation |
| [6] ZTE | Proposal 11: For unicast LTE-MTC transmission scheduled by DCI, 2 bits in the DCI are used to dynamically override the resource reservation during the transmission period for the following cases: - The reserved resources cannot be used by the unicast transmission. - The reserved resources during the first half part can be used by the unicast transmission. - The reserved resources during the second half part can be used by the unicast transmission. - The reserved resources can be used by the unicast transmission. Proposal 12: For LTE-MTC transmission, dynamic signalling in DCI can be used to indicate information of one-shot downlink resource reservation. - The LTE-MTC UE assumes the downlink LTE-MTC transmissions in the reserved resources are punctured. Proposal 13: For LTE-MTC transmission, a dynamic dedicated control information is used to indicate information of one-shot uplink resource reservation. - The uplink LTE-MTC transmission in the reserved resources can be punctured/muted or postponed. |
| [7] Futurewei | Proposal 5: The eNB can indicate resource reservation in DCI. The field in the DCI is 2 bits. |
| [8] Qualcomm Incorporated | Proposal 5: For unicast transmission, dynamic DCI signalling can be used to indicate a switch between the legacy subframe level resource reservation and finer granularity reserved resources for the scheduled LTE-MTC transmission |

Given the proposals in the contributions and the discussion that has taken place in the RAN1#99 meeting, the following proposals can be considered:

- Discuss the DCI options listed in [10], and consider potential differing aspects between NB-IoT and LTE-MTC (e.g. the DL frequency-domain resource reservation).

The table below summarizes proposals related to the UL dropping mechanism.

| [2] Ericsson | Proposal 19: For unicast LTE-M transmissions, PUSCH scrambled by C-RNTI or SPS-C-RNTI that would fall into the reserved resource are dropped (i.e., punctured) for symbol-level and slot-level reserved resources. |
| --- | --- |
| [8] Qualcomm Incorporated | Proposal 1: For unicast, at least for PUCCH and PUSCH scrambled by C-RNTI and SPS-C-RNTI with N_rep>1 or N>1, where N is the transmission duration, transmission on the reserved resources are dropped for symbol and slot-level reserved resources |

Given the proposals in the contributions and the discussion that has taken place in the RAN1#99 meeting, the following proposals can be considered:

- For unicast, at least in the repetition case, symbols of PUCCH and PUSCH scrambled by C-RNTI and SPS-C-RNTI that would fall into the reserved resources are dropped (meaning punctured) for symbol- and slot-level reserved resources.

- Discuss whether some special handling is needed in the non-repetition case.

The table below summarizes proposals related to handling of DL/UL DMRS and CSI-RS

| [2] Ericsson | Proposal 20: LTE-M resource reservation is not applied to symbols that carry DMRS. Proposal 21: Symbols carrying LTE-M CSI-RS can be reserved. |
| --- | --- |
| [4] Nokia, Nokia Shanghai Bell | Proposal 4: Rel-16 UE cannot expect DMRS to be transmitted in reserved resource. Proposal 5: Resource reservation applies to CSI-RS. |
| [6] ZTE | Proposal 7: For symbol-level resource reservation in UL LTE-MTC, DMRS symbol cannot be reserved if other symbols in corresponding slot are not reserved. Proposal 8: For slot-level reservation in UL LTE-MTC, DMRS transmission in a reserved slot is dropped. |
| [8] Qualcomm Incorporated | Proposal 2: LTE-MTC DL symbols that carry DMRS are not configured as reserved resources Proposal 3: CSI-RS REs are not reserved Proposal 4: At least the PRB level frequency domain resource reservation is supported, and LTE-MTC MPDCCH/PDSCH transmission including DMRS on these reserved PRBs is dropped or punctured |

Given the proposals in the contributions and the discussion that has taken place in the RAN1#99 meeting, the following proposals can be considered:

- Discuss and decide whether CSI-RS REs can be reserved in DL frequency-domain and DL time-domain resource reservation.

- In DL frequency-domain and DL time-domain resource reservation, DMRS REs can be reserved if and only if all other non-CRS REs in the same slot and PRB are also reserved.

- In UL time-domain resource reservation, DMRS REs can be reserved if and only if all other REs in the same slot and PRB are also reserved.

# References

- RP-191356, “Revised WID: Additional MTC enhancements for LTE”

- R1-1911833, “Coexistence of LTE-MTC with NR”, Ericsson

- R1-1911920, “On eMTC co-existence with NR”, Huawei, HiSilicon

- R1-1911983, “Coexistence of eMTC with NR”, Nokia, Nokia Shanghai Bell

- R1-1912377, “Discussion on coexistence of LTE-MTC with NR”, LG Electronics

- R1-1912412, “Coexistence of LTE-MTC with NR”, ZTE

- R1-1912631, “Resource reservation for eMTC”, Futurewei

- R1-1912680, “Coexistence of LTE-MTC with NR”, Qualcomm Incorporated

- R1-1913230, “Feature lead summary for Coexistence of LTE-MTC with NR”

- R1-1913515, “Feature lead summary #2 of Coexistence of NB-IoT with NR”