3GPP TSG-RAN WG1 Meeting #99	R1-1913455

Reno, USA, November 18th – 22th, 2019

Agenda Item:	7.2.5

Source:	Thales

Title:	Text proposal for Section 6.1 of TR38.821

Type: 	pCR

Document for:	Approval

# Introduction

Since last RAN1 meeting, a new version of TR38.821 [1] has been approved.

In this document, we provide a FL TP on Link-Level and System-Level Evaluations for NTN. It aims to

- Enrich the current texts of Section 6.1 based on already captured subsections/agreements

- Fix the remaining inconsistencies

- Capture the simulation results

Endorse the TP captured in this contribution and include it in TR38.821

# Text Proposal

======================== Start of TP for TR 38.821 =============================

## 6.1	Link-Level and System-Level Evaluations

Both multi-satellite and single satellite simulations should be considered for calibration and performance evaluation.

### 6.1.1	System level simulations

#### 6.1.1.1 Simulation assumptions

The following tables representing two sets of satellite parameters are considered as the baseline for system level simulator calibration:

Table 6.1.1.1-1: Set-1 satellite parameters for system level simulator calibration

============================== Rows Omitted ================================

Table 6.1.1.1-2: Set-2 satellite parameters for system level simulator calibration

============================== Rows Omitted ================================

The following table is agreed for UE characteristics for System Level Simulations

Table 6.1.1.1-3: UE characteristics for system level simulations

============================== Rows Omitted ================================

The following table is agreed for the beam layout definition for a single satellite simulation

Table 6.1.1.1-4: Beam layout definition for single satellite simulation

============================== Rows Omitted ================================

Table 6.1.1.1-5: System Level Simulation assumptions for calibration

| ============================== Rows Omitted ================================ |  |
| --- | --- |
| UE distribution | Base-line for calibration: at least X=10 UEs per beam with uniform distribution in all the Voronoi cell area associated to each beam. The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers.(Note 3) |
| ============================== Rows Omitted ================================ |  |
| Note 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. Note 2: For the calibration purpose, - The ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). - The atmospheric absorptions loss shall be considered. CC={c0, c1, …, cn}ckkkVkckCC |  |

Traditionally, the wrap around mechanism used in system level simulations refers to a mirroring effect of the surrounding cells/beams so that the computational load of the simulations can be reduced. However, these types of schemes are not applicable in NTN context except in the specific case of central beam at nadir (90° elevation). Therefore, for the evaluation, all the additional surrounding beams should be simulated independently.

As a consequence, a new wrap around mechanism is introduced in case of single satellite simulation for intra-satellite interference modelling based on additional bore-sight beam directions which should be computed based on the methodology captured in Table 6.1.1-4.

In particular, the following wrap-around mechanism should be adopted for calibration:

- For FRF = 1, two additional tiers of beams are considered in the simulation surrounding the 19-beam layout (cf. Figure 6.1.1.1-1).

- For FRF > 1, four additional tiers of beams are considered in the simulation surrounding the 19-beam layout (cf. Figure 6.1.1.1-1).

- Considering a UE attached to a beam, in the DL/UL all remaining beams are treated as interference as long as these beams are sharing the same frequency band/polarization (cf. Figure 6.1.1.1-2).

- For the metrics statistic (e.g. coupling loss, geometry), only the UEs placed in the inner-19 beams are considered.

Figure 6.1.1.1-1 : Illustrations of the additional tiers of beams to be wrapped around based on the FRF configurations

Figure 6.1.1.1-2 : Definition of the interfering beams to be considered

The beam layout parameters captured in the following table are adopted as a starting point for single satellite simulations.

Table 6.1.1.1-6: Beam layout parameters for single satellite simulation

============================== Rows Omitted ================================

For handheld terminal, the following association between antenna port and antenna element is assumed for calibration, evaluations and link budgets calculation:

- 1 TX with one antenna element associated to one Tx branch.

- 2 RX with each antenna element associated to one Rx branch.

Note: Implementations without the above association can be evaluated in addition

As a consequence, the following considerations are assumed for handheld use cases :

- For downlink transmission, a combination of the two Rx branches allows to prevent depolarization loss. (cf. Figure 6.1.1.1-3)

- For uplink transmissions,

  - A 3dB depolarization loss should be taken into account assuming polarization reuse is applied and satellite reception implements circular polarization (e.g., when frequency reuse option 3 is considered ) (cf. Figure 6.1.1.1-4 – configuration A)

  - A 0dB depolarization loss can be assumed when satellite reception implements dual polarization per beam (e.g., for the frequency reuse 1 and 3 cases) (cf. Figure 6.1.1.1-4 – configuration B)

Figure 6.1.1.1-3 : Example of DL RX/TX configuration for Handheld use cases

Figure 6.1.1.1-4 Example of UL RX/TX configurations for Handheld use cases

For VSAT use cases, calibration results and link budgets should be computed assuming no depolarization loss.

Table 6.1.1.1-7: SLS parameters for performance evaluation

============================== Rows Omitted ================================

The following table captures the impairments introduced on the RF signal due to the satellite payload and movement for link level simulations.

Table 6.1.1.1-8: Impairments due to satellite payload and satellite movement

|  | S-band | Ka-band |
| --- | --- | --- |
| Phase noise model (Note 2) | Optional | Phase noise profile according to TR38.803 |
| On-board oscillator long-term drift (Note 3) | [0.5] ppm | [0.5] ppm |
| ============================== Rows Omitted ================================ |  |  |
| Max Doppler shift if pre/post compensation mechanism is assumed at satellite payload side | Scenario A: n/a Scenario C2/D2: - Satellite alt. = 1200 km : - beam diameter = 90 km (Set 1 - S-band) : 0.91 ppm - beam diameter = 40 km (Set 1 - Ka-band): 0.40 ppm - beam diameter = 190 km (Set 2 - S-band) : 1.91 ppm - beam diameter = 90 km (Set 2 - Ka-band) : 0.91ppm - beam diameter = 1000 km (Max beam foot print size) : 9.17ppm - Satellite alt. = 600 km : - beam diameter = 50 km (Set 1 - S-band) : 1.05 ppm - beam diameter = 20 km (Set 1 - Ka-band) : 0.42 ppm - beam diameter = 90 km (Set 2 - S-band) : 1.88 ppm - beam diameter = 50 km (Set 2 - Ka-band) : 1.05 ppm - beam diameter = 1000 km (max beam foot print size) : 15.82 ppm |  |
| ============================== Rows Omitted ================================ |  |  |
| Note 1 : Min. Elevation angle for both sat- user equipment is equal to 10 degree. Note 2 : For regenerative scenario, this can be considered as the phase noise model for the gNB. For transparent scenarios, it should be considered as an additional phase noise w.r.t the phase noise generated by the gNB and the UE. Note 3 : These values are provided for information only and have not been considered in Radio Layer 1 analysis for the following reasons : - In the cases where transparent satellite payload are considered, it is assumed that the gNB can detect the frequency shift due to on -board oscillator long-term drift and compensate for it. - In the cases where regenerative satellite payload are considered, it is assumed that the on -board oscillator drift is sufficiently decreased to become negligible. |  |  |

Table 6.1.1.1-9: List of calibration study cases

============================== Rows Omitted ================================

#### 6.1.1.2 Calibration results

Based on System Level Simulation assumptions for calibration described in Table 6.1.1-5, the results of CL, Geometry SIR and Geometry SINR simulated on DL and UL transmissions are reported in Table 6.1.1-10 and Table 6.1.1-11. These results are representative of the average performance reported by the different companies in [R1-1913404] [R1-1913405].

Table 6.1.1.2-1: Calibration results on DL transmissions

|  | DL Coupling Loss |  |  | DL Geometry SIR |  |  | DL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| SC1 | 109.3 | 113.6 | 117.9 | -3.0 | -1.0 | 1.2 | -3.2 | -1.2 | 1.0 |
| SC2 | 109.2 | 113.6 | 118.0 | 8.4 | 9.0 | 9.2 | 5.5 | 7.4 | 8.4 |
| SC3 | 109.3 | 113.7 | 118.0 | 9.7 | 10.0 | 10.2 | 6.0 | 8.1 | 9.2 |
| SC4 | 138.0 | 140.3 | 142.5 | -3.1 | -1.1 | 1.1 | -4.0 | -2.1 | -0.1 |
| SC5 | 138.0 | 140.3 | 142.5 | 8.0 | 8.9 | 9.2 | 1.5 | 3.3 | 4.9 |
| SC6 | 96.2 | 97.5 | 98.9 | -3.0 | -1.1 | 1.1 | -3.2 | -1.2 | 0.8 |
| SC7 | 96.2 | 97.5 | 98.9 | 8.3 | 9.0 | 9.2 | 6.6 | 7.6 | 7.8 |
| SC8 | 96.2 | 97.5 | 98.9 | 9.7 | 9.9 | 10.1 | 7.5 | 8.1 | 8.5 |
| SC9 | 123.7 | 125.3 | 127.0 | -3.0 | -1.1 | 1.1 | -3.1 | -1.1 | 1.0 |
| SC10 | 123.7 | 125.3 | 127.0 | 8.3 | 9.0 | 9.2 | 7.3 | 8.2 | 8.5 |
| SC11 | 102.2 | 103.5 | 105.0 | -3.0 | -1.1 | 1.1 | -3.2 | -1.2 | 0.8 |
| SC12 | 102.2 | 103.5 | 105.0 | 8.3 | 9.0 | 9.2 | 6.6 | 7.6 | 7.8 |
| SC13 | 102.2 | 103.5 | 105.0 | 9.7 | 9.9 | 10.1 | 7.5 | 8.1 | 8.5 |
| SC14 | 129.8 | 131.3 | 133.0 | -3.0 | -1.1 | 1.1 | -3.1 | -1.1 | 1.0 |
| SC15 | 129.8 | 131.4 | 133.0 | 8.3 | 9.0 | 9.2 | 7.3 | 8.2 | 8.5 |
| SC16 | 117.3 | 121.7 | 126.0 | -3.0 | -1.0 | 1.2 | -4.3 | -2.2 | 0.0 |
| SC17 | 117.3 | 121.7 | 126.0 | 8.3 | 9.0 | 9.2 | -0.3 | 3.2 | 6.0 |
| SC18 | 117.4 | 121.8 | 126.1 | 9.7 | 10.0 | 10.1 | -0.2 | 3.4 | 6.4 |
| SC19 | 143.4 | 145.8 | 148.1 | -3.1 | -1.1 | 1.0 | -5.9 | -4.0 | -2.1 |
| SC20 | 143.4 | 145.8 | 148.1 | 8.3 | 9.1 | 9.5 | -3.3 | -1.2 | 0.9 |
| SC21 | 100.3 | 105.5 | 111.1 | -3.0 | -1.1 | 1.1 | -4.5 | -2.3 | -0.1 |
| SC22 | 100.3 | 105.5 | 111.1 | 8.3 | 9.0 | 9.2 | -1.3 | 3.4 | 6.4 |
| SC23 | 100.4 | 105.6 | 111.1 | 9.7 | 9.9 | 10.1 | -1.2 | 3.5 | 6.9 |
| SC24 | 129.8 | 131.5 | 133.3 | -3.0 | -1.0 | 1.2 | -3.3 | -1.4 | 0.7 |
| SC25 | 129.8 | 131.5 | 133.3 | 8.3 | 8.9 | 9.3 | 5.0 | 6.2 | 6.9 |
| SC26 | 106.1 | 111.6 | 117.3 | -3.0 | -1.1 | 1.1 | -4.6 | -2.3 | 0.0 |
| SC27 | 106.1 | 111.6 | 117.2 | 8.3 | 9.0 | 9.2 | -1.4 | 3.3 | 6.5 |
| SC28 | 106.2 | 111.6 | 117.3 | 9.7 | 9.9 | 10.1 | -1.4 | 3.5 | 7.0 |
| SC29 | 135.8 | 137.6 | 139.4 | -3.0 | -1.0 | 1.2 | -3.3 | -1.4 | 0.7 |
| SC30 | 135.8 | 137.6 | 139.4 | 8.3 | 8.9 | 9.3 | 5.0 | 6.2 | 6.9 |
| Note: Geometry SINR = -10log10(I/C + N/C). where C. I and N equals the carrier. interferer and noise power levels measured over the configured signal bandwidth. |  |  |  |  |  |  |  |  |  |

Table 6.1.1.2-2: Calibration results on UL transmissions

|  | UL Coupling Loss |  |  | UL Geometry SIR |  |  | UL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| SC1 | 109.2 | 113.5 | 117.8 | -6.9 | -1.3 | 4.4 | -7.0 | -1.5 | 1.0 |
| SC2 | 109.2 | 113.5 | 117.8 | 3.6 | 8.3 | 12.9 | 3.1 | 7.6 | 12.2 |
| SC3 | 109.2 | 113.5 | 117.8 | 4.5 | 9.3 | 14.1 | 3.5 | 8.1 | 12.7 |
| SC4 | 137.9 | 140.2 | 142.5 | -4.6 | -1.0 | 3.3 | -9.8 | -7.3 | -5.0 |
| SC5 | 138.0 | 140.3 | 142.5 | 8.0 | 8.9 | 9.2 | 1.5 | 3.3 | 4.9 |
| SC6 | 96.1 | 97.4 | 98.8 | -3.9 | -1.1 | 2.6 | -3.9 | -1.1 | 2.6 |
| SC7 | 96.1 | 97.4 | 98.8 | 6.1 | 8.3 | 10.4 | 6.1 | 8.3 | 10.4 |
| SC8 | 96.1 | 97.4 | 98.8 | 6.9 | 9.3 | 11.6 | 6.9 | 9.3 | 11.5 |
| SC9 | 123.7 | 125.3 | 127.1 | -4.0 | -0.8 | 3.2 | -4.1 | -1.1 | 2.7 |
| SC10 | 123.7 | 125.4 | 127.1 | 6.9 | 9.0 | 11.1 | 5.2 | 7.1 | 9.0 |
| SC11 | 102.2 | 103.4 | 104.8 | -4.0 | -1.1 | 2.6 | -4.0 | -1.2 | 2.5 |
| SC12 | 102.2 | 103.4 | 104.8 | 5.9 | 8.2 | 10.4 | 5.9 | 8.2 | 10.3 |
| SC13 | 102.2 | 103.4 | 104.8 | 6.7 | 9.2 | 11.6 | 6.7 | 9.2 | 11.5 |
| SC14 | 129.7 | 131.4 | 133.1 | -4.0 | -0.8 | 3.2 | -4.5 | -1.7 | 1.5 |
| SC15 | 129.7 | 131.4 | 133.1 | 6.9 | 9.0 | 11.1 | 2.3 | 4.1 | 5.8 |
| SC16 | 117.2 | 121.5 | 125.8 | -6.7 | -1.2 | 4.5 | -7.6 | -2.6 | 2.5 |
| SC17 | 117.2 | 121.5 | 125.8 | 3.7 | 8.3 | 12.9 | 0.9 | 5.3 | 9.6 |
| SC18 | 117.2 | 121.5 | 125.8 | 4.5 | 9.3 | 14.1 | 0.3 | 4.7 | 9.0 |
| SC19 | 143.4 | 145.7 | 148.1 | -4.6 | -0.9 | 3.5 | -13.9 | -11.6 | -9.3 |
| SC20 | 143.4 | 145.7 | 148.1 | 6.3 | 9.1 | 11.8 | -13.6 | -11.2 | -8.9 |
| SC21 | 100.1 | 105.4 | 111.0 | -8.3 | -1.6 | 4.9 | -8.3 | -1.6 | 4.9 |
| SC22 | 100.1 | 105.4 | 111.0 | 2.2 | 8.0 | 13.7 | 2.1 | 7.9 | 13.7 |
| SC23 | 100.1 | 105.4 | 111.0 | 3.0 | 9.0 | 14.9 | 3.0 | 8.9 | 14.8 |
| SC24 | 129.8 | 131.5 | 133.4 | -3.9 | -0.7 | 3.5 | -4.5 | -1.6 | 1.7 |
| SC25 | 129.8 | 131.6 | 133.4 | 7.1 | 9.3 | 11.6 | 2.1 | 4.0 | 5.9 |
| SC26 | 106.0 | 111.4 | 117.1 | -8.5 | -1.6 | 5.0 | -8.5 | -1.7 | 4.9 |
| SC27 | 106.0 | 111.4 | 117.1 | 2.0 | 8.0 | 13.9 | 1.9 | 7.9 | 13.8 |
| SC28 | 106.0 | 111.4 | 117.1 | 2.9 | 9.0 | 15.1 | 2.7 | 8.7 | 14.8 |
| SC29 | 135.8 | 137.6 | 139.5 | -4.0 | -0.7 | 3.5 | -6.0 | -3.6 | -1.2 |
| SC30 | 135.8 | 137.6 | 139.5 | 7.2 | 9.4 | 11.7 | -2.8 | -0.9 | 0.9 |
| Note: Geometry SINR = -10log10(I/C + N/C). where C. I and N equals the carrier. interferer and noise power levels measured over the configured signal bandwidth. |  |  |  |  |  |  |  |  |  |

#### 6.1.1.3 System Level Simulation Evaluation Results

Two companies have provided DL user throughput performance results based on single satellite SLS assumptions, see Section 6.1.1.1.

One source provided results for 8 different study cases for 20% and ~50% target resource utilization (RU), see Table 6.1.1.3-1 Additional assumptions are 10UEs per cell, proportional fair scheduling. LOS probability is according to Table 6.6.1-1 in TR 38.811. The used deployment scenario is rural. The number of HARQ processes is assumed to be RTT/T_slot.

Table 6.1.1.3-1 SLS UE throughput performance in Mbit/s [R1-1911858]

|  |  | GEO, Ka-band |  | LEO-600, Ka-band |  | LEO-600, S-band |  | LEO-1200, S-band |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Study case |  | 1 | 2 | 6 | 7 | 9 | 10 | 14 | 15 |
| RU=20% | 5% | 0.96 | 0.48 | 2.04 | 1.02 | 0.11 | 0.03 | 0.11 | 0.05 |
|  | 50% | 2.7 | 2.05 | 4.08 | 2.78 | 0.31 | 0.15 | 0.3 | 0.19 |
|  | 95% | 4.93 | 3.9 | 6.43 | 4.7 | 0.52 | 0.34 | 0.52 | 0.41 |
| RU~50% | 5% | 5.34 | 4.03 | 6.91 | 4.76 | 0.44 | 0.3 | 0.4 | 0.31 |
|  | 50% | 8.53 | 6.61 | 10.03 | 7.75 | 0.74 | 0.58 | 0.78 | 0.6 |
|  | 95% | 11.99 | 9.99 | 13.7 | 11.22 | 1.09 | 0.88 | 1.09 | 0.9 |

One source provided results for LEO-1200 S-Band with no frequency reuse (Study case 14) with 20% resource utilization, see Table 6.1.1.3-2. Additional assumptions are 10UEs per cell, proportional fair scheduling and 16 parallel HARQ processes. LOS probability is according to Table 6.6.1-1 in TR 38.811. The used deployment scenario is rural.

Table 6.1.1.3-2 SLS UE throughput performance in Mbit/s [R1-1913244]

|  |  | LEO-1200, S-band |
| --- | --- | --- |
| Study case |  | 14 |
| RU=20% | 5% | 0.2 |
|  | 50% | 0.31 |
|  | 95% | 0.44 |

performancessumptions

### 6.1.2	Link level simulations

The following table provides the LLS parameters for DL synchronization performance evaluation

Table 6.1.2-1: LLS parameters for DL synchronization evaluation

============================== Rows Omitted ================================

Note: The SNR range to be evaluated should be based on the link budget analysis for each channel

The following table provides the LLS parameters for PRACH performance evaluation.

Table 6.1.2-2: LLS parameters for PRACH performance evaluation

| ============================== Rows Omitted ================================ |  |  |
| --- | --- | --- |
| Channel Model | Baseline TDL/CDL-D model in [2], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban at corresponding elevation angle for each case |  |
| ============================== Rows Omitted ================================ |  |  |
| Antenna Configuration at the UE | Omni-directional antenna with single linearly polarized antenna element | VSAT with 60 cm equivalent aperture diameter |
| ============================== Rows Omitted ================================ |  |  |
| UE speed | 3 km/h | 0 km/h, 1200 km/h |
| Timing Offset | A uniform distribution in [0 max differential delay] shall be assumed. Note 1: Ideal common delay compensation is assumed. Note 2 : The maximal differential delay values that should be supported for NTN are provided in Table 4.2-2. The max differential delays expected for specific cases can be computed based on the half power beam width and the target elevation angle. |  |
| Phase noise model | S-band phase noise modelling (optional) Ka-band phase noise modelling : phase noise profile according to in TR38.803 |  |
| PRACH design | Each company should provide details on configuration (i.e. format, SCS, N_CS, …). New formats are not precluded. |  |
| Metric | PRACH detection rate, FAR (Based on the preamble pool size is not less than 64), CDF of estimation error for frequency/timing, |  |
| Receiver | Companies are encouraged to report the receiver for PRACH detection. |  |

Table 6.1.2-3 PRACH study cases

|  | Elevation angle | Differential delay | UL Frequency offset (Both S- and Ka-band) (with compensation of common Doppler) | Beam Set at satellite |
| --- | --- | --- | --- | --- |
| Case 1 | 90 degree for LEO | Small | Large | Set-2 |
| Case 2 | 45 degree for LEO | Medium | Medium | Set-2 |
| Case 3 | 10 degree for GEO and 30 degree for LEO | Large | Small | Set-2 |
| Case 4 | With both open loop timing and frequency compensation | Small | Small | Set-2 |
| Note 1: As the baseline, the number of UEs that simultaneously access the network in a single random access occasion (RO) is 2. The two UEs may have different timing offsets/Doppler, which are randomly picked within the [0 Max_differential_delay]/[-max_UL_frequency_offset max_UL_frequency_offset] per case; Note 2: Fixed power offset between UEs is 3dB. Note 3: The SINR of the stronger UE for simulation is based on the SNR from link budget (with bandwidth for UL = 1MHz for VSAT in Ka, and Handheld for S) with additional offset of [-6-10 log10(Bandwidth [MHz])] dB) per case where the -6 dB degradation is introduced as additional margin. |  |  |  |  |

The following table provides the LLS parameters for data transmission performance evaluation.

Table 6.1.2-4: LLS parameters for data transmission performance evaluation

============================== Rows Omitted ================================

HPA non-linearity modelling is not considered as part of the baseline for link level simulations.

PAPR optimizations for downlink channels are not necessary to be specified for NTN at least for Rel-17.

### 6.1.3	Link Budget Analysis

#### 6.1.3.1		Link Budget Calculation

Carrier-to-noise-and-interference ratio (CNIR) of transmission link between satellite and UE can be derived by carrier-to-noise ratio (CNR) and carrier-to-interference ratio (CIR) as follows

|  | CNIR dB=-10log1010-0.1CNR [dB]+10-0.1CIR [dB] | (6.1.3.1-1) |
| --- | --- | --- |

The formula for CNR calculation is

| CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ] | (6.1.3.1-2) |
| --- | --- |

where EIRP is effective isotropic radiated power (EIRP), GT is antenna-gain-to-noise-temperature, k is Boltzmann constant and equals to -228.6 dBW/K/Hz,  PLFS is free space path loss, PLA is atmospheric path loss due to gases and rain fades, PLSM is shadowing margin, PLSL[dB]  is scintillation loss, PLAD is additional loss, for example degradation due to feeder links in case of non-regenerative systems, and B is channel bandwidth.

Antenna-gain-to-noise-temperature GT can be derived by [2]

|  | GTdB=GR dBi-Nf [dB]-10log10T0 [K]+Ta K-T0 [K]10-0.1Nf [dB] | (6.1.3.1-3) |
| --- | --- | --- |

where GR is receive antenna gain,  Nf is noise figure, T0 is ambient temperature and Ta is antenna temperature. Receive antenna gain GR can be obtained by

|  | GR dBi=GR,edBi+10log10NR,a-Lp[dB], &array antenna10log10η∙π2∙D[m]2λ[m]2, &parabolic antenna | (6.1.3.1-4) |
| --- | --- | --- |

where GR,e is receive antenna element gain, NR,a is the number of receive antenna elements, Lp is polarization loss, η is the antenna aperture efficiency (a dimensionless parameter with typical values for parabolic antennas from 0.55 to 0.70), D is the equivalent antenna diameter, and λ is the wavelength.

EIRP can be calculated by

|  | EIRP dBW=PT dBW-LC dB+GT [dBi] | (6.1.3.1-5) |
| --- | --- | --- |

where PT is antenna transmit power, LC is cable loss, and GT is transmit antenna gain and can be derived by

|  | GT dBi=GT,e dBi+10log10NT,a, &array antenna10log10η∙π2∙D[m]2λ[m]2, &parabolic antenna | (6.1.3.1-6) |
| --- | --- | --- |

where GT,e is transmit antenna element gain and NT,a is the number of transmit antenna elements.

#### 6.1.3.2		Parameters

The parameter configuration for link budget analysis is listed in Table 6.1.3.2-1.

Table 6.1.3.2-1 Parameter configuration for link budget analysis

| Parameters | Notes |
| --- | --- |
| Target elevation angle | 30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2) |
| ============================== Rows Omitted ================================ |  |
| Average CIR within a satellite beam based on logarithmic mean | Based on single satellite system-level calibration methodology, statistics for average CIR are only collected for the UEs located in the central beam of the 19-beamlayout. The central beam boresight direction is computed based on the target elevation angle assumption. When the generated beam has a partial or full coverage outside the earth, it is discarded. For DL calibration, - CIR is computed by averaging CIR over UEs randomly distributed over the reference beam (UE distribution assumption of Table 6.1.1-5 in TR 38.821).(See Figure 6.1.3.2-1 for UE bandwidth allocation, and Figure 6.1.1.1-1 and Figure 6.1.1.1-2 for beam deployment). For UL calibration, - For Handheld device, the channel bandwidth is 360 kHz. - For VSAT, the channel bandwidth equals the system bandwidth allocated to each beam divided by 10. - The devices in one beam are allocated on adjacent frequency resources. The same resource allocation is assumed for all the beams. - CIR is computed by averaging over 10 simultaneously transmitting UEs randomly distributed over the reference beam (UE distribution assumption of Table 6.1.1-5 in TR 38.821). (See Figure 6.1.3.2-2 for UE bandwidth allocation, and Figure 6.1.1.1-1 and Figure 6.1.1.1-2 for beam deployment) - The averaging should be performed over multiple realizations. |
| ============================== Rows Omitted ================================ |  |
| Polarization loss | The considerations of Section 6.1.1 on Polarization loss apply. |
| ============================== Rows Omitted ================================ |  |

Figure 6.1.3.2-1 :Illustration of UE bandwidth allocation in DL calibration

Figure 6.1.3.2-2 : Illustration of UE bandwidth allocation in UL calibration

PAPR optimizations for downlink channels are not necessary to be specified for NTN at least for Rel-17.

#### 6.1.3.3		Link Budget Results

Table 6.1.3.3-1 Link budgets results

| Case | Transmission mode | Frequency [GHz] | TX: EIRP [dBm] | RX: G/T [dB/T] | Bandwidth [MHz] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC1 | DL | 20.0 | 96.0 | 15.9 | 400.0 | 210.6 | 1.2 | 0.0 | 1.1 | 0.0 | 0.0 | 11.6 |
|  | UL | 30.0 | 76.2 | 28.0 | 400.0 | 214.1 | 1.1 | 0.0 | 1.1 | 0.0 | 0.0 | 0.5 |
| SC2 | DL | 20.0 | 91.2 | 15.9 | 133.3 | 210.6 | 1.2 | 0.0 | 1.1 | 0.0 | 0.0 | 11.6 |
|  | UL | 30.0 | 76.2 | 28.0 | 133.3 | 214.1 | 1.1 | 0.0 | 1.1 | 0.0 | 0.0 | 5.2 |
| SC3 | DL | 20.0 | 93.0 | 15.9 | 200.0 | 210.6 | 1.2 | 0.0 | 1.1 | 0.0 | 0.0 | 11.6 |
|  | UL | 30.0 | 76.2 | 28.0 | 200.0 | 214.1 | 1.1 | 0.0 | 1.1 | 0.0 | 0.0 | 3.5 |
| SC4 | DL | 2.0 | 103.8 | -31.6 | 30.0 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | 0.0 |
|  | UL | 2.0 | 23.0 | 19.0 | 0.4 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | -10.9 |
| SC5 | DL | 2.0 | 99.0 | -31.6 | 10.0 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | 0.0 |
|  | UL | 2.0 | 23.0 | 19.0 | 0.4 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | -10.9 |
| SC6 | DL | 20.0 | 60.0 | 15.9 | 400.0 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 8.5 |
|  | UL | 30.0 | 76.2 | 13.0 | 400.0 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 18.4 |
| SC7 | DL | 20.0 | 55.2 | 15.9 | 133.3 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 8.5 |
|  | UL | 30.0 | 76.2 | 13.0 | 133.3 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 23.1 |
| SC8 | DL | 20.0 | 57.0 | 15.9 | 200.0 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 8.5 |
|  | UL | 30.0 | 76.2 | 13.0 | 200.0 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 21.4 |
| SC9 | DL | 2.0 | 78.8 | -31.6 | 30.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 6.6 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 2.8 |
| SC10 | DL | 2.0 | 74.0 | -31.6 | 10.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 6.6 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 2.8 |
| SC11 | DL | 20.0 | 66.0 | 15.9 | 400.0 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 9.1 |
|  | UL | 30.0 | 76.2 | 13.0 | 400.0 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 13.0 |
| SC12 | DL | 20.0 | 61.2 | 15.9 | 133.3 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 9.1 |
|  | UL | 30.0 | 76.2 | 13.0 | 133.3 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 17.8 |
| SC13 | DL | 20.0 | 63.0 | 15.9 | 200.0 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 9.1 |
|  | UL | 30.0 | 76.2 | 13.0 | 200.0 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 16.0 |
| SC14 | DL | 2.0 | 84.8 | -31.6 | 30.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 7.2 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -2.6 |
| SC15 | DL | 2.0 | 80.0 | -31.6 | 10.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 7.2 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -2.6 |
| SC16 | DL | 20.0 | 88.0 | 15.9 | 400.0 | 210.4 | 0.8 | 0.0 | 0.5 | 0.0 | 0.0 | 4.8 |
|  | UL | 30.0 | 76.2 | 20.0 | 400.0 | 213.9 | 0.7 | 0.0 | 0.5 | 0.0 | 0.0 | -6.3 |
| SC17 | DL | 20.0 | 83.2 | 15.9 | 133.3 | 210.4 | 0.8 | 0.0 | 0.5 | 0.0 | 0.0 | 4.8 |
|  | UL | 30.0 | 76.2 | 20.0 | 133.3 | 213.9 | 0.7 | 0.0 | 0.5 | 0.0 | 0.0 | -1.6 |
| SC18 | DL | 20.0 | 85.0 | 15.9 | 200.0 | 210.4 | 0.8 | 0.0 | 0.5 | 0.0 | 0.0 | 4.8 |
|  | UL | 30.0 | 76.2 | 20.0 | 200.0 | 213.9 | 0.7 | 0.0 | 0.5 | 0.0 | 0.0 | -3.3 |
| SC19 | DL | 2.0 | 98.3 | -31.6 | 30.0 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -5.2 |
|  | UL | 2.0 | 23.0 | 14.0 | 0.4 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -15.7 |
| SC20 | DL | 2.0 | 93.5 | -31.6 | 10.0 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -5.2 |
|  | UL | 2.0 | 23.0 | 14.0 | 0.4 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -15.7 |
| SC21 | DL | 20.0 | 52.0 | 15.9 | 400.0 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 0.5 |
|  | UL | 30.0 | 76.2 | 5.0 | 400.0 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 10.4 |
| SC22 | DL | 20.0 | 47.2 | 15.9 | 133.3 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 0.5 |
|  | UL | 30.0 | 76.2 | 5.0 | 133.3 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 15.1 |
| SC23 | DL | 20.0 | 49.0 | 15.9 | 200.0 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 0.5 |
|  | UL | 30.0 | 76.2 | 5.0 | 200.0 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 13.4 |
| SC24 | DL | 2.0 | 72.8 | -31.6 | 30.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 0.6 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -3.2 |
| SC25 | DL | 2.0 | 68.0 | -31.6 | 10.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 0.6 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -3.2 |
| SC26 | DL | 20.0 | 58.0 | 15.9 | 400.0 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 1.1 |
|  | UL | 30.0 | 76.2 | 5.0 | 400.0 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 5.0 |
| SC27 | DL | 20.0 | 53.2 | 15.9 | 133.3 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 1.1 |
|  | UL | 30.0 | 76.2 | 5.0 | 133.3 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 9.8 |
| SC28 | DL | 20.0 | 55.0 | 15.9 | 200.0 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 1.1 |
|  | UL | 30.0 | 76.2 | 5.0 | 200.0 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 8.0 |
| SC29 | DL | 2.0 | 78.8 | -31.6 | 30.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 1.2 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -8.6 |
| SC30 | DL | 2.0 | 74.0 | -31.6 | 10.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 1.2 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -8.6 |
| Note : The link budget calculations including CIR and CINR results contributed by the companies are available in [R1 1913351]. |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.3		Uplink timing advance/RACH procedure

============================== Text Omitted ================================

| . . . ... .. |  |  |  |  |
| --- | --- | --- | --- | --- |

============================== Text Omitted ================================

========================= End of TP for TR 38.821 =============================

# Text proposal motivations

## Impairments due to satellite payload and satellite movement

It is proposed to update Table 6.1.1-8. The proposed updates are the followings :

- Remove the FFS statement for phase noise modeling and replace it by the statements used in the tables of Section 6.1.2.

- Add a note about on-board oscillator long-term drift to clarify that it has been provided for information but it has not been considered in RAN1 analysis for the time being.

- Provide Max Doppler shift values if pre/post compensation mechanism is assumed and max beam foot print size is considered.

## Depolarization loss

During last meeting. the following agreements have been achieved in order to clarify in which configuration depolarization loss should be considered :

Agreement:

Calibration results and link budgets should be computed assuming no depolarization loss.

- For handheld use cases. on the downlink. it is assumed that a combination of the two Rx branches allows to prevent depolarization loss.

Agreement:

For handheld use cases. on the uplink.

- A 3dB depolarization loss should be taken into account assuming polarization reuse is applied and satellite reception implements circular polarization (e.g.. for frequency reuse 4 case)

- A 0dB depolarization loss can be assumed when satellite reception implements dual polarization per beam (e.g.. for the frequency reuse 1 and 3 cases)

It is proposed to capture these agreements in the TP.

## Wrap around mechanism

A wrap around mechanism has been adopted as baseline for system level calibration. It is proposed to explain and illustrate this mechanism in the TP.

## Voronoï cell definition

During one of the last meetings. the following agreement has been captured in Table 6.1.1-5 :

Agreement : For uplink. the cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers.

One part of the text proposal aims to explain the concept of Voronoi cell.

## LLS parameters for PRACH performance evaluation

In the current version of TR38.821 (v0.8.0). there are two sections where LLS parameters are defined for PRACH performance evaluation :

- Section 6.1.2 – Table 6.1.2-2

- Section 6.3 – Table 6.3.1-1

In both tables. we can find assumptions about metrics. channel model. frequency offsets. It has been reported that this formatting can lead to misalignment in the simulation assumptions. Therefore. it proposed propose re-arrange the content of the tables and to put them in the same section .

## Link budget analysis and CIR computations

During last meeting. the following agreements have been made concerning CIR computations :

Agreement:

The computation of the CIR as per the previous agreement is clarified as follows.

- Compute CIR for uplink by averaging over 10 simultaneously transmitting UEs (applies to both handheld and VSAT cases) randomly distributed over the reference beam (UE coverage assumption of table 6.1.1-5 in TR 38.821).

  - The averaging should be performed over multiple realizations.

- Compute CIR for downlink averaging over UEs randomly distributed over the reference beam.

Agreement:

For the CIR/CNR/CNIR value computation for link budgets. the minimum elevation angle of the reference beam boresight should be set to 12.5° for the GEO Set-1 case. to ensure that the reference beam footprint is completely on Earth.

Agreement:

For the CIR/CNR/CNIR value computation for link budgets. the minimum elevation angle of the reference beam boresight should be set to 20° for the GEO Set-2 case. to ensure that the reference beam footprint is completely on Earth.

It is proposed to update the link budget assumptions to capture these agreements.

# Reference

| [1] | “TR 38.821 V0.9.0 Solutions for NR to support non-terrestrial networks (NTN)” |
| --- | --- |