3GPP TSG RAN WG1#99                                      R1-1913459

Reno, USA, November 18th – 22th, 2019

Source:	vivo

Title:	Summary#2 of offline discussion on ULFPTx

Agenda Item:	7.2.8.4

Document for:	Discussion and Decision

No consensus on the proposal below

For 4TX Mode-2 partial coherent UEs, for the codebook-based UL PUSCH of a UE configured with a 2-port SRS resource and a 4-port SRS resource for full Tx power transmission

- If UE is configured with nonCoherent codebook for 4TX, the DCI indicates a TPMI from the nonCoherent subset if the SRI indicates a 2-port SRS resource

- If UE is configured with partialAndNonCoherent codebook for 4TX, the DCI can indicate a 2Tx TPMI from the fullyAndPartialAndNonCoherent or nonCoherent 2Tx subset if the SRI indicates a 2-port SRS resource

  - Support of fullyAndPartialAndNonCoherent or nonCoherent subset depends on UE capability

Possible Agreement

For 4 ports, max number of bits to indicate TPMI(s) which can deliver UL full power:

  - Alt 1: bitmap based scheme, max 10 or 11 bits

Support: Futurewei, Huawei/HiSi, Apple, ZTE/Sanechips. OPPO, DOCOMO, LGE

Concern: Samsung, Intel, CATT, IDC, QC, CMCC Nokia/NSB, Ericsson (9)

  - Alt 2:

    - Non Coherent 2 bits

    - Partial coherent 3 bits

|  | TPMI Groups |
| --- | --- |
| G0 | Rank 1 {TPMI=0} |
| G1 | Rank 1 {TPMI=0,1} |
| G2 | Rank 1 {TPMI=0,1,2} |
| G3 | Rank 1 {TPMI=0,1,2,3} |
| G4 | Rank 1 {TPMI=4-7} |
| G5 | Rank 1 {TPMI=8-11} |
| G6 | Rank 2 {TPMI=0-5} |

| 4Tx, nonCoherent | 4Tx, partial coherent |
| --- | --- |
| G0 | G0 |
| G1 | G1 |
| G2 | G2 |
| G3 | G3 |
|  | {G4,G6} |
|  | {G5,G6} |

Definition of G0~G6 can be found in the table below.

| TPMI Groups |  |
| --- | --- |
| G0 | 121000 |
| G1 | {121000,120100} {1210010000} |
| G2 | {121000,120100,120010} : {1210000100,1210010000, 1200100100} : 12100010001000 Note: Rank2, 3 full power TPMIs are implicitly derived from rank1 TPMIs |
| G3 | {121000,120100,120010,120001} |
| G4 | {121010,1210-10,1210j0,1210-j0} |
| G5 | {120101,12010-1,12010j,12010-j} |
| G6 | {1210000100,1200100001,1210010000,1210000001, 1200100100,1200001001} |