3GPP TSG RAN WG1 #99									 R1-1913503

Reno, USA, November 18th – 22nd, 2019

Agenda Item:	7.2.3.1

Source:	AT&T

Title:	Summary #3 of 7.2.3.1 – Mechanisms for resource multiplexing among backhaul and access links

Document for:	Discussion/Approval

# Introduction

This contribution provides a summary of section 7.2.3.1 on potential enhancements to support multiplexing of access and backhaul links and provides proposals synthesized from the views expressed in contributions listed in the Appendix.

# Resource multiplexing among backhaul and access links

Observations and proposals in this section are primarily related to the following objectives from the IAB WID:

- Specification of mechanisms for resource multiplexing among backhaul and access links. This includes:

  - Specification of semi-static configuration for IAB-node/IAB-donor DU resources in case of TDM operation subject to half-duplex constraint. This shall be forward compatible to allow the support of half-duplex scenarios with FDM and SDM resource sharing among backhaul and access links.

  - Specification of time resource types for the DU’s child links: DL hard, DL soft, UL hard, UL soft, Flexible hard, Flexible soft, Not Available

  - Specification of dynamic indication (L1 signalling) of the availability of soft resources for a child IAB-node DU

  - Specification of required transmission/reception rules for IAB-nodes and associated behaviours regarding time resource utilization as discussed in TR 38.874 clause 7.3.3.

## IAB-node MT resource configuration

Background from RAN1#98bis:

Agreements:

Dynamic indication of UL-Flexible-DL (intended to dynamically update the flexible symbols) is supported for IAB-node MTs

- The dynamic indication of slot formats of UL-Flexible-DL should use DCI format 2_0 and reserved entries in Table 11.1.1-1 in 38.213.

Background from RAN1#99:

Agreements:

For IAB-MTs, the following new slot formats are defined and indicated by SFI in DCI Format 2_0:

| U | U | U | U | U | U | U | U | U | U | U | U | U | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U | U | U | U | U | U | U | U | U | U | U | U | F | F |
| U | U | U | U | U | U | U | U | U | U | U | F | F | F |
| U | U | U | U | U | U | U | U | U | U | F | F | F | F |
| U | U | U | U | U | U | U | U | U | F | F | F | F | F |
| U | U | U | U | U | U | U | U | F | F | F | F | F | F |
| U | U | U | U | U | U | U | F | F | F | F | F | F | F |
| U | U | U | U | U | U | F | F | F | F | F | F | F | F |
| U | U | U | U | U | F | F | F | F | F | F | F | F | F |
| U | U | U | U | F | F | F | F | F | F | F | F | F | F |
| U | U | U | F | F | F | F | F | F | F | F | F | F | F |
| U | U | F | F | F | F | F | F | F | F | F | F | F | F |
| U | F | F | F | F | F | F | F | F | F | F | F | F | F |
| U | F | F | F | F | F | F | F | F | F | F | F | F | D |
| U | U | F | F | F | F | F | F | F | F | F | F | F | D |
| U | U | U | F | F | F | F | F | F | F | F | F | F | D |
| U | F | F | F | F | F | F | F | F | F | F | F | D | D |
| U | U | F | F | F | F | F | F | F | F | F | F | D | D |
| U | U | U | F | F | F | F | F | F | F | F | F | D | D |
| U | F | F | F | F | F | F | F | F | F | F | D | D | D |
| U | U | F | F | F | F | F | F | F | F | F | D | D | D |
| U | U | U | F | F | F | F | F | F | F | F | D | D | D |
| U | U | U | U | U | U | U | U | U | U | U | U | F | D |
| U | U | U | U | U | U | U | U | U | U | U | F | F | D |
| U | U | U | U | U | U | U | U | U | U | F | F | F | D |
| U | U | U | U | U | U | U | U | U | U | U | F | D | D |
| U | U | U | U | U | U | U | U | U | U | F | F | D | D |
| U | U | U | U | U | U | U | U | U | F | F | F | D | D |
| U | F | D | D | D | D | D | D | D | D | D | D | D | D |
| U | U | F | D | D | D | D | D | D | D | D | D | D | D |
| U | U | U | F | D | D | D | D | D | D | D | D | D | D |
| U | F | F | D | D | D | D | D | D | D | D | D | D | D |
| U | U | F | F | D | D | D | D | D | D | D | D | D | D |
| U | U | U | F | F | D | D | D | D | D | D | D | D | D |
| U | F | F | F | D | D | D | D | D | D | D | D | D | D |
| U | U | F | F | F | D | D | D | D | D | D | D | D | D |
| U | U | U | F | F | F | D | D | D | D | D | D | D | D |
| U | U | U | U | U | U | U | U | U | F | F | F | F | D |
| U | U | U | U | U | U | F | F | F | F | F | F | D | D |
| U | U | U | U | U | U | F | F | D | D | D | D | D | D |
| U | U | U | U | U | U | U | D | D | D | D | D | D | D |

Agreements:

Update the parameter TDD-UL-DL-Config-IAB-MT to TDD-UL-DL-ConfigDedicated-IAB-MT

If the IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter TDD-UL-DL-ConfigDedicated-IAB-MT overrides all symbols (with a limitation that effectively only flexible symbols can be overwritten in Rel-16) per slot over the number of slots as provided by TDD-UL-DL-ConfigurationCommon.

The TDD-UL-DL-ConfigDedicated-IAB-MT provides

-	a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT

-	for each slot configuration from the set of slot configurations

-	a slot index for a slot provided by slotIndex

-	a set of symbols for a slot by symbols where

-	if symbols = allDownlink, all symbols in the slot are downlink

-	if symbols = allUplink, all symbols in the slot are uplink

-	if symbols = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible

-   if symbols = explicit-[IAB-MT], nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible.

For each slot having a corresponding index provided by slotIndex, the IAB-MT applies a format provided by a corresponding symbols.

Company proposals:

| Huawei | Proposal 3: If an IAB node MT is additionally provided with tdd-UL-DL-ConfigurationDedicated, the parameter tdd-UL-DL-ConfigurationDedicated overrides all symbols per slot provided by tdd-UL-DL-ConfigurationCommon. Proposal 4: An IAB node MT can transmit or receive cell specific signals/channels in the set of symbols regardless of the slot format provided by the enhanced tdd-UL-DL-ConfigurationDedicated. The list of cell-specific signals/channels includes: - SS/PBCH block - CORESET for Type0-PDCCH CSS set - PRACH |
| --- | --- |
| Vivo | Proposal 2: New DCI Format is used for explicit indication of DU-IA. Further down-select additional slot format for DU resource from Table 1 in R1-1912019 |
| LG | Proposal 1: For IAB MT, support slot formats in Table A-1 in the Annex which change DL into UL and UL into DL from the existing slot formats. Proposal 2: Slot formats with following sequence order should be considered for SDM/FDM among DU and MT. - Flexible-UL-Flexible-DL-Flexible - Flexible-DL-Flexible-UL-Flexible |
| Samsung | Proposal 1: Slot formats in Table 1 is adopted for slot formats of UL-Flexible-DL in IAB. Table 1. Slot formats of the UL-Flexible-DL 56; U; F; F; F; F; F; F; F; F; F; F; F; F; D. 57; U; U; F; F; F; F; F; F; F; F; F; F; F; D. 58; U; U; U; F; F; F; F; F; F; F; F; F; F; D. 59; U; F; F; F; F; F; F; F; F; F; F; F; D; D. 60; U; U; F; F; F; F; F; F; F; F; F; F; D; D. 61; U; U; U; F; F; F; F; F; F; F; F; F; D; D. 62; U; F; F; F; F; F; F; F; F; F; F; D; D; D. 63; U; U; F; F; F; F; F; F; F; F; F; D; D; D. 64; U; U; U; F; F; F; F; F; F; F; F; D; D; D. 65; U; U; U; U; U; U; U; U; U; U; U; U; F; D. 66; U; U; U; U; U; U; U; U; U; U; U; F; F; D. 67; U; U; U; U; U; U; U; U; U; U; F; F; F; D. 68; U; U; U; U; U; U; U; U; U; U; U; F; D; D. 69; U; U; U; U; U; U; U; U; U; U; F; F; D; D. 70; U; U; U; U; U; U; U; U; U; F; F; F; D; D. 71; U; F; D; D; D; D; D; D; D; D; D; D; D; D. 72; U; U; F; D; D; D; D; D; D; D; D; D; D; D. 73; U; U; U; F; D; D; D; D; D; D; D; D; D; D. 74; U; F; F; D; D; D; D; D; D; D; D; D; D; D. 75; U; U; F; F; D; D; D; D; D; D; D; D; D; D. 76; U; U; U; F; F; D; D; D; D; D; D; D; D; D. 77; U; F; F; F; D; D; D; D; D; D; D; D; D; D. 78; U; U; F; F; F; D; D; D; D; D; D; D; D; D. 79; U; U; U; F; F; F; D; D; D; D; D; D; D; D. 80; U; U; U; U; U; U; U; U; U; F; F; F; F; D. 81; U; U; U; U; U; U; F; F; F; F; F; F; D; D. 82; U; U; U; U; U; U; F; F; D; D; D; D; D; D. |
| CMCC | Proposal 1: The flexible symbols to cover round trip time between downlink and uplink symbols should be considered when TDD-UL-DL-ConfigurationCommon configures the DL-Flexible-UL slot format and TDD-UL-DL-Config-IAB-MT overrides the flexible resources by the UL-Flexible-DL slot format. Proposal 2: The flexible symbols to cover round trip time between downlink and uplink symbols should be considered for the design of SFI table for the new UL-Flexible-DL format. Inserting flexible symbols before and after the UL-Flexible-DL symbols should be considered in the SFI table design. |
| AT&T | Proposal 1: For IAB-MTs, in addition to the existing slot configuration period definition in 38.213, a new slot configuration period which supports UL-F-D patterns should be defined as the following: A slot configuration period of msec includes slots with SCS configuration . From the slots, a first slots include only uplink symbols and a last slots include only downlink symbols. The symbols after the first slots are uplink symbols. The symbols before the last slots are downlink symbols. The remaining are flexible symbols. Proposal 2: The upper layer parameter TDD-UL-DL-Config-IAB-MT should indicate whether the legacy slot configuration period definition or a new slot configuration period supporting UL-F-D patterns should be assumed by the IAB-MT. Proposal 3: The following new slot formats should be defined for use by IAB-nodes (DUs and MTs) in the semi-static F1-AP and RRC configurations, and as part the SFI in DCI Format 2_0: F; F; F; F; F; F; F; U; U; U; U; U; U; U. U; U; U; U; U; U; U; F; F; F; F; F; F; F. U; U; U; U; U; U; U; D; D; D; D; D; D; D. U; U; U; U; U; U; U; F; U; U; U; U; U; U. U; U; U; U; U; U; U; F; F; U; U; U; U; U. U; U; U; U; U; U; U; F; F; F; U; U; U; U. U; U; U; U; U; U; U; F; F; F; F; U; U; U. U; U; U; U; U; U; U; F; F; F; F; F; U; U. U; U; U; U; U; U; U; F; F; F; F; F; F; U. U; U; U; U; U; U; U; F; U; U; U; U; U; U. U; U; U; U; U; U; U; F; F; U; U; U; U; U. U; U; U; U; U; U; U; F; U; U; U; U; U; U. U; U; U; U; U; U; U; D; F; U; U; U; U; U. U; U; U; U; U; U; U; D; D; F; U; U; U; U. U; U; U; U; U; U; U; D; D; D; F; U; U; U. U; U; U; U; U; U; U; D; F; F; U; U; U; U. U; U; U; U; U; U; U; D; D; F; F; U; U; U. U; U; U; U; U; U; U; D; D; D; F; F; U; U. U; U; U; U; U; U; U; D; F; F; F; U; U; U. U; U; U; U; U; U; U; D; D; F; F; F; U; U. U; U; U; U; U; U; U; D; D; D; F; F; F; U. |
| Nokia | Proposal 10: Use DCI format 2_0 and reserved entries of Table 11.1.1-1 in 38.213 to indicate SFI with sequence order UL-Flexible-DL - The exact slot formats with sequence order UL-Flexible-DL are FFS. - Slot formats 246-254 are used for conveying the AI Availability Combination Table in the case of joint indication of SFI and AI |

## IAB-node DU resource configuration

Background from RAN#97:

- The resources are configured on a per DU (cell) basis

  - FFS: indication of additional supplemental per-link resource configurations of child DUs

Background from RAN1#98bis:

Agreements:

The donor CU and the parent node can be made aware of the multiplexing capability between MT and DU (TDM required, TDM not required) of an IAB node to for any {MT CC, DU cell} pair.

- Signaling details up to RAN2/RAN3.

- Background from RAN1#99:

Agreements:

- The indication of the multiplexing capability for the case of no-TDM between IAB MT and IAB DU is additionally provided with respect to each transmission-direction combination (per MT CC/DU cell pair):

- MT-TX/DU-TX

- MT-TX/DU-RX

- MT-RX/DU-TX

- MT-RX/DU-RX

- Note: This agreement does not require any additional specification impact in RAN1 in Rel-16, i.e. in Rel-16 the behaviour of the IAB node is only defined for TDM cases. The behaviour for no-TDM is left to IAB-node/network implementation in Rel-16.

Agreements:

A parent IAB node/donor can be provided with the full D/U/F + H/S/NA resource configuration of each child IAB-DU.

- Note: the new parameter for the signalling is already endorsed; the above agreements provide additional details for the signalling

- No additional signaling optimization is required from a RAN1 perspective

Agreements:

A parent IAB node/donor can be provided with cell-specific signals/channels configurations (as listed in the previous agreements copied below) of each child IAB-DU. How/whether to use the information to handle any potential conflict at the parent IAB node/donor is left to network implementation

  - NOTE: This overturns the last sub-bullet of the following RAN1#97 agreement:

Agreements:

If a DU NA or Soft resource is configured with cell-specific signals/channels, the resource is treated as if it were a Hard DU resource (Alt. 2 from RAN1#96bis).

  - The list of cell-specific signals/channels includes:

- resources for SSB transmission at DU, including both CD-SSB and non-CD-SSB;

- configured RACH occasions for receiving at the DU

- periodic CSI-RS transmission at the DU

- scheduled resource for receving SR at DU

- The parent does not need to be aware of the cell-specific signals/channel configurations of the child DU

Working assumption:

For paired spectrum, the H/S/NA DU resource configuration framework is extended with the following:

- Two separate H/S/NA per-cell DU semi-static configurations are provided (with slot level resolution) for downlink and uplink

- FFS: whether the definition of half-duplex operation has to be modified for IAB nodes operating in paired spectrum

Company proposals:

| Huawei | Proposal 1: For TDM operation, an IAB node MT can determine a set of NULL resources - The IAB node MT does not monitor PDCCH candidates in search space sets for DCI scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI(s) in the NULL resources - The IAB node MT does not expect to receive PDSCH or CSI-RS in the NULL resources - The IAB node MT does not expect to transmit PUSCH, PUCCH or SRS in the NULL resources Proposal 2: The parent node need to be aware of the cell-specific signals/channels configurations of the IAB node DU, i.e. CU can provide following parameters of IAB node DU to its parent node: STC, MIB, and RMSI. Proposal 1: Donor CU needs to provide two sets of resource configurations to each IAB DU cell: a basic resource configuration for access UEs and one additional resource configuration for child IAB node MTs, and the IAB node DU cell should maintain two sets of DU resource configurations simultaneously. Proposal 2: H/S/NA attributes for the per-cell DU resource configuration is based on the TDD-UL-DL configuration for access UEs. |
| --- | --- |
| Ericsson | - Proposal 1: - The indication of the multiplexing capability between IAB MT and IAB DU is with respect to each transmission-direction combination, i.e., TDM/no-TDM for MT-TX/DU-TX, MT-TX/DU-RX, MT-RX/DU-TX, and MT-RX/DU-RX. |
| ZTE | Proposal 2: The “resource treated as if it were a Hard DU resource” in the RAN1 #97 agreement has the minimum granularity of “per resource type (D/U/F) in single slot”. Proposal 3: RAN1 clarifies the meaning of “resource treated as if it were a Hard DU resource” in the RAN1 #97 agreement, by e.g. one of following two interpretations. • Interpretation #1: the attribute of the corresponding resource is still NA or INA, but the resource is used in the same way as Hard. • Interpretation #2: the attribute of the corresponding resource is changed to Hard. |
| CMCC | Proposal 4: The parent node should have the knowledge of resources that treated as hard at child IAB. Proposal 5: Supporting only per cell basis DU resource configurations works for Rel-16. Proposal 6: Additional configuration information indicating NA resources on a per-child IAB node basis can optionally be provided to an IAB DU. Proposal 7: No default H/S/NA resource type is defined for IAB. |
| Nokia | Proposal 1: Support per-link resource configurations of child DUs in the case of dual connectivity Proposal 2: Extend the current IAB resource configuration rules to support different dual connectivity scenarios in Rel-17. Proposal 3: Default resource types for IAB node are the following: - MT part: Flexible - DU part: Flexible Soft |
| NTT DOCOMO | Proposal 1: Full DU resource configuration information (D/U/F/H/S/NA) of its child IAB nodes is always necessary for an IAB node. Proposal 2: Parent IAB-node has the ability to be aware of the hard DU resource treated by its child IAB-node. Proposal 3: The hard DU resource treated by its child IAB-node is separately indicated from the semi-static DU resource configuration. |
| Qualcomm | Proposal 1: An IAB node, given its DU per-cell resource configuration, determines a compatible Rel-15 cell specific TDD configuration, compatible dedicated TDD configurations for UEs and children IAB MTs, and required dynamic SFI procedures to realize such DU per-cell resource configuration. Proposal 2: One H or S or NA attribute value is provided for all symbols of the same type (D, or U, or F) within a slot, for all slots of the DU resource configuration. Proposal 4: The necessary information at an IAB node about its child DU resource configuration is the H/S/NA portion. The D/U/F portion of the configuration is not necessarily needed in all cases and hence some signalling optimization is possible. Proposal 5: The H/S/NA DU resource configuration framework is extended to paired spectrum with the following: - Two separate H/S/NA per-cell DU semi-static configurations are provided (with slot level resolution) for downlink and uplink - DCI format 4_0 is extended to provide the resource availability indication for both downlink and uplink (similarly to how DCI format 2_0 is used in paired spectrum). Proposal 6: Additional configuration information indicating NA resources on a per-child IAB node basis is provided to an IAB DU (IAB-node DU and IAB donor DU) - A per-child IAB node NA configuration overrides any H or S configuration in the IAB-node DU configuration for the communication to the corresponding child IAB node. - The granularity of the per-child IAB node NA configuration is the same as the granularity for the H/S/NA IAB-node DU configuration. - A per-child IAB node NA configuration makes the corresponding resources of the IAB-donor DU not available for the communication to the corresponding child IAB node. |

Offline Proposal 2.2-1:

The IAB-DU-Resource-Configuration-TDD-Config has a periodicity of:

ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}

Offline Proposal 2.2-2:

The IAB-DU-Resource-Configuration-H/S/NA-Config has a periodicity of:

ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, [ms80], [ms160]}

Offline Conclusion 2.2-3:

Default values for IAB-DU-Resource-Configuration-TDD-Config and IAB-DU-Resource-Configuration-H/S/NA-Config (if needed) are left to RAN3 to decide (update the Upper Layer Parameters list with this as a note).

## Guard symbols for MT/DU Tx/Rx transitions

Background from RAN1#98bis:

Agreements:

A parent IAB node can be made aware of the number of symbols Ng the child IAB node would like the parent IAB node not to use at the edge (beginning or end) of a slot when there is a transition between child MT and child DU. Separately or additionally, the child IAB node can be made aware of the number of guard symbols that the parent IAB node will provide.

- Ng can be provided for each of the [8] possible transitions with potential overlap:

| MT to DU | DL Tx | UL Rx |
| --- | --- | --- |
| DL Rx |  |  |
| UL Tx |  |  |
| DU to MT | DL Rx | UL Tx |
| DL Tx |  |  |
| UL Rx |  |  |

- If Ng is not provided it is assumed to be 0

NOTE: this agreement does not introduce any performance requirement on IAB nodes.

Background from RAN1#99:

Agreements:

Desired Guard Symbols and Provided Guard Symbols are provided per cell and use 3 bits for each of the 8 transitions to indicate the number of guard symbols.

- In Rel-16, a range of 0-4 symbols are supported for each transition. Additional entries are reserved for future use

- A new parameter GuardSymbol-SCS is also provided which indicates the reference SCS (FR1: {15kHz, 30kHz, 60kHz}, FR2: {60kHz, 120kHz}) to be used for the guard symbols.

Company proposals:

| Huawei | Proposal 3: The IAB node should report the number of guard symbols for all of the eight switching patterns between MT and DU. Proposal 4: A reference subcarrier spacing is provided together with the guard symbols for transitions between the MT and DU. |
| --- | --- |
| Intel | Proposal 1: There are three options to generate the Ng table: - Option 1: Use a single Ng table for all carrier frequencies and SCSs. - Option 2: Generate two tables for FR1 and FR2, respectively. - Option 3: Generate Four tables for SCS = {15kHz, 30kHz, 60kHz, 120kHz}, respectively. Option 2 is preferred. Proposal 2: Parent DU can be made aware of IAB DU’s dynamic scheduling information for better resource utilization. |
| LG | Proposal 3: The number of symbols Ng can have minus value(s) for reporting/providing the number of additional MT symbols after/before MT and DU transition. Proposal 4: Ng can be provided for 4 transition cases where the transition cases include only one of two transition directions (MT to DU or DU to MT). |
| CMCC | Proposal 3: The bracket around the number of possible transitions should be removed. The agreement is updated as follows: A parent IAB node can be made aware of the number of symbols Ng the child IAB node would like the parent IAB node not to use at the edge (beginning or end) of a slot when there is a transition between child MT and child DU. Separately or additionally, the child IAB node can be made aware of the number of guard symbols that the parent IAB node will provide. - Ng can be provided for each of the 8 possible transitions with potential overlap: MT to DU; DL Tx; UL Rx. DL Rx. UL Tx. DU to MT; DL Rx; UL Tx. DL Tx. UL Rx. - If Ng is not provided it is assumed to be 0 NOTE: this agreement does not introduce any performance requirement on IAB nodes. |
| Nokia | Proposal 4: RAN1 is asked to confirm the range for the number of guard symbols for each possible transition as shown in the table below. MT to DU; DL Tx; UL Rx. DL Rx; [1,2]; [1,4]. UL Tx; [0] **); [0,2]. DU to MT; DL Rx; UL Tx. DL Tx; [0,1]; [0,3]. UL Rx; [0] **); [0,2]. **) Only negative values, i.e. no overlap of slots. Proposal 5: Two-bit information is used to indicate the number of guard symbols per each transition resulting in max 16 information. For possible optimization, the transitions having the only zero could be left out from the signalled information. |

## Indication of soft resource availability

Background from RAN1#98bis:

Agreements:

The minimum granularity of explicit indication of DU-IA for IAB DU is per resource type (D/U/F) in a slot.

Agreements:

The explicit indication of DU-IA is provided using the DCI Format 2_0 framework.

- FFS the following alternatives (to be down-selected):

  - Alt 1:  Reuse existing DCI Format 2_0, select/reinterpret current and, possibly, some reserved entries in Table 11.1.1-1 in 38.213 to indicate DU-IA.

  - Alt 2:  Reuse existing DCI Format 2_0, select reserved entries of Table 11.1.1-1 in 38.213 to indicate DU-IA.

  - Alt 3:  New DCI Format.

Agreements:

A DCI following the DCI Format 2_0 structure is used to indicate DU-IA to an IAB node using a new IA-RNTI different from SFI-RNTI.

This DCI contains one or multiple fields (similar to SFI-index fields in DCI Format 2_0), each field value is used as the index in a RRC configured AI (Availability Indicator) AvailabilityCombination table (similar to the SFI SlotFormatCombination table).

Each entry in the AI AvailabilityCombination table indicates the resource availability for a set of consecutive slots.

Each element of each entry in the AI AvailabilityCombination table indicates the resource availability in a slot.

The resource availability can take 8 values:

| Value | Meaning |
| --- | --- |
| 0 | No resources available |
| 1 | D resources available |
| 2 | U resources available |
| 3 | D and U resources available |
| 4 | F resources available |
| 5 | D and F resources available |
| 6 | U and F resources available |
| 7 | All resources available |

The maximum number of entries in the AI AvailabilityCombination table is 512.

Agreements:

The minimum granularity of explicit indication of DU-IA is per child IAB-DU resource type (D/U/F) in a slot.

- This is an update to the agreement from RAN1#98bis:

Agreements:

The minimum granularity of explicit indication of DU-IA for IAB DU is per resource type (D/U/F) in a slot.

Agreements:

The resource availability indicated by an AI index field value applies to Soft resources configured for the child IAB-DU in a number of slots, where the number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 4_0.

Agreements:

Update the agreement from RAN1#98bis as the following:

The resource availability can take 8 values:

| Value | Meaning |
| --- | --- |
| 0 | D soft resources: no indication of availability U soft resources: no indication of availability F soft resources: no indication of availability |
| 1 | D soft resources: indicated available U soft resources: no indication of availability F soft resources: no indication of availability |
| 2 | D soft resources: no indication of availability U soft resources: indicated available F soft resources: no indication of availability |
| 3 | D soft resources: indicated available U soft resources: indicated available F soft resources: no indication of availability |
| 4 | D soft resources: no indication of availability U soft resources: no indication of availability F soft resources: indicated available |
| 5 | D soft resources: indicated available U soft resources: no indication of availability F soft resources: indicated available |
| 6 | D soft resources: no indication of availability U soft resources: indicated available F soft resources: indicated available |
| 7 | D soft resources: indicated available U soft resources: indicated available F soft resources: indicated available |

Agreements:

Adopt the following regarding the IAB behaviour as it relates to soft DU resources:

The IAB node may use a soft DU resource for transmission and/or reception in line with D/U/F configuration of the resource, if and only if

- the use of the resource does not change the IAB-MT behavior relative to the IAB-MT behavior if the resource was instead configured as unavailable type

or

- the IAB node has received a DCI with DCI format 4_0 indicating that the resource is available

Company proposals:

| Huawei | Proposal 5: The IAB node ignores the dynamic indication of availability on Hard and NA resources. Proposal 6: The IAB node can ignore the dynamic indication of an AI-DCI on the Soft resource which has been indicated by a previous AI-DCI. |
| --- | --- |
| Vivo | Proposal 1: For IAB-node, the processing times between a downlink indication for DU child link availability from parent node and PDSCH/PUSCH transmission on DU child link should be defined. |
| Ericsson | Proposal 2: Adopt the following regarding the IAB behaviour as it relates to soft DU resources: The IAB node can use a soft DU resource for transmission and/or reception in line with D/U/F configuration of the resource, if and only if - the use of the resource does not change the IAB node MT behavior relative to the behavior if the resource was instead configured as Not Available or - the IAB node has received a DCI with DCI format 4_0 indicating that the resource is available Proposal 3: If the IAB MT detects more than one DCI Format 4_0 indicating resource availability for the same slot, the IAB MT expects each of the more than one DCI Format 4_0 to indicate the same AI value for the slot. |
| LG | Proposal 5: It is considerable that an IAB node can request its DU soft resource availability to its parent node. |
| ZTE | Proposal 1: To down-select from Alt-1 and Alt-2 for DU-IA indication. Value; Alt-1 definition; Alt-2 definition. 0; No soft resources (if any) available; There is no soft resource of any D/F/U type, or there are soft resources but none of them is available. 1; Soft D resources (if any) available; Soft D resources available. 2; Soft U resources (if any) available; Soft U resources available. 3; Soft D and soft U resources (if any) available; Soft D and soft U resources available. 4; Soft F resources (if any) available; Soft F resources available. 5; Soft D and soft F resources (if any) available; Soft D and soft F resources available. 6; Soft U and soft F resources (if any) available; Soft U and soft F resources available. 7; All soft resources (if any) available; All soft resources available. |
| Lenovo | Proposal: Support explicit RRC signaling to configure the offset between DCI 2_0 transmission and application. |
| Samsung | Proposal 2: Further discussion is necessary whether Availability indicator is applied only for soft resource. |
| Nokia | Proposal 6: When operating according to implicit indication, MT determines the availability of flexible resources according to Rel-15 rules defined in TS 38.213 (Section 11.1) and based on received DCI and/or higher layer configuration. - If Flexible MT resource is not assigned for Parent BH link it is considered as “IA” – i.e. DU resource is implicitly indicated as available - otherwise, if Flexible MT resource is assigned for Parent BH DL or Parent BH UL link it is considered as “INA” - i.e. DU resource is implicitly indicated as not available. Proposal 7: Define priorities related to AI-RNTI in the following way: - AI-RNTI is prioritized over implicit determination - When SFI-RNTI and AI-RNTI are conflicting with each other, operate according to DU INA. Proposal 8: Support a configuration option where entries of the AI Availability Combination Table can be mapped to reserved entries of Table 11.1.1-1 in TS 38.213. Proposal 9: Use Rel-15 Type-3 CSS for configuring PDCCH monitoring for DCI format configured with SFI-RNTI or AI-RNTI. Proposal 11: For the usage of soft resources at a child DU, use processing times defined for flexible resources in Rel-15 as the basis to determine when certain soft resource at the latest be made available for the DU. |
| Qualcomm | Proposal 3: The D, U, or F in the explicit indication of resource availability from an IAB node to a child IAB node refer to the child node MT slot configuration (cell specific + dedicated + dynamic SFI). |
| CEWiT | Proposal 1: Indication of IA should be only for the resource type configured as soft Observation 2: Based on the semi-static configuration of symbols of a slot, all values of resource availability may not be applicable for all slots of DU of IAB node Proposal 2: The IA/INA signal is based on any one of the following strategies. Alt 1: Parent node signals IA/INA only for the resource type configured as soft Alt 2: Parent node signals IA/INA for any resource type irrespective of the semi-static configuration - IA/INA signal over rides semi-static configuration of DU of child node - Child node ignores the IA/INA signal for the semi-statically configured resource type |

Offline proposal 2.4-5:

Alt 1. If the IAB node receives multiple DCI Format 4_0 indicating resource availability for a given slot, the IAB node follows the indication given by the latest received indication.

The parent DU may assume the indication of DCI Format 4_0 may be applied by the child IAB-DU no later than N_AI slots after the slot in which DCI Format 4_0 is transmitted, where N_AI is given by the following:

| SCS | N_AI |
| --- | --- |
| 15kHz | 1 |
| 30kHz | 1 |
| 60kHz | 2 |
| 120kHz | 3 |

Alt 2. The IAB node does not expect to receive multiple DCI Format 4_0 with different AI index fields for a given slot.