3GPP TSG-RAN WG1 Meeting #99	R1-1913533

Reno, USA, 18th – 22nd November 2019

Agenda Item:	6.2.1.3

Source:	Ericsson

Title:	Feature lead summary #4 for Scheduling of multiple DL/UL transport blocks for LTE-MTC

Document for:	Discussion and Decision

# 1	Introduction

In the Rel-16 work item on “Additional MTC enhancements for LTE” [1], one of the objectives is to specify support for scheduling of multiple DL/UL transport blocks.

The objective is to specify the following set of improvements for machine-type communications for BL/CE UEs. […] Scheduling enhancement: - Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast [RAN1, RAN2] - Enhancement of SPS can be discussed.

RAN1 and RAN2 agreements made in earlier meetings for this topic are summarized in [2] and [3]. The most recent L1 parameter list and UE feature list can be found in [4] and [5]. This document provides a summary and recommendations based on the contributions in [6] – [16] and the revised contributions in [17] – [19]. This is an updated version of the earlier feature lead summaries provided in [20] – [22].

# 2	DCI design for CE mode A

Based on discussion on email reflector discussion, the following can be considered.

- For unicast in CE mode A, select one of the following DCI design options.

- Option 1: Support FH for 1, 2, and 8 HARQs

| Bits | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  | 9 | 10 | 11 | 12 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 HARQ | 0 | 0 | 0 | 0 | 0 | HARQ ID |  |  |  | NDI | RV1 |  |  | FH1 |  |
| 2 HARQS | 0 | 0 | HARQ2 Index +8 |  |  |  |  |  |  | N1 | N2 | RV2-1 | RV/FH2-2 |  |  |
| 4 HARQS | 0 | HARQ4 index + 36 |  |  |  |  |  |  |  | N1 | N2 | N3 | N4 |  |  |
| 6 HARQS | HARQ6 Index +27 |  |  |  |  |  | N1 |  | N2 | N3 | N4 | N5 | N6 |  |  |
| 8 HARQS | 1 | 1 | 1 | N1 | N2 | N3 | N4 | N5 |  | N6 | N7 | N8 | RV/FH8 |  |  |

Where Hi is the scheduled HARQ ID in the set {0-7}

HARQ2 Index = nChoosek(8- H1,2)-H2+H1

HARQ4 Index = nChoosek (7- H1,4)+nChoosek (7- H2,3)+nChoosek(8- H3,2)- H4+H3

HARQ6 Index = nChoosek(8- H1,2)-H2+H1     H1 and H2 are the HARQ ID’s not scheduled.

RV1 is RV for H1 in the set {0,1,2,3}

FH1: Same behaviour as legacy frequency hopping bit. Set to 0 when frequency hopping and 64QAM are disabled

RV2-1 indicates the RV for H1 in the set {0,2}

RV/FH2-2: If Repetition=1 and frequency hopping is enabled and 64QAM is not enabled then

RV/FH2-2 indicates the RV for H2 in the set {0,2}

Else if Repetition=1 and 64QAM is enabled then

RV/FH2-2 indicates the extension to MCS table for 64QAM and RV for H2 is 0

Else if Repetition>1 and frequency hopping is enabled then

RV/FH2-2 indicates if H1 and H2 are frequency hopped and RV for H2 starts at 0

Else

RV/FH2-2 indicates the RV for H2 in the set {0,2}

End

RV/FH8: If Repetition=1 and frequency hopping is enabled and 64QAM is not enabled then

RV/FH8 indicates the RV for H1 to H8 in the set {0,2}

Else if Repetition=1 and 64QAM is enabled then

RV/FH8 indicates the extension to MCS table for 64QAM and RV for H1 to H8 is 0

Else if Repetition>1 and frequency hopping is enabled then

RV/FH8 indicates if H1 to H8 are frequency hopped and RV for H1 to H8 starts at 0

Else

RV/FH8 indicates the RV for H1 to H8 in the set {0,2}

End

- Option 2: Support FH for 1 and 8 HARQs

| Bits | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  | 9 | 10 | 11 | 12 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 HARQ | 0 | 0 | 0 | 0 | 0 | HARQ ID |  |  |  | NDI | RV1 |  |  | FH1 |  |
| 2 HARQS | 0 | 0 | HARQ2 Index +8 |  |  |  |  |  |  | N1 | N2 | RV2-1 | RV2-2 |  |  |
| 4 HARQS | 0 | HARQ4 index + 36 |  |  |  |  |  |  |  | N1 | N2 | N3 | N4 |  |  |
| 6 HARQS | HARQ6 Index +27 |  |  |  |  |  | N1 |  | N2 | N3 | N4 | N5 | N6 |  |  |
| 8 HARQS | 1 | 1 | 1 | N1 | N2 | N3 | N4 | N5 |  | N6 | N7 | N8 | FH8 |  |  |

Where Hi is the scheduled HARQ ID in the set {0-7}

HARQ2 Index = nChoosek(8- H1,2)-H2+H1

HARQ4 Index = nChoosek(7- H1,4)+nChoosek (7- H2,3)+nChoosek(8- H3,2)- H4+H3

HARQ6 Index = nChoosek(8- H1,2)-H2+H1  (H1 and H2 are the HARQ ID’s not scheduled)

RV1 is RV for H1 in the set {0,1,2,3}

FH1: Same behaviour as legacy frequency hopping bit. Set to 0 when frequency hopping and 64QAM are disabled

RV2-1 indicates the RV for H1 in the set {0,2}

RV2-2 indicates the RV for H2 in the set {0,2}

FH8: Same behaviour as legacy frequency hopping bit. Set to 0 when frequency hopping and 64QAM are disabled

For 4, 6 and 8 HARQ processes, RV for all the HARQ processes are set to 0.

# 3	Early termination

The following can be considered.

- For unicast, use DCI codewords for early termination according to below.

For CE mode B:

| Bits | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 HARQ | 0 | 0 | 0 | MCS |  |  |  | HARQ1 |  | N1 |
| 2 HARQS | 0 | MCS+3 |  |  |  | HARQ2 Index |  |  | N1 | N2 |
| 3 HARQS | MCS+15 |  |  |  |  | HARQ3 |  | N2 | N3 | N4 |
| 4 HARQS | MCS +52 |  |  |  |  |  | N1 | N2 | N3 | N4 |

Use the only one spare state for Bits 1-6: 11 11 11.

For CE mode A, assuming this is the design:

| Bits | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 8 |  | 9 | 10 | 11 | 12 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 HARQ | 0 | 0 | 0 | 0 | 0 | HARQ ID |  |  |  |  | NDI | RV1 |  |  | FH1 |  |
| 2 HARQS | 0 | 0 | HARQ2 Index +8 |  |  |  |  |  |  |  | N1 | N2 | RV2-1 | RV/FH2-2 |  |  |
| 4 HARQS | 0 | HARQ4 index + 36 |  |  |  |  |  |  |  |  | N1 | N2 | N3 | N4 |  |  |
| 6 HARQS | HARQ6 Index +27 |  |  |  |  |  |  | N1 |  | N2 | N3 | N4 | N5 | N6 |  |  |
| 8 HARQS | 1 | 1 | 1 | N1 | N2 | N3 | N4 |  | N5 |  | N6 | N7 | N8 | RV/FH8 |  |  |

Use one of the spare states in bits 1-6 between 11 01 10 and 11 11 11, e.g. 11 01 11.

# 4	Interleaving

RAN1#99 has made the following agreements:

Agreement

For unicast interleaved transmission without frequency hopping, the interleaving granularity is 1 repetition (RU) of a TB in CE mode A and 4 repetitions (RUs) of a TB in CE mode B in FDD and TDD

- No changes to the legacy RV rules

Agreement

For unicast interleaved transmission, the interleaving is supported in a way that aims to ensure that in case of frequency hopping, each TB is mapped to more than one frequency location.

Based on the above agreements, the following can be considered.

- For unicast interleaved transmission with frequency hopping, the same interleaving is used as without frequency hopping.

# References

[1]	RP-191356, “Revised WID: Additional MTC enhancements for LTE”

[2]	R1-1911573, “RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE”

[3]	R2-1914102, “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”

[4]	R1-1911574, “Summary of RRC parameters for L1 configuration for Rel-16 LTE-MTC”

[5]	R1-1911750, “RAN1 UE features list for Rel-16 LTE”

[6]	R1-1911832, “Scheduling of multiple DL/UL transport blocks in LTE-MTC”, Ericsson

[7]	R1-1911919, “Scheduling of multiple transport blocks”, Huawei, HiSilicon

[8]	R1-1911982, “Scheduling of multiple DL/UL transport blocks”, Nokia, Nokia Shanghai Bell

[9]	R1-1912188, “Scheduling of multiple DL/UL TBs for eMTC”, Intel Corporation

[10]	R1-1912334, “On scheduling of multiple DL/UL transport blocks”, Sony

[11]	R1-1912365, “Discussion on the remaining issues of scheduling of multiple DL/UL TBs”, Beijing Xiaomi Software Tech

[12]	R1-1912369, “LTE-M Multiple Transport Block Grant Design Considerations”, Sierra Wireless, S.A.

[13]	R1-1912376, “Discussion on multiple transport blocks scheduling in MTC”, LG Electronics

[14]	R1-1912411, “Consideration on scheduling enhancement for MTC”, ZTE

[15]	R1-1912679, “Scheduling of multiple DL/UL transport blocks”, Qualcomm Incorporated

[16]	R1-1912701, “Scheduling of multiple DL/UL transport blocks”, Sequans Communications

[17]	R1-1913246, “Scheduling of multiple DL/UL transport blocks”, Qualcomm Incorporated

[18]	R1-1913318, “Consideration on scheduling enhancement for MTC”, ZTE

[19]	R1-1913403, “Scheduling of multiple transport blocks”, Huawei, HiSilicon

[20]	R1-1913229, “Feature lead summary for Scheduling of multiple DL/UL transport blocks for LTE-MTC”

[21]	R1-1913371, “Feature lead summary #2 for Scheduling of multiple DL/UL transport blocks for LTE-MTC”

[22]	R1-1913448, “Feature lead summary #3 for Scheduling of multiple DL/UL transport blocks for LTE-MTC”