# 3GPP TSG-RAN WG1 Meeting #99																R1-1913586

# Reno, NV, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.201 | CR | 0001 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of V2X, NR-based access to unlicensed spectrum, integrated access backhaul for NR and remote interference management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core, NR_unlic-Core, NR_IAB-Core, NR_CLI_RIM-Core |  |  |  |  |  | Date: |  |  | 2019-11-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of V2X, NR-based access to unlicensed spectrum, integrated access backhaul for NR and remote interference management |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of V2X, NR-based access to unlicensed spectrum, integrated access backhaul for NR and remote interference management |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for V2X, NR-based access to unlicensed spectrum, integrated access backhaul for NR and remote interference management |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.1.1, 4.2.2, 4.2.4, 5.1, 5.4, 5.5, 5.6, 5.7, 5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

UE	User Equipment

IAB	Integrated access and backhaul

DU	Distributed Unit

MT	Mobile Termination

MAC	Medium Access Control

RRC	Radio Resource Control

SAP	Service Access Point

RLC	Radio Link Control

FEC	Forward Error Correction

HARQ	Hybrid Automatic Repeat Request

MIMO	Multiple Input Multiple Output

OFDM	Orthogonal Frequency Division Multiplexing

CP	Cyclic Prefix

DFT-s-OFDM	Discrete Fourier Transform-spread-Orthogonal Frequency Division Multiplexing

FDD	Frequency Division Duplex

TDD	Time Division Duplex

PDSCH	Physical Downlink Shared Channel

PDCCH	Physical Downlink Control Channel

PBCH	Physical Broadcast Channel

PRACH	Physical Random Access Channel

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PSBCH	Physical Sidelink Broadcast Channel

PSCCH	Physical Sidelink Control Channel

PSFCH	Physical Sidelink Feedback Channel

PSSCH	Physical Sidelink Shared Channel

BPSK	Binary Phase Shift Keying

QPSK	Quadrature Phase Shift Keying

QAM	Quadrature Amplitude Modulation

LDPC	Low Density Parity Check

E-UTRA	Evolved Universal Terrestrial Radio Access

SRS	Sounding reference signal

4	General description of layer 1

4.1	Relation to other layers

4.1.1	General protocol architecture

The radio interface described in this specification covers the interface between the User Equipment (UE) and gNB, between gNBs, between IAB-node DU and IAB-node MT/UE, and between UEs. The radio interface is composed of the Layer 1, 2 and 3. The TS 38.200 series describes the Layer 1 (Physical Layer) specifications. Layers 2 and 3 are described in the 38.300 series.

Figure 1: Radio interface protocol architecture around the physical layer

Figure 1 shows the NR radio interface protocol architecture around the physical layer (Layer 1). The physical layer interfaces the Medium Access Control (MAC) sub-layer of Layer 2 and the Radio Resource Control (RRC) Layer of Layer 3. The circles between different layer/sub-layers indicate Service Access Points (SAPs). The physical layer offers a transport channel to MAC. The transport channel is characterized by how the information is transferred over the radio interface. MAC offers different logical channels to the Radio Link Control (RLC) sub-layer of Layer 2. A logical channel is characterized by the type of information transferred.

4.1.2	Service provided to higher layers

The physical layer offers data transport services to higher layers. The access to these services is through the use of a transport channel via the MAC sub-layer. Details are specified in [2].

4.2	General description of layer 1

4.2.1	Multiple access

The multiple access scheme for the NR physical layer is based on Orthogonal Frequency Division Multiplexing (OFDM) with a cyclic prefix (CP). For uplink, Discrete Fourier Transform-spread-OFDM (DFT-s-OFDM) with a CP is also supported. To support transmission in paired and unpaired spectrum, both Frequency Division Duplex (FDD) and Time Division Duplex (TDD) are enabled.

The Layer 1 is defined in a bandwidth agnostic way based on resource blocks, allowing the NR Layer 1 to adapt to various spectrum allocations. A resource block spans 12 sub-carriers with a given sub-carrier spacing.

The radio frame has a duration of 10ms and consists of 10 sub-frames with a sub-frame duration of 1ms. A sub-frame is formed by one or multiple adjacent slots, each having 14 adjacent symbols. Further details on the frame structure are specified in [2].

4.2.2	Physical channels and modulation

The physical channels defined in the downlink are:

-	the Physical Downlink Shared Channel (PDSCH),

-	the Physical Downlink Control Channel (PDCCH),

-	the Physical Broadcast Channel (PBCH),

The physical channels defined in the uplink are:

-	the Physical Random Access Channel (PRACH),

-	the Physical Uplink Shared Channel (PUSCH),

-	and the Physical Uplink Control Channel (PUCCH).

The physical channels defined in the sidelink are:

-	the Physical Sidelink Broadcast Channel (PSBCH),

-	the Physical Sidelink Control Channel (PSCCH),

-	the Physical Sidelink Feedback Channel (PSFCH),

-	and the Physical Sidelink Shared Channel (PSSCH).

In addition, signals are defined as reference signals, primary and secondary synchronization signals.

The modulation schemes supported are

-	in the downlink, QPSK, 16QAM, 64QAM, and 256QAM,

-	in the uplink, QPSK, 16QAM, 64QAM and 256QAM for OFDM with a CP and π/2-BPSK, QPSK, 16QAM, 64QAM and 256QAM for DFT-s-OFDM with a CP

4.2.3	Channel coding

The channel coding scheme for transport blocks is quasi-cyclic LDPC codes with 2 base graphs and 8 sets of parity check matrices for each base graph, respectively. One base graph is used for code blocks larger than certain sizes or with initial transmission code rate higher than thresholds; otherwise, the other base graph is used. Before the LDPC coding, for large transport blocks, the transport block is segmented into multiple code blocks with equal size. The channel coding scheme for PBCH and control information is Polar coding based on nested sequences. Puncturing, shortening and repetition are used for rate matching. Further details of channel coding schemes are specified in [4].

4.2.4	Physical layer procedures

There are several Physical layer procedures involved. Such procedures covered by the physical layer are;

-	Cell search

-	Power control

-	Uplink synchronisation and Uplink timing control

-	Random access related procedures

-	HARQ related procedures

-	Beam management and CSI related procedures

-	Sidelink related procedures

-	Channel access procedures

Through the control of physical layer resources in the frequency domain as well as in the time and power domains, implicit support of interference coordination is provided in NR.

4.2.5	Physical layer measurements

Radio characteristics are measured by the UE and the network and reported to higher layers. These include, e.g. measurements for intra- and inter-frequency handover, inter RAT handover, timing measurements, and measurements for RRM.

Measurements for inter-RAT handover are defined in support of handover to E-UTRA.

5	Document structure of physical layer specification

5.1	Overview

The physical layer specification consists of a general document (TS 38.201), and seven documents (TS 38.202, 38.211 through 38.215, and 37.213). The relation between the physical layer specifications in the context of the higher layers is shown in Figure 2.

Figure 2: Relation between Physical Layer specifications

5.2	TS 38.201: Physical layer; General description

The scope is to describe:

-	The contents of the Layer 1 documents (TS 38.200 series);

-	Where to find information;

5.3	TS 38.202: Physical layer services provided by the physical layer

The scope is to describe services provided by the physical layer, and to specify:

-	Services and functions of the physical layer;

-	Model of physical layer of the UE;

-	Parallel transmission of simultaneous physical channels and SRS;

-	Measurements provided by the physical layer.

5.4	TS 38.211: Physical channels and modulation

The scope is to establish the characteristics of the Layer-1 physical channels, generation of physical layer signals and modulation, and to specify:

-	Definition of the uplink, downlink and sidelink physical channels;

-	Frame structure and physical resources;

-	Modulation mapping (BPSK, QPSK, etc);

-	OFDM signal generation;

-	Scrambling, modulation and upconversion;

-	Layer mapping and precoding;

-	Physical shared channel in uplink, downlink and sidelink;

-	Reference signal in uplink, downlink, and sidelink;

-	Physical random access channel;

-	Primary and secondary synchronization signals.

5.5	TS 38.212: Multiplexing and channel coding

The scope is to describe the transport channel and control channel data processing, including multiplexing, channel coding and interleaving, and to specify:

-	Channel coding schemes;

-	Rate matching;

-	Uplink transport channels and control information;

-	Downlink transport channels and control information;

-	Sidelink transport channels and control information.

5.6	TS 38.213: Physical layer procedures for control

The scope is to establish the characteristics of the physical layer procedures for control, and to specify:

-	Synchronization procedures;

-	Uplink power control;

-	Random access procedure;

-	UE procedure for reporting control information;

-	UE procedure for receiving control information.

5.7	TS 38.214: Physical layer procedures for data

The scope is to establish the characteristics of the physical layer procedures for data, and to specify:

-	Power control;

-	Physical downlink shared channel related procedures;

-	Physical uplink shared channel related procedure;

-	Physical sidelink shared channel related procedure.

5.8	TS 38.215: Physical layer measurements

The scope is to establish the characteristics of the physical layer measurements, and to specify:

-	Control of UE/NG-RAN measurements;

-	Measurement capabilities for NR.

5.9	TS 37.213: Physical layer procedures for shared spectrum channel access

The scope is to establish the characteristics of the physical layer procedures for shared spectrum channel, and to specify:

-	Downlink channel access procedures;

-	Uplink channel access procedures.