Agreements up to RAN1#99 on Multi-Beam1 (AI 7.2.8.3)

UL transmit beam selection for multi-panel operation that facilitates panel-specific beam selection

Agreement@RAN1#95

In Rel-16, an identifier (ID) that can be used at least for indicating panel-specific UL transmission is supported, where detailed usages for the panel-specific UL transmission are FFS

- The ID should be defined considering the possibility to reuse/modification of Rel-15 specification support or introducing new ID

- Note: RAN1 to avoid unnecessary specification support requiring UE to explicitly disclose its UL antenna panel implementation

- FFS: Whether UE capability signalling is introduced for panel-specific UL transmission

Agreement@RAN1#AH1901

An identifier (ID), agreed in RAN1#95, that can be used at least for indicating panel-specific UL transmission is to be down-selected or merged from the following alternatives in next RAN1 meeting:

- Alt.1: an SRS resource set ID, where FFS on further association to other RS (if needed)

- Alt.2: an ID, which is directly associated to a reference RS resource and/or resource set

- Alt.3: an ID, which can be assigned for a target RS resource or resource set

- Alt.4: an ID which is additionally configured in spatial relation info

For purpose of further discussion on this topic for RAN1#96 and future meetings

Following multi-panel UE (MPUE) categories can be used for discussions on possible enhancements over Rel-15, if needed.

- MPUE-Assumption1: Multiple panels are implemented on a UE and only one panel can be activated at a time, with panel switching/activation delay of [X] ms

- MPUE-Assumption2: Multiple panels are implemented on a UE and multiple panels can be activated at a time and one or more panels can be used for transmission

- MPUE-Assumption3: Multiple panels are implemented on a UE and multiple panels can be activated at a time but only one panel can be used for transmission

Note: Above does not imply the support of either one or both of the categories but is only for efficient discussions at least for this meeting, which may also be updated further. Whether to support either one or both categories will depend on subsequent discussions

Note: There is no consensus among the companies in RAN1 whether MPUE-Assumption2 is in the work scope of Rel-16 WI

Agreement@RAN1#96

If RAN1 cannot agree on the support of at least one of MPUE-Assumption1, MPUE-Assumption2, MPUE-Assumption3, enhancements on panel-specific beam selection for uplink will not be supported in Rel-16.

Deadline for decision: RAN1#96bis

Agreement@RAN1#96bis

In Rel-16, only introduce specification enhancement for MPUE-Assumption3

- MPUE-Assumption3: Multiple panels are implemented on a UE and multiple panels can be activated at a time but only one panel can be used for transmission.

  - Note that this does not require a UE to always activate multi-panels simultaneously

  - Note: UE can control the panel activation/deactivation

- Possible use cases at least include

  - (General) UL coverage enhancement for FR2 considering the UE power consumption

- Discussion topics in Rel-16 include:

  - Details on the identification for a panel and corresponding panel definition

- Any enhancement introduced in Rel-16 should take further enhancement of simultaneous transmission across multiple panels for future releases into account.

This is a UE optional feature

Working Assumption@RAN1#96bis

The agreed ID (not excluding to reuse existing ID) for a panel can be used for panel-selection-based transmission of PUSCH, PUCCH and SRS, among multiple activated panels.

- FFS details, including an explicit/implicit indication of the panel, also considering beam correspondence at UE.

- FFS on whether the ID can be used for panel-specific PRACH transmission, if supported.

Agreement@RAN1#97

Select one of the following alternatives in RAN1#98. Companies should take into account the maturity, forward compatibility to future releases, efficient use of SRS resource usage, and extension to simultaneous transmission across multiple panels of each alternatives for completion within the intended Rel-16 schedule. If there is no consensus in RAN1#98, UL multi-panel enhancement will not be specified in Rel-16.

gNB can configure/indicate panel-specific transmission for UL transmission, via

- Alt.2: Introduce a UL-TCI framework in Rel-16 and support UL-TCI based signaling analogous to DL beam indication supported in Rel-15, e.g., as illustrated below.

  - A new panel ID may or may not be introduced.

  - A panel specific signaling is performed using UL-TCI state

- Alt.3: a new panel-ID is introduced, which can be implicitly/explicitly applied to the transmission for a target RS resource or resource set, for PUCCH resource, for SRS resource, FFS for PRACH

  - A panel specific signaling is performed using the new panel-ID implicitly (e.g., by DL beam reporting enhancement) or explicitly.

  - If explicitly signaled, the ID can be configured in the target RS/channel or reference RS(e.g., in the DL RS resource configuration or in spatial relation info).

  - No new MAC CE is specified for the purpose of introducing the ID.

(For example) Alt.2 UL-TCI states

| Valid UL-TCI state Configuration | Source (reference) RS | (target) UL RS | [qcl-Type ] |
| --- | --- | --- | --- |
| 1 | SRS resource (for BM) + [panel ID] | DM-RS for PUCCH or SRS or PRACH | Spatial-relation |
| 2 | DL RS(a CSI-RS resource or a SSB) + [panel ID] | DM-RS for PUCCH or SRS or PRACH | Spatial-relation |
| 3 | DL RS(a CSI-RS resource or a SSB) + [panel ID] | DM-RS for PUSCH | Spatial-relation + [port(s)-indication] |
| 4 | DL RS(a CSI-RS resource or a SSB) and SRS resource + [panel ID] | DM-RS for PUSCH | Spatial-relation + [port(s)-indication] |
| 5 | SRS resource + [panel ID] | DM-RS for PUSCH | Spatial-relation + [port(s)-indication] |
| 6 | UL RS(a SRS for BM) and SRS resource + [panel ID] | DM-RS for PUSCH | Spatial-relation + [port(s)-indication] |

Agreement@RAN1#97

Select one of the following alternatives in RAN1#98. Companies should take into account the maturity, forward compatibility to future releases, efficient use of SRS resource usage, and extension to simultaneous transmission across multiple panels of each alternatives for completion within the intended Rel-16 schedule. If there is no consensus in RAN1#98, UL multi-panel enhancement will not be specified in Rel-16.

Conclusion@RAN1#98

- From RAN1 point of view, a “UE panel” would be a logical entity and how to map physical UE antennas to the logical entity is up to UE implementation.

- (Informative) For certain condition(s), gNB can assume the mapping between UE’s physical antennas to the logical entity “UE panel” activated for transmission will not be changed

  - FFS: Whether “UE panel” is transparent to gNB

  - FFS: UE capability includes at least the number of “UE panels”.

  - FFS: Whether/how to define the certain condition(s)

    - For example: The duration of time over which the gNB assumes there will be no change

    - For example: Until next update or report from UE

- (Informative) Depending on UE’s own implementation, a “UE panel” can have at least the following functionality as an operational role of

  - Unit of antenna group to control its Tx beam independently

Conclusion@RAN1#98

On the issue of uplink transmit beam selection that facilitates UE panel specific uplink beam selection, there is no consensus in RAN1 to introduce additional specification support in RAN1. No further discussion for Rel-16 from RAN1 perspective.

Enhancements on UL and/or DL transmit beam selection specified in Rel-15 to reduce latency and overhead

Spatial relation updates for AP-SRS via MAC CE

Working Assumption@RAN1#96

For UL beam management latency and overhead reduction, support MAC CE based spatial relation update for aperiodic SRS per resource level

- FFS: Whether this is a UE optional feature

- Note: Qualcomm prefers to have this as a UE optional feature

Agreement@RAN1#96bis

The working assumption made in RAN1#96 is confirmed

For UL beam management latency and overhead reduction, support MAC CE based spatial relation update for aperiodic SRS per resource level

- FFS: Whether this is a UE optional feature

FFS: Whether above is applicable regardless of the aperiodic SRS target use

Agreement@RAN1#97

The supported feature of MAC CE based spatial relation update for aperiodic SRS per resource level is applicable to at least 3 supported usages as codebook-based UL, non-codebook-based UL, beam management.

Working Assumption@RAN1#97

The supported feature of MAC CE based spatial relation update for aperiodic SRS is applicable to the usage of antenna switching per SRS resource level

Agreement@RAN1#98

The following working assumption is confirmed with modification (in red):

The supported feature of MAC CE based spatial relation update for aperiodic SRS is applicable to the usage of antenna switching per SRS resource level.

- In case of antenna switching, UE does not expect to be configured with different spatial relation within the same set

Updating pathloss reference RS for PUSCH/SRS via MAC CE

Agreement@RAN1#97

Decide in RAN1#98 whether to support updating path loss reference RSs for power control for PUSCH and SRS via MAC-CE.

- FFS: Condition that the RS for PL will follows the downlink RS in spatial relation.

- FFS: When the spatial relation of AP-SRS for CB/NCB UL is activated by MAC-CE, UL power control parameters for PUSCH can be activated via the MAC-CE.

Agreement@RAN1#98

Continue discussion on the support of updating pathloss RSs for power control for PUSCH and SRS via MAC-CE, including the following candidates until RAN1#98bis:

- Option 1: For codebook based PUSCH transmission, the pathloss RS follows DL RS in spatial relation associated with SRI indicated in scheduling DCI, if the pathloss RS is not configured and periodic DL RS is configured in the spatial relation.

  - FFS: the cases of non-codebook based PUSCH, SRS

- Option 2: Pathloss RS is associated/configured for downlink RS in spatial relation info.

  - gNB can configure more than 4 pathloss RSs.

- Option 3: At least the pathloss RSs for SRS or PUSCH can be explicitly activated/updated by the MAC-CE

  - FFS: The other power control parameters including P0, alpha, and a closed loop process index are also activated by the MAC-CE

  - FFS on whether to support the number of configured pathloss RSs are more than four.

  - Note: The MAC-CE is the activation MAC CE for ap-SRS/sp-SRS.

- Option 4: Support updating TCI state for periodic CSI-RS by MAC CE.

  - Note: The periodic CSI-RS is used for pathloss reference.

- Option 5: Support semi-persistent CSI-RS for pathloss reference RS.

- Note: Baseline is that the same transmission power is applied within SRS resource set (same as Rel-15).

Agreement@RAN1#98bis

Select one among the following alternatives on whether/how to revise the existing mechanism on higher layer filtered RSRP for pathloss measurement, when a pathloss RS is updated by MAC CE (if agreed) in RAN1#99.

- Alt.1: L1-RSRP based pathloss measurement is applied when the pathloss RS is updated by MAC CE.

- Alt.2: Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Note, before the higher layer filtered RSRP is applied, UE uses L1-RSRP for pathloss estimation.

- Alt.3: Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Note: Filtered RSRP value for previous pathloss RS will be used before the application time.

- Alt.4: Reuse higher layer filtered RSRP for pathloss measurement, with the same behavior with higher layer filtered RSRP as in Rel-15.

  - Note: UE is expected to track all the RRC-configured candidate pathloss RSs.

  - Note: The maximum configurable pathloss RSs by RRC is up to UE capability.

- Note: If there is no consensus to the down-selection above, the specification in Rel-16 only allows the maximum configurable pathloss RSs by RRC is 4 (same as Rel-15). For less than or equal to X candidate pathloss RSs configured by RRC, the existing mechanism on higher layer filtered RSRP for pathloss measurement can be reused.

  - Baseline is X=4.

  - The pathloss RS selected by the MAC CE, if agreed, is among the X RRC configured candidate pathloss RSs.

- Note: For Alt.1, Alt.2, Alt.3, and Alt.4, the maximum configurable pathloss RSs by RRC is increased from Rel-15, e.g., 8, 16, or 64.

  - For Alt.1, Alt.2, Alt.3, such pathloss reference signals are for configuration purpose only.

Working assumption:@RAN1#98bis

Pathloss reference RS for PUSCH can be activated/updated via a MAC CE

- The MAC CE message can activate/update the value of PUSCH-PathlossReferenceRS-Id corresponding to sri-PUSCH-PowerControlId.

  - Note(Informative): In TS38.331, the mapping is given by SRI-PUSCH-PowerControl, in which the linkage is between sri-PUSCH-PowerControlId and PUSCH-PathlossReferenceRS-Id.

- Further signaling details are up to RAN2.

- Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot after the 5th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

    - This is only applicable for UEs supporting the number of RRC-configurable pathloss RSs larger than 4, and this is only for the case that the activated PL RS by the MAC CE is not tracked.

    - UE is only required to track the activated PL RS(s) if the configured PL RSs by RRC is greater than 4.

    - It is up to UE whether to update the filtered RSRP value for previous PL RS 3ms after sending ACK for the MAC CE.

- Send an LS to RAN4 asking opinion on this working assumption.

Working assumption:@RAN1#98bis

Pathloss reference RS for AP-SRS/SP-SRS can be activated/updated via a MAC CE.

- A UE can be configured with multiple pathloss RSs by RRC and one of them can be activated/updated via the MAC CE for a SRS resource set.

- Further signaling details are up to RAN2.

- Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot after the 5th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

    - This is only applicable for UEs supporting the number of RRC-configurable pathloss RSs larger than 4, and this is only for the case that the activated PL RS by the MAC CE is not tracked.

    - UE is only required to track the activated PL RS if the configured PL RSs by RRC is greater than 4.

    - It is up to UE whether to update the filtered RSRP value for previous PL RS 3ms after sending ACK for the MAC CE.

- Send an LS to RAN4 asking opinion on this working assumption.

Agreement (RRC impact)@RAN1#99

On power control for PUSCH, PUCCH, and SRS, the total number of maximum configurable pathloss RSs, in including those supported in Rel-15, by RRC is 64

- Note: Such pathloss reference signals are for configuration purpose only, and UE is still only required to track up to 4 pathloss RSs for any PUSCH, PUCCH, and SRS transmissions.

  - “Up to 4 pathloss RSs” applies the total number of pathloss RSs for PUSCH, PUCCH, and SRS

Agreement (RRC impact)@RAN1#99

For the feature of MAC CE based pathloss RS updates for PUSCH/SRS in Rel-16,

- Introduce one new RRC parameter to enable the feature of MAC CE based pathloss RS updates for PUSCH/SRS in Rel-16, i.e.,

  - enablePLRSupdateForPUSCHSRS

Agreement@RAN1#99

When enablePLRSupdateForPUSCHSRS is configured, if a grant-based or grant-free PUSCH transmission is scheduled/activated by DCI format 0_1 that does not include a SRI field, the RS resource index qd corresponding to the PUSCH-PathlossReferenceRS-Id mapped with sri-PUSCH-PowerControlId = 0 is used for path-loss measurement of PUSCH transmission. In this case, UE expects to be configured with sri-PUSCH-PowerControl

Simultaneous spatial relation update for multiple PUCCH resources

Agreement@RAN1#96

For signaling overhead reduction on updating/configuring spatial relation for PUCCH, support simultaneous spatial relation update/configuration for multiple PUCCH resources

- FFS signaling details to be decided in next meeting, including down-selection/merging among the following options

  - Spatial relation update for all PUCCH resources in a CC by one MAC CE

  - Spatial relation update per Rel-15 PUCCH resource set

  - Spatial relation update per group of PUCCH (which might need to be introduced for Rel-16)

  - PUCCH spatial relation info configured in a BWP could be applied across different BWP or different cells

  - Other options are not precluded.

Agreement@RAN1#96bis

Simultaneous update/indication of a single spatial relation per group of PUCCH is supported by using one MAC CE

- As a starting point, the group should correspond to all the PUCCHs in a BWP when a single active spatial relation is applied before and after activation

- If there is no consensus on the details of the grouping, only one group per BWP will be supported in Rel-16 which will correspond to all the PUCCHs in a BWP

Detailed design on the MAC CE is up to RAN2

Working Assumption@RAN1#97

For the supported feature of simultaneous update/indication of a single spatial relation per group of PUCCH by using one MAC CE, the following configuration options for the group are supported:

- At least up to two groups per BWP

  - FFS: Details on configuring the groups including whether to use implicit method or explicit method

    - For example, each corresponding to different TRP/panel, at least for multi-TRP/panel case

    - Another example, each corresponding to different active spatial relation at least for single TRP case

  - If there is no consensus to support more than two groups, only up to two groups will be supported in Rel-16

Agreement@RAN1#98bis

Confirm the working assumption made in RAN1#97, with the following updates.

For the supported feature of simultaneous update/indication of a single spatial relation per group of PUCCH by using one MAC CE, the following configuration options for the group are supported:

- At least up to two groups per BWP.

  - FFS: Details on configuring the groups including whether to use implicit method or explicit method

    - For example, each corresponding to different TRP/panel, at least for multi-TRP/panel case

    - Another example, each corresponding to different active spatial relation at least for single TRP case

  - If there is no consensus to support more than two groups within RAN1#98bis, only up to two groups will be supported in Rel-16

  - Note: the terminology of “group” may or may not be in specifications.

Agreement@RAN1#98bis

At least for the agreed feature of simultaneous update/indication of a single spatial relation per group of PUCCH resources by using MAC CE, explicit higher layer signalling on PUCCH resource grouping is supported.

  - Signalling details are up to RAN2

Agreement@RAN1#98bis

For the agreed feature of simultaneous update/indication of a single spatial relation per group of PUCCH resources by using MAC CE,

- Support up to 4 groups per BWP.

Agreement@RAN1#99

Send an LS to RAN2 to indicate that from RAN1 perspective, the following examples have be studied and RAN2 should consider this in their work.

For the agreed feature of simultaneous update/indication of a single spatial relation per group of PUCCH resources by using MAC CE, the detail on explicit higher layer signalling on PUCCH resource grouping is determined by

- Example 1: Introducing a group identifier inside each PUCCH resource

- Example 2: Defining a new PUCCH group, which contains identities of PUCCH resources

Draft LS to be prepared by Jonghyun (LGE)

R1-1913323	[DRAFT] LS on explicit higher layer signalling on PUCCH resource grouping for simultaneous spatial relation updates	LG Electronics

Agreement@RAN1#99

The LS is endorsed in R1-1913423

Default spatial relation for PUCCH/SRS in FR2

For further discussion

Study the spatial relation for the PUCCH/SRS to follow a TCI-state/QCL of PDCCH/CSI-RS/SSB if spatial relation info of PUCCH/SRS is not configured in FR2

Agreement@RAN1#98

At least for UEs supporting beam correspondance, if spatial relation info for dedicated-PUCCH/SRS, except for SRS with usage = 'BeamManagement', is not configured in FR2, a default spatial relation for dedicated-PUCCH/SRS is applied

- FFS: Detail on the default spatial relation

Agreement@RAN1#98

At least for UEs supporting beam correspondence, if spatial relation info for dedicated-PUCCH/SRS, except for SRS with usage = 'BeamManagement', is not configured in FR2, the applied default spatial relation for the dedicated-PUCCH/SRS is down-selected from the followings in RAN1#98bis

- Alt.1: default TCI state or QCL assumption of PDSCH (e.g. the most recent slot and the lowest CORESET ID)

- Alt.2: one of an active TCI state of CORESET

  - FFS: details of which TCI state

- Alt.3: TCI state of scheduling PDCCH for A-SRS/PUCCH, and default TCI state or QCL assumption of PDSCH for other than A-SRS/PUCCH

- Alt.4: CORESET#0 QCL assumption

- Alt.5: pathloss reference RS

  - FFS: details of which pathloss reference RS

- FFS: whether to apply the above for UEs not supporting beam correspondence

Agreement@RAN1#98bis

For SRS resource(s) configured in an SRS resource set with usage = ‘nonCodebook’ with configuration of associatedCSI-RS, the agreed feature of a default spatial relation is not applied.

- Note: Accordingly, the previous agreement made in RAN1#98 is updated as follows (RED):

  - At least for UEs supporting beam correspondence, if spatial relation info for dedicated-PUCCH/SRS, except for SRS with usage = 'BeamManagement' and except for SRS with usage = ‘nonCodebook’ with configuration of associatedCSI-RS, is not configured in FR2, a default spatial relation for the dedicated-PUCCH/SRS is applied.

Working Assumption@RAN1#98bis

The default spatial relation for dedicated-PUCCH/SRS for a CC in FR2, at least when no pathloss RSs are configured by RRC is determined by

- Default TCI state or QCL assumption of PDSCH, i.e.,

  - in case when CORESET(s) are configured on the CC, the CORESET with the lowest ID in the most recent monitored downlink slot, or

  - in case when any CORESETs are not configured on the CC, the activated TCI state with the lowest ID applicable to PDSCH in the active DL-BWP of the CC

- Above applies at least for UEs supporting beam correspondence

- Above applies at least for the single TRP case

- FFS: Details on UE behavior in the absence of the activated TCI state

- FFS: Details on default spatial relation in multicarrier scenario

- FFS: Details on which RS to use for pathloss measurement

- FFS: Details on how to handle this issue in case pathloss RSs are configured

Agreement@RAN1#99

The following working assumption is confirmed with revision in red

The default spatial relation for dedicated-PUCCH/SRS for a CC in FR2, at least when no pathloss RSs are configured by RRC is determined by

- Default TCI state or QCL assumption of PDSCH, i.e.,

- in case when CORESET(s) are configured on the CC, the TCI state / QCL assumption of the CORESET with the lowest ID, or

  - The PL RS to be used is the QCL-TypeD RS of the same TCI state / QCL assumption of the CORESET with the lowest ID

  - Note: The PL RS should be periodic RS

- in case when any CORESETs are not configured on the CC, the activated TCI state with the lowest ID applicable to PDSCH in the active DL-BWP of the CC

- Above applies at least for UEs supporting beam correspondence

- Above applies at least for the single TRP case

- FFS: Details on UE behavior in the absence of the activated TCI state

- FFS: Details on default spatial relation in multicarrier scenario

- FFS: Details on which RS to use for pathloss measurement

- FFS: Details on how to handle this issue in case pathloss RSs are configured

Agreement@RAN1#99

If PL RS for SRS is not configured

- When CORESET is not configured and at least 1 active TCI state is configured for PDSCH

  - PL RS for SRS is the QCL-TypeD RS of the active TCI state of the lowest ID for PDSCH

    - Note: The PL RS should be periodic RS.

- When CORESET is not configured and absence of active TCI state on the CC for PDSCH

  - No change from Rel-15 UE behavior

Agreement@RAN1#99

Support default spatial relation and default pathloss RS of PUSCH scheduled by DCI format 0_0 when there is no PUCCH resources configured on the active UL BWP CC in FR2 and in RRC-connected mode, for UEs supporting the feature of the default spatial relation for dedicated-PUSCH in Rel-16

- The default spatial relation is the TCI state / QCL assumption of the CORESET with the lowest ID.

- The default pathloss RS is the QCL-TypeD RS of the same TCI state / QCL assumption of the CORESET with the lowest ID

  - Note: The PL RS should be periodic RS.

Agreement@RAN1#99

In Rel-16, support to schedule PUSCH by DCI format 0_0 on the CC in FR2 and in RRC-connected mode with PUCCH resource(s) configured, where all the configured PUCCH resource(s) are not configured with any spatial relation.

- Note: The spatial relation and the PL RS for the PUSCH scheduled by DCI format 0_0 follow those for the PUCCH resource(s), which are the default spatial relation and the default pathloss RS, respectively.

Agreement (RRC impact)@RAN1#99

For enabling the agreed UE behaviors in Rel-16 on the default spatial relation and pathloss RS,

- Introduce three new RRC parameters to enable the default spatial relation and PL RS behaviors, i.e.,

  - enableDefaultBeamPlForPUSCH0_0, for PUSCH scheduled by DCI format 0_0

  - enableDefaultBeamPlForPUCCH, for dedicated PUCCH

  - enableDefaultBeamPlForSRS, for dedicated SRS

Simultaneous TCI states activation/selection across multiple CCs/BWPs

For further discussion

Study beam indication/activation for a group of CCs

Agreement@RAN1#98

For latency/overhead reduction across multiple CCs/BWPs, support single MAC-CE to activate at least the same set of PDSCH TCI state IDs for multiple CCs/BWPs

- Example 1: Reuse Rel-15 MAC-CE to activate same set of TCI state IDs for all active BWPs in same band or cell group(s) on FR2

  - Support of this mode can be indicated by UE capability

  - To operate in this mode, UE may expect the same QCL-TypeD RS is configured for same TCI state ID for all BWPs in each band or cell group(s)

  - For activation MAC-CE received on any active BWP in a band or cell group(s), indicated activated TCI state IDs will be applied to every active BWP in that band or cell group(s)

- Example 2: Reuse Rel-15 MAC-CE to activate one set of TCI state IDs (including both QCL Type-A and Type-D RSs) for an active BWP of the CC indicated by the MAC-CE to be applied to all active BWPs in same band or cell group(s) on FR2

  - Note: The QCL Type A RS(s) applied to each CC/BWP is that corresponding to the same resource ID(s) indicated by the TCI state IDs

- FFS: operation/signaling details including the possibility to activate different sets of PDSCH TCI state IDs for multiple CCs/BWPs

- Note: QCL type-A comes from the BWP where the TCI state is applied

Conclusion@RAN1#98bis

For the agreed feature of single MAC-CE to activate at least the same set of PDSCH TCI state IDs for multiple CCs/BWPs,

- In the RRC perspective, the candidate (up to 128) TCI-states are still independently configurable by RRC for each CC/BWP.

Agreement@RAN1#98bis

When a set of TCI-state IDs for PDSCH are activated by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the same set of TCI-state IDs are applied for the all BWPs in the indicated CCs.

- Further signaling details are up to RAN2.

- Whether to support the inter-band CA for this feature will be decided in RAN1#99.

- Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same set of PDSCH TCI state IDs for multiple CCs/BWPs is up to capability discussion.

  - FFS on the UE capability signaling details

- Note: This at least applies to single TRP case.

- FFS: How many combinations of CCs can be configured by RRC and relevant UE capability

Agreement@RAN1#98bis

When a TCI-state ID is activated for a CORESET by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the TCI-state ID is applied for the CORESET(s) with the same CORESET ID for all the BWPs in the indicated CCs.

- Further signaling details are up to RAN2.

- Whether to support the inter-band CA for this feature will be decided in RAN1#99.

- Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same PDCCH TCI state IDs for multiple CCs/BWPs is up to capability discussion.

  - FFS on the UE capability signaling details

- Note: This at least applies to single TRP case.

Agreement@RAN1#98bis

For the purpose of simultaneous TCI state activation across multiple CCs/BWPs,

- Up to 2 lists of CCs can be configured by RRC per UE, and the applied list is determined by the indicated CC in the MAC CE.

  - UE expect no overlapped CC in multiple RRC-configured lists of CCs.

  - Send LS to RAN2 to inform above

Working assumption:@RAN1#98bis

When a Spatial Relation Info is activated for a SP/AP SRS resource by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the Spatial Relation Info is applied for the SP/AP SRS resource(s) with the same SRS resource ID for all the BWPs in the indicated CCs.

- Further signaling details are up to RAN2.

- Whether to support the inter-band CA for this feature will be decided in RAN1#99.

- Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same SRS resource IDs for multiple CCs/BWPs is up to capability discussion.

- FFS on the UE capability signaling details

- Note: This at least applies to single TRP case.

- FFS on the power control details (without RAN2 impact)

Working assumption:@RAN1#98bis

For the purpose of simultaneous Spatial Relation update across multiple CCs/BWPs,

- Up to 2 lists of CCs can be configured by RRC per UE, and the applied list is determined by the indicated CC in the MAC CE.

  - UE expect no overlapped CC in multiple RRC-configured lists of CCs.

  - The lists are independent from those for simultaneous TCI state activation

  - Send LS to RAN2 to inform above

Agreement@RAN1#99

The following working assumption is confirmed with revision in red

(Working assumption #1 @RAN1#98bis) When a Spatial Relation Info is activated for a SP/AP SRS resource by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the Spatial Relation Info is applied for the SP/AP SRS resource(s) with the same SRS resource ID for all the BWPs in the indicated CCs.

- Further signaling details are up to RAN2.

- Whether to support the inter-band CA for this feature will be decided in RAN1#99.

- Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same SRS resource IDs for multiple CCs/BWPs is up to capability discussion.

- FFS on the UE capability signaling details

- Note: This at least applies to single TRP case.

- FFS on the power control details (without RAN2 impact)

Agreement@RAN1#99

- For the supported feature of simultaneous spatial relation update across multiple CCs/BWPs by MAC CE, Additional restriction for SRS, restricted to only with the same ‘usage’

  - NO (i.e., no need to change from the part of the WA that “… the Spatial Relation Info is applied for the SP/AP SRS resource(s) with the same SRS resource ID for all the BWPs in the indicated CCs”)

    - i.e., no need to introduce further restriction in relation to the same usage, e.g., gNB can handle this

Agreement@RAN1#99

The following working assumption is confirmed

Working assumption#2 @RAN1#98bis

For the purpose of simultaneous Spatial Relation update across multiple CCs/BWPs,

- Up to 2 lists of CCs can be configured by RRC per UE, and the applied list is determined by the indicated CC in the MAC CE.

  - UE expect no overlapped CC in multiple RRC-configured lists of CCs.

  - The lists are independent from those for simultaneous TCI state activation

Agreement@RAN1#99

On simultaneous TCI-state-ID set activation for PDSCH via MAC CE (agreed in RAN1#98bis) where the same set of TCI-state IDs is applied for the all BWPs in the configured CCs, inter-band CA is not prohibited from the signaling perspective

- Note: Depending on the TCI state configuration across different CCs (configured via RRC), the set of TCI-state IDs may not refer to the same set of TCI states for different CCs

Agreement@RAN1#99

On simultaneous TCI-state-ID activation for CORESET via MAC CE (agreed in RAN1#98bis) where the same TCI-state ID is applied for the all BWPs in the configured CCs, inter-band CA is not prohibited from the signaling perspective

- Note: Depending on the TCI state configuration across different CCs (configured via RRC), the TCI-state ID may not refer to the same  TCI state for different CCs

Agreement@RAN1#99

On simultaneous spatial relation update for SP/AP-SRS resources via MAC CE (agreed in RAN1#98bis) where the same spatial relation is applied for the all BWPs in the configured CCs, inter-band CA is not prohibited from the signaling perspective

MPE related issues for beam management enhancements

Agreement@RAN1#97

Down-select in RAN1#98 from the following options for beam management enhancements:

- Alt1. Support UE to report CRI/SSBRI where the CRI/SSBRI refers to a preferred spatial relation RS for UL transmission

  - FFS: Whether to support SRI in addition to CRI/SSBRI

  - FFS on details of the reporting configuration (e.g. separate or joint reporting with existing DL beam reporting, introduction of new information from UE such as MPR)

- Alt2. Support SRI field in the DCI can be used to indicate multiple SRS resources and UE’s autonomous selection of one SRS resource for PUSCH beam determination out of the multiple

- Alt3: Reuse Rel-15 beam specific PHR reporting to determine beam-specific MPE impact transparently, i.e., by difference value between Pc,max (which is calculated based on P-MPR) and the required transmission power.

  - FFS: Enhancement on UL beam configuration for virtual PHR.

- Alt4: No enhancements considering MPE issues in Rel-16 RAN1 specifications. It is up to UE implementation in conjunction to RAN4 specicfiation support.

If no consensus in RAN1#98, no further discussion in RAN1.

Conclusion@RAN1#98

On the issue of beam management enhancements to address MPE, there is no consensus in RAN1 to introduce additional specification support in RAN1. No further discussion for Rel-16 from RAN1 perspective.

Other DL/UL beam management enhancements

Agreement@RAN1#AH1901

For latency and overhead reduction for DL beam management,

- No new CSI-RS design and no new term such as ‘sub-time unit’ or ‘sub-symbol’ are introduced in Rel-16, i.e., no support of sub-time unit for beam management RS shorter than 1 OFDM symbol

Companies can provide further evaluation results and proposals for faster DL beam operation other than those requiring sub-time unit

Agreement@RAN1#AH1901

For UL beam management latency reduction in controlling PUCCH spatial relation, the maximum RRC configurable number of spatial relations for PUCCH (i.e., maxNrofSpatialRelationInfos) is increased to be 64 per BWP.

- FFS: RRC and/or MAC CE signaling overhead reduction related to this.

Agreement@RAN1#96

In RAN1#96bis, determine whether to support the configuration of up to 64 candidate beams for BFR by RRC signaling.

- FFS signaling details including whether MAC-CE message can choose a subset of the candidate beams as active resources for new beam identification in Rel-16

Agreement@RAN1#96bis

Support the configuration of up to 64 candidate beams for BFR by RRC signalling, without introducing additional MAC CE signalling for down-selecting a subset of beams.

- The total number of RSs for new beam identification and layer 1 RSRP measurement are part of UE capability signaling

This applies per BWP.