# Introduction

In this summary, the agreements made on Rel.16 NR_eMIMO MU-MIMO CSI to-date are compiled and systematically arranged ([1][2][3][4][5]). The main purpose is to identify missing parts, if any, as well as to aid specification editors. Therefore:

- Only agreements pertinent to specification are included.

- For a given topic/component, the captured agreement represents the most mature form. Redundancies (including most FFS items that have been agreed) are removed.

Summary

## Structure of DFT-based compression

For Rel-16 NR, agree on DFT-based compression as the adopted Type II rank 1-2 overhead reduction (compression) scheme

- Wf is determined from a set of predefined DFT vectors.

- PMI feedback: W1 (wideband), W2, possibly a small number of parameters for Wf (wideband)

- Detailed formulation is given below

- The same DFT-based compression scheme is extended for Type II port selection codebook as follows:

  - For rank 1-2, reuse Rel.15 Type II W1 port selection matrix for Rel.16 Type II port selection codebook

    - Only L=2 and 4 are supported

  - For rank 3-4, support extension with simple reuse of Rel.15 W1 matrix (i.e. layer-common W1)

Table 1 Structure

- Precoders for a layer is given by size-P×N3 matrix W=W1W2WfH - P=2N1N2= #SD dimensions - N3= #FD dimensions - Precoder normalization: the precoding matrix for given rank and unit of N3 is normalized to norm 1/sqrt(rank) - Spatial domain (SD) compression - L spatial domain basis vectors (mapped to the two polarizations, so 2L in total) selected - Compression in spatial domain using W1=v0v1…vL-100v0v1…vL-1, where vii=0L-1 are N1N2×1 orthogonal DFT vectors (same as Rel. 15 Type II) - Frequency-domain (FD) compression - Compression via Wf=fk0 fk1…fkM-1, where fkmm=0M-1 are M size-N3×1 orthogonal DFT vectors - Number of FD-components M is configurable - Note: The terminology “SD-compression” and “FD-compression” are for discussion purposes only and are not intended to be captured in the specification

Codebook subset restriction

Support scheme 1 as mandatory and scheme 2 as optional analogous to Rel.15 Type II codebook.

- Scheme 1: Analogous to Rel.15 Type II (SD beam group restriction + per coefficient hard amplitude restriction)

  - Four beam groups are selected via higher-layer configured bitmap B1

  - For each spatial beam in each of the four beam groups, hard restriction (maximum amplitude of 0 or 1) is applied to any of the coefficients associated with the beam (the restriction is applied for both polarizations of the beam). This maximum amplitude restriction is higher-layer configured with four bitmaps

- Scheme 2: Type II SD beam group restriction + soft sum-power restriction,

  - Four beam groups are selected via higher-layer configured bitmap B1

  - For each beam  in each of the four beam groups, power ratio threshold  is configured, the following criterion should be satisfied: 1KNZ(λ,k,l0)×m=0M-1pres,l02λ,m,k≤γl0 for each  where KNZ(λ,k,l0) denotes the number of NZCs associated with (λ,k,l0).

    - The value of power threshold γl0 is configured from the Rel.15 2-bit amplitude restriction table

    - The number of beam-groups is the same as Rel.15 Type II CBSR

- Cf. Notes in UE capability agreement in RAN1#98bis regarding “mandatory” and “optional” (cf. section 2.5)

On the UE behavior for Rel.16 Type II CBSR:

- When the UE does not support soft amplitude restriction as indicated by the respective UE capability signaling, the UE is not expected to be configured with amplitude restriction values other than “0” and “1”

## Basic compression parameters

Spatial domain compression parameters

On the values of L, support

- L={2,4}

- L=6 for the following combinations of p and beta

  - p value equals to 1/4, beta value equals to {1/4, ½, 3/4}

  - p value equals to 1/2, beta value equals to ¼

  - Above applies only for the case of 32 ports, rank 1 or 2, R=1

  - Note that the payload size for L=6 should not exceed that of Rel-15 type-2 codebook

  - The above feature is UE optional

- [Inferred from late agreement on (L,p)] The value of L is higher-layer configured

Inherited from Rel.15: N1, N2 (along with the values) are higher-layer configured. (O1,O2)=(4,4).

Frequency domain compression parameters

On the choice of oversampling factor O3, agree on O3=1

- The rotation factor q3 is therefore not needed

On FD compression unit, agree on {PMI subband size = CQI subband size} as the default, along with {PMI subband size = CQI subband size / R} as secondary

- The value of R is fixed to 2

- The FD compression unit parameter R is higher-layer configured

On the value of M (the number of FD compression units), M=p×N3R where.

- The value of M is higher-layer configured [via R and p]

Values of N3: For R R∈1,2and NSB (the number of CQI subbands), , is higher-layer configured

Parameter setting across layers and RI values

The parameter R is layer-common and RI-common

For the higher-layer setting of SD/FD basis parameters (L, p):

- The parameter p=v0 for RI=3-4 is higher-layer configured in conjunction with the parameter p=y0 for RI=1-2.

- The parameters (y0, v0) can take value from {(1/2,1/4),(1/4,1/4),(1/4,1/8)}

Table 2 (L,p) setting12,14,14,14,14,18

| RI | Layer | L | p |
| --- | --- | --- | --- |
| 1 | 0 | x0 | y0 |
| 2 | 0 |  |  |
|  | 1 |  |  |
| 3 | 0 |  | v0 |
|  | 1 |  |  |
|  | 2 |  |  |
| 4 | 0 |  |  |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |

Subset selection (as a means of compression)

Terms:

- “FD basis subset selection” refers to the selection of M out of N3 FD DFT vectors

- “Coefficient subset selection” refers to the selection of KNZ (# non-zero coefficients) out of 2LM where KNZ ≤ K0

On basis/coefficient subset selection for RI=1, the following is supported:

- Common selection for all beams with size-K0 subset of 2LM reported

  - Note: Wf=fk0 fk1…fkM-1, i.e. Mi=M ∀i and ki,mm=0Mi-1 are identical (i.e., ki,m=km, i=0,…,2L-1), and kmm=0M-1 are all selected from the index set {0,1,…,N3-1} from the same orthogonal basis group

  - The value of K0 is configured via higher-layer signaling

    - The number of reported non-zero coefficients can be smaller than or equal to K0

- Size-K0 subset design: Unrestricted (polarization-independent) subset selection which requires a size-2LM bitmap in UCI part 2

- The value of K0:  where  (the value of K0 is higher-layer configured via ).β∈18,14,12,34

On subset selection for RI=2:

- SD basis selection (selection of L out of N1N2 SD DFT vectors) is layer-common

- Layer-independent FD basis subset selection

- Layer-independent coefficient subset selection

- The size-K0 subset design for layer 0 is also applied to layer 1

- K0 is the maximum number of non-zero coefficients for each layer.

On RI=3-4 extension:

- SD basis subset selection is layer-common

- Layer-independent FD basis subset selection

- Layer-independent coefficient subset selection

- With the agreed total max # NZ coefficients across all layers ≤ 2K0 where the K0 value (hence β) set for RI{1,2}, KNZ,i≤K0 (the number of non-zero coefficients per layer shall be less than or equal to K0) as long as i=0RI-1KNZ,i≤2K0

  - No further restriction on the maximum number of non-zero coefficients per layer

The following FD basis subset selection scheme is supported:

- For N3≤19N3≤19, one-step free selection is used:

  - FD basis subset selection indicator is per layer where it is log2N3-1Mi-1-bit indicator, (i=0,1,…,(RI-1))

- For N3>19, IntS is window-based and fully parameterized with MinitialMinitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …,  where

  - is selected by the UE and reported in UCI part 2modMinitial+n,N3, n=0,1,⋯N3'-1

- The 2nd step subset selection is indicated by an X2-bit combinatorial indicator (for each layer) in UCI part 2

Supported parameter combinations

Table 3 Parameter combination L,p,β,α

| Parameter combination | L | p = y0 (RI= 1-2) | p = v0 (RI= 3-4) | β | Restriction (if any) |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | ¼ | 1/8 | ¼ |  |
| 2 | 2 | ¼ | 1/8 | ½ |  |
| 3 | 4 | ¼ | 1/8 | ¼ |  |
| 4 | 4 | ¼ | 1/8 | ½ |  |
| 5 | 4 | ¼ | ¼ | ¾ |  |
| 6 | 4 | ½ | ¼ | ½ |  |
| 7 | 6 | ¼ | - | ½ | RI=1-2, 32 ports |
| 8 | 6 | ¼ | - | ¾ | RI=1-2, 32 ports |

## LCC quantization

On LCC quantization, denote the LC coefficient associated with beam l ∈0,1,…,2L-1 and frequency unit m ∈0,1,…,M-1 as cl,m, and the strongest coefficient (out of the KNZ non-zero coefficients reported using the bitmap) as cl*,m*.

UE reports the following for the quantization of the non-zero coefficients in W2

- (For RI=1) A log2KNZ-bit indicator for the strongest coefficient index l*,m*

  - Strongest coefficient cl*,m*=1 (hence its amplitude/phase are not reported)

- Two polarization-specific reference amplitudes:

  - For the polarization associated with the strongest coefficient cl*,m*=1, since the reference amplitude = 1, it is not reported

  - For the other polarization, reference amplitude is quantized to 4 bits

    - The alphabet is {1,1214,1414,1814,…,121414 , 0} (-1.5dB step size)

- For cl,m,  l,m≠(l*,m*):

  - For each polarization, differential amplitudes of the coefficients calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

    - The alphabet is {1,12,12,122,14,142,18,182} (-3dB step size)

    - Note: The final quantized amplitude pl,m is preflL×pdiffl,m and the coefficient is given by cl,m=preflL×pdiffl,m×φ(l,m)

- Each phase is quantized to 16PSK (4-bit)

On “zero” in the reference amplitude value set, “zero” is removed and the associated code point is designated as “reserved”.

- Note: “Reserved” implies that the associated code point is not used in reference amplitude reporting or, at least in Rel-16, any other purpose(s)

For RI∈2,3,4, different layers are independently quantized.

## UCI parameters

UCI content

The UCI consists of two parts:

- Information pertaining to the number(s) of non-zero coefficients is reported in UCI part 1

  - Note: This does not imply whether this information consists of single or multiple values

- The payload of UCI part 1 remains the same for different RI value(s)

- Bitmap is used to indicate non-zero coefficient indices

Table 4 UCI parameter list

| Parameter | Location | Details/description |
| --- | --- | --- |
| # NZ coefficients | UCI part 1 | RI ({1,…, RIMAX}) and KNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOT {1,2,…, 2K0} are reported in UCI part 1 |
| Wideband CQI | UCI part 1 | Same as R15 |
| Subband CQI | UCI part 1 | Same as R15 |
| Bitmap per layer | UCI part 2 | RI=1-2: for layer l, size-2LM For RI=3-4, bitmaps, each with size-2LMi (i=0,1,…, RI-1, where i denotes the i-th layer) are reported in UCI part 2 |
| Strongest coefficient indicator (SCI) | UCI part 2 | RI=1: the existing agreement on SCI: “A log2KNZ-bit indicator for the strongest coefficient index l*,m*” RI>1: See Table below |
| SD basis subset selection indicator | UCI part 2 | SD basis subset selection indicator is a -bit indicator. Details follow Rel.15 |
| FD basis subset selection indicator | UCI part 2 | See Table below |
| LC coefficients: phase | UCI part 2 | Quantized independently across layers |
| LC coefficients: amplitude | UCI part 2 | Quantized independently across layers (including reference amplitude for weaker polarization, for each layer) |
| SD oversampling (rotation) factor q1, q2 | UCI part 2 | Values of q1, q2 follow Rel.15 |

Table 5 SCI and FD basis indication

| SCI and FD basis subset selection indicator |  |
| --- | --- |
| SCI for RI>1 | Per-layer SCI, where SCIi is a log22L–bit (i=0,1,…,(RI-1)) indicator. The location (index) of the strongest LC coefficient for layer i before index remapping is (li*,mi*), SCIiindicates li*, and mi* is not reported |
| Index remapping | For layer i, the index mi of each nonzero LC coefficient cli,mi is remapped with respect to mi* to mi such that mi*=0. The FD basis index kmi associated to each nonzero LC coefficient cli,mi is remapped with respect to kmi* to kmi such that kmi*=0. The sets cli,mi≠cli*,0 and kmi≠0 are reported. Informative note (for the purpose of reference procedure): The index li,mi of nonzero LC coefficients is remapped as (li,mi)→(li,mi-mi*modMi). The codebook index associated with nonzero LC coefficient index li,mi is remapped as kmi→kmi-kmi*modN3. |
| Combinatorial indicator for N3≤19 | log2N3-1Mi-1 bits |
| Combinatorial indicator for N3>19 | log2N3'-1Mi-1 bits |
| Minitial | Reported in UCI part 2, , bits |

UCI omission

Note: CSI omission occurs when the allocated UL resource for UCI is not sufficient for full CSI reporting.

Denote the non-zero LC coefficient (NZC) associated with layer , beam , and FD-basis  as . The associated bitmap component (including zero(s)) is.

For the purpose of UCI omission, the parameters in UCI Part 2 is divided into 3 groups where Group n is of a higher priority than Group (n+1), n=0, 1.

When the UE is configured to report NRep CSI reports,

- Group 0 includes: SD rotation factors, SD indicator, and SCI(s) for all the NRep reports

- Priority level definition: If priority levels of two LCCs and are such that Prio(,l2,m2)< Prio(,l1,m1), LCC  has a higher priority over

  - Non-zero LC coefficients and bits of bitmap  are prioritized/ordered from high to low priority according to (,l,m) with the same priority function Prio(,l,m)

- The priority level is calculated as Prio(,l,m)=2L.RI.P(m)+RI.l+ where P(m) maps the index m according to the following order of the corresponding FD components (if selected): 0, N3-1, 1, N3-2, 2, ....

- For each of the NRep reports, Group 1 includes: reference amplitude(s) for weaker polarization, FD indicator, the  highest priority non-zero LC coefficients  () and the  highest priority bits of bitmap

- For each of the NRep reports, Group 2 includes: the  lowest priority non-zero LC coefficients  () and the  lowest priority bits of bitmap

- Note: G1 and G2 exclude the indices associated with the strongest coefficient(s)

For Rel.16 Type II codebook, the mapping order for UCI bits can be described by the following example text for TS38.212:

Table 6.3.2.1.2-5A: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeIIr16 or typeIIr16-PortSelection

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Tables 6.3.2.1.2-1A/2A, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: i2,3,l:l=1,…,ν, i1,5, i1,6,l and (KNZ, TOT2-ν)×3 highest priority bits of i2,4,l:l=1,…,ν,(KNZ, TOT/2-ν)×4 highest priority bits of {i2,5,l:l=1,…,ν} and ν*2LM-KNZ, TOT/2 highest priority bits of i1,7,l:l=1,…,ν, in decreasing order of priority based on function Pril,i,f defined in section 5.2.3 of TS38.214, if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: KNZ, TOT/2×3 lowest priority bits of i2,4,l:l=1,…,ν, KNZ, TOT/2×4 lowest priority bits of i2,5,l:l=1,…,ν and KNZ, TOT/2 lowest priority bits of i1,7,l:l=1,…,ν, in decreasing order of priority based on function Pril,i,f defined in section 5.2.3 of TS38.214, if reported |

Further details on CSI reporting for Rel.16 Type II codebook

SP CSI reporting of Part-1-only CSI on PUCCH format 3 and 4 is not supported

When R=2:

- When the actual CQI sub-band size is equal to the configured CQI sub-band size, the CQI sub-band is split into two equal parts wherein each part corresponds to a PMI sub-band.

- For edge CQI sub-bands:

  - When an edge CQI sub-band size is less than or equal to the half of the configured CQI sub-band size, there is only one PMI sub-band with the same size as the edge CQI sub-band.

  - Otherwise, when an edge CQI sub-band size is greater than half of the configured CQI sub-band size, there are two PMI sub-bands wherein the size of the edge PMI sub-band is smaller than the other PMI sub-band (whose size is half of the configured CQI sub-band size).

## UE capability related issues

Note: for discussion purposes:

- “Mandatory” implies that the (sub-)feature is always supported when the UE is capable of Rel.16 Type II codebook. In other words, this feature is considered basic. Rel.16 Type II codebook is a UE optional feature.

- “Optional” implies that a separate UE (sub-)capability is needed (hence not necessarily supported) even when the UE is capable of Rel.16 Type II codebook. In other words, this feature is considered advanced.

For a UE capable of Rel.16 Type II codebook, agree on the following:

- Mandatory support for L=2, 4

  - Supported without additional UE capability signaling

- Mandatory support for maximum rank of 1 and 2

  - Supported without additional UE capability signaling

- The support for maximum rank of 3 is optional

- The support for maximum rank of 4 is optional

- Separate UE capabilities for the “regular” Rel.16 Type II and Rel.16 Type II port selection codebooks

- The support for parameter combinations 1, 2, 3, 4, 5, and 6 is mandatory, while the support for parameter combinations 7 and 8 is optional.

- On the # of PMI subbands:

  - Mandatory support for R=1

  - Optional support for N3>19

  - Whether it is mandatory or optional for R=2 and N3<=19 is to be discussed during the Rel-16 UE capability discussions

In Rel-16, for CSI report other than beam report:

- Increase the maximum number of configured aperiodic CSI Report Settings a UE can report per BWP (RRC parameter maxNumberAperiodicCSI-PerBWP-ForCSI-Report) up to 8, subject to UE capability

  - The maximum number of configured aperiodic CSI Report Settings a UE supports is part of UE capability

- Note: This increases the value range of the associated UE capability signaling from 1...4 to 1...8

- Note: FFS for FR2 whether UE can report a value larger than 4

# References

- RP-182320, RAN#82, Samsung, Status report to TSG: Enhancements on MIMO for NR

- RP-190149, RAN#83, Samsung, Status report to TSG: Enhancements on MIMO for NR

- RP-191187, RAN#84, Samsung, Status report to TSG: Enhancements on MIMO for NR

- 3GPP RAN1, RAN1#98, Chairman Notes

- 3GPP RAN1, RAN1#98bis, Chairman Notes

- 3GPP RAN1, RAN1#99, Chairman Notes