# Agreements upto RAN1#99

UL full power transmission scheme

Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs

- This specification support is a UE optional feature

Note: UE capability 1, 2, 3 agreed in RAN1#AH1901 mean the PA architectures.

- UE capability 1: for the UE to support full Tx power in UL transmission, full rated PAs on each Tx chain is supported with a new UE capability

- UE capability 2: for the UE to support full Tx power in UL transmission, no Tx chain is assumed to deliver full power with the new UE capability

- UE capability 3: for the UE to support full Tx power in UL transmission, subset of Tx chains with full rated PAs is supported with a new UE capability

Support following scheme for UL full power Tx for UE capability 2 and 3:

- A UE can be configured for one of two modes of full power operation to support ‘Capability 2’ and ‘Capability 3’ subject to UE capability

- A UE can be configured by the network to support full power transmission

- Mode 1: The UE can be configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within an SRS resource set which usage is set to ‘codebook’

  - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission.

  - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15

    - For mode 1, 2Tx non-coherent UE, the new codebook subset at least includes rank=1 TPMI=2 defined in Rel-15 which can be used for UL full power transmission

      - For rank=1, TPMI=2, TPMI=0, TPMI=1 are included in new codebook subset for non-coherent UEs with power scaling defined as in [38.213] Rel-15

      - For rank=2, TPMI=0 is included in the new codebook subset

    - For mode 1, 4Tx non-coherent UE, the new codebook subset at least includes, rank 1 TPMI= 13 defined in Rel-15, at least includes, rank 2 TPMI=6 defined in Rel-15, at least includes, rank 3 TPMI=1 defined in Rel-15 which can be used for UL full power transmission

    - For Mode 1 4TX, for non-full power uplink transmission, antenna selection precoders are included in the new codebook subset following Rel-15 power scaling factor

    - For full power uplink transmission Mode 1, 4TX partial-coherent, the new codebook subset includes

      - Rank1(CP-OFDM): TPMI = 12,13,14,15

      - Rank1(DFT-s-OFDM): TPMI = 12,13,14,15

        - FFS: TPMI=16, 17, 18, 19

- Mode 2: The UE can be configured with one SRS resource or multiple SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’

  - UE transmits SRS and PUSCH in same manner, whether antenna virtualization is used or not

  - Rel-15 codebooks and codebook subsets are used

  - Note: Antenna selection precoder can be used to enable full power related PA(s) to produce full power transmission for Capability-3 UE.

  - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI

    - For mode 2, in case of non-coherent with 2 ports, support following TPMI indication for rank 1 which support UL full power transmission:

      - Rank 1: support {TPMI=0} and {TPMI=1}

      - For 2 ports, number of bits to indicate TPMI(s) which can deliver UL full power:

        - 2 bits (bitmap)

      - Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions

    - For mode 2, for 4 ports, number of bits to indicate TPMI(s) which can deliver UL full power:

      - Non Coherent 2 bits

      - Partial coherent 4 bits

        - Additional entries on top of existing entries may be added to table 1 and table 2

      - Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions

  - For 4 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set

    - Depending on UE capability, either up to 2 or 4 SRS resources are supported

  - For 2 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set

    - Depending on UE capability, either up to 2 or 4 SRS resources are supported

  - For mode 2 UEs, up to 2 different spatial relation info can be configured for all SRS resources with usage set to ‘codebook’

  - The size of precoding information and number of layers field in DCI is determined by the maximum number of ports among the SRS resources in the SRS resource set with usage of codebook.

    - If the number of ports for a configured SRS resource is less than the maximum SRS port number among the configured SRS resources, the most significant bit(s) shall be reserved.

Power scaling

For the the 2TX and 4TX case, the linear value of power after power scaling, is divided equally among the non-zero PUSCH ports

- The above applies for the cases including when UE transmitting at P_c_max

For a capability 1 UE working with full power operations, for PUSCH power control, power scaling factor is fixed to 1

For Mode1,

- 2Tx, non-coherent UEs with power scaling defined as in [38.213] Rel-15

- 4TX, for non-full power uplink transmission, antenna selection precoders are included in the new codebook subset following Rel-15 power scaling factor

For Mode2

- Power scaling factor is equal to 1 for the reported TPMI precoders that supports full power Tx

- for the other TPMI precoders, if only one SRS resource is configured, the power scaling factor is determined by #non-zero-PUSCH-port divided by #SRS-ports

- for the other TPMI precoders, the power scaling factor is determined by #non-zero PUSCH port/#SRS ports in the SRS resource indicated by SRI

UE capability

- At least for PC3, UE capability 1, 3 can support full power transmission.

- Regardless of UE capability 1, 2, or 3, signaling of “UL full power tx capability” is supported for UEs with full power uplink transmission capability

- The signaling of above information does not imply any specific UE PA architecture implementation.

- Note: Support of Mode 1, Mode 2 have separate UE capability

- Note: UE capability 1, 2, 3 agreed in RAN1#AH1901 mean the PA architectures.

RRC configuration

- Support RRC configuration to operate in Mode1 or Mode2 subject to UE capability

- RRC parameters ULFPTx, ULFPTxModes are configured per UL BWP

# Previous agrements per meeting

RAN1#94bis agreements

Agreement

Consider the following potential solutions and other solutions (such as combination of the solutions below) for UL full power transmission. Decision will be made in RAN1#95:

Option 1: Refinement/adjustment of UL codebook is supported

  - 1-1: Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs

  - 1-2: Introduce additional scaling factor for uplink codebook

Option 2: UE transparently apply a small cyclic or linear delay

Option 3: Power control mechanism to be modified to support UL full power transmission without precluding the use of full rated PA(s)

  - Note: Full rated PA refers to a PA having power not lower than that of the power class

Option 4: Up to UE implementation (no specification impact)

RAN1#95 agreements

Agreement

Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs

- This specification support is a UE optional feature

- FFS: Whether this applies for the entire codebook or subset of codebook

Agreement

Full TX power UL transmission, one additional option (option 5) is added

Option5: For the precoders with 0 entries, the linear value PPUSCH,b,f,c(i,j,qd,l) of a PUSCH transmission power is scaled by a ratio Rel-16.  The value of Rel-16 is selected up to UE implementation within the range of [Rel-15, 1],  where Rel-15 is the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the number of configured antenna ports for the PUSCH transmission scheme as defined in NR Rel-15 specification.

- UE is required to maintain consistent Rel-16αRel-16 value on different occasions of PUSCH transmissions with the same precoder for PUSCH

Agreement

Full TX power UL transmission, option 4 is updated as follows

Option 4: Up to UE implementation with UE capability signalling of full power transmission in UL (no specification impact)

For guidance in future RAN1 discussions:

Understanding of antenna virtualization for ease of discussion (not for specification):

- A UE forms an antenna port by transmitting on one or more TX chains (each with a power amplifier)

RAN1#AH1901 agreements

Agreement

Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs. The support of this feature is indicated by the UE as part of UE capability signalling. For power class 3:

- UE capability 1: for the UE to support full Tx power in UL transmission, full rated PAs on each Tx chain is supported with a new UE capability

  - FFS: detailed power scaling description

  - Note: Full Tx power means UE delivers total power of 23dBm for PC3

- UE capability 2: for the UE to support full Tx power in UL transmission, no Tx chain is assumed to deliver full power with the new UE capability

  - FFS: detailed design

- UE capability 3: for the UE to support full Tx power in UL transmission, subset of Tx chains with full rated PAs is supported with a new UE capability

FFS: Whether all three capabilities will be specified or a subset will be specified

FFS: UE capability signalling/reporting details

Note: Two or more of the above capabilities could be merged depending on the further details

Send LS to RAN4 to provide their view on PC 2 applicability of the new UE capability (Rakesh, vivo).

The LS is endorsed in principle with revisions. LS is endorsed in R1-1901440

RAN1#96 agreements

Agreement

Note: UE capability 1, 2, 3 agreed in RAN1#AH1901 mean the PA architectures.

At least for PC3, UE capability 1, 3 can support full power transmission.

Working assumption: For PC3, UE capability 2 can support full power transmission.

- Companies to check for any implementation issues and/or performance of Rel-16 full power transmission compared to Rel-15 non-coherent codebook subset uplink transmission)

Agreement

Down select among the following two alternatives by RAN1#96bis. As part of UE capabilities signalled the following is included:

Alt1: UE capability signaling of supported one or group of TPMI precoder(s)

Alt3: UE capability signaling of power scaling schemes for full uplink power transmission

- Note: This does not imply any restriction on UE antenna virtualization

- FFS: Whether full uplink power transmission needs to be supported for all precoders

RAN1#96bis agreements

Agreement

For the the 2TX and 4TX case, the linear value of power after power scaling, is divided equally among the non-zero PUSCH ports

- The above applies for the cases including when UE transmitting at P_c_max

Agreement

Supported UE capabilities and supported scheme for UE capability 1

- Option 3

- FFS: Whether to additionally support Option 1-2

Agreement

Regardless of UE capability 1, 2, or 3, signalling of “UL full power tx capability” is supported for UEs with full power uplink transmission capability

- FFS: For UE capability 1, if any other information is necessary

- For UE capability 2 and UE capability 3, in addition to signalling “UL full power tx capability”, further information on UE capability are signalled if needed

  - FFS: Details such as support of UE capability signaling of supported one or group of TPMI precoder(s) for full power transmission, support different number of SRS ports for resources for codebook, and other UE capability signaling can be introduced

  - FFS: Whether full uplink TX power capability can be explicitly/implicitly derived from the TPMI/TPMI group precoders for full power transmission

UEs with full power uplink transmissions are those Rel-16 UEs which can transmit at full power at least for rank1

The signalling of above information does not imply any specific UE PA architecture implementation.

Agreement

RAN1 will select one of the alternative solutions below to support UE capability 2. Further clarification or details are needed for Alt1, Alt3-1, Alt3-2, and Alt5. Email discussion by 17th of April for companies to provide clarification on Alt1, Alt3-1, Alt3-2, and Alt5. To be coordinated by Rakesh (vivo).

- Alt1: Option1-1 (Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs, e.g. for 2Tx the new codeboookSubset is all non-antenna selection TPMIs or with only TPMI [1 1] for rank 1)

- Alt3-1: Option3+Option2 (Multiple SRS resources with different number of SRS port(s) in each resource)

  - FFS: Whether to additionally support Option 1-2

- Alt3-2: Option3+Option2+ Option1-1 (Multiple SRS resources with different number of SRS port(s) in each resource)

- Alt5: FDM multi-port simultaneous transmission

RAN1#97 agreements

Working Assumption

Support following scheme for UL full power Tx for UE capability 2 and 3:

- A UE can be configured for one of two modes of full power operation to support ‘Capability 2’ and ‘Capability 3’ subject to UE capability

- A UE can be configured by the network to support full power transmission

- Mode 1: The UE can be configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within an SRS resource set which usage is set to ‘codebook’

  - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission.

  - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15

    - FFS: At least a subset of the non-antenna selection TPMI precoder(s) is(are) supported

    - FFS: Additional support of antenna selection TPMI precoders

  - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI

- Mode 2: The UE can be configured with one SRS resource or multiple SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’

  - UE transmits SRS and PUSCH in same manner, whether antenna virtualization is used or not

  - Rel-15 codebooks and codebook subsets are used

    - Note: Antenna selection precoder can be used to enable full power related PA(s) to produce full power transmission for Capability-3 UE.

  - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI

    - A set of TPMIs that deliver full power can be signalled by the UE in order to support at least  UEcap3, for SRS resource with more than 1 ports,

    - e.g. For SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted with full power in same manner as single port SRS, if SRI indicating SRS resource with multiple ports is signalled based on Rel-15 MIMO behaviour (transmission rank indicator, TPMI indicator, etc) except the power scaling aspects

    - The following cases are not precluded

      - For example, for 4TX on UE side (with 20+20+17+17dBm) virtualized as 2 SRS ports, full uplink power transmission can be enabled by precoder [1 0] or [0 1]

    - FFS: number of SRS resources supported

      - 2

      - 3

    - FFS: for 4 Tx, how many different TPMIs/TPMI groups support full power

  - FFS: any rules for spatial filter update for the SRS resources with different number ports

Note: How to capture the behaviour for ‘Mode 1’ and ‘Mode 2’ in specifications is TBD

Note: For single port, there is no SRI and TPMI

Note: Support of Mode 1, Mode 2 have separate UE capability

RAN1#98 agreements

Agreement

For mode 1, 2Tx non-coherent UE, the new codebook subset at least includes rank=1 TPMI=2 defined in Rel-15 which can be used for UL full power transmission

Agreement

For mode 1, 4Tx non-coherent UE, the new codebook subset at least includes, rank 1 TPMI= 13 defined in Rel-15 which can be used for UL full power transmission

- FFS for the case that part of ports can deliver full power transmission

Agreement

For mode 1, 4Tx non-coherent UE, the new codebook subset

- at least includes, rank 2 TPMI=6 defined in Rel-15

- at least includes, rank 3 TPMI=1 defined in Rel-15

Agreement

For mode 2, in case of non-coherent with 2 ports, support following TPMI indication for rank 1 which support UL full power transmission:

- Rank 1: support {TPMI=0} and {TPMI=1}

- FFS: Details on UE capability signalling

Conclusion

For mode 2, no additional rule for spatial filter update for SRS resources with different number ports

Agreement

For a capability 1 UE working with full power operations, for PUSCH power control, power scaling factor is fixed to 1

Agreement

For a UE working with Mode1 operation, for PUSCH power control, down-select or merge from the following alternatives in RAN1#98bis

- Alt1: reuse Rel-15 power scaling mechanism.

- Alt2: power scaling factor is configured.

- Alt3: power scaling factor is determinded by #non-zero-PUSCH-port divided by #SRS-ports in the SRS resource indicated by SRI.

- Alt4: A UE can scale its transmit power by  to reach full power, where

  - Kρ,m,ν>1 is a scale factor associated with  ρ∈{1,2,4}SRS ports corresponding to the PUSCH transmission and an optional mth TPMI with rank v.

    - If a TPMI is not associated with Kρ,m,ν, then  is determined without regard to m and v.

    - If Kρ,m,ν is not configured by higher layers, a set of fixed values are defined for K(ρ).

  - ρ0is the number of non-zero PUSCH ports being transmitted

- Alt5: For the precoders in the new codebook subset for full power transmission, the power scaling factor is 1.

Agreement

For a UE working with Mode2 operation, for PUSCH power control, down-select or merge from the following alternatives in RAN1#98bis

- Alt1: power scaling factor is determinded by the reported TPMI precoders.

- Alt2: power scaling factor is configured.

- Alt3: power scaling factor is determinded by #non-zero-PUSCH-port divided by #SRS-ports in the SRS resource indicated by SRI.

- Alt4: A UE can scale its transmit power by min⁡(1,K(ρ,m,ν)ρ0/ρ) to reach full power, where

  - Kρ,m,ν>1 is a scale factor associated with ρ∈{1,2,4} SRS ports corresponding to the PUSCH transmission and an optional mth TPMI with rank v.

    - If a TPMI is not associated with Kρ,m,ν, then Kρ,m,ν is determined without regard to m and v.

    - If Kρ,m,ν is not configured by higher layers, a set of fixed values are defined for K(ρ).

  - is the number of non-zero PUSCH ports being transmitted

Agreement

- For 4 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set

  - Depending on UE capability, either up to 2 or 4 SRS resources are supported

- For 2 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set

  - Depending on UE capability, either up to 2 or 4 SRS resources are supported

- For mode 2 UEs, up to 2 different spatial relation info can be configured for all SRS resources with usage set to ‘codebook’

Note: it does not mean to support simultaneous transmission of multiple SRS resources usage is set to ‘codebook’

RAN1#98bis agreements

Agreement

- Support RRC configuration to operate in Mode1 or Mode2 subject to UE capability

  - For UE capabilty-2 and-3, gNB can configure a UE to operate in Mode 1 or Mode 2 subject to UE capability

    - Note : if UE only supports Mode 1 gNB cannot configure this UE to operate in Mode 2, if UE only supports Mode 2 gNB cannot configure this UE to operate in Mode 1

  - FFS: UE capability signaling discussion

  - Note: capability-1 UE can be configured with RRC parameter “ULFPTx” to deliver UL full power has been agreed, exact parameter name is up to RAN2

- If gNB does not configure UE for Rel-16 full power UL transmission, Rel-16 UEs operate in Rel-15 behavior

Agreement

For 2Tx in mode 1,

- For rank=1, TPMI=2, TPMI=0, TPMI=1 are included in new codebook subset for non-coherent UEs with power scaling defined as in [38.213] Rel-15

- For rank=2, TPMI=0 is included in the new codebook subset

Agreement

For Mode 1 4TX, for non-full power uplink transmission, antenna selection precoders are included in the new codebook subset following Rel-15 power scaling factor

- FFS: Whether to include antenna selection precoders for full power uplink transmission

Agreement

For full power uplink transmission Mode 1, 4TX partial-coherent, the new codebook subset includes

- Rank1(CP-OFDM): TPMI = 12,13,14,15

- Rank1(DFT-s-OFDM): TPMI = 12,13,14,15

  - FFS: TPMI=16, 17, 18, 19

- FFS: Whether clarification on which port pairs are coherent is needed

Agreement

For Mode2,

- Power scaling factor is equal to 1 for the reported TPMI precoders that supports full power Tx

- for the other TPMI precoders, if only one SRS resource is configured, the power scaling factor is determined by #non-zero-PUSCH-port divided by #SRS-ports

- for the other TPMI precoders, the power scaling factor is determined by #non-zero PUSCH port/#SRS ports in the SRS resource indicated by SRI

RAN1#99 agreements

Agreement

The size of precoding information and number of layers field in DCI is determined by the maximum number of ports among the SRS resources in the SRS resource set with usage of codebook.

  - If the number of ports for a configured SRS resource is less than the maximum SRS port number among the configured SRS resources, the most significant bit(s) shall be reserved.

Agreement

- RRC parameters ULFPTx, ULFPTxModes are configured per UL BWP

Agreement

For 2 ports, number of bits to indicate TPMI(s) which can deliver UL full power:

- 2 bits (bitmap)

- Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions

Agreement

For 4 ports, number of bits to indicate TPMI(s) which can deliver UL full power:

  - Non Coherent 2 bits

  - Partial coherent 4 bits

    - Additional entries on top of existing entries may be added to table 1 and table 2

  - Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions

Table 1.

| 4Tx, nonCoherent | 4Tx, partial coherent (4bit) |
| --- | --- |
| G0 | G0 |
| G1 | G1 |
| G2 | G2 |
| G3 | G3 |
|  | G4 |
|  | G5 |
|  | G6 |

Definition of G0~G6 can be found in the table below.

Table 2.