3GPP TSG-RAN WG1 Meeting #99	R1-1913620

# Reno, USA, November 18-22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0330 | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-16 DL MIMO EE features in 36.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FUTUREWEI |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_DL_MIMO_EE-Core |  |  |  |  |  | Date: |  |  | 2019-12-5 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of Rel-16 DL MIMO EE features in 36.212 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of enhancements for SRS capacity and coverage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No introduction of Rel-16 DL MIMO EE features in 36.212 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.6, 5.3.3.1.7, 5.3.3.1.7A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.211, TS 36.213 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.3.3.1.6	Format 3

DCI format 3 is used for the transmission of TPC commands for PUCCH, SPUCCH corresponding to semi-persistently scheduled PDSCH with slot/subslot duration, semi-persistently scheduled PUSCH with slot/subslot duration, PUSCH with subframe duration with 2-bit power adjustments, and additional SRS symbols.

The following information is transmitted by means of the DCI format 3:

- TPC command number 1, TPC command number 2,…, TPC command number N

where , and where  is equal to the payload size of format 0 before CRC attachment when format 0 is mapped onto the common search space, including any padding bits appended to format 0. The parameter tpc-Index or tpc-Index-PUCCH-SCell-r13 or tpc-Index-additionalSRS provided by higher layers determines the index to the TPC command for a given UE.

If , a bit of value zero shall be appended to format 3.

For BL/CE UE, Lformat 0 and format 0 are replaced by Lformat 6-0A and format 6-0A, respectively, in the description above.

##### 5.3.3.1.7	Format 3A

DCI format 3A is used for the transmission of TPC commands for PUCCH, SPUCCH corresponding to semi-persistently scheduled PDSCH with slot/subslot duration, semi-persistently scheduled PUSCH with slot/subslot duration, PUSCH with subframe duration with single bit power adjustments, and additional SRS symbols.

The following information is transmitted by means of the DCI format 3A:

-	TPC command number 1, TPC command number 2,…, TPC command number M

where , and where  is equal to the payload size of format 0 before CRC attachment when format 0 is mapped onto the common search space, including any padding bits appended to format 0. The parameter tpc-Index or tpc-Index-PUCCH-SCell-r13 or tpc-Index-additionalSRS provided by higher layers determines the index to the TPC command for a given UE.

For BL/CE UE, Lformat 0 and format 0 are replaced by Lformat 6-0A and format 6-0A, respectively, in the description above.

##### 5.3.3.1.7A	Format 3B

DCI format 3B is used for the transmission of a group of TPC commands for SRS transmissions by one or more UEs. Along with a TPC command, a SRS request may also be transmitted.

The following information is transmitted by means of the DCI format 3B:

-	block number 1, block number 2, …, block number

where the starting position of a block is determined by the parameter startingBitOfFormat3B or startingBitOfFormat3B-additionalSRS provided by higher layers for the UE configured with the block.

If a UE has more than 5 TDD SCells configured without PUCCH and without PUSCH, one block is configured for the UE by higher layers, with the following fields defined for the block:

-	SRS request – 0 or 2 bits. This field is present, and if present interpreted, according to the definition in Subclause 8.2 of [3].

-	TPC command number 1, TPC command number 2, …, TPC command number n

The n TPC command fields correspond to a set of n TDD SCells without PUCCH and without PUSCH, with the set indicated by the SRS request field or determined by higher layers if there is no SRS request field present. A TPC command field has 1 bit if the parameter fieldTypeFormat3B or fieldTypeFormat3B-additionalSRS provided by higher layers has a value of 1 or 3, and 2 bits if the parameter fieldTypeFormat3B or fieldTypeFormat3B-additionalSRS has a value of 2 or 4.

If a UE has up to 5 TDD SCells configured without PUCCH and without PUSCH, one or more blocks each corresponding to an SCell are configured by higher layers, with the following fields defined for each block:

- SRS request - 0, 1, or 2 bits, where the number of bits is determined in Subclause 8.2 of [3].

- TPC command - 1 or 2 bits, where the number of bits is 1 if the parameter fieldTypeFormat3B or fieldTypeFormat3B-additionalSRS provided by higher layers has a value of 1 or 3, and 2 if the parameter fieldTypeFormat3B or fieldTypeFormat3B-additionalSRS has a value of 2 or 4.

The size of format 3B is equal to , and where  is equal to the payload size of format 0 before CRC attachment when format 0 is mapped onto the common search space, including any padding bits appended to format 0.