3GPP TSG-RAN WG1 Meeting #99	R1-1913636

# Reno, NV, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0025 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of integrated access and backhaul for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2019-12-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of integrated access and backhaul for NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhancing PRACH resource mapping to support intergrated access and backhaul |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for integrated access and backhaul |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.1, 6.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.202, 38.212, 38.213, 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

BWP	Bandwidth part

CCE	Control channel element

CORESET	Control resource set

CRB	Common resource block

CSI	Channel-state information

CSI-RS	CSI reference signal

DCI	Downlink Control Information

DM-RS	Demodulation reference signal

FR1	Frequency range 1 as defined in [8, TS 38.104]

FR2	Frequency range 2 as defined in [8, TS 38.104]

IAB	Integrated access and backhaul

IAB-MT	IAB mobile termination

IE	Information element

PBCH	Physical broadcast channel

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PRACH	Physical random-access channel

PRB	Physical resource block

PSS	Primary synchronization signal

PT-RS	Phase-tracking reference signal

PUCCH	Physical uplink control channel

PUSCH	Physical uplink shared channel

REG	Resource-element group

SRS	Sounding reference signal

SSS	Secondary synchronization signal

VRB	Virtual resource block

## 4.1	General

Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed in time units  where Δfmax=480∙103 Hz and . The constant  where ,  and .

Throughout this specification, unless otherwise noted, statements using the term “UE” in clauses 4, 5, 6, or 7 are equally applicable to the IAB-MT part of an IAB node.

#### 6.3.3.2	Mapping to physical resources

The preamble sequence shall be mapped to physical resources according to

where  is an amplitude scaling factor in order to conform to the transmit power specified in [5, TS38.213], and  is the antenna port. Baseband signal generation shall be done according to clause 5.3 using the parameters in Table 6.3.3.1-1 or Table 6.3.3.1-2 with  given by Table 6.3.3.2-1.

Random access preambles can only be transmitted in the time resources given by the higher-layer parameter prach-ConfigurationIndex according to Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104].

For the IAB-MT part of an IAB node, the following applies to Tables 6.3.3.2-2 to 6.3.3.2-4 if all the higher layer parameters prach-ConfigurationFrameOffset, prach-ConfigurationPeriodScaling, and prach-ConfigurationSOffset are configured:

-	nSFN mod x=y in Tables 6.3.3.2-2 to 6.3.3.2-4 shall be replaced by nSFN mod xIAB=y+Δy mod xIAB where Δy∈0,1,…, xIAB-1 is given by the higher-layer parameter prach-ConfigurationFrameOffset, and xIAB=δx where δ is given by the higher-layer parameter prach-ConfigurationPeriodScaling;

-	the subframe number in Tables 6.3.3.2-2 to 6.3.3.2-3 and the slot number in Table 6.3.3.2-4 shall be replaced by sn+Δs mod L where sn is the slot or subframe number, Δs∈0,1,…,L-1 is given by the higher-layer parameter prach-ConfigurationSOffset,  and L is the number of subframes in a frame in Tables 6.3.3.2-2 to 6.3.3.2-3 and the number of slots in a frame in Table 6.3.3.2-4

Random access preambles can only be transmitted in the frequency resources given by the higher-layer parameter msg1-FrequencyStart. The PRACH frequency resources nRA∈0,1,…,M-1, where M equals the higher-layer parameter msg1-FDM, are numbered in increasing order within the initial uplink bandwidth part during initial access, starting from the lowest frequency. Otherwise, nRA are numbered in increasing order within the active uplink bandwidth part, starting from the lowest frequency.

For the purpose of slot numbering in the tables, the following subcarrier spacing shall be assumed:

-	15 kHz for FR1

-	60 kHz for FR2.

For handover purposes to a target cell in paired or unpaired spectrum where the target cell uses Lmax=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if the association pattern period in Section 8.1 of [5, TS 38.213] is not equal to 10 ms.

For inter frequency handover purposes where the source cell is either in paired or unpaired spectrum and the target cell is in unpaired spectrum and uses Lmax=8, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 76800Ts.

Table 6.3.3.2-1: Supported combinations of  and , and the corresponding value of .

|  | for PRACH | for PUSCH | , allocation expressed in number of RBs for PUSCH |  |
| --- | --- | --- | --- | --- |
| 839 | 1.25 | 15 | 6 | 7 |
| 839 | 1.25 | 30 | 3 | 1 |
| 839 | 1.25 | 60 | 2 | 133 |
| 839 | 5 | 15 | 24 | 12 |
| 839 | 5 | 30 | 12 | 10 |
| 839 | 5 | 60 | 6 | 7 |
| 139 | 15 | 15 | 12 | 2 |
| 139 | 15 | 30 | 6 | 2 |
| 139 | 15 | 60 | 3 | 2 |
| 139 | 30 | 15 | 24 | 2 |
| 139 | 30 | 30 | 12 | 2 |
| 139 | 30 | 60 | 6 | 2 |
| 139 | 60 | 60 | 12 | 2 |
| 139 | 60 | 120 | 6 | 2 |
| 139 | 120 | 60 | 24 | 2 |
| 139 | 120 | 120 | 12 | 2 |

Table 6.3.3.2-2: Random access configurations for FR1 and paired spectrum/supplementary uplink.

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 16 | 1 | 1 | 0 | - | - | 0 |
| 1 | 0 | 16 | 1 | 4 | 0 | - | - | 0 |
| 2 | 0 | 16 | 1 | 7 | 0 | - | - | 0 |
| 3 | 0 | 16 | 1 | 9 | 0 | - | - | 0 |
| 4 | 0 | 8 | 1 | 1 | 0 | - | - | 0 |
| 5 | 0 | 8 | 1 | 4 | 0 | - | - | 0 |
| 6 | 0 | 8 | 1 | 7 | 0 | - | - | 0 |
| 7 | 0 | 8 | 1 | 9 | 0 | - | - | 0 |
| 8 | 0 | 4 | 1 | 1 | 0 | - | - | 0 |
| 9 | 0 | 4 | 1 | 4 | 0 | - | - | 0 |
| 10 | 0 | 4 | 1 | 7 | 0 | - | - | 0 |
| 11 | 0 | 4 | 1 | 9 | 0 | - | - | 0 |
| 12 | 0 | 2 | 1 | 1 | 0 | - | - | 0 |
| 13 | 0 | 2 | 1 | 4 | 0 | - | - | 0 |
| 14 | 0 | 2 | 1 | 7 | 0 | - | - | 0 |
| 15 | 0 | 2 | 1 | 9 | 0 | - | - | 0 |
| 16 | 0 | 1 | 0 | 1 | 0 | - | - | 0 |
| 17 | 0 | 1 | 0 | 4 | 0 | - | - | 0 |
| 18 | 0 | 1 | 0 | 7 | 0 | - | - | 0 |
| 19 | 0 | 1 | 0 | 1,6 | 0 | - | - | 0 |
| 20 | 0 | 1 | 0 | 2,7 | 0 | - | - | 0 |
| 21 | 0 | 1 | 0 | 3,8 | 0 | - | - | 0 |
| 22 | 0 | 1 | 0 | 1,4,7 | 0 | - | - | 0 |
| 23 | 0 | 1 | 0 | 2,5,8 | 0 | - | - | 0 |
| 24 | 0 | 1 | 0 | 3, 6, 9 | 0 | - | - | 0 |
| 25 | 0 | 1 | 0 | 0,2,4,6,8 | 0 | - | - | 0 |
| 26 | 0 | 1 | 0 | 1,3,5,7,9 | 0 | - | - | 0 |
| 27 | 0 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | - | - | 0 |
| 28 | 1 | 16 | 1 | 1 | 0 | - | - | 0 |
| 29 | 1 | 16 | 1 | 4 | 0 | - | - | 0 |
| 30 | 1 | 16 | 1 | 7 | 0 | - | - | 0 |
| 31 | 1 | 16 | 1 | 9 | 0 | - | - | 0 |
| 32 | 1 | 8 | 1 | 1 | 0 | - | - | 0 |
| 33 | 1 | 8 | 1 | 4 | 0 | - | - | 0 |
| 34 | 1 | 8 | 1 | 7 | 0 | - | - | 0 |
| 35 | 1 | 8 | 1 | 9 | 0 | - | - | 0 |
| 36 | 1 | 4 | 1 | 1 | 0 | - | - | 0 |
| 37 | 1 | 4 | 1 | 4 | 0 | - | - | 0 |
| 38 | 1 | 4 | 1 | 7 | 0 | - | - | 0 |
| 39 | 1 | 4 | 1 | 9 | 0 | - | - | 0 |
| 40 | 1 | 2 | 1 | 1 | 0 | - | - | 0 |
| 41 | 1 | 2 | 1 | 4 | 0 | - | - | 0 |
| 42 | 1 | 2 | 1 | 7 | 0 | - | - | 0 |
| 43 | 1 | 2 | 1 | 9 | 0 | - | - | 0 |
| 44 | 1 | 1 | 0 | 1 | 0 | - | - | 0 |
| 45 | 1 | 1 | 0 | 4 | 0 | - | - | 0 |
| 46 | 1 | 1 | 0 | 7 | 0 | - | - | 0 |
| 47 | 1 | 1 | 0 | 1,6 | 0 | - | - | 0 |
| 48 | 1 | 1 | 0 | 2,7 | 0 | - | - | 0 |
| 49 | 1 | 1 | 0 | 3,8 | 0 | - | - | 0 |
| 50 | 1 | 1 | 0 | 1,4,7 | 0 | - | - | 0 |
| 51 | 1 | 1 | 0 | 2,5,8 | 0 | - | - | 0 |
| 52 | 1 | 1 | 0 | 3,6,9 | 0 | - | - | 0 |
| 53 | 2 | 16 | 1 | 1 | 0 | - | - | 0 |
| 54 | 2 | 8 | 1 | 1 | 0 | - | - | 0 |
| 55 | 2 | 4 | 0 | 1 | 0 | - | - | 0 |
| 56 | 2 | 2 | 0 | 1 | 0 | - | - | 0 |
| 57 | 2 | 2 | 0 | 5 | 0 | - | - | 0 |
| 58 | 2 | 1 | 0 | 1 | 0 | - | - | 0 |
| 59 | 2 | 1 | 0 | 5 | 0 | - | - | 0 |
| 60 | 3 | 16 | 1 | 1 | 0 | - | - | 0 |
| 61 | 3 | 16 | 1 | 4 | 0 | - | - | 0 |
| 62 | 3 | 16 | 1 | 7 | 0 | - | - | 0 |
| 63 | 3 | 16 | 1 | 9 | 0 | - | - | 0 |
| 64 | 3 | 8 | 1 | 1 | 0 | - | - | 0 |
| 65 | 3 | 8 | 1 | 4 | 0 | - | - | 0 |
| 66 | 3 | 8 | 1 | 7 | 0 | - | - | 0 |
| 67 | 3 | 4 | 1 | 1 | 0 | - | - | 0 |
| 68 | 3 | 4 | 1 | 4 | 0 | - | - | 0 |
| 69 | 3 | 4 | 1 | 7 | 0 | - | - | 0 |
| 70 | 3 | 4 | 1 | 9 | 0 | - | - | 0 |
| 71 | 3 | 2 | 1 | 1 | 0 | - | - | 0 |
| 72 | 3 | 2 | 1 | 4 | 0 | - | - | 0 |
| 73 | 3 | 2 | 1 | 7 | 0 | - | - | 0 |
| 74 | 3 | 2 | 1 | 9 | 0 | - | - | 0 |
| 75 | 3 | 1 | 0 | 1 | 0 | - | - | 0 |
| 76 | 3 | 1 | 0 | 4 | 0 | - | - | 0 |
| 77 | 3 | 1 | 0 | 7 | 0 | - | - | 0 |
| 78 | 3 | 1 | 0 | 1,6 | 0 | - | - | 0 |
| 79 | 3 | 1 | 0 | 2,7 | 0 | - | - | 0 |
| 80 | 3 | 1 | 0 | 3,8 | 0 | - | - | 0 |
| 81 | 3 | 1 | 0 | 1,4,7 | 0 | - | - | 0 |
| 82 | 3 | 1 | 0 | 2,5,8 | 0 | - | - | 0 |
| 83 | 3 | 1 | 0 | 3, 6, 9 | 0 | - | - | 0 |
| 84 | 3 | 1 | 0 | 0,2,4,6,8 | 0 | - | - | 0 |
| 85 | 3 | 1 | 0 | 1,3,5,7,9 | 0 | - | - | 0 |
| 86 | 3 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | - | - | 0 |
| 87 | A1 | 16 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 88 | A1 | 16 | 1 | 4 | 0 | 2 | 6 | 2 |
| 89 | A1 | 8 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 90 | A1 | 8 | 1 | 4 | 0 | 2 | 6 | 2 |
| 91 | A1 | 4 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 92 | A1 | 4 | 1 | 4,9 | 0 | 1 | 6 | 2 |
| 93 | A1 | 4 | 0 | 4 | 0 | 2 | 6 | 2 |
| 94 | A1 | 2 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 95 | A1 | 2 | 0 | 1 | 0 | 2 | 6 | 2 |
| 96 | A1 | 2 | 0 | 4 | 0 | 2 | 6 | 2 |
| 97 | A1 | 2 | 0 | 7 | 0 | 2 | 6 | 2 |
| 98 | A1 | 1 | 0 | 4 | 0 | 1 | 6 | 2 |
| 99 | A1 | 1 | 0 | 1,6 | 0 | 1 | 6 | 2 |
| 100 | A1 | 1 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 101 | A1 | 1 | 0 | 1 | 0 | 2 | 6 | 2 |
| 102 | A1 | 1 | 0 | 7 | 0 | 2 | 6 | 2 |
| 103 | A1 | 1 | 0 | 2,7 | 0 | 2 | 6 | 2 |
| 104 | A1 | 1 | 0 | 1,4,7 | 0 | 2 | 6 | 2 |
| 105 | A1 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 6 | 2 |
| 106 | A1 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 6 | 2 |
| 107 | A1 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 6 | 2 |
| 108 | A1/B1 | 2 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 109 | A1/B1 | 2 | 0 | 4 | 0 | 2 | 7 | 2 |
| 110 | A1/B1 | 1 | 0 | 4 | 0 | 1 | 7 | 2 |
| 111 | A1/B1 | 1 | 0 | 1,6 | 0 | 1 | 7 | 2 |
| 112 | A1/B1 | 1 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 113 | A1/B1 | 1 | 0 | 1 | 0 | 2 | 7 | 2 |
| 114 | A1/B1 | 1 | 0 | 7 | 0 | 2 | 7 | 2 |
| 115 | A1/B1 | 1 | 0 | 1,4,7 | 0 | 2 | 7 | 2 |
| 116 | A1/B1 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 7 | 2 |
| 117 | A2 | 16 | 1 | 2,6,9 | 0 | 1 | 3 | 4 |
| 118 | A2 | 16 | 1 | 4 | 0 | 2 | 3 | 4 |
| 119 | A2 | 8 | 1 | 2,6,9 | 0 | 1 | 3 | 4 |
| 120 | A2 | 8 | 1 | 4 | 0 | 2 | 3 | 4 |
| 121 | A2 | 4 | 0 | 2,6,9 | 0 | 1 | 3 | 4 |
| 122 | A2 | 4 | 0 | 4 | 0 | 2 | 3 | 4 |
| 123 | A2 | 2 | 1 | 2,6,9 | 0 | 1 | 3 | 4 |
| 124 | A2 | 2 | 0 | 1 | 0 | 2 | 3 | 4 |
| 125 | A2 | 2 | 0 | 4 | 0 | 2 | 3 | 4 |
| 126 | A2 | 2 | 0 | 7 | 0 | 2 | 3 | 4 |
| 127 | A2 | 1 | 0 | 4 | 0 | 1 | 3 | 4 |
| 128 | A2 | 1 | 0 | 1,6 | 0 | 1 | 3 | 4 |
| 129 | A2 | 1 | 0 | 4,9 | 0 | 1 | 3 | 4 |
| 130 | A2 | 1 | 0 | 1 | 0 | 2 | 3 | 4 |
| 131 | A2 | 1 | 0 | 7 | 0 | 2 | 3 | 4 |
| 132 | A2 | 1 | 0 | 2,7 | 0 | 2 | 3 | 4 |
| 133 | A2 | 1 | 0 | 1,4,7 | 0 | 2 | 3 | 4 |
| 134 | A2 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 3 | 4 |
| 135 | A2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 3 | 4 |
| 136 | A2 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 3 | 4 |
| 137 | A2/B2 | 2 | 1 | 2,6,9 | 0 | 1 | 3 | 4 |
| 138 | A2/B2 | 2 | 0 | 4 | 0 | 2 | 3 | 4 |
| 139 | A2/B2 | 1 | 0 | 4 | 0 | 1 | 3 | 4 |
| 140 | A2/B2 | 1 | 0 | 1,6 | 0 | 1 | 3 | 4 |
| 141 | A2/B2 | 1 | 0 | 4,9 | 0 | 1 | 3 | 4 |
| 142 | A2/B2 | 1 | 0 | 1 | 0 | 2 | 3 | 4 |
| 143 | A2/B2 | 1 | 0 | 7 | 0 | 2 | 3 | 4 |
| 144 | A2/B2 | 1 | 0 | 1,4,7 | 0 | 2 | 3 | 4 |
| 145 | A2/B2 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 3 | 4 |
| 146 | A2/B2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 3 | 4 |
| 147 | A3 | 16 | 1 | 4,9 | 0 | 1 | 2 | 6 |
| 148 | A3 | 16 | 1 | 4 | 0 | 2 | 2 | 6 |
| 149 | A3 | 8 | 1 | 4,9 | 0 | 1 | 2 | 6 |
| 150 | A3 | 8 | 1 | 4 | 0 | 2 | 2 | 6 |
| 151 | A3 | 4 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 152 | A3 | 4 | 0 | 4 | 0 | 2 | 2 | 6 |
| 153 | A3 | 2 | 1 | 2,6,9 | 0 | 2 | 2 | 6 |
| 154 | A3 | 2 | 0 | 1 | 0 | 2 | 2 | 6 |
| 155 | A3 | 2 | 0 | 4 | 0 | 2 | 2 | 6 |
| 156 | A3 | 2 | 0 | 7 | 0 | 2 | 2 | 6 |
| 157 | A3 | 1 | 0 | 4 | 0 | 1 | 2 | 6 |
| 158 | A3 | 1 | 0 | 1,6 | 0 | 1 | 2 | 6 |
| 159 | A3 | 1 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 160 | A3 | 1 | 0 | 1 | 0 | 2 | 2 | 6 |
| 161 | A3 | 1 | 0 | 7 | 0 | 2 | 2 | 6 |
| 162 | A3 | 1 | 0 | 2,7 | 0 | 2 | 2 | 6 |
| 163 | A3 | 1 | 0 | 1,4,7 | 0 | 2 | 2 | 6 |
| 164 | A3 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 2 | 6 |
| 165 | A3 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 2 | 6 |
| 166 | A3 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 2 | 6 |
| 167 | A3/B3 | 2 | 1 | 2,6,9 | 0 | 2 | 2 | 6 |
| 168 | A3/B3 | 2 | 0 | 4 | 0 | 2 | 2 | 6 |
| 169 | A3/B3 | 1 | 0 | 4 | 0 | 1 | 2 | 6 |
| 170 | A3/B3 | 1 | 0 | 1,6 | 0 | 1 | 2 | 6 |
| 171 | A3/B3 | 1 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 172 | A3/B3 | 1 | 0 | 1 | 0 | 2 | 2 | 6 |
| 173 | A3/B3 | 1 | 0 | 7 | 0 | 2 | 2 | 6 |
| 174 | A3/B3 | 1 | 0 | 1,4,7 | 0 | 2 | 2 | 6 |
| 175 | A3/B3 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 2 | 6 |
| 176 | A3/B3 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 2 | 6 |
| 177 | B1 | 16 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 178 | B1 | 16 | 1 | 4 | 0 | 2 | 7 | 2 |
| 179 | B1 | 8 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 180 | B1 | 8 | 1 | 4 | 0 | 2 | 7 | 2 |
| 181 | B1 | 4 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 182 | B1 | 4 | 1 | 4,9 | 0 | 1 | 7 | 2 |
| 183 | B1 | 4 | 0 | 4 | 0 | 2 | 7 | 2 |
| 184 | B1 | 2 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 185 | B1 | 2 | 0 | 1 | 0 | 2 | 7 | 2 |
| 186 | B1 | 2 | 0 | 4 | 0 | 2 | 7 | 2 |
| 187 | B1 | 2 | 0 | 7 | 0 | 2 | 7 | 2 |
| 188 | B1 | 1 | 0 | 4 | 0 | 1 | 7 | 2 |
| 189 | B1 | 1 | 0 | 1,6 | 0 | 1 | 7 | 2 |
| 190 | B1 | 1 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 191 | B1 | 1 | 0 | 1 | 0 | 2 | 7 | 2 |
| 192 | B1 | 1 | 0 | 7 | 0 | 2 | 7 | 2 |
| 193 | B1 | 1 | 0 | 2,7 | 0 | 2 | 7 | 2 |
| 194 | B1 | 1 | 0 | 1,4,7 | 0 | 2 | 7 | 2 |
| 195 | B1 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 7 | 2 |
| 196 | B1 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 7 | 2 |
| 197 | B1 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 7 | 2 |
| 198 | B4 | 16 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 199 | B4 | 16 | 1 | 4 | 0 | 2 | 1 | 12 |
| 200 | B4 | 8 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 201 | B4 | 8 | 1 | 4 | 0 | 2 | 1 | 12 |
| 202 | B4 | 4 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 203 | B4 | 4 | 0 | 4 | 0 | 2 | 1 | 12 |
| 204 | B4 | 4 | 1 | 4,9 | 0 | 2 | 1 | 12 |
| 205 | B4 | 2 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 206 | B4 | 2 | 0 | 1 | 0 | 2 | 1 | 12 |
| 207 | B4 | 2 | 0 | 4 | 0 | 2 | 1 | 12 |
| 208 | B4 | 2 | 0 | 7 | 0 | 2 | 1 | 12 |
| 209 | B4 | 1 | 0 | 1 | 0 | 2 | 1 | 12 |
| 210 | B4 | 1 | 0 | 4 | 0 | 2 | 1 | 12 |
| 211 | B4 | 1 | 0 | 7 | 0 | 2 | 1 | 12 |
| 212 | B4 | 1 | 0 | 1,6 | 0 | 2 | 1 | 12 |
| 213 | B4 | 1 | 0 | 2,7 | 0 | 2 | 1 | 12 |
| 214 | B4 | 1 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 215 | B4 | 1 | 0 | 1,4,7 | 0 | 2 | 1 | 12 |
| 216 | B4 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 1 | 12 |
| 217 | B4 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 1 | 12 |
| 218 | B4 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 1 | 12 |
| 219 | C0 | 8 | 1 | 4 | 0 | 2 | 7 | 2 |
| 220 | C0 | 4 | 1 | 4,9 | 0 | 1 | 7 | 2 |
| 221 | C0 | 4 | 0 | 4 | 0 | 2 | 7 | 2 |
| 222 | C0 | 2 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 223 | C0 | 2 | 0 | 1 | 0 | 2 | 7 | 2 |
| 224 | C0 | 2 | 0 | 4 | 0 | 2 | 7 | 2 |
| 225 | C0 | 2 | 0 | 7 | 0 | 2 | 7 | 2 |
| 226 | C0 | 1 | 0 | 4 | 0 | 1 | 7 | 2 |
| 227 | C0 | 1 | 0 | 1,6 | 0 | 1 | 7 | 2 |
| 228 | C0 | 1 | 0 | 4,9 | 0 | 1 | 7 | 2 |
| 229 | C0 | 1 | 0 | 1 | 0 | 2 | 7 | 2 |
| 230 | C0 | 1 | 0 | 7 | 0 | 2 | 7 | 2 |
| 231 | C0 | 1 | 0 | 2,7 | 0 | 2 | 7 | 2 |
| 232 | C0 | 1 | 0 | 1,4,7 | 0 | 2 | 7 | 2 |
| 233 | C0 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 7 | 2 |
| 234 | C0 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 7 | 2 |
| 235 | C0 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 7 | 2 |
| 236 | C2 | 16 | 1 | 4,9 | 0 | 1 | 2 | 6 |
| 237 | C2 | 16 | 1 | 4 | 0 | 2 | 2 | 6 |
| 238 | C2 | 8 | 1 | 4,9 | 0 | 1 | 2 | 6 |
| 239 | C2 | 8 | 1 | 4 | 0 | 2 | 2 | 6 |
| 240 | C2 | 4 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 241 | C2 | 4 | 0 | 4 | 0 | 2 | 2 | 6 |
| 242 | C2 | 2 | 1 | 2,6,9 | 0 | 2 | 2 | 6 |
| 243 | C2 | 2 | 0 | 1 | 0 | 2 | 2 | 6 |
| 244 | C2 | 2 | 0 | 4 | 0 | 2 | 2 | 6 |
| 245 | C2 | 2 | 0 | 7 | 0 | 2 | 2 | 6 |
| 246 | C2 | 1 | 0 | 4 | 0 | 1 | 2 | 6 |
| 247 | C2 | 1 | 0 | 1,6 | 0 | 1 | 2 | 6 |
| 248 | C2 | 1 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 249 | C2 | 1 | 0 | 1 | 0 | 2 | 2 | 6 |
| 250 | C2 | 1 | 0 | 7 | 0 | 2 | 2 | 6 |
| 251 | C2 | 1 | 0 | 2,7 | 0 | 2 | 2 | 6 |
| 252 | C2 | 1 | 0 | 1,4,7 | 0 | 2 | 2 | 6 |
| 253 | C2 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 2 | 6 |
| 254 | C2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 2 | 6 |
| 255 | C2 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 2 | 6 |

Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum.

| PRACH Configuration Index | Preamble format |  |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 16 | 1 | 9 | 0 | - | - | 0 |
| 1 | 0 | 8 | 1 | 9 | 0 | - | - | 0 |
| 2 | 0 | 4 | 1 | 9 | 0 | - | - | 0 |
| 3 | 0 | 2 | 0 | 9 | 0 | - | - | 0 |
| 4 | 0 | 2 | 1 | 9 | 0 | - | - | 0 |
| 5 | 0 | 2 | 0 | 4 | 0 | - | - | 0 |
| 6 | 0 | 2 | 1 | 4 | 0 | - | - | 0 |
| 7 | 0 | 1 | 0 | 9 | 0 | - | - | 0 |
| 8 | 0 | 1 | 0 | 8 | 0 | - | - | 0 |
| 9 | 0 | 1 | 0 | 7 | 0 | - | - | 0 |
| 10 | 0 | 1 | 0 | 6 | 0 | - | - | 0 |
| 11 | 0 | 1 | 0 | 5 | 0 | - | - | 0 |
| 12 | 0 | 1 | 0 | 4 | 0 | - | - | 0 |
| 13 | 0 | 1 | 0 | 3 | 0 | - | - | 0 |
| 14 | 0 | 1 | 0 | 2 | 0 | - | - | 0 |
| 15 | 0 | 1 | 0 | 1,6 | 0 |  |  | 0 |
| 16 | 0 | 1 | 0 | 1,6 | 7 | - | - | 0 |
| 17 | 0 | 1 | 0 | 4,9 | 0 | - | - | 0 |
| 18 | 0 | 1 | 0 | 3,8 | 0 | - | - | 0 |
| 19 | 0 | 1 | 0 | 2,7 | 0 | - | - | 0 |
| 20 | 0 | 1 | 0 | 8,9 | 0 | - | - | 0 |
| 21 | 0 | 1 | 0 | 4,8,9 | 0 | - | - | 0 |
| 22 | 0 | 1 | 0 | 3,4,9 | 0 | - | - | 0 |
| 23 | 0 | 1 | 0 | 7,8,9 | 0 | - | - | 0 |
| 24 | 0 | 1 | 0 | 3,4,8,9 | 0 | - | - | 0 |
| 25 | 0 | 1 | 0 | 6,7,8,9 | 0 | - | - | 0 |
| 26 | 0 | 1 | 0 | 1,4,6,9 | 0 | - | - | 0 |
| 27 | 0 | 1 | 0 | 1,3,5,7,9 | 0 | - | - | 0 |
| 28 | 1 | 16 | 1 | 7 | 0 | - | - | 0 |
| 29 | 1 | 8 | 1 | 7 | 0 | - | - | 0 |
| 30 | 1 | 4 | 1 | 7 | 0 | - | - | 0 |
| 31 | 1 | 2 | 0 | 7 | 0 | - | - | 0 |
| 32 | 1 | 2 | 1 | 7 | 0 | - | - | 0 |
| 33 | 1 | 1 | 0 | 7 | 0 | - | - | 0 |
| 34 | 2 | 16 | 1 | 6 | 0 | - | - | 0 |
| 35 | 2 | 8 | 1 | 6 | 0 | - | - | 0 |
| 36 | 2 | 4 | 1 | 6 | 0 | - | - | 0 |
| 37 | 2 | 2 | 0 | 6 | 7 | - | - | 0 |
| 38 | 2 | 2 | 1 | 6 | 7 | - | - | 0 |
| 39 | 2 | 1 | 0 | 6 | 7 | - | - | 0 |
| 40 | 3 | 16 | 1 | 9 | 0 | - | - | 0 |
| 41 | 3 | 8 | 1 | 9 | 0 | - | - | 0 |
| 42 | 3 | 4 | 1 | 9 | 0 | - | - | 0 |
| 43 | 3 | 2 | 0 | 9 | 0 | - | - | 0 |
| 44 | 3 | 2 | 1 | 9 | 0 | - | - | 0 |
| 45 | 3 | 2 | 0 | 4 | 0 | - | - | 0 |
| 46 | 3 | 2 | 1 | 4 | 0 | - | - | 0 |
| 47 | 3 | 1 | 0 | 9 | 0 | - | - | 0 |
| 48 | 3 | 1 | 0 | 8 | 0 | - | - | 0 |
| 49 | 3 | 1 | 0 | 7 | 0 | - | - | 0 |
| 50 | 3 | 1 | 0 | 6 | 0 | - | - | 0 |
| 51 | 3 | 1 | 0 | 5 | 0 | - | - | 0 |
| 52 | 3 | 1 | 0 | 4 | 0 | - | - | 0 |
| 53 | 3 | 1 | 0 | 3 | 0 | - | - | 0 |
| 54 | 3 | 1 | 0 | 2 | 0 | - | - | 0 |
| 55 | 3 | 1 | 0 | 1,6 | 0 | - | - | 0 |
| 56 | 3 | 1 | 0 | 1,6 | 7 | - | - | 0 |
| 57 | 3 | 1 | 0 | 4,9 | 0 | - | - | 0 |
| 58 | 3 | 1 | 0 | 3,8 | 0 | - | - | 0 |
| 59 | 3 | 1 | 0 | 2,7 | 0 | - | - | 0 |
| 60 | 3 | 1 | 0 | 8,9 | 0 | - | - | 0 |
| 61 | 3 | 1 | 0 | 4,8,9 | 0 | - | - | 0 |
| 62 | 3 | 1 | 0 | 3,4,9 | 0 | - | - | 0 |
| 63 | 3 | 1 | 0 | 7,8,9 | 0 | - | - | 0 |
| 64 | 3 | 1 | 0 | 3,4,8,9 | 0 | - | - | 0 |
| 65 | 3 | 1 | 0 | 1,4,6,9 | 0 | - | - | 0 |
| 66 | 3 | 1 | 0 | 1,3,5,7,9 | 0 | - | - | 0 |
| 67 | A1 | 16 | 1 | 9 | 0 | 2 | 6 | 2 |
| 68 | A1 | 8 | 1 | 9 | 0 | 2 | 6 | 2 |
| 69 | A1 | 4 | 1 | 9 | 0 | 1 | 6 | 2 |
| 70 | A1 | 2 | 1 | 9 | 0 | 1 | 6 | 2 |
| 71 | A1 | 2 | 1 | 4,9 | 7 | 1 | 3 | 2 |
| 72 | A1 | 2 | 1 | 7,9 | 7 | 1 | 3 | 2 |
| 73 | A1 | 2 | 1 | 7,9 | 0 | 1 | 6 | 2 |
| 74 | A1 | 2 | 1 | 8,9 | 0 | 2 | 6 | 2 |
| 75 | A1 | 2 | 1 | 4,9 | 0 | 2 | 6 | 2 |
| 76 | A1 | 2 | 1 | 2,3,4,7,8,9 | 0 | 1 | 6 | 2 |
| 77 | A1 | 1 | 0 | 9 | 0 | 2 | 6 | 2 |
| 78 | A1 | 1 | 0 | 9 | 7 | 1 | 3 | 2 |
| 79 | A1 | 1 | 0 | 9 | 0 | 1 | 6 | 2 |
| 80 | A1 | 1 | 0 | 8,9 | 0 | 2 | 6 | 2 |
| 81 | A1 | 1 | 0 | 4,9 | 0 | 1 | 6 | 2 |
| 82 | A1 | 1 | 0 | 7,9 | 7 | 1 | 3 | 2 |
| 83 | A1 | 1 | 0 | 3,4,8,9 | 0 | 1 | 6 | 2 |
| 84 | A1 | 1 | 0 | 3,4,8,9 | 0 | 2 | 6 | 2 |
| 85 | A1 | 1 | 0 | 1,3,5,7,9 | 0 | 1 | 6 | 2 |
| 86 | A1 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 7 | 1 | 3 | 2 |
| 87 | A2 | 16 | 1 | 9 | 0 | 2 | 3 | 4 |
| 88 | A2 | 8 | 1 | 9 | 0 | 2 | 3 | 4 |
| 89 | A2 | 4 | 1 | 9 | 0 | 1 | 3 | 4 |
| 90 | A2 | 2 | 1 | 7,9 | 0 | 1 | 3 | 4 |
| 91 | A2 | 2 | 1 | 8,9 | 0 | 2 | 3 | 4 |
| 92 | A2 | 2 | 1 | 7,9 | 9 | 1 | 1 | 4 |
| 93 | A2 | 2 | 1 | 4,9 | 9 | 1 | 1 | 4 |
| 94 | A2 | 2 | 1 | 4,9 | 0 | 2 | 3 | 4 |
| 95 | A2 | 2 | 1 | 2,3,4,7,8,9 | 0 | 1 | 3 | 4 |
| 96 | A2 | 1 | 0 | 2 | 0 | 1 | 3 | 4 |
| 97 | A2 | 1 | 0 | 7 | 0 | 1 | 3 | 4 |
| 98 | A2 | 2 | 1 | 9 | 0 | 1 | 3 | 4 |
| 99 | A2 | 1 | 0 | 9 | 0 | 2 | 3 | 4 |
| 100 | A2 | 1 | 0 | 9 | 9 | 1 | 1 | 4 |
| 101 | A2 | 1 | 0 | 9 | 0 | 1 | 3 | 4 |
| 102 | A2 | 1 | 0 | 2,7 | 0 | 1 | 3 | 4 |
| 103 | A2 | 1 | 0 | 8,9 | 0 | 2 | 3 | 4 |
| 104 | A2 | 1 | 0 | 4,9 | 0 | 1 | 3 | 4 |
| 105 | A2 | 1 | 0 | 7,9 | 9 | 1 | 1 | 4 |
| 106 | A2 | 1 | 0 | 3,4,8,9 | 0 | 1 | 3 | 4 |
| 107 | A2 | 1 | 0 | 3,4,8,9 | 0 | 2 | 3 | 4 |
| 108 | A2 | 1 | 0 | 1,3,5,7,9 | 0 | 1 | 3 | 4 |
| 109 | A2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 9 | 1 | 1 | 4 |
| 110 | A3 | 16 | 1 | 9 | 0 | 2 | 2 | 6 |
| 111 | A3 | 8 | 1 | 9 | 0 | 2 | 2 | 6 |
| 112 | A3 | 4 | 1 | 9 | 0 | 1 | 2 | 6 |
| 113 | A3 | 2 | 1 | 4,9 | 7 | 1 | 1 | 6 |
| 114 | A3 | 2 | 1 | 7,9 | 7 | 1 | 1 | 6 |
| 115 | A3 | 2 | 1 | 7,9 | 0 | 1 | 2 | 6 |
| 116 | A3 | 2 | 1 | 4,9 | 0 | 2 | 2 | 6 |
| 117 | A3 | 2 | 1 | 8,9 | 0 | 2 | 2 | 6 |
| 118 | A3 | 2 | 1 | 2,3,4,7,8,9 | 0 | 1 | 2 | 6 |
| 119 | A3 | 1 | 0 | 2 | 0 | 1 | 2 | 6 |
| 120 | A3 | 1 | 0 | 7 | 0 | 1 | 2 | 6 |
| 121 | A3 | 2 | 1 | 9 | 0 | 1 | 2 | 6 |
| 122 | A3 | 1 | 0 | 9 | 0 | 2 | 2 | 6 |
| 123 | A3 | 1 | 0 | 9 | 7 | 1 | 1 | 6 |
| 124 | A3 | 1 | 0 | 9 | 0 | 1 | 2 | 6 |
| 125 | A3 | 1 | 0 | 2,7 | 0 | 1 | 2 | 6 |
| 126 | A3 | 1 | 0 | 8,9 | 0 | 2 | 2 | 6 |
| 127 | A3 | 1 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 128 | A3 | 1 | 0 | 7,9 | 7 | 1 | 1 | 6 |
| 129 | A3 | 1 | 0 | 3,4,8,9 | 0 | 1 | 2 | 6 |
| 130 | A3 | 1 | 0 | 3,4,8,9 | 0 | 2 | 2 | 6 |
| 131 | A3 | 1 | 0 | 1,3,5,7,9 | 0 | 1 | 2 | 6 |
| 132 | A3 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 7 | 1 | 1 | 6 |
| 133 | B1 | 4 | 1 | 9 | 2 | 1 | 6 | 2 |
| 134 | B1 | 2 | 1 | 9 | 2 | 1 | 6 | 2 |
| 135 | B1 | 2 | 1 | 7,9 | 2 | 1 | 6 | 2 |
| 136 | B1 | 2 | 1 | 4,9 | 8 | 1 | 3 | 2 |
| 137 | B1 | 2 | 1 | 4,9 | 2 | 2 | 6 | 2 |
| 138 | B1 | 1 | 0 | 9 | 2 | 2 | 6 | 2 |
| 139 | B1 | 1 | 0 | 9 | 8 | 1 | 3 | 2 |
| 140 | B1 | 1 | 0 | 9 | 2 | 1 | 6 | 2 |
| 141 | B1 | 1 | 0 | 8,9 | 2 | 2 | 6 | 2 |
| 142 | B1 | 1 | 0 | 4,9 | 2 | 1 | 6 | 2 |
| 143 | B1 | 1 | 0 | 7,9 | 8 | 1 | 3 | 2 |
| 144 | B1 | 1 | 0 | 1,3,5,7,9 | 2 | 1 | 6 | 2 |
| 145 | B4 | 16 | 1 | 9 | 0 | 2 | 1 | 12 |
| 146 | B4 | 8 | 1 | 9 | 0 | 2 | 1 | 12 |
| 147 | B4 | 4 | 1 | 9 | 2 | 1 | 1 | 12 |
| 148 | B4 | 2 | 1 | 9 | 0 | 1 | 1 | 12 |
| 149 | B4 | 2 | 1 | 9 | 2 | 1 | 1 | 12 |
| 150 | B4 | 2 | 1 | 7,9 | 2 | 1 | 1 | 12 |
| 151 | B4 | 2 | 1 | 4,9 | 2 | 1 | 1 | 12 |
| 152 | B4 | 2 | 1 | 4,9 | 0 | 2 | 1 | 12 |
| 153 | B4 | 2 | 1 | 8,9 | 0 | 2 | 1 | 12 |
| 154 | B4 | 2 | 1 | 2,3,4,7,8,9 | 0 | 1 | 1 | 12 |
| 155 | B4 | 1 | 0 | 1 | 0 | 1 | 1 | 12 |
| 156 | B4 | 1 | 0 | 2 | 0 | 1 | 1 | 12 |
| 157 | B4 | 1 | 0 | 4 | 0 | 1 | 1 | 12 |
| 158 | B4 | 1 | 0 | 7 | 0 | 1 | 1 | 12 |
| 159 | B4 | 1 | 0 | 9 | 0 | 1 | 1 | 12 |
| 160 | B4 | 1 | 0 | 9 | 2 | 1 | 1 | 12 |
| 161 | B4 | 1 | 0 | 9 | 0 | 2 | 1 | 12 |
| 162 | B4 | 1 | 0 | 4,9 | 2 | 1 | 1 | 12 |
| 163 | B4 | 1 | 0 | 7,9 | 2 | 1 | 1 | 12 |
| 164 | B4 | 1 | 0 | 8,9 | 0 | 2 | 1 | 12 |
| 165 | B4 | 1 | 0 | 3,4,8,9 | 2 | 1 | 1 | 12 |
| 166 | B4 | 1 | 0 | 1,3,5,7,9 | 2 | 1 | 1 | 12 |
| 167 | B4 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 1 | 12 |
| 168 | B4 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 2 | 1 | 1 | 12 |
| 169 | C0 | 16 | 1 | 9 | 2 | 2 | 6 | 2 |
| 170 | C0 | 8 | 1 | 9 | 2 | 2 | 6 | 2 |
| 171 | C0 | 4 | 1 | 9 | 2 | 1 | 6 | 2 |
| 172 | C0 | 2 | 1 | 9 | 2 | 1 | 6 | 2 |
| 173 | C0 | 2 | 1 | 8,9 | 2 | 2 | 6 | 2 |
| 174 | C0 | 2 | 1 | 7,9 | 2 | 1 | 6 | 2 |
| 175 | C0 | 2 | 1 | 7,9 | 8 | 1 | 3 | 2 |
| 176 | C0 | 2 | 1 | 4,9 | 8 | 1 | 3 | 2 |
| 177 | C0 | 2 | 1 | 4,9 | 2 | 2 | 6 | 2 |
| 178 | C0 | 2 | 1 | 2,3,4,7,8,9 | 2 | 1 | 6 | 2 |
| 179 | C0 | 1 | 0 | 9 | 2 | 2 | 6 | 2 |
| 180 | C0 | 1 | 0 | 9 | 8 | 1 | 3 | 2 |
| 181 | C0 | 1 | 0 | 9 | 2 | 1 | 6 | 2 |
| 182 | C0 | 1 | 0 | 8,9 | 2 | 2 | 6 | 2 |
| 183 | C0 | 1 | 0 | 4,9 | 2 | 1 | 6 | 2 |
| 184 | C0 | 1 | 0 | 7,9 | 8 | 1 | 3 | 2 |
| 185 | C0 | 1 | 0 | 3,4,8,9 | 2 | 1 | 6 | 2 |
| 186 | C0 | 1 | 0 | 3,4,8,9 | 2 | 2 | 6 | 2 |
| 187 | C0 | 1 | 0 | 1,3,5,7,9 | 2 | 1 | 6 | 2 |
| 188 | C0 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 8 | 1 | 3 | 2 |
| 189 | C2 | 16 | 1 | 9 | 2 | 2 | 2 | 6 |
| 190 | C2 | 8 | 1 | 9 | 2 | 2 | 2 | 6 |
| 191 | C2 | 4 | 1 | 9 | 2 | 1 | 2 | 6 |
| 192 | C2 | 2 | 1 | 9 | 2 | 1 | 2 | 6 |
| 193 | C2 | 2 | 1 | 8,9 | 2 | 2 | 2 | 6 |
| 194 | C2 | 2 | 1 | 7,9 | 2 | 1 | 2 | 6 |
| 195 | C2 | 2 | 1 | 7,9 | 8 | 1 | 1 | 6 |
| 196 | C2 | 2 | 1 | 4,9 | 8 | 1 | 1 | 6 |
| 197 | C2 | 2 | 1 | 4,9 | 2 | 2 | 2 | 6 |
| 198 | C2 | 2 | 1 | 2,3,4,7,8,9 | 2 | 1 | 2 | 6 |
| 199 | C2 | 8 | 1 | 9 | 8 | 2 | 1 | 6 |
| 200 | C2 | 4 | 1 | 9 | 8 | 1 | 1 | 6 |
| 201 | C2 | 1 | 0 | 9 | 2 | 2 | 2 | 6 |
| 202 | C2 | 1 | 0 | 9 | 8 | 1 | 1 | 6 |
| 203 | C2 | 1 | 0 | 9 | 2 | 1 | 2 | 6 |
| 204 | C2 | 1 | 0 | 8,9 | 2 | 2 | 2 | 6 |
| 205 | C2 | 1 | 0 | 4,9 | 2 | 1 | 2 | 6 |
| 206 | C2 | 1 | 0 | 7,9 | 8 | 1 | 1 | 6 |
| 207 | C2 | 1 | 0 | 3,4,8,9 | 2 | 1 | 2 | 6 |
| 208 | C2 | 1 | 0 | 3,4,8,9 | 2 | 2 | 2 | 6 |
| 209 | C2 | 1 | 0 | 1,3,5,7,9 | 2 | 1 | 2 | 6 |
| 210 | C2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 8 | 1 | 1 | 6 |
| 211 | A1/B1 | 2 | 1 | 9 | 2 | 1 | 6 | 2 |
| 212 | A1/B1 | 2 | 1 | 4,9 | 8 | 1 | 3 | 2 |
| 213 | A1/B1 | 2 | 1 | 7,9 | 8 | 1 | 3 | 2 |
| 214 | A1/B1 | 2 | 1 | 7,9 | 2 | 1 | 6 | 2 |
| 215 | A1/B1 | 2 | 1 | 4,9 | 2 | 2 | 6 | 2 |
| 216 | A1/B1 | 2 | 1 | 8,9 | 2 | 2 | 6 | 2 |
| 217 | A1/B1 | 1 | 0 | 9 | 2 | 2 | 6 | 2 |
| 218 | A1/B1 | 1 | 0 | 9 | 8 | 1 | 3 | 2 |
| 219 | A1/B1 | 1 | 0 | 9 | 2 | 1 | 6 | 2 |
| 220 | A1/B1 | 1 | 0 | 8,9 | 2 | 2 | 6 | 2 |
| 221 | A1/B1 | 1 | 0 | 4,9 | 2 | 1 | 6 | 2 |
| 222 | A1/B1 | 1 | 0 | 7,9 | 8 | 1 | 3 | 2 |
| 223 | A1/B1 | 1 | 0 | 3,4,8,9 | 2 | 2 | 6 | 2 |
| 224 | A1/B1 | 1 | 0 | 1,3,5,7,9 | 2 | 1 | 6 | 2 |
| 225 | A1/B1 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 8 | 1 | 3 | 2 |
| 226 | A2/B2 | 2 | 1 | 9 | 0 | 1 | 3 | 4 |
| 227 | A2/B2 | 2 | 1 | 4,9 | 6 | 1 | 2 | 4 |
| 228 | A2/B2 | 2 | 1 | 7,9 | 6 | 1 | 2 | 4 |
| 229 | A2/B2 | 2 | 1 | 4,9 | 0 | 2 | 3 | 4 |
| 230 | A2/B2 | 2 | 1 | 8,9 | 0 | 2 | 3 | 4 |
| 231 | A2/B2 | 1 | 0 | 9 | 0 | 2 | 3 | 4 |
| 232 | A2/B2 | 1 | 0 | 9 | 6 | 1 | 2 | 4 |
| 233 | A2/B2 | 1 | 0 | 9 | 0 | 1 | 3 | 4 |
| 234 | A2/B2 | 1 | 0 | 8,9 | 0 | 2 | 3 | 4 |
| 235 | A2/B2 | 1 | 0 | 4,9 | 0 | 1 | 3 | 4 |
| 236 | A2/B2 | 1 | 0 | 7,9 | 6 | 1 | 2 | 4 |
| 237 | A2/B2 | 1 | 0 | 3,4,8,9 | 0 | 1 | 3 | 4 |
| 238 | A2/B2 | 1 | 0 | 3,4,8,9 | 0 | 2 | 3 | 4 |
| 239 | A2/B2 | 1 | 0 | 1,3,5,7,9 | 0 | 1 | 3 | 4 |
| 240 | A2/B2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 6 | 1 | 2 | 4 |
| 241 | A3/B3 | 2 | 1 | 9 | 0 | 1 | 2 | 6 |
| 242 | A3/B3 | 2 | 1 | 4,9 | 2 | 1 | 2 | 6 |
| 243 | A3/B3 | 2 | 1 | 7,9 | 0 | 1 | 2 | 6 |
| 244 | A3/B3 | 2 | 1 | 7,9 | 2 | 1 | 2 | 6 |
| 245 | A3/B3 | 2 | 1 | 4,9 | 0 | 2 | 2 | 6 |
| 246 | A3/B3 | 2 | 1 | 8,9 | 0 | 2 | 2 | 6 |
| 247 | A3/B3 | 1 | 0 | 9 | 0 | 2 | 2 | 6 |
| 248 | A3/B3 | 1 | 0 | 9 | 2 | 1 | 2 | 6 |
| 249 | A3/B3 | 1 | 0 | 9 | 0 | 1 | 2 | 6 |
| 250 | A3/B3 | 1 | 0 | 8,9 | 0 | 2 | 2 | 6 |
| 251 | A3/B3 | 1 | 0 | 4,9 | 0 | 1 | 2 | 6 |
| 252 | A3/B3 | 1 | 0 | 7,9 | 2 | 1 | 2 | 6 |
| 253 | A3/B3 | 1 | 0 | 3,4,8,9 | 0 | 2 | 2 | 6 |
| 254 | A3/B3 | 1 | 0 | 1,3,5,7,9 | 0 | 1 | 2 | 6 |
| 255 | A3/B3 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 2 | 1 | 2 | 6 |

Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum.

| PRACH Config. Index | Preamble format |  |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | A1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 1 | A1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 2 | A1 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 6 | 2 |
| 3 | A1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 4 | A1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 5 | A1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 |
| 6 | A1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 7 | A1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 8 | A1 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 6 | 2 |
| 9 | A1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 |
| 10 | A1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 |
| 11 | A1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 12 | A1 | 1 | 0 | 19,39 | 7 | 1 | 3 | 2 |
| 13 | A1 | 1 | 0 | 3,5,7 | 0 | 1 | 6 | 2 |
| 14 | A1 | 1 | 0 | 24,29,34,39 | 7 | 1 | 3 | 2 |
| 15 | A1 | 1 | 0 | 9,19,29,39 | 7 | 2 | 3 | 2 |
| 16 | A1 | 1 | 0 | 17,19,37,39 | 0 | 1 | 6 | 2 |
| 17 | A1 | 1 | 0 | 9,19,29,39 | 0 | 2 | 6 | 2 |
| 18 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 |
| 19 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 3 | 2 |
| 20 | A1 | 1 | 0 | 3,5,7,9,11,13 | 7 | 1 | 3 | 2 |
| 21 | A1 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 3 | 2 |
| 22 | A1 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 6 | 2 |
| 23 | A1 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 24 | A1 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 | 2 | 3 | 2 |
| 25 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 3 | 2 |
| 26 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 |
| 27 | A1 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 6 | 2 |
| 28 | A1 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 3 | 2 |
| 29 | A2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 30 | A2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 31 | A2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 32 | A2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 33 | A2 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 3 | 4 |
| 34 | A2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 |
| 35 | A2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 36 | A2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 37 | A2 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 3 | 4 |
| 38 | A2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 |
| 39 | A2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 |
| 40 | A2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 41 | A2 | 1 | 0 | 19,39 | 5 | 1 | 2 | 4 |
| 42 | A2 | 1 | 0 | 3,5,7 | 0 | 1 | 3 | 4 |
| 43 | A2 | 1 | 0 | 24,29,34,39 | 5 | 1 | 2 | 4 |
| 44 | A2 | 1 | 0 | 9,19,29,39 | 5 | 2 | 2 | 4 |
| 45 | A2 | 1 | 0 | 17,19,37,39 | 0 | 1 | 3 | 4 |
| 46 | A2 | 1 | 0 | 9, 19, 29, 39 | 0 | 2 | 3 | 4 |
| 47 | A2 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 3 | 4 |
| 48 | A2 | 1 | 0 | 23,27,31,35,39 | 5 | 1 | 2 | 4 |
| 49 | A2 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 50 | A2 | 1 | 0 | 3,5,7,9,11,13 | 5 | 1 | 2 | 4 |
| 51 | A2 | 1 | 0 | 3,5,7,9,11,13 | 0 | 1 | 3 | 4 |
| 52 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 5 | 1 | 2 | 4 |
| 53 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 |
| 54 | A2 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 5 | 2 | 2 | 4 |
| 55 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 5 | 1 | 2 | 4 |
| 56 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 |
| 57 | A2 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 3 | 4 |
| 58 | A2 | 1 | 0 | 0,1,2,…,39 | 5 | 1 | 2 | 4 |
| 59 | A3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 60 | A3 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 61 | A3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 62 | A3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 63 | A3 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 2 | 6 |
| 64 | A3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 65 | A3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 66 | A3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 67 | A3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 68 | A3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 69 | A3 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 70 | A3 | 1 | 0 | 19,39 | 7 | 1 | 1 | 6 |
| 71 | A3 | 1 | 0 | 3,5,7 | 0 | 1 | 2 | 6 |
| 72 | A3 | 1 | 0 | 9,11,13 | 2 | 1 | 2 | 6 |
| 73 | A3 | 1 | 0 | 24,29,34,39 | 7 | 1 | 1 | 6 |
| 74 | A3 | 1 | 0 | 9,19,29,39 | 7 | 2 | 1 | 6 |
| 75 | A3 | 1 | 0 | 17,19,37,39 | 0 | 1 | 2 | 6 |
| 76 | A3 | 1 | 0 | 9,19,29,39 | 0 | 2 | 2 | 6 |
| 77 | A3 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 2 | 6 |
| 78 | A3 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 1 | 6 |
| 79 | A3 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 80 | A3 | 1 | 0 | 3,5,7,9,11,13 | 0 | 1 | 2 | 6 |
| 81 | A3 | 1 | 0 | 3,5,7,9,11,13 | 7 | 1 | 1 | 6 |
| 82 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 83 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 1 | 6 |
| 84 | A3 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 | 2 | 1 | 6 |
| 85 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 1 | 6 |
| 86 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 87 | A3 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 2 | 6 |
| 88 | A3 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 1 | 6 |
| 89 | B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 90 | B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 91 | B1 | 8 | 1,2 | 9,19,29,39 | 2 | 2 | 6 | 2 |
| 92 | B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 93 | B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 2 | 6 | 2 |
| 94 | B1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 95 | B1 | 1 | 0 | 19,39 | 8 | 1 | 3 | 2 |
| 96 | B1 | 1 | 0 | 3,5,7 | 2 | 1 | 6 | 2 |
| 97 | B1 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 | 2 |
| 98 | B1 | 1 | 0 | 9,19,29,39 | 8 | 2 | 3 | 2 |
| 99 | B1 | 1 | 0 | 17,19,37,39 | 2 | 1 | 6 | 2 |
| 100 | B1 | 1 | 0 | 9,19,29,39 | 2 | 2 | 6 | 2 |
| 101 | B1 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 6 | 2 |
| 102 | B1 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 103 | B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 104 | B1 | 1 | 0 | 3,5,7,9,11,13 | 8 | 1 | 3 | 2 |
| 105 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 | 2 |
| 106 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 107 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 108 | B1 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 8 | 2 | 3 | 2 |
| 109 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 110 | B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 6 | 2 |
| 111 | B1 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 | 2 |
| 112 | B4 | 16 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 113 | B4 | 16 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 114 | B4 | 8 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 115 | B4 | 8 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 116 | B4 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 1 | 12 |
| 117 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 118 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 119 | B4 | 4 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 120 | B4 | 2 | 1 | 7,15,23,31,39 | 2 | 2 | 1 | 12 |
| 121 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 122 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 123 | B4 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 124 | B4 | 1 | 0 | 19, 39 | 2 | 2 | 1 | 12 |
| 125 | B4 | 1 | 0 | 17, 19, 37, 39 | 0 | 1 | 1 | 12 |
| 126 | B4 | 1 | 0 | 24,29,34,39 | 2 | 1 | 1 | 12 |
| 127 | B4 | 1 | 0 | 9,19,29,39 | 2 | 2 | 1 | 12 |
| 128 | B4 | 1 | 0 | 9,19,29,39 | 0 | 2 | 1 | 12 |
| 129 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 1 | 12 |
| 130 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 2 | 1 | 12 |
| 131 | B4 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 132 | B4 | 1 | 0 | 23,27,31,35,39 | 2 | 2 | 1 | 12 |
| 133 | B4 | 1 | 0 | 9,11,13,15,17,19 | 0 | 1 | 1 | 12 |
| 134 | B4 | 1 | 0 | 3,5,7,9,11,13 | 2 | 1 | 1 | 12 |
| 135 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 136 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 1 | 12 |
| 137 | B4 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 2 | 2 | 1 | 12 |
| 138 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 139 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 | 12 |
| 140 | B4 | 1 | 0 | 3, 5, 7, …, 23,25 | 2 | 1 | 1 | 12 |
| 141 | B4 | 1 | 0 | 3, 5, 7, …, 23,25 | 0 | 2 | 1 | 12 |
| 142 | B4 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 1 | 12 |
| 143 | B4 | 1 | 0 | 0, 1, 2,…, 39 | 2 | 1 | 1 | 12 |
| 144 | C0 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 |
| 145 | C0 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 146 | C0 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 147 | C0 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 148 | C0 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 7 | 2 |
| 149 | C0 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 150 | C0 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 |
| 151 | C0 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 152 | C0 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 7 | 2 |
| 153 | C0 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 154 | C0 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 |
| 155 | C0 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 156 | C0 | 1 | 0 | 19,39 | 8 | 1 | 3 | 2 |
| 157 | C0 | 1 | 0 | 3,5,7 | 0 | 1 | 7 | 2 |
| 158 | C0 | 1 | 0 | 24,29,34,39 | 8 | 1 | 3 | 2 |
| 159 | C0 | 1 | 0 | 9,19,29,39 | 8 | 2 | 3 | 2 |
| 160 | C0 | 1 | 0 | 17,19,37,39 | 0 | 1 | 7 | 2 |
| 161 | C0 | 1 | 0 | 9,19,29,39 | 0 | 2 | 7 | 2 |
| 162 | C0 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 163 | C0 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 7 | 2 |
| 164 | C0 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 165 | C0 | 1 | 0 | 3,5,7,9,11,13 | 8 | 1 | 3 | 2 |
| 166 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 | 2 |
| 167 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 |
| 168 | C0 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 8 | 2 | 3 | 2 |
| 169 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 170 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 |
| 171 | C0 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 7 | 2 |
| 172 | C0 | 1 | 0 | 0,1,2,…,39 | 8 | 1 | 3 | 2 |
| 173 | C2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 174 | C2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 175 | C2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 176 | C2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 177 | C2 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 2 | 6 |
| 178 | C2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 179 | C2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 180 | C2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 181 | C2 | 2 | 1 | 7,15,23,31,39 | 2 | 2 | 2 | 6 |
| 182 | C2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 |
| 183 | C2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 |
| 184 | C2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 185 | C2 | 1 | 0 | 19,39 | 2 | 1 | 2 | 6 |
| 186 | C2 | 1 | 0 | 3,5,7 | 0 | 1 | 2 | 6 |
| 187 | C2 | 1 | 0 | 24,29,34,39 | 7 | 1 | 1 | 6 |
| 188 | C2 | 1 | 0 | 9,19,29,39 | 7 | 2 | 1 | 6 |
| 189 | C2 | 1 | 0 | 17,19,37,39 | 0 | 1 | 2 | 6 |
| 190 | C2 | 1 | 0 | 9,19,29,39 | 2 | 2 | 2 | 6 |
| 191 | C2 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 2 | 6 |
| 192 | C2 | 1 | 0 | 3,5,7,9,11,13 | 7 | 1 | 1 | 6 |
| 193 | C2 | 1 | 0 | 23,27,31,35,39 | 7 | 2 | 1 | 6 |
| 194 | C2 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 195 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 2 | 1 | 6 |
| 196 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 197 | C2 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 | 2 | 1 | 6 |
| 198 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 1 | 6 |
| 199 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |
| 200 | C2 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 2 | 6 |
| 201 | C2 | 1 | 0 | 0,1,2,…,39 | 7 | 1 | 1 | 6 |
| 202 | A1/B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 203 | A1/B1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 204 | A1/B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 205 | A1/B1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 206 | A1/B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 207 | A1/B1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 208 | A1/B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 209 | A1/B1 | 1 | 0 | 19,39 | 8 | 1 | 3 | 2 |
| 210 | A1/B1 | 1 | 0 | 9,19,29,39 | 8 | 1 | 3 | 2 |
| 211 | A1/B1 | 1 | 0 | 17,19,37,39 | 2 | 1 | 6 | 2 |
| 212 | A1/B1 | 1 | 0 | 9,19,29,39 | 2 | 2 | 6 | 2 |
| 213 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 | 2 |
| 214 | A1/B1 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 6 | 2 |
| 215 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 216 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 | 2 |
| 217 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 |
| 218 | A1/B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 6 | 2 |
| 219 | A1/B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 6 | 2 |
| 220 | A2/B2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 221 | A2/B2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 222 | A2/B2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 223 | A2/B2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 224 | A2/B2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 225 | A2/B2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 226 | A2/B2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 227 | A2/B2 | 1 | 0 | 19,39 | 6 | 1 | 2 | 4 |
| 228 | A2/B2 | 1 | 0 | 9,19,29,39 | 6 | 1 | 2 | 4 |
| 229 | A2/B2 | 1 | 0 | 17,19,37,39 | 2 | 1 | 3 | 4 |
| 230 | A2/B2 | 1 | 0 | 9,19,29,39 | 2 | 2 | 3 | 4 |
| 231 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 6 | 1 | 2 | 4 |
| 232 | A2/B2 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 3 | 4 |
| 233 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 234 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 6 | 1 | 2 | 4 |
| 235 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 |
| 236 | A2/B2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 3 | 4 |
| 237 | A2/B2 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 3 | 4 |
| 238 | A3/B3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 239 | A3/B3 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 240 | A3/B3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 241 | A3/B3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 242 | A3/B3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 243 | A3/B3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 244 | A3/B3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 245 | A3/B3 | 1 | 0 | 19,39 | 2 | 1 | 2 | 6 |
| 246 | A3/B3 | 1 | 0 | 9,19,29,39 | 2 | 1 | 2 | 6 |
| 247 | A3/B3 | 1 | 0 | 17,19,37,39 | 2 | 1 | 2 | 6 |
| 248 | A3/B3 | 1 | 0 | 9,19,29,39 | 2 | 2 | 2 | 6 |
| 249 | A3/B3 | 1 | 0 | 7,15,23,31,39 | 2 | 1 | 2 | 6 |
| 250 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 251 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 2 | 2 | 2 | 6 |
| 252 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 2 | 6 |
| 253 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 2 | 6 |
| 254 | A3/B3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 2 | 6 |
| 255 | A3/B3 | 1 | 0 | 1,3,5,7,…,37,39 | 2 | 1 | 2 | 6 |