3GPP TSG-RAN WG1 Meeting #99	R1-1913669

# Reno, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0078 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of multiple LTE CRS rate matching patterns |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2019-12-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Extend the LTE CRS rate mathching to apply when multiple LTE CRS rate matching patterns are configured for one NR carrier |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce LTE-CRS-PatternList-r16, that includes multiple LTE-CRS-toMatchAround configurations, to the LTE CRS pattern rate matching |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Only one LTE CRS rate matching pattern for an NR carrier is supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10, 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 10	UE procedure for receiving control information

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG except for PDCCH monitoring in Type0/0A/2-PDCCH CSS sets where the UE is not required to apply the procedures in this clause for the SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

A UE monitors a set of PDCCH candidates in one or more CORESETs on the active DL BWP on each activated serving cell configured with PDCCH monitoring according to corresponding search space sets where monitoring implies decoding each PDCCH candidate according to the monitored DCI formats.

For monitoring of a PDCCH candidate in a slot

-	If the UE has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if the UE does not monitor PDCCH candidates in a Type0-PDCCH CSS set and at least one RE for a PDCCH candidate overlaps with at least one RE corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate.

-	If a UE has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if the UE does not monitor PDCCH candidates in a Type0-PDCCH CSS set and at least one RE for a PDCCH candidate overlaps with at least one RE corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate.

-	If the UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in Subclause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell.

-	If at least one RE of a PDCCH candidate on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList-r16, the UE is not required to monitor the PDCCH candidate.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than 4 serving cells, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates the UE can monitor per slot when the UE is configured for carrier aggregation operation over more than 4 cells. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates per slot that corresponds to  downlink cells, where

-	 is the number of configured downlink cells if the UE does not provide pdcch-BlindDetectionCA

-	otherwise,  is the value of pdcch-BlindDetectionCA

When a UE is configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates per slot that corresponds to  downlink cells for the MCG where  is provided by pdcch-BlindDetection for the MCG and determines a capability to monitor a maximum number of PDCCH candidates per slot that corresponds to  downlink cells for the SCG where  is provided by pdcch-BlindDetection for the SCG. When the UE is configured for carrier aggregation operation over more than 4 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per slot a number of PDCCH candidates that is larger than the maximum number as derived from the corresponding value of .

When a UE is configured for NR-DC operation with a total of downlink cells on both the MCG and the SCG, the UE expects to be provided pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG with values that satisfy

-	pdcch-BlindDetection for the MCG + pdcch-BlindDetection for the SCG <= pdcch-BlindDetectionCA, if the UE reports pdcch-BlindDetectionCA, or

-	pdcch-BlindDetection for the MCG + pdcch-BlindDetection for the SCG <= , if the UE does not report pdcch-BlindDetectionCA.

For NR-DC operation, the UE may indicate, through pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG.

If the UE reports pdcch-BlindDetectionCA,

-	the value range of pdcch-BlindDetectionMCG-UE or of pdcch-BlindDetectionSCG-UE is [1, …, pdcch-BlindDetectionCA-1], and

-	pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA.

Otherwise,  if  is a maximum total number of downlink cells that the UE can be configured on both the MCG and the SCG as described in [10, TS 38.133],

-	the value range of pdcch-BlindDetectionMCG-UE or of pdcch-BlindDetectionSCG-UE is [1, 2, 3],

-	pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= .

## 10.1	UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI or a TC-RNTI on the primary cell

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, or TPC-SRS-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, or CS-RNTI(s).

For a DL BWP, if a UE is not provided searchSpaceSIB1 for Type0-PDCCH CSS set by PDCCH-ConfigCommon, the UE does not monitor PDCCH candidates for a Type0-PDCCH CSS set on the DL BWP. The Type0-PDCCH CSS set is defined by the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level given in Table 10.1-1. If the active DL BWP and the initial DL BWP have same SCS and same CP length and the active DL BWP includes all RBs of the CORESET with index 0, or the active DL BWP is the initial DL BWP, the CORESET configured for Type0-PDCCH CSS set has CORESET index 0 and the Type0-PDCCH CSS set has search space set index 0.

For a DL BWP, if the UE is not provided searchSpaceOtherSystemInformation for Type0A-PDCCH CSS set, the UE does not monitor PDCCH for Type0A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type0A-PDCCH CSS set are given in Table 10.1-1.

For a DL BWP and a Type1-PDCCH CSS set, a UE is provided a configuration for a search space by ra-SearchSpace. If the UE has not been provided a Type3-PDCCH CSS set or a USS set and the UE has received a C-RNTI, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI in the Type1-PDCCH CSS set.

If a UE is not provided pagingSearchSpace for Type2-PDCCH CSS set, the UE does not monitor PDCCH for Type2-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type2-PDCCH CSS set are given in Table 10.1-1.

If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in Subclause 13, and the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of

-	a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or

-	a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure

If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set based on the search space set associated with the value of searchSpaceID.

The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the CORESET configured by pdcch-ConfigSIB1 in MIB and for corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain, QCL-TypeA, and QCL-TypeD properties, when applicable [6, TS 38.214], and if the UE is not provided a TCI state indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in the CORESET. The value for the DM-RS scrambling sequence initialization is the cell ID. A SCS is provided by subCarrierSpacingCommon in MIB.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set does not have same QCL-TypeD properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and

-	a C-RNTI, an MCS-C-RNTI, or a CS-RNTI,

the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.

Table 10.1-1: CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for CSS sets configured by searchSpaceSIB1

| CCE Aggregation Level | Number of Candidates |
| --- | --- |
| 4 | 4 |
| 8 | 2 |
| 16 | 1 |

For each DL BWP configured to a UE in a serving cell, a UE can be provided by higher layer signalling with  CORESETs. For each CORESET, the UE is provided the following by ControlResourceSet:

-	a CORESET index , , by controlResourceSetId;

-	a DM-RS scrambling sequence initialization value by pdcch-DMRS-ScramblingID;

-	a precoder granularity for a number of REGs in the frequency domain where the UE can assume use of a same DM-RS precoder by precoderGranularity;

-	a number of consecutive symbols provided by duration;

-	a set of resource blocks provided by frequencyDomainResources;

-	CCE-to-REG mapping parameters provided by cce-REG-MappingType;

-	an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by TCI-State, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in a respective CORESET;

-	an indication for a presence or absence of a transmission configuration indication (TCI) field for DCI format 1_1 transmitted by a PDCCH in CORESET , by tci-PresentInDCI.

When precoderGranularity = allContiguousRBs, a UE does not expect

-	to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency

-	any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround, or from LTE-CRS-PatternList-r16, or with any RE of a SS/PBCH block.

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. The bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  PRBs with starting common RB position  where the first common RB of the first group of 6 PRBs has common RB index .

For a CORESET other than a CORESET with index 0,

-	if a UE has not been provided a configuration of TCI state(s) by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET, or has been provided initial configuration of more than one TCI states for the CORESET by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure;

-	if a UE has been provided a configuration of more than one TCI states by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET as part of Reconfiguration with sync procedure as described in [12, TS 38.331] but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

For a CORESET with index 0, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with

-	the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or

-	a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure.

For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI state. For a CORESET with index 0, the UE expects that QCL-TypeD of a CSI-RS in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block

-	if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and  is the SCS configuration for the PUCCH. The active BWP is defined as the active BWP in the slot when the activation command is applied.

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with  search space sets where, for each search space set from the  search space sets, the UE is provided the following by SearchSpace:

-	a search space set index , , by searchSpaceId

-	an association between the search space set  and a CORESET  by controlResourceSetId

-	a PDCCH monitoring periodicity of  slots and a PDCCH monitoring offset of  slots, by monitoringSlotPeriodicityAndOffset

-	a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot

-	a duration of  slots indicating a number of slots that the search space set  exists by duration

-	a number of PDCCH candidates  per CCE aggregation level  by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively

-	an indication that search space set  is either a CSS set or a USS set by searchSpaceType

-	if search space set  is a CSS set

-	an indication by dci-Format0-0-AndFormat1-0 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0

-	an indication by dci-Format2-0 to monitor one or two PDCCH candidates for DCI format 2_0 and a corresponding CCE aggregation level

-	an indication by dci-Format2-1 to monitor PDCCH candidates for DCI format 2_1

-	an indication by dci-Format2-2 to monitor PDCCH candidates for DCI format 2_2

-	an indication by dci-Format2-3 to monitor PDCCH candidates for DCI format 2_3

-	if search space set  is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1

If the monitoringSymbolsWithinSlot indicates to a UE to monitor PDCCH in a subset of up to three consecutive symbols that are same in every slot where the UE monitors PDCCH for all search space sets, the UE does not expect to be configured with a PDCCH SCS other than 15 kHz if the subset includes at least one symbol after the third symbol.

A UE does not expect to be provided a first symbol and a number of consecutive symbols for a CORESET that results to a PDCCH candidate mapping to symbols of different slots.

A UE does not expect any two PDCCH monitoring occasions on an active DL BWP, for a same search space set or for different search space sets, in a same CORESET to be separated by a non-zero number of symbols that is smaller than the CORESET duration.

A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set , the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number  [4, TS 38.211] in a frame with number  if nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set  for  consecutive slots, starting from slot , and does not monitor PDCCH candidates for search space set  for the next  consecutive slots.

A USS at CCE aggregation level  is defined by a set of PDCCH candidates for CCE aggregation level .

If a UE is configured with CrossCarrierSchedulingConfig for a serving cell the carrier indicator field value corresponds to the value indicated by CrossCarrierSchedulingConfig.

For an active DL BWP of a serving cell on which a UE monitors PDCCH candidates in a USS, if the UE is not configured with a carrier indicator field, the UE monitors the PDCCH candidates without carrier indicator field. For an active DL BWP of a serving cell on which a UE monitors PDCCH candidates in a USS, if a UE is configured with a carrier indicator field, the UE monitors the PDCCH candidates with carrier indicator field.

A UE does not expect to monitor PDCCH candidates on an active DL BWP of a secondary cell if the UE is configured to monitor PDCCH candidates with carrier indicator field corresponding to that secondary cell in another serving cell. For the active DL BWP of a serving cell on which the UE monitors PDCCH candidates, the UE monitors PDCCH candidates at least for the same serving cell.

For a search space set  associated with CORESET , the CCE indexes for aggregation level  corresponding to PDCCH candidate  of the search space set in slot  for an active DL BWP of a serving cell corresponding to carrier indicator field value  are given by

where

for any CSS, ;

for a USS, , ,  for ,  for ,  for , and ;

;

is the number of CCEs, numbered from 0 to , in CORESET ;

is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, ;

, where  is the number of PDCCH candidates the UE is configured to monitor for aggregation level  of a search space set  for a serving cell corresponding to ;

for any CSS, ;

for a USS,  is the maximum of  over all configured  values for a CCE aggregation level  of search space set  ;

the RNTI value used for  is the C-RNTI.

A UE that

-	is configured for operation with carrier aggregation, and

-	indicates support of search space sharing through searchSpaceSharingCA-UL or through searchSpaceSharingCA-DL, and

-	has a PDCCH candidate with CCE aggregation level  in CORESET  for a DCI format 0_1 or a DCI format 1_1 having a first size and associated with serving cell ,

can receive a corresponding PDCCH through a PDCCH candidate with CCE aggregation level  in CORESET  for a DCI format 0_1 or a DCI format 1_1, respectively, having a second size and associated with serving cell  if the first size and the second size are same.

A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP.

A PDCCH candidate with index  for a search space set  using a set of CCEs in a CORESET  on the active DL BWP for serving cell  is not counted for monitoring if there is a PDCCH candidate with index  for a search space set , or if there is a PDCCH candidate with index  and , in the CORESET  on the active DL BWP for serving cell  using a same set of CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index  is counted for monitoring.

Table 10.1-2 provides the maximum number of monitored PDCCH candidates, , for a DL BWP with SCS configuration  for a UE per slot for operation with a single serving cell.

Table 10.1-2: Maximum number  of monitored PDCCH candidates per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of monitored PDCCH candidates per slot and per serving cell |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 10.1-3 provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration  that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell.

CCEs for PDCCH candidates are non-overlapped if they correspond to

-	different CORESET indexes, or

-	different first symbols for the reception of the respective PDCCH candidates.

Table 10.1-3: Maximum number  of non-overlapped CCEs per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of non-overlapped CCEs per slot and per serving cell |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

If a UE is configured with  downlink cells with DL BWPs having SCS configuration  where , the UE is not required to monitor, on the active DL BWP of the scheduling cell, more than  PDCCH candidates or more than  non-overlapped CCEs per slot for each scheduled cell.

If a UE is configured with  downlink cells with DL BWPs having SCS configuration , where , a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than  PDCCH candidates or more than  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the  downlink cells.

For each scheduled cell, the UE is not required to monitor on the active DL BWP with SCS configuration  of the scheduling cell more than  PDCCH candidates or more than  non-overlapped CCEs per slot.

A UE does not expect to be configured CSS sets that result to corresponding total, or per scheduled cell, numbers of monitored PDCCH candidates and non-overlapped CCEs per slot that exceed the corresponding maximum numbers per slot.

For same cell scheduling or for cross-carrier scheduling where a scheduling cell and scheduled cell(s) have DL BWPs with same SCS configuration , a UE does not expect a number of PDCCH candidates, and a number of corresponding non-overlapped CCEs per slot on a secondary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the secondary cell per slot.

For cross-carrier scheduling, the number of PDCCH candidates for monitoring and the number of non-overlapped CCEs per slot are separately counted for each scheduled cell.

For all search space sets within a slot , denote by  a set of CSS sets with cardinality of  and by  a set of USS sets with cardinality of . The location of USS sets , , in  is according to an ascending order of the search space set index.

Denote by , , the number of counted PDCCH candidates for monitoring for CSS set  and by , , the number of counted PDCCH candidates for monitoring for USS set .

For the CSS sets, a UE monitors  PDCCH candidates requiring a total of  non-overlapping CCEs in a slot.

The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration  in slot  according to the following pseudocode. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring.

Denote by  the set of non-overlapping CCEs for search space set  and by  the cardinality of  where the non-overlapping CCEs for search space set  are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets , .

Set

Set

Set

while  AND

allocate  PDCCH candidates for monitoring to USS set

;

;

;

end while

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) of one or more cells

the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs having same QCL-TypeD properties as the CORESET, on the active DL BWP of a cell from the one or more cells

-	the CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index

-	the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

-	for the purpose of determining the CORESET, a SS/PBCH block is considered to have different QCL-TypeD properties than a CSI-RS

-	for the purpose of determining the CORESET, a first CSI-RS associated with a SS/PBCH block in a first cell and a second CSI-RS in a second cell that is also associated with the SS/PBCH block are assumed to have same QCL-TypeD properties

-	the allocation of non-overlapping CCEs and of PDCCH candidates for PDCCH monitoring is according to all search space sets associated with the multiple CORESETs on the active DL BWP(s) of the one or more cells

-	the number of active TCI states is determined from the multiple CORESETs

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs where none of the CORESETs has TCI-states with 'QCL-TypeD',

the UE is required to monitor PDCCH candidates in overlapping PDCCH monitoring occasions for search space sets associated with different CORESETs.

For a scheduled cell and at any time, a UE expects to have received at most 16 PDCCHs for DCI formats 1_0 or 1_1 with CRC scrambled by C-RNTI, CS-RNTI, or MCS-RNTI scheduling 16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol and at most 16 PDCCHs for DCI formats 0_0 or 0_1 with CRC scrambled by C-RNTI, CS-RNTI, or MCS-RNTI scheduling 16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol.

If a UE

-	is not configured for NR-DC operation and indicates through pdcch-BlindDetectionCA a capability to monitor PDCCH candidates for  downlink cells and the UE is configured with  downlink cells or  uplink cells, or

-	is configured with NR-DC operation and for a cell group with  downlink cells or  uplink cells

the UE expects to have respectively received at most  PDCCHs for

-	DCI formats 1_0 or 1_1 with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-RNTI scheduling  PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all  downlink cells

-	DCI formats 0_0 or 0_1 with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-RNTI scheduling  PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all  uplink cells

If a UE

-	is configured to monitor a first PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a CSS set and a second PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a USS set in a CORESET with index zero on an active DL BWP, and

-	the DCI formats 0_0/1_0 associated with the first PDCCH candidate and the DCI formats 0_0/1_0 associated with the second PDCCH candidate have same size, and

-	the UE receives the first PDCCH candidate and the second PDCCH candidate over a same set of CCEs, and

-	the first PDCCH candidate and the second PDCCH candidate have identical scrambling, and

-	the DCI formats 0_0/1_0 for the first PDCCH candidate and the DCI formats 0_0/1_0 for the second PDCCH candidate have CRC scrambled by either C-RNTI, or MCS-C-RNTI, or CS-RNTI

the UE decodes only the DCI formats 0_0/1_0 associated with the first PDCCH candidate.

If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format.

A UE configured with a bandwidth part indicator in DCI formats 0_1 or 1_1 determines, in case of an active DL BWP or of an active UL BWP change, the DCI information applicable to the new active DL BWP or UL BWP, respectively, as described in Subclause 12.

For unpaired spectrum operation, if a UE is not configured for PUSCH/PUCCH transmission on serving cell , the UE does not expect to monitor PDCCH on serving cell  if the PDCCH overlaps in time with SRS transmission (including any interruption due to uplink or downlink RF retuning time [10, TS 38.133]) on serving cell  and if the UE is not capable of simultaneous reception and transmission on serving cell and serving cell .

If a UE is provided resourceBlocks and symbolsInResourceBlock in RateMatchPattern, or if the UE is additionally provided periodicityAndPattern in RateMatchPattern, the UE can determine a set of RBs in symbols of a slot that are not available for PDSCH reception as described in [6, TS 38.214]. If a PDCCH candidate in a slot is mapped to one or more REs that overlap with REs of any RB in the set of RBs in symbols of the slot, the UE does not expect to monitor the PDCCH candidate.