3GPP TSG RAN WG1 Ad-hoc Meeting	R1-151010

Paris, France, 24th – 26th March 2015

Source:	Sharp

Title:	Coexistence evaluation results for DL only LAA in indoor scenario

Agenda Item:	2.1

Document for:	Discussion and Decision

- Introduction

RAN1 initiated the study item phase for Licensed-Assisted Access (LAA) [1] in RAN1#78bis. In RAN1#79 [2], some details of evaluation methodologies were agreed. According to section 8.1 in TR 36.889 [3], the following evaluation methodology is followed in the WiFi + LAA coexistence case.

In the Wi-Fi-LAA coexistence case, the following evaluation methodology is followed.

- For each UE and eNB/AP drop

  - Step 1: Performance metrics for two Wi-Fi networks coexisting in a given evaluation scenario are evaluated and recorded.

  - Step 2: Wi-Fi is replaced with LAA for the group of eNBs and UEs served by one of the Wi-Fi operators. Performance metrics of the Wi-Fi network coexisting with the LAA network are evaluated and recorded.

A comparison of the performance metrics between the two steps for the Wi-Fi network that was not replaced with LAA can be used to evaluate coexistence between LAA and Wi-Fi in an unlicensed band.

Furthermore, some agreements were reached with regard to the kind of LBT schemes that should be evaluated [4].

Agreements:

- Classify the evaluated LBT schemes according to the following categories:

  - Category 1: No LBT

  - Category 2: LBT without random back-off

  - Category 3: LBT with random back-off with fixed size of contention window

  - Category 4: LBT with random back-off with variable size of contention window

Note: Contention window is the maximum possible random back-off value

Note: Category classification does not restrict a LBT design investigation

Note: Company is encouraged to evaluate many categories as much as possible

- Illustrative examples

  - FBE procedure as defined in EN BRAN V1.8.0 belongs to category 2

  - LBE procedure with a fixed q for the contention window as defined in EN BRAN V1.8.0 belongs to category 3

  - LBE procedure Op A with a variable q for the contention window as defined in EN BRAN V1.8.0 belongs to category 4

In this contribution we provide simulation results of coexistence between LAA and WiFi for downlink.

- Listen before talk

In our evaluation, we study three different listen-before-talk (LBT) algorithms that are applied to LAA eNBs as follows:

LBT scheme 1 (category 2): Fixed CCA timing without random back-off

Under LBT scheme 1, we assume that LAA eNBs start CCA at a fixed time instant, and the CCA duration is also fixed. That time instant is around 20 us before a subframe boundary.

If the channel is busy, the eNB waits for the next subframe to perform a new CCA.

If the LAA eNB judges the channel to be idle, the LAA eNB can start a burst transmission spanning 3 subframes and 11 symbols. After that, the LAA eNB performs CCA again in the last 3 OFDM symbols of the fourth subframe, and if it doesn't detect any signal in the channel, the LAA eNB transmits a new burst from the first OFDM symbol of the next subframe (if the LAA eNB has still DL buffer).

In our simulation, the maximum burst transmission time is 4 ms (3 contiguous subframes and 11 OFDM symbols can be used to transmit PDSCH). The LAA eNB performs CCA in the last three OFDM symbols to guarantee idle period of at least 5% of the actual occupancy time.

Figure 1: Mechanism of LBT scheme 1 for LAA

LBT scheme 2 (category 3): Fixed CCA timing with random back-off

Under LBT scheme 2, we assume that LAA eNBs can only occupy the channel in the last three OFDM symbols of a subframe (which is the contention window).

Three OFDM symbols last about 214 us, while the CCA check needs to be performed for at least 20 us for the eNB to be able to consider a channel clear. Given the nature of LAA, we consider it wise to segment these last three OFDM symbols and define some time instants in which a CCA check can be performed. For example, the channel could be segmented in 10 instants (CCA slots) separated by 21.4 us, which could be referred to by an index Taccess = [1, 10] (more granular segmentation would also be possible, in which case a 20 us CCA check would span multiple CCA slots). Different eNBs could then be assigned a random integer in this range, which would be their random back-off counter.

Each eNB decrements the value of its back-off counter at every CCA slot instant. When the back-off counter reaches 0, if the channel is busy, the eNB will wait until the next contention window. If the channel is considered idle the eNB starts transmitting (either the initial signal or directly PDSCH if the CCA slot is the one immediately preceding a subframe boundary).

An eNB can keep transmitting until OFDM symbol #10 of the fourth subframe. This way the 4 ms length limit is respected even if the eNB started transmitting right after the first CCA slot, and the process can be repeated in those last three OFDM symbols, making it possible to chain transmissions if no other device contends for the channel and thus increasing the system's efficiency.

Figure 2: Mechanism of LBT scheme 2 for LAA

LBT scheme 3 (category 3): Flexible CCA timing with random back-off

Under LBT scheme 3, we assume that LAA eNBs can only occupy the channel at any moment within the first three OFDM symbols after a channel becomes idle (which is this scheme's flexible contention window).

As in LBT scheme 2, some CCA slots are defined within a contention window of 3 OFDM symbols. The difference is that this contention window is not fixed to any subframe structure; it starts right after the channel is considered idle.

Each LAA node is assigned a random number in the range of the CCA slots, decreasing the value of its back-off counter at every CCA slot.

When the back-off counter reaches 0, if the channel is busy, the eNB will wait until the next contention window. If the channel is considered idle the eNB starts transmitting (either the initial signal or directly PDSCH if the CCA slot is the one immediately preceding a subframe boundary).

An eNB can keep transmitting for a total duration of 4 ms, freeing the channel afterwards. The next contention window starts 20 us after the end of the transmission (if no other device occupies the channel before).

Figure 3: Mechanism of LBT scheme 3 for LAA

- Simulation results

We evaluated the UPT performances by system level simulation in indoor scenarios according to the evaluation methodologies agreed in RAN1#79 [2] and the LBT categories agreed in RAN1#80 [4]. Note that the detailed simulation assumptions are listed in the Annex. In our assumption, operator A operates WiFi APs in both steps, and operator B replaces WiFi APs in step 1 with LAA eNBs in step 2. Operation in the licensed carrier is not included in the simulations.

Table 1 shows the simulation results for an "indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic", classified into low, middle and high traffic conditions according to buffer occupancy (BO) [5].

Table 1: indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

| LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| LBT 1 (cat 2) | UPT CDF [Mbps] | 5% | 16.37 | 19.35 | 27.14 | 3.63 | 11.55 | 18.07 | 0.19 | 0.79 | 1.34 |
|  |  | 50% | 46.16 | 53.13 | 56.62 | 31.29 | 44.35 | 47.12 | 4.91 | 24.53 | 25.16 |
|  |  | 95% | 68.18 | 72.24 | 80.01 | 59.75 | 68.68 | 71.48 | 32.46 | 53.09 | 56.50 |
|  |  | Mean | 44.96 | 49.96 | 55.38 | 31.01 | 42.82 | 46.57 | 9.06 | 25.37 | 26.16 |
|  | Delay CDF [s] | 5% | 0.048 | 0.048 | 0.045 | 0.048 | 0.048 | 0.045 | 0.093 | 0.052 | 0.046 |
|  |  | 50% | 0.096 | 0.079 | 0.074 | 0.158 | 0.097 | 0.097 | 0.589 | 0.186 | 0.214 |
|  |  | 95% | 0.443 | 0.358 | 0.329 | 1.284 | 0.548 | 0.505 | 4.432 | 1.545 | 2.312 |
|  |  | Mean | 0.154 | 0.131 | 0.113 | 0.351 | 0.187 | 0.163 | 1.165 | 0.417 | 0.533 |
|  | 𝜌 |  | 1.01 | 1.01 | 1.01 | 0.97 | 0.99 | 1.00 | 0.83 | 0.95 | 0.98 |
|  | BO |  | 0.18 | 0.16 | 0.14 | 0.34 | 0.22 | 0.21 | 0.70 | 0.44 | 0.44 |
|  | 𝜆 |  | 0.6 |  |  | 0.7 |  |  | 0.9 |  |  |
| LBT 2 (cat 3) | UPT CDF [Mbps] | 5% | 16.37 | 19.02 | 26.94 | 3.63 | 7.73 | 16.37 | 0.19 | 0.62 | 2.20 |
|  |  | 50% | 46.16 | 51.41 | 55.90 | 31.29 | 41.78 | 47.45 | 4.91 | 18.75 | 25.14 |
|  |  | 95% | 68.18 | 72.18 | 79.63 | 59.75 | 67.53 | 71.94 | 32.46 | 52.75 | 55.41 |
|  |  | Mean | 44.96 | 49.11 | 55.05 | 31.01 | 40.24 | 46.25 | 9.06 | 21.70 | 26.74 |
|  | Delay CDF [s] | 5% | 0.048 | 0.048 | 0.045 | 0.048 | 0.048 | 0.045 | 0.093 | 0.052 | 0.046 |
|  |  | 50% | 0.096 | 0.083 | 0.074 | 0.158 | 0.108 | 0.096 | 0.589 | 0.243 | 0.200 |
|  |  | 95% | 0.443 | 0.380 | 0.289 | 1.284 | 0.705 | 0.502 | 4.432 | 2.127 | 1.826 |
|  |  | Mean | 0.154 | 0.139 | 0.110 | 0.351 | 0.226 | 0.160 | 1.165 | 0.546 | 0.454 |
|  | 𝜌 |  | 1.01 | 1.01 | 1.01 | 0.97 | 0.98 | 1.00 | 0.83 | 0.94 | 0.98 |
|  | BO |  | 0.18 | 0.17 | 0.14 | 0.34 | 0.25 | 0.21 | 0.70 | 0.50 | 0.41 |
|  | 𝜆 |  | 0.6 |  |  | 0.7 |  |  | 0.9 |  |  |
| LBT 3 (cat 3) | UPT CDF [Mbps] | 5% | 16.37 | 17.72 | 38.00 | 3.63 | 5.02 | 27.22 | 0.19 | 0.47 | 12.15 |
|  |  | 50% | 46.16 | 46.56 | 63.87 | 31.29 | 37.11 | 56.50 | 4.91 | 14.82 | 38.02 |
|  |  | 95% | 68.18 | 68.87 | 84.19 | 59.75 | 64.96 | 78.54 | 32.46 | 45.24 | 64.44 |
|  |  | Mean | 44.96 | 45.04 | 63.17 | 31.01 | 36.31 | 55.15 | 9.06 | 18.13 | 38.11 |
|  | Delay CDF [s] | 5% | 0.048 | 0.048 | 0.043 | 0.048 | 0.048 | 0.043 | 0.093 | 0.055 | 0.044 |
|  |  | 50% | 0.096 | 0.095 | 0.064 | 0.158 | 0.129 | 0.076 | 0.589 | 0.317 | 0.113 |
|  |  | 95% | 0.443 | 0.405 | 0.186 | 1.284 | 0.879 | 0.260 | 4.432 | 2.336 | 0.652 |
|  |  | Mean | 0.154 | 0.153 | 0.081 | 0.351 | 0.267 | 0.104 | 1.165 | 0.631 | 0.214 |
|  | 𝜌 |  | 1.01 | 1.01 | 1.01 | 0.97 | 0.98 | 1.00 | 0.83 | 0.93 | 1.00 |
|  | BO |  | 0.18 | 0.19 | 0.11 | 0.34 | 0.28 | 0.16 | 0.70 | 0.55 | 0.28 |
|  | 𝜆 |  | 0.6 |  |  | 0.7 |  |  | 0.9 |  |  |

Figure 4 shows the gains of mean WiFi UPT in the WiFi and LAA coexistence case compared to mean WiFi UPT in the WiFi and WiFi coexistence case in indoor deployment. Compared to the WiFi + WiFi coexistence case with low traffic, the results of WiFi + LAA with LBT scheme 1 case show 11% improvement in the mean UPT of WiFi and 18% improvement in the 5%ile UPT of WiFi; the results of WiFi + LAA with LBT scheme 2 case show 9% improvement in the mean UPT of WiFi and 16% improvement in the 5%ile UPT of WiFi; and the results of WiFi + LAA with LBT scheme 3 case show no degradation in the mean UPT of WiFi and 8% improvement in the 5%ile UPT of WiFi.

This positive interaction accelerates as the load increases. In the high load case we observe mean UPT gains of 180%, 140%, and 100% for LBT schemes 1, 2, and 3, respectively; in the 5%ile case we observe gains of 316%, 228%, and 147% for the respective schemes.

The physical layer of LTE is designed to be robust against interferences, which becomes more relevant under high load. The UPT of WiFi coexisting with LAA improves as the UPT of LAA increases.

Figure 4: gain of mean WiFi UPT in the WiFi and LAA coexistence case compared to mean WiFi UPT in the WiFi and WiFi coexistence case in indoor deployment

These simulation results show that all the identified LBT design options have no negative impact on WiFi, and therefore that LAA can coexist with WiFi.

Observation 1: Both FBE-based approach (LBT category 2) and LBE-based approach (LBT category 3) are appropriate for co-existence, as their implementation in LAA does not degrade WiFi operation.

- On the contrary, a better throughput can be observed in the WiFi nodes as well.

Table 2 shows the simulation results for "indoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic".

Table 2: indoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic

| LAA LBT cat. | Reported parameters |  | Low load BO range : 10%~25% |  | Medium load BO range : 35%~50% |  | High load BO range : above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| LBT 1 (cat. 2) | UPT CDF [Mbps] | 5% | 27.74 | 29.24 | 2.23 | 2.74 | 0.22 | 0.12 |
|  |  | 50% | 55.65 | 57.46 | 28.91 | 30.99 | 9.70 | 10.20 |
|  |  | 95% | 78.32 | 81.10 | 66.35 | 68.08 | 59.82 | 58.98 |
|  |  | Mean | 54.67 | 56.56 | 30.59 | 32.77 | 16.61 | 18.76 |
|  | Delay CDF [s] | 5% | 0.045 | 0.045 | 0.046 | 0.046 | 0.047 | 0.047 |
|  |  | 50% | 0.074 | 0.070 | 0.179 | 0.159 | 0.470 | 0.375 |
|  |  | 95% | 0.412 | 0.361 | 2.275 | 2.089 | 3.929 | 3.969 |
|  |  | Mean | 0.142 | 0.121 | 0.522 | 0.475 | 1.073 | 1.025 |
|  | 𝜌 |  | 1.01 | 1.00 | 0.97 | 0.98 | 0.89 | 0.88 |
|  | BO |  | 0.13 | 0.18 | 0.46 | 0.44 | 0.68 | 0.66 |
|  | 𝜆 |  | 0.8 |  | 1.1 |  | 1.3 |  |
| LBT 2 (cat. 3) | UPT CDF [Mbps] | 5% | 31.13 | 29.80 | 4.84 | 4.63 | 0.43 | 0.54 |
|  |  | 50% | 53.36 | 54.02 | 30.99 | 30.77 | 13.67 | 15.88 |
|  |  | 95% | 74.18 | 73.99 | 58.53 | 59.60 | 48.68 | 47.40 |
|  |  | Mean | 53.10 | 53.67 | 31.55 | 31.36 | 17.49 | 18.25 |
|  | Delay CDF [s] | 5% | 0.045 | 0.045 | 0.046 | 0.046 | 0.055 | 0.055 |
|  |  | 50% | 0.079 | 0.077 | 0.161 | 0.160 | 0.340 | 0.297 |
|  |  | 95% | 0.337 | 0.321 | 1.561 | 1.392 | 3.537 | 3.734 |
|  |  | Mean | 0.128 | 0.116 | 0.378 | 0.363 | 0.850 | 0.808 |
|  | 𝜌 |  | 1.01 | 1.00 | 0.99 | 0.99 | 0.93 | 0.94 |
|  | BO |  | 0.19 | 0.18 | 0.42 | 0.41 | 0.62 | 0.61 |
|  | 𝜆 |  | 0.8 |  | 1.1 |  | 1.3 |  |
| LBT 3 (cat. 3) | UPT CDF [Mbps] | 5% | 33.00 | 33.02 | 8.53 | 8.22 | 0.89 | 0.76 |
|  |  | 50% | 54.90 | 55.19 | 34.48 | 32.88 | 17.00 | 17.07 |
|  |  | 95% | 75.65 | 75.42 | 60.66 | 58.88 | 47.51 | 47.91 |
|  |  | Mean | 54.85 | 54.70 | 34.82 | 33.48 | 19.59 | 19.05 |
|  | Delay CDF [s] | 5% | 0.043 | 0.043 | 0.043 | 0.043 | 0.056 | 0.056 |
|  |  | 50% | 0.078 | 0.078 | 0.142 | 0.150 | 0.291 | 0.291 |
|  |  | 95% | 0.280 | 0.295 | 1.065 | 1.171 | 2.965 | 2.996 |
|  |  | Mean | 0.107 | 0.112 | 0.285 | 0.304 | 0.687 | 0.696 |
|  | 𝜌 |  | 1.01 | 1.00 | 1.00 | 0.99 | 0.97 | 0.96 |
|  | BO |  | 0.17 | 0.18 | 0.36 | 0.37 | 0.56 | 0.58 |
|  | 𝜆 |  | 0.8 |  | 1.1 |  | 1.3 |  |

The results shown in the table are somehow misleading in that there seems to be no fairness issues between the different LAA operators. However, this is not the reality. The results in the table correspond to an average of the individual results of multiple drops; if each drop is considered individually it can be observed that an LAA operator tends to hoard the channel.

This issue occurs when operators are not synchronized, leading to a situation in which the contention window of one operator happens before the contention window of the other. Figure 5 shows an example in which the channel is always taken by a node of LAA operator 1, resulting in nodes of LAA operator 2 always seeing the channel as busy.

Figure 5: LAA operator 1 can hoard the channel if its contention window occurs earlier than that of LAA operator 2

This is particularly excruciating with LBT category 2 schemes (such as the proposed LBT scheme 1), but category 3 can also incur in it if not designed carefully (the proposed LBT scheme 2 has a similar issue, while LBT scheme 3 is able to deal with it and ensure fairness between operators).

Observation 2: RAN1 should carefully consider how to ensure fairness between different LAA operators, as under the current conditions an operator may be able to continuously occupy the channel.

- Conclusion

We provided an observation for coexistence between LAA and WiFi.

Observation 1: Both FBE-based approach (LBT category 2) and LBE-based approach (LBT category 3) are appropriate for co-existence, as their implementation in LAA does not degrade WiFi operation.

- On the contrary, a better throughput can be observed in the WiFi nodes as well.

Observation 2: RAN1 should carefully consider how to ensure fairness between different LAA operators, as under the current conditions an operator may be able to continuously occupy the channel.

- References

- RP-141664, “Study on Licensed-Assisted Access using LTE,” Ericsson et al., RAN#65.

- "Chairman's Notes RAN1#79", November 2014.

- R1-145474, TR 36.889 v0.1.1.

- "Chairman's Notes RAN1#80", February 2015.

- R1-145336, “Way Forward on LAA Evaluation Assumptions: Load Factor,” Intel, Samsung, Qualcomm, Ericsson, RAN1#79

- Annex

  - Simulation assumptions

Table A1, A2, and A3 show the details of the simulation assumption.

Table A1: General evaluation assumptions

| Parameters | Assumptions / Values |
| --- | --- |
| Deployment | Indoor scenario |
| Number of nodes | 4 per operator |
| Carrier frequency | 5 GHz for unlicensed band |
| UE bandwidth | 20 MHz for unlicensed band |
| Number of carriers | Single carrier |
| Transmission power | 18 dBm |
| Channel model | ITU InH |
| Antenna pattern | 2D Omni-directional |
| Antenna gain + connector loss | 5 dBi |
| Number of UEs | 10 UEs per operator |
| Traffic model | FTP model 3 with packet size of 0.5 Mbytes |
| UE receiver | MMSE |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Cell selection | Best RSRP/RSS-based with 0 dB handover margin |
| Network synchronization | Small cells of the same operator: synchronized and time-aligned Small cells between different operators: unsynchronized |
| Simulation step | 1 us |

Table A2: WiFi system evaluation assumptions

| Parameters | Assumptions / Values |
| --- | --- |
| MCS | 802.11ac MCS table (with 256QAM) |
| Antenna configuration | 1Tx 2Rx |
| Channel coding | LDPC |
| Frame aggregation | A-MPDU |
| MPDU size | 1.5 kbytes |
| Max PPDU duration | 4 ms |
| MAC coordination | Basic DCF SIFS (16 us), DIFS (34 us) |
| Slot size | 9 us |
| RTS/CTS | N/A |
| Contention window | Min: 15 slot, Max 1023 slot |
| CCA-CS | -82 dBm |
| CCA-ED | -62 dBm |
| ACK modeled | Yes Error free reception ACK transmission using MCS 0 (BPSK, r = 1/2) |
| DL/UL duplexing | DL traffic only |
| Rate control | Open loop target BLER: 0.1 |
| OFDM symbol length | 4 us |
| UE scheduler | Round-robin with full bandwidth allocation |
| File scheduler | FIFO |

Table A3: LAA system evaluation assumptions

| Parameters | Assumptions / Values |
| --- | --- |
| PCI planning | Planned |
| MCS | QPSK/16QAM/64QAM/256QAM |
| Antenna configuration | 1Tx 2Rx |
| Transmission schemes | DMRS-based transmission |
| Channel estimation | Ideal |
| Max burst transmission duration | 4 ms |
| CSI feedback | Ideal |
| CCA-ED | -62 dBm |
| CCA time slot | 20 us |
| Cyclic Prefix | Normal CP |
| HARQ scheme | Chase combining Maximum retransmission number = 4 |
| Overhead assumption | PDCCH: 1 OFDM symbol CRS: 1 port (antenna port 0) DMRS: 1 port (antenna port 7) |
| UE scheduler | Proportional Fairness |
| File scheduler | FIFO |

- Appendix

  - C.D.F. of UPT

Figure B1: C.D.F. of UPT in Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

Figure B2: C.D.F. of UPT in LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic

  - C.D.F. of delay

Figure B3: C.D.F. of delay in Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

Figure B4: C.D.F. of delay in LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic