# 3GPP TSG RAN WG1 LAA Meeting (Ad Hoc)	R1-151070

# Paris, France, 24th – 26th March 2015

# Title: 	Evaluation results for LAA and LAA Coexistence with DL-only LAA

# Source: 	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

# Agenda Item:	2.1

# Document for: 	Discussion/Decision

# Introduction

In RAN1 #80, LBT schemes were classified into four categories for LAA channel access evaluations [1].

- Classify the evaluated LBT schemes according to the following categories:

  - Category 1: No LBT

  - Category 2: LBT without random back-off

  - Category 3: LBT with random back-off with fixed size of contention window

  - Category 4: LBT with random back-off with variable size of contention window

Note: Contention window is the maximum possible random back-off value

Note: Category classification does not restrict a LBT design investigation

Note: Company is encouraged to evaluate many categories as much as possible

- Illustrative examples

  - FBE procedure as defined in EN BRAN V1.8.0 belongs to category 2

  - LBE procedure with a fixed q for the contention window as defined in EN BRAN V1.8.0 belongs to category 3

  - LBE procedure Op A with a variable q for the contention window as defined in EN BRAN V1.8.0 belongs to category 4

Also, the Buffer Occupancy was agreed as an output metric for at least DL only LAA evaluation.

- Confirm the working assumption of Buffer Occupancy (BO) as an output metric for at least DL only LAA evaluation

  - The modifications of BO for uplink transmission is FFS

- Packet arrival rate for the measured BO of the non-replaced Wi-Fi network in Wi-Fi/Wi-Fi coexistence scenario is used as the packet arrival rate in Wi-Fi/LAA coexistence evaluations

  - Recommend to report BO both for LAA and Wi-Fi

- The corresponding BO range for DL only transmission is:

  - Low load: 10%~25%

  - Medium load: 35~50%

  - High load: above 55%

- Report a ratio of  mean served cell throughput and offered cell throughput

In this contribution, some simulation results of LAA-LAA coexistence performance for DL only LAA are provided based on the agreed assumptions.

# Evaluation Assumptions

The following LAA-LAA coexistence scenarios are evaluated as agreed in [2]:

- Indoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic (LAA without licensed carrier)

- Indoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic

- Outdoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic (LAA without licensed carrier)

- Outdoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic

In this contribution, the LAA frame structures of FBE and LBE based on Alt.2 in [3] are evaluated for LAA transmission. The FTP-3 traffic model is used for all simulation cases. The detailed LBT parameters and additional simulation assumptions can be found in the appendix. In addition, evaluation results for LAA-Wi-Fi coexistence are given in our companion contribution [4].

# Simulation Results

In this section, simulation results are provided for indoor and outdoor coexistence scenarios. The average user perceived throughput (UPT), latency, the ratio between the mean served cell throughput to the mean offered cell throughput (𝜌) and the mean buffer occupancy (BO) are considered as the performance metrics which are agreed in [2].

# Indoor Scenario

Table 1 and Table 2 show LAA-LAA coexistence simulation results of LBE-based LBT (category 3) in the indoor scenario without and with licensed carrier support, respectively.

Table 1 LAA-LAA coexistence results for indoor scenario, without licensed CC, LBT category 3

| Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 43.62 | 43.21 | 4.84 | 2.96 | 1.33 | 0.82 |
|  | 50% | 73.71 | 74.33 | 48.64 | 48.74 | 21.46 | 26.15 |
|  | 95% | 95.31 | 94.69 | 76.55 | 78.66 | 64.39 | 69.55 |
|  | Mean | 72.28 | 71.80 | 45.92 | 45.41 | 27.13 | 29.74 |
| Delay CDF [s] | 5% | 0.036 | 0.036 | 0.036 | 0.036 | 0.036 | 0.036 |
|  | 50% | 0.055 | 0.055 | 0.097 | 0.092 | 0.192 | 0.170 |
|  | 95% | 0.190 | 0.202 | 1.361 | 1.120 | 2.532 | 2.722 |
|  | Mean | 0.078 | 0.079 | 0.302 | 0.291 | 00.595 | 0.577 |
| 𝜌 |  | 0.88 | 0.83 | 0.72 | 0.74 | 0.59 | 0.60 |
| BO |  | 0.12 | 0.12 | 0.37 | 0.35 | 0.61 | 0.57 |
| 𝜆 |  | 0.7 |  | 0.90 |  | 1.0 |  |
| Additional comments |  | LBE, q Value:32, Maximum Occupancy Time: 10 ms, LAA without licensed carrier |  |  |  |  |  |

Table 2 LAA-LAA coexistence results for indoor scenario, with licensed CC, LBT category 3

| Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 107.56 | 110.11 | 98.39 | 96.65 | 94.60 | 97.22 |
|  | 50% | 132.48 | 136.73 | 124.31 | 130.28 | 121.31 | 127.08 |
|  | 95% | 150.44 | 152.64 | 142.51 | 147.01 | 142.51 | 146.85 |
|  | Mean | 131.37 | 135.72 | 123.27 | 127.49 | 120.73 | 125.70 |
| Delay CDF [s] | 5% | 0.024 | 0.024 | 0.024 | 0.024 | 0.024 | 0.024 |
|  | 50% | 0.025 | 0.025 | 0.028 | 0.026 | 0.030 | 0.027 |
|  | 95% | 0.056 | 0.053 | 0.065 | 0.060 | 0.068 | 0.065 |
|  | Mean | 0.033 | 0.032 | 0.036 | 0.034 | 0.037 | 0.036 |
| 𝜌 |  | 0.91 | 0.86 | 0.82 | 0.87 | 0.80 | 0.82 |
| BO |  | 0.06 | 0.06 | 0.08 | 0.08 | 0.09 | 0.09 |
| 𝜆 |  | 0.7 |  | 0.90 |  | 1.0 |  |
| Additional comments |  | LBE, q Value:32, Maximum Occupancy Time: 10 ms, LAA with licensed carrier |  |  |  |  |  |

Table 3 and Table 4 show LAA-LAA coexistence simulation results of FBE-based LBT (category 2) in the indoor scenario without and with licensed carrier support, respectively.

Table 3 LAA-LAA coexistence results for indoor scenario, without licensed CC, LBT category 2

| Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 51.79 | 51.61 | 7.04 | 7.04 | 2.24 | 2.49 |
|  | 50% | 74.03 | 73.40 | 49.77 | 45.84 | 15.48 | 18.09 |
|  | 95% | 92.40 | 92.43 | 74.49 | 77.36 | 53.43 | 60.86 |
|  | Mean | 73.33 | 73.22 | 46.50 | 45.83 | 20.66 | 23.20 |
| Delay CDF [s] | 5% | 0.037 | 0.037 | 0.037 | 0.037 | 0.040 | 0.037 |
|  | 50% | 0.049 | 0.054 | 0.090 | 0.096 | 0.428 | 0.356 |
|  | 95% | 0.186 | 0.183 | 1.198 | 1.582 | 5.000 | 4.007 |
|  | Mean | 0.076 | 0.075 | 0.278 | 0.328 | 1.101 | 0.994 |
| 𝜌 |  | 0.84 | 0.79 | 0.69 | 0.71 | 0.48 | 0.47 |
| BO |  | 0.14 | 0.14 | 0.42 | 0.43 | 0.83 | 0.83 |
| 𝜆 |  | 0.85 |  | 1.1 |  | 1.5 |  |
| Additional comments |  | FBE frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms) |  |  |  |  |  |

Table 4 LAA-LAA coexistence results for indoor scenario, with licensed CC, LBT category 2

| Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 101.93 | 105.20 | 95.90 | 94.06 | 73.83 | 75.08 |
|  | 50% | 128.93 | 135.55 | 118.57 | 124.58 | 104.80 | 106.91 |
|  | 95% | 145.15 | 153.47 | 138.28 | 145.45 | 126.26 | 136.05 |
|  | Mean | 127.33 | 133.82 | 117.85 | 123.77 | 103.73 | 106.31 |
| Delay CDF [s] | 5% | 0.025 | 0.024 | 0.025 | 0.024 | 0.025 | 0.024 |
|  | 50% | 0.026 | 0.024 | 0.029 | 0.027 | 0.037 | 0.036 |
|  | 95% | 0.058 | 0.057 | 0.070 | 0.066 | 0.089 | 0.094 |
|  | Mean | 0.035 | 0.033 | 0.039 | 0.037 | 0.047 | 0.048 |
| 𝜌 |  | 0.87 | 0.85 | 0.80 | 0.82 | 0.73 | 0.73 |
| BO |  | 0.08 | 0.07 | 0.10 | 0.10 | 0.16 | 0.16 |
| 𝜆 |  | 0.85 |  | 1.1 |  | 1.5 |  |
| Additional comments |  | FBE frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms) |  |  |  |  |  |

Based on the simulation results from the indoor deployment, it is observed that in different traffic situations, both LBE based and FBE based LBT algorithms could achieve the co-existence fairness between two LAA operators. With higher traffic load, the performance degrades due to the increased transmission latency caused by channel congestion. With respect to the performance comparison between LBE and FBE, it is observed that FBE performs better than LBE with the same traffic load (e.g.: average UPT is around 73 Mbps for category 2 when λ is 0.85, whereas average UPT is around 45 Mbps for category 3 when λ is 0.9). LAA-FBE outperforms LAA-LBE mainly due to the intra-operator frequency reuse one and slightly quicker channel access with shorter (e)CCA time,. Hence, it is important to study further whether it is possible to support intra-operator frequency reuse one with LBE.

Observation 1: Co-existence fairness of two LTE operators can be achieved with both LBE-based and FBE-based LBT algorithms.

Observation 2: LAA-FBE outperforms LAA-LBE mainly due to the intra-operator frequency reuse factor one and slightly quicker channel access with shorter (e)CCA time.

Proposal 1: It should be studied further whether it is possible to support LBE intra-operator frequency reuse one with LBE.

# Outdoor Scenario

Table 5 - Table 8 show the LAA-LAA coexistence simulation results with LBE-based and FBE-based LBT schemes in the outdoor scenario without and with licensed carrier support.

Table 5 LAA-LAA coexistence results for outdoor scenario, without licensed CC, LBT category 3

| Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 27.67 | 22.00 | 1.82 | 1.90 | 0.08 | 0.42 |
|  | 50% | 72.79 | 72.29 | 29.44 | 30.10 | 6.46 | 9.44 |
|  | 95% | 100.40 | 100.44 | 67.68 | 70.27 | 42.03 | 38.45 |
|  | Mean | 69.64 | 69.60 | 31.67 | 32.10 | 12.08 | 13.98 |
| Delay CDF [s] | 5% | 0.036 | 0.036 | 0.037 | 0.037 | 0.055 | 0.057 |
|  | 50% | 0.055 | 0.055 | 0.146 | 0.145 | 0.382 | 0.344 |
|  | 95% | 0.214 | 0.224 | 1.861 | 1.680 | 4.041 | 3.225 |
|  | Mean | 0.109 | 0.097 | 0.446 | 0.390 | 0.905 | 0.802 |
| 𝜌 |  | 0.84 | 0.87 | 0.64 | 0.66 | 0.41 | 0.51 |
| BO |  | 0.10 | 0.11 | 0.44 | 0.42 | 0.78 | 0.75 |
| 𝜆 |  | 0.50 |  | 0.7 |  | 1.0 |  |
| Additional comments |  | LBE, q Value:32, Maximum Occupancy Time: 10 ms, LAA without licensed carrier |  |  |  |  |  |

Table 6 LAA-LAA coexistence results for outdoor scenario, with licensed CC, LBT category 3

| Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 86.96 | 94.31 | 74.58 | 84.92 | 56.22 | 60.78 |
|  | 50% | 133.39 | 137.22 | 122.62 | 128.35 | 105.40 | 114.80 |
|  | 95% | 153.85 | 156.98 | 145.28 | 151.79 | 135.95 | 143.74 |
|  | Mean | 130.60 | 134.69 | 119.33 | 126.24 | 103.68 | 109.95 |
| Delay CDF [s] | 5% | 0.024 | 0.024 | 0.024 | 0.024 | 0.024 | 0.024 |
|  | 50% | 0.025 | 0.025 | 0.029 | 0.027 | 0.037 | 0.034 |
|  | 95% | 0.056 | 0.055 | 0.067 | 0.064 | 0.090 | 0.086 |
|  | Mean | 0.033 | 0.032 | 0.037 | 0.035 | 0.045 | 0.043 |
| 𝜌 |  | 0.89 | 0.91 | 0.83 | 0.85 | 0.72 | 0.77 |
| BO |  | 0.04 | 0.04 | 0.06 | 0.06 | 0.10 | 0.10 |
| 𝜆 |  | 0.50 |  | 0.7 |  | 1.0 |  |
| Additional comments |  | LBE, q Value:32, Maximum Occupancy Time: 10 ms, LAA with licensed carrier |  |  |  |  |  |

Table 7 LAA-LAA coexistence results for outdoor scenario, without licensed CC, LBT category 2

| Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 18.68 | 15.38 | 7.28 | 8.83 | 2.76 | 3.69 |
|  | 50% | 72.29 | 71.53 | 38.59 | 37.85 | 27.99 | 29.21 |
|  | 95% | 94.91 | 97.12 | 73.55 | 73.70 | 70.72 | 71.46 |
|  | Mean | 67.38 | 67.65 | 40.91 | 39.67 | 31.94 | 32.86 |
| Delay CDF [s] | 5% | 0.037 | 0.037 | 0.037 | 0.037 | 0.037 | 0.037 |
|  | 50% | 0.056 | 0.054 | 0.117 | 0.126 | 0.182 | 0.168 |
|  | 95% | 0.317 | 0.629 | 1.584 | 1.854 | 3.155 | 2.631 |
|  | Mean | 0.140 | 0.257 | 0.357 | 0.392 | 0.699 | 0.576 |
| 𝜌 |  | 0.75 | 0.79 | 0.65 | 0.69 | 0.55 | 0.61 |
| BO |  | 0.16 | 0.16 | 0.39 | 0.42 | 0.56 | 0.56 |
| 𝜆 |  | 0.60 |  | 0.90 |  | 1.0 |  |
| Additional comments |  | FBE frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms) |  |  |  |  |  |

Table 8 LAA-LAA coexistence results for outdoor scenario, with licensed CC, LBT category 2

| Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 84.73 | 94.11 | 68.21 | 65.14 | 64.55 | 67.66 |
|  | 50% | 126.69 | 137.28 | 113.06 | 123.23 | 110.19 | 116.67 |
|  | 95% | 150.92 | 159.23 | 142.51 | 146.85 | 139.66 | 146.31 |
|  | Mean | 124.63 | 133.27 | 111.09 | 117.89 | 108.00 | 113.97 |
| Delay CDF [s] | 5% | 0.024 | 0.024 | 0.025 | 0.024 | 0.025 | 0.024 |
|  | 50% | 0.026 | 0.024 | 0.035 | 0.031 | 0.035 | 0.034 |
|  | 95% | 0.058 | 0.056 | 0.079 | 0.074 | 0.078 | 0.076 |
|  | Mean | 0.035 | 0.033 | 0.041 | 0.038 | 0.042 | 0.040 |
| 𝜌 |  | 0.83 | 0.87 | 0.74 | 0.83 | 0.72 | 0.77 |
| BO |  | 0.05 | 0.05 | 0.09 | 0.09 | 0.10 | 0.10 |
| 𝜆 |  | 0.60 |  | 0.90 |  | 1.0 |  |
| Additional comments |  | FBE frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms) |  |  |  |  |  |

The results of the outdoor case in the above tables follow a similar trend as those of the indoor scenario. Comparing the indoor and outdoor scenarios, it can be observed that the indoor scenario has better performance (higher UPT and lower latency) due to good interference isolation from other neighbor clusters.

# Conclusion

In this contribution, simulation results of LAA-LAA coexistence performance are provided for DL only LAA system. Based on the simulation results, the following conclusions are made:

Observation 1: Co-existence fairness of multiple LTE operators can be achieved for both FBE-based and LBE-based LBT schemes in both indoor and outdoor deployment.

Observation 2: LAA-FBE outperforms LAA-LBE mainly due to the intra-operator frequency reuse factor one and slightly quicker channel access with shorter (e)CCA time.

Proposal 1: It should be studied further whether it is possible to support LBE intra-operator frequency reuse one with LBE.

# References

- 3GPP RAN1#80 Chairman’s note, Feb. 2015.

- 3GPP R1-150890, “Table templates for DL only LAA coexistence evaluation result”, Ericsson, Feb. 2015.

- 3GPP R1-150189, “Coexistence Evaluation Results for DL-only LAA”, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, Feb. 2015.

- 3GPP R1-151069, “Evaluation results for LAA and Wi-Fi Coexistence with DL-only LAA”, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, Mar. 2015.

# Appendix: Simulation Assumptions

Table 9 LTE additional simulation parameters

| Parameter | value |
| --- | --- |
| Outdoor scenario layout | Based on SCE# 2a + unlicensed band X=4, Y=1; 10 UEs per operator per carrier; 10 m for min. distance between small cells of different operators |
| Indoor scenario layout | Based on SCE#3 + unlicensed band X=4, Y=1; 10 UEs per operator per carrier; 3 m minimal distance between small cells of different operators |
| Traffic model | FTP Model 3 file size: 0.5 Mbytes |
| Network synchronization | For the same operator, the network is ideally synchronized Small cells of different operators are not synchronized |
| Total BS TX power | 18 dBm |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized; adaptive stream |
| CCA-ED | -73 dBm/MHz+23-PH / (1 MHz) |
| Link adaptation | Realistic |
| Transmission schemes | TM10, QPSK/16QAM/64QAM (no 256QAM) |
| Scheduler | Proportional fair |
| UE receiver | MMSE-IRC |
| Scheduling delay for LAA | 4 ms (Channel ON Status) |
| CQI/RI/PMI Feedback for LAA | Sub-band feedback with 1 ms feedback period and 1ms feedback delay |
| MAC HARQ | Number of maximum ReTx: 3; minimal interval of ReTx: 8 ms (Channel ON Status) |
| Outer loop link adaptation | Enabled |