3GPP TSG RAN WG1 Meeting AH_NR#3      	                          R1-1715528

Nagoya, Japan, 18th – 21st, September 2017

Source:               ZTE, Sanechips

Title:                    Analysis of URLLC reliability for uplink control channel

Agenda item:      6.3.2.5

Document for:    Discussion and Decision

# Introduction

Ultra-reliable and low latency communications (URLLC) have been discussed for several RAN1 meetings, but mainly on low latency part. It is scheduled to raise the priority of the ultra-reliable part of URLLC after RAN#76 meeting. In this contribution, we mainly present our considerations on URLLC reliability for uplink control channel. Note, this contribution is revised from R1-1712457.

# Reliability model of URLLC

As specified in [1], a general URLLC reliability requirement for one transmission of a packet is 1-10-5 for 32 bytes with a user plane latency of 1ms. To achieve such stringent reliability requirement, several HARQ retransmissions may be needed. In [2], the probability for successful transmission for K transmissions for one HARQ process case is given,

P=j=1KR(j), where Rj=k=0j-1Cj-1k1-Pc∙PDTXj-1-kPck+1Pd,(k+1)i=1kPN∙1-Pd,i       (1)

which also taking the PUCCH reliability into account. Here R(j) is the probability that the jth transmission is successfully received by UE. Pc is probability of successful PDCCH transmission and Pd,k is probability of successful transmission of k data transmission with HARQ combining at receiver. Here PDTX= Prob{DTX or NACK is detected | UE sends DTX}, is the probability that DTX is successfully received or is detected as NACK by eNB, given the UE fails to detect PDCCH. That is PDTX represents a data retransmission without HARQ combining at UE side. PN=Prob{DTX or NACK is detected | UE sends NACK}, is the probability that the NACK is successfully received or is detected as DTX by eNB, given the UE successfully detects PDCCH for the transmission but fails to decode PDSCH. In other words, PN is the probability that a combinable data retransmission is triggered.

Proposal 1: RAN1 should define a unified model taken into account UL/DL control and data channel into account to evaluate the reliability of URLLC. PUCCH reliability should support the whole reliability of URLLC packet under this model.

In the next section, from the perspective of PUCCH reliability, we give some suggested combinations of channel reliabilities that can achieve the overall reliability target of 99.999%.

# Analysis of PUCCH reliability

Among all the factors mentioned above, two parameters PDTX and PN are related to PUCCH. To meet the 99.999% requirement of P, PDTX and PN should be as large as possible. That is the probability PDTX = Prob{ACK | UE sends DTX} and PN= Prob{ACK | UE sends NACK} shall be as small as possible. Note, in LTE, the target of false alarm PDTX is 0.01, and the target of PN is 0.001.

Short PUCCH should be the baseline for downlink HARQ-ACK feedback of URLLC because of the low latency requirement. In the following context, sequence selection based short PUCCH without NACK resources sharing for up to 2bits UCI is used to evaluate the reliability of PUCCH.

- Analysis of PDTX and PN

In Figure 1, the performance of 1/2-bit ACK/NACK transmission in 1-symbol sequence selection based PUCCH is shown. In the simulation, three performance metrics with targets including ACK missing probability, NACK-to-ACK probability and DTX-to-ACK probability are applied. The simulation assumptions are given in Table 1 in the Appendix.

Assume the targets of ACK missing probability and NACK-to-ACK probability are the same as in LTE, i.e., 1% and 0.1% respectively. It can be observed that the minimum required SNR for both 1/2-bit UCI depends on the ACK missing probability, and about 1.5dB enhancement is needed with the DTX-to-ACK probability increasing from 0.01 up to 0.001.

Observation 1: About 1.5dB boosting on sequence selection based short PUCCH is needed to improve the DTX-to-ACK probability from 0.01 up to 0.001.

Figure 1 Performance of 1-symbol PUCCH with PDTX  equals 0.01 and 0.001

Since the ACK missing probability PMissed has no direct influence on the overall reliability, the target can be kept as 0.01. However, at the minimum required SNR, PN can reach to about 10-4 when PDTX =0.01, and about 10-5 for PDTX =0.001. Therefore, we propose that,

Proposal 2: PUCCH reliability metrics can be defined as one of the following combinations,

- Combination 1: PMissed=0.01,  PDTX =0.01, PN =10-4, i.e. PMissed=0.99, PDTX=0.99, PN=0.9999

- Combination 2: PMissed=0.01, PDTX =0.001, PN =10-5, i.e. PMissed=0.99, PDTX=0.999, PN=0.99999

Then, based on (1), for a DL HARQ process of maximum 2 transmissions, the overall reliability target of 99.999% can be achieved by any of following combinations of channel reliabilities:

- With PDTX=0.99, PN=0.9999

- Pc=0.9999, Pd1=0.99, Pd2=0.9999.

- Pc=0.9999, Pd1=0.999, Pd2=0.999.

- With  PDTX=0.999, PN=0.99999

- Pc=0.999, Pd1=0.999, Pd2=0.999.

For a DL HARQ process of maximum 3 transmissions, the overall reliability target of 99.999% can be achieved by any of following combinations of channel reliabilities:

- With PDTX=0.99, PN=0.9999

- Pc=0.9999, Pd1=Pd2=Pd3=0.99.

- With  PDTX=0.999, PN=0.99999

- Pc=0.999, Pd1=Pd2=Pd3=0.99.

- Pc=0.999, Pd1=0.9, Pd2=0.99, Pd3=0.999.

According to the simulation result of Figure A.2 in [2], it can be found that about 2dB enhancement on PDCCH is needed to improve the PDCCH reliability from 99% to 99.9%, and 4dB for 99% to 99.99%. Compared to the 1.5 dB enhancement on PUCCH for improving the DTX-to-ACK probability from 0.01 to 0.001, the enhancement needed for PDCCH is larger. However, short PUCCH is power limited always the bottleneck of the cell coverage. Therefore, both combination may have use cases and need to be investigated under whole consideration of channels..

# Conclusion

This contribution provides the following observations and proposals:

Observation 1: About 1.5dB boosting on sequence selection based short PUCCH is needed to improve the DTX-to-ACK probability from 0.01 up to 0.001.

Proposal 1: RAN1 should define a unified model taken into account UL/DL control and data channel into account to evaluate the reliability of URLLC. PUCCH reliability should support the whole reliability of URLLC packet under this model.

Proposal 2: PUCCH reliability metrics can be defined as one of the following combinations,

- Combination 1: PMissed=0.01,  PDTX =0.01, PN =10-4, i.e. PMissed=0.99, PDTX=0.99, PN=0.9999

- Combination 2: PMissed=0.01, PDTX =0.001, PN =10-5, i.e. PMissed=0.99, PDTX=0.999, PN=0.99999

# References

- 3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies; Release 14.

- 3GPP RAN1 #88, R1-1701595 Analysis of URLLC reliability for HARQ, ZTE.

# Appendix

Table 1 Simulation parameters for link-level simulation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 15Khz |
| System bandwidth | 20 MHz |
| Number of symbol | 1 symbol |
| Number of PRB | 1 PRB |
| Channel model | TDL-C, Delay Spread(DS)= 300ns |
| UE speed | 3km/h |
| Antenna configuration | 1Tx(UE), 2 Rx(eNB) |
| CP length | Normal |
| UCI bits | 1,2 |
| Modulation mode | Sequence selection |
| Shortened PUCCH design | As shown in [4]. |
| Channel estimation | Practical channel estimation |
| Performance metrics | For HARQ-ACK: ACK missed detection probability (1%), NACK-to-ACK error probability (0.1%); DTX-to-ACK probability (1% or 0.1%). |