3GPP TSG WG1 NR Ad Hoc Meeting	R1- 1715575

Nagoya, Japan, 18–21 September 2017

Agenda Item:	6.3.3.7

Source:	Huawei, HiSilicon

Title:	HARQ operation and feedback indication design for UL grant-free transmission

Document for:	Discussion and decision

# Introduction

In RAN1 #90, the following agreements were achieved regarding HARQ-ACK channel for UL transmission without grant ([1]):

Agreements:

- If HARQ feedback is supported, to indicate HARQ feedback of UL transmission without grant, following options and related UE behavior should be further studied.

  - Option 1: Based on UL grant to indicate “ACK”

  - Option 2: Group-common DCI

    - 2-1: Only ACK

    - 2-2: ACK and NACK

  - Option 3: Define a Timer, UE assumes following, when the Timer expires

    - 3-1: ACK if an NACK is not received after the K repetitions

    - 3-2: NACK if an ACK is not received

- FFS: Option 1, Option 2 and Option 3-2 can be used during and after the K repetition

- Note: UL grant for the same TB initially transmitted without grant can indicate “NACK”

This contribution focuses on discussing the HARQ operation and whether an acknowledgement/indication of successful receiving of that TB from gNB is needed as well as the possible design. The system level simulation results are provided as well.

# Termination condition and HARQ-feedback indication design

## Termination condition and necessity of HARQ-ACK feedback channel

To support URLLC service, the number of maximum repetitions can be determined according to the delay budget, numerology and slot duration. In fact, it is a tradeoff to select a proper K for a UE. If K is too large, users in good quality channel may waste resources to transmit unnecessary repetitions which may cause interference to other grant-free UEs sharing the same resources. However, if K is too small, the reliability may not be satisfied unless using retransmission by UL grant which further introduces some delay.  Considering the tolerance of collisions among UEs for UL transmission scheme without UL grant we aim to achieve from the beginning of the design and the importance of low latency feature (reliability aspect could also be further enhanced by other means at a later stage), the good choice would be to configure a relatively large value of K to instruct the UE to fully utilize available UL transmission opportunities and allow network to stop the repetitions of a grant-free transmission by indicating ACK if it has been successfully decoded during the repetitions.  More simulation results and observations can be found in section 2.3 and section 3.

Proposal 1: NR should support the network to send an HARQ feedback indicating ACK to a UE performing UL transmission without UL grant with repetitions.

## HARQ feedback indication design for UL GF

In general, HARQ feedback indication for UL GF can be conveyed by a specific channel, e.g., PHICH-like channel as discussed in [3], or included in DCI which will be discussed in details in this section.

If HARQ-feedback indication is included in DCI, the DCI could be a UE specific DCI or a group DCI.

    - HARQ feedback indication in UE specific DCI

UE specific DCI is usually for scheduling PUSCH/PDSCH, which includes many bits for resource allocation related. There should be some way to indicate the ACK in the DCI for UL GF.  One solution is to re-use a regular DCI grant of a new TB to implicitly indicating the ACK, which is the “option 1” in meeting#90’s agreement. However, for low latency application and/or ACK termination of a continuous repetition, gNB cannot wait till there are new data to be scheduled for the UE to send the grant to indicate the ACK. Therefore, even though there is no PUSCH/PDSCH to be scheduled, the full size DCI needs to be transmitted to convey the ACK indication. Apparently, the overhead is too large.

Alternatively, ACK indication in UE specific DCI could be a new DCI format (e.g., ACK-DCI) different from normal DCI for scheduling data. ACK-DCI may only include one bit for ACK indication for one specific UE. However, CRC will be attached to the 1 bit payload. Therefore, the CRC overhead and the coding gain might be a concern for this option.

In addition, many UEs may share the same resources for UL GF transmission, which may result in gNB transmitting a lot of UE specific DCIs to just confirm the successful receiving of the TB to each of UEs.

Observation 1: UE specific DCI conveying ACK indication for UL GF is inefficient in terms of large overhead.

    - HARQ-feedback indication in group DCI

A group DCI which address a group of UEs can include more than one bit (e.g., 10 ~ 20 bits), in which, for example, 1 bit per UE is to indicate HARQ-feedback for UL GF transmission.

A new format for the group DCI of HARQ feedback indication needs to be defined. UE grouping, i.e., which UEs are grouped into a group DCI needs to be configured or defined. The group can be identified using a GF group RNTI, which can be derived based on predefined association with transmission resources or configured and is used to scramble the CRC for the DCI. Moreover, UE needs to identify which bit is targeting for that UE in the group DCI.  The group DCI can employ a bit map format with multiple HARQ feedback bits indication corresponding to multiple UEs with minimum overhead.

There may be at least two different ways to implement group DCI for HARQ feedback indication. One is to have a format similar to LTE DCI format 3/3A, where the location of each UE is identified using the UE position index. If multiple HARQ processes are used for each UE, the HARQ process ID information can also be potentially included in the UE position index for group DCI.

Another approach is to have the GF group RNTI associated with the grant-free transmission resources (e.g. as a function of at least the time unit). For this scenario, the UEs access the same grant-free resources are considered in the group for group DCI. The group DCI may still use a bit map format. The advantage of such an approach is that, unlike the other approach where the group of UEs are preconfigured in RRC, the UE group does not have to be defined in advance, which match better with actual grant-free transmission as the group of UEs access the same grant-free resources may change over time.

In group DCI, each UE may be assigned only 1 bit indicating the HARQ feedback. For example, a bit value of “1” means the grant-free transmission of the UE is successful.  The UE behavior with respect to the different bit status should be specified.

Observation 2: A group DCI conveying HARQ-feedback indication for UL GF is more efficient comparing to UE specific DCI.

## Power ramping of RS

For grant free uplink transmission, the uplink transmission power is based on open loop power control. The power may not be sufficient on gNB detection for RS. Hence, power ramping of RS for grant free uplink transmission can be considered.

gNB can indicate UEs whether RS is detected or not, and in case of RS of grant free transmission has not been detected, the retransmission power of RS  can be ramped up to accelerate the gNB to detect the grant free uplink transmissions. In case of the corresponding RS is detected but the data is still not received correctly, the transmission power of RS can keep the same as the previous transmission..

# System level simulation results

In this section, we compare the following schemes using system level simulations to demonstrate the advantage of early termination of K repetitions:

Scheme 1:  GF with configured K repetitions, no early stopping.

Scheme 2:  GF configured with maximum K repetitions, but repetition is early stopped by ACK.

Figure 1 shows the system level simulation performance of scheme 2 and scheme 1 with different configured K’s in terms of percentage of UEs that satisfies the URLLC reliability and latency requirement. The simulation assumptions are summarized in Table 1 as in appendix.

For scheme 2, the K is set to be 7, which is the maximum number of repetitions that the UE can perform under the latency requirement of URLLC. It can be seen that the preconfigured K repetitions with ACK stopping scheme significantly outperforms all the preconfigured K repetitions without early stopping schemes.

For scheme 1 with different K values, they may suffer from performance degradation in comparison to scheme 2 due to different reasons: A large preconfigured K (e.g. K=7) may cause too much unnecessary interference to other UEs, while a small preconfigured K may not be able to meet the reliability requirement of URLLC.  Preconfigured maximum K repetition with early stopping provide a natural way to adapt to each transmission and provide the optimal number of repetitions needed for each transmission and therefore provide the best performance.

Observation 3: Early termination of repetitions of a GF transmission can significantly improve system performance for URLLC application.

Figure 1: Percentage of UEs satisfying the URLLC requirements

Based above analysis and simulation results, the following proposals are given:

Proposal 2: At least a group DCI format should be supported for an acknowledgement/indication of successfully receiving TB(s) in a grant-free transmission.

Proposal 3: The group DCI design for HARQ feedback indication can consider the following mechanism: group RNTI associated with grant-free resources.

# Conclusion

The necessity of early terminating the repetitions of UL GF transmission by an acknowledgement or indication of successful receiving of that TB from gNB is discussed in this contribution. The possible design for the ACK indication of successful receiving is given and the system level simulation is provided as well, which lead to the following observations:

Observation 1: UE specific DCI conveying ACK indication for UL GF is inefficient in terms of large overhead.

Observation 2: A group DCI conveying HARQ-feedback indication for UL GF is more efficient comparing to UE specific DCI.

Observation 3: Early termination of repetitions of a GF transmission can significantly improve system performance for URLLC application

The following proposals are given based the analysis and observations:

Proposal 1: NR should support the network to send an HARQ feedback indicating ACK to a UE performing UL transmission without UL grant with repetitions.

Proposal 2: At least a group DCI format should be supported for an acknowledgement/indication of successfully receiving TB(s) in a grant-free transmission.

Proposal 3: The group DCI design for HARQ feedback indication can consider the following mechanism: group RNTI associated with grant-free resources.

# References

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #90.

R1-1703332, “Necessity of PHICH-like channel in NR”, Huawei, HiSilicon, Athens, Greece, February 13 - 18, 2017.

# Appendix

Table 1: SLS simulation parameters

| Attributes | Values or assumptions |
| --- | --- |
| Layout | Urban Macro: Hex. Grid, 57 cells wrap around; 500m ISD |
| Carrier Frequency | 4 GHz |
| Duplexing Mode | FDD |
| System BW | 20 MHz |
| Number of RBs | 25 RBs |
| Number of GF RB partitions | 5 |
| Sub-carrier spacing | 60 kHz |
| SLOT length | 125 μs |
| MCS | QPSK, rate 1/3 (fixed for GF) |
| OFDM symbols per SLOT | 7 Symbols (NCP) |
| Channel model | 3D-UMa; user speed = 3km/h (Following TR 36.873) |
| UE TX power | 23 dBm |
| OL Power Control | P0 = -85 dBm , PL Compensation = 0.93 |
| BBU Receiver Noise Figure | 5dB |
| PHY Packet size | 32 bytes |
| BS Antenna Configuration | 4 Rx (Following TR 38.802) |
| BS Antenna Pattern | Following TR 36.873 |
| BS Antenna Gain + Connector Loss | Following TR 38.802 |
| UE Antenna Configuration | 2 Tx (Following TR 38.802) |
| UL Tx mode | SFBC |
| Traffic Model | FTP3 |
| Fraction of Indoor UEs | 0.8 |
| RB Allocation | GF: Random Selection of partition (for both new and re-transmission) |
| ACK Feedback assumption | Ideal |
| Channel estimation | Ideal |
| Latency bound | 1ms |
| Target Reliability | 10-5 |
| Decoding Delay | 1 SLOT ( BBU and UE) |