3GPP TSG RAN WG1 NR Ad-Hoc#3	 R1-1715596

Nagoya, Japan, 18-21 September 2017

Agenda Item:	6.2.3.8

Source:	Huawei, HiSilicon

Title:	SRS antenna switching

Document for:	Discussion and decision

# Introduction

In RAN1#90 [1], the following agreements were made for SRS antenna switching:

Agreements: - Support antenna switching for SRS transmission within a carrier for the following UE antenna configuration: - UE configured with 2Tx (transmit antenna ports) - UE configured with 1Tx (transmit antenna ports) - Note: 1Tx and 2Tx is dependent on UE’s capability for SRS transmission - FFS: details

In this contribution, we provide our views on detailed SRS antenna switching design.

# Discussion on Antennas Switching for SRS transmission

In RAN1#90 [1], it was agreed to support antenna switching for SRS transmission for CSI acquisition for UE configured with 1Tx and 2Tx. With antenna switching for SRS transmission, gNB can obtain the full DL CSI based on the measurement of SRS for the UE different DL and UL capabilities, e.g., 2T4R. It is known that 1Tx2 ports antenna switching for SRS was supported when 1T2R case in LTE. Then, in the case of 4Rx at UE side in NR, if the UE only has 1Tx, antenna switching for SRS transmission with 1Tx4 ports should be supported, if the UE has 2Tx, antenna switching for SRS transmission with 2Tx4 ports should be supported.

## Discussion on 1Tx2 ports antennas switching for SRS transmission

For the antenna switching for the case of 1Tx2, there is an antenna switching scheme defined in LTE, where SRS can be transmitted in each antenna with equally chance. In NR, the LTE-like antenna switching scheme for 1Tx2 can be reused.

## Discussion on 2Tx4 ports antennas switching for SRS transmission

In the following, we give the detailed design of 2Tx4 ports antenna switching for SRS transmission. To support SRS transmission with 2Tx SRS antenna switching for 4Rx, SRS hopping for 4TX switching should be defined in the specification. Since 1Tx2 ports antenna switching for SRS transmission is supported in LTE, it is straightforward to design 2Tx4 ports antenna switching similar to LTE’s design. In particular, if the whole configured bandwidth for SRS transmission needs to be covered by K SRS transmissions, 2K SRS transmissions are needed for 2Tx4 ports antenna switching to allow each antenna to cover the configured bandwidth for SRS. Moreover, in the K SRS transmissions, the opportunity of SRS transmission for each antenna is needed to be as equal as possible.

According to the above discussion, the SRS hopping with 2Tx4 ports antenna switching is given as follows. UE can use 2 antennas for SRS transmission simultaneously, and these 2 antennas compose an antenna group. Therefore, the SRS transmission with 2Tx SRS antenna switching for 4Rx becomes switching among antenna groups, e.g., among 2 antenna groups. Similar to LTE, when frequency hopping is disabled, the antenna group index for SRS transmission, of the UE antenna group that transmits the SRS at time  is given by

,                                                          (1)

where   counts the number of UE-specific SRS transmission.

When frequency hopping is enabled, the index , of the UE antenna group that transmits the SRS at time  is given by

(2)

where is the minimum needed number of SRS transmissions to cover the whole configured bandwidth for SRS, and the antennas in a group have the capability to transmit SRS simultaneously. The association between antenna group index and antenna index is defined in Table 1.

Table 1 Association between antenna group and antenna

| Antenna group index | Antenna index |
| --- | --- |
| Antenna group | {0, 1} |
| Antenna group | {2, 3} |

Based on the proposed scheme in (1) and (2), we show the antenna switching pattern for SRS transmission in Table 2. In the example, SRS hopping is enabled with K = 2. Antenna group 0 is composed by antenna {0, 1} and antenna group 1 is composed by antenna {2, 3}, which is shown in Table 1. It can be found that the scheme above is just re-using the antenna switching method in LTE for 1T2R, therefore basic design principle of SRS antenna switching can be guaranteed.

Table 2 An example of SRS hopping with K=2

|  | First part of SRS bandwidth | Second part of SRS bandwidth |
| --- | --- | --- |
| 0 | Antenna group Antenna indexes {0,1} |  |
| 1 |  | Antenna group Antenna indexes {2,3} |
| 2 | Antenna group Antenna indexes {2,3} |  |
| 3 |  | Antenna group Antenna indexes {0,1} |

Proposal 1: For 2Tx4 ports antenna switching for SRS transmission, adopt LTE-like antenna switching scheme, i.e., (1),  (2) and Table 1, with 2Tx in an antenna group.

## Discussion on 1Tx4 ports antennas switching for SRS transmission

In order to support 1Tx4 ports antenna switching, the following design principles should be satisfied, which are similar to 1Tx2 ports antenna switching design in LTE

- The whole configured bandwidth for SRS needs to be covered by K SRS transmissions

- In the K SRS transmissions, the opportunity of SRS transmission for each antenna is needed to be as equal as possible

- Within 4K SRS transmissions, the whole configured bandwidth for SRS is covered by each antenna

According to the above discussion, the SRS hopping for 1Tx4 ports antenna switching is given as follows.

When frequency hopping is disabled, the antenna index for SRS transmission, of the UE antenna that transmits the SRS at time  is given by

.                                                          (3)

where nSRS  counts the number of UE-specific SRS transmission.

When frequency hopping is enabled, the index , of the UE antenna that transmits the SRS at time  is given by

(4)

where is the minimum needed number of SRS transmissions to cover the whole configured bandwidth for SRS.  and are used to guarantee that the whole configured bandwidth for SRS can be covered by 4K SRS transmissions for each antenna and the opportunity for SRS transmission is as equal as possible for each antenna in K SRS transmission.

Based on the proposed scheme in (3) and (4), we show the antenna switching pattern for SRS transmission in Table 3. In the example, SRS hopping is enabled with K = 4. Each SRS transmission will cover one fourth of the whole configured bandwidth for SRS. K=4 SRS transmissions covers the whole configured bandwidth for SRS.

Table 3 An example of SRS hopping with K=4

| nSRS | First part of SRS bandwidth | Second part of SRS bandwidth | Third part of SRS bandwidth | Fourth part of SRS bandwidth |
| --- | --- | --- | --- | --- |
| 0 | Antenna index 0 |  |  |  |
| 1 |  | Antenna index 1 |  |  |
| 2 |  |  | Antenna index 2 |  |
| 3 |  |  |  | Antenna index 3 |
| 4 | Antenna index 1 |  |  |  |
| 5 |  | Antenna index 2 |  |  |
| 6 |  |  | Antenna index 3 |  |
| 7 |  |  |  | Antenna index 0 |
| 8 | Antenna index 2 |  |  |  |
| 9 |  | Antenna index 3 |  |  |
| 10 |  |  | Antenna index 0 |  |
| 11 |  |  |  | Antenna index 1 |
| 12 | Antenna index 3 |  |  |  |
| 13 |  | Antenna index 0 |  |  |
| 14 |  |  | Antenna index 1 |  |
| 15 |  |  |  | Antenna index 2 |

Proposal 2: For 1Tx4 ports antenna switching for SRS transmission, adopt antenna switching scheme shown in (3) and (4).

# Performance evaluation of antennas switching for SRS transmission

In this section, we will provide some simulation results to show the benefits of 2Tx4 ports antenna switching for SRS transmission. In all of the simulation below, the impact of time delay and insertion loss of SRS antenna switching are considered. Moreover, FTP 1 traffic model is used in the simulation and other simulation assumptions can be found in Appendix-A.

In Figure 1, we provide the performance evaluation of the 2Tx4 ports antenna switching with insertion loss. The baseline is 2TX SRS transmission without antenna switching. For both of the cases in the simulation (i.e., 2Tx without switching and 2Tx4 ports antenna switching), 2T4R is assumed at UE side. In the simulation, there is no insertion loss in the baseline (2TX without antenna switching), and 0.5dB~2dB for the 2Tx4 ports antenna switching.

Figure 1 Performance gain 2Tx4 ports SRS antenna switching scheme in (1) and (2) over 2T4R without switching

As illustrated in Figure 1, it shows that 2Tx4 ports antenna switching can obtain 16%~23% cell average gain over 2T4R without antenna switching. For performance gain for cell edge, 15%~23% gain over 2T4R without antenna switching can be obtained with different insertion loss assumption. Due to the performance gain in the simulation results, we propose to support at least 2Tx4 ports antenna switching.

Observation 1: With 2Tx4 ports antenna switching scheme for SRS transmission in (1) and (2), significant gain can be obtained in the case of 2T4R at UE side, where 16%~23% average gain can be observed.

Furthermore, we provide the simulation result of 1Tx4 ports antenna switching for SRS transmission in the following. In the simulation, 1T4R is assumed at UE side.

Figure 2 Performance gain of 1Tx4ports SRS antenna switching over 1T4R without switching.

As illustrated in Figure 2, it shows that 1Tx4 ports antenna switching can obtain 19%~26% cell average gain over 1T4R without antenna switching. For performance gain for cell edge, 15%~18% gain over 1T4R without antenna switching can be obtained with different insertion loss assumption. Moreover, a detailed design of 1Tx4 ports antenna switching can be found in Appendix-B.

Observation 2: With 1Tx4 ports antenna switching for SRS transmission, significant gain can be obtained in the case of 1T4R at UE side, where 19%~26% average gain can be observed.

# Conclusions

Based the discussions above, we have the following observations and proposals:

Observation 1: With 2Tx4 ports antenna switching for SRS transmission, significant gain can be obtained in the case of 2T4R at UE side, where 16%~23% average gain can be observed.

Observation 2: With 1Tx4 ports antenna switching for SRS transmission, significant gain can be obtained in the case of 1T4R at UE side, where 19%~26% average gain can be observed.

Proposal 1: For 2Tx4 ports antenna switching for SRS transmission, adopt LTE-like antenna switching scheme, i.e., (1), (2) and Table 1, with 2Tx in an antenna group.

Proposal 2: For 1Tx4 ports antenna switching for SRS transmission, adopt antenna switching scheme shown in (3) and (4).

# References

Chairman’s notes, RAN1#90, Aug. 2017.

# Appendix-A: Simulation Assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | TDD |
| Inter-BS distance | 200m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 10MHz |
| Channel model | UMi |
| BS Tx power | 41dBm |
| BS antenna configuration | (M, N, P, Mg, Ng) = (8, 4, 2, 1, 1); (dH,dV) = (0.8, 0.5)λ |
| BS TXRU mapping | (MTXRU, NTXRU, P, Mg, Ng) = (2, 4, 2, 1, 1) |
| UE antenna configurations | 2Rx, 4Rx, Cross-polarized with 0, 90deg |
| UE antenna height | Follow TR36.873 |
| UE antenna gain | Follow TR36.873 |
| UE receiver noise figure | 9 dB |
| Traffic model | Non-Full buffer, FTP model 1, 500KB packet size |
| UE distribution | 80% Indoor, 3km/h, 20% Outdoor, 30km/h |
| Scheduler | PF |
| HARQ scheme | CC with up to 3 retransmissions |
| UE receiver type | MMSE-IRC |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |
| MIMO mode | MU-MIMO with rank adaptation, max rank =4 |
| SRS hopping bandwidth | 24 PRB |
| SRS hopping period | 2 ms |