3GPP TSG RAN WG1 NR Ad-Hoc#3	R1-1715599

Nagoya, Japan, 18-21 September 2017

Agenda Item:	6.2.3.8

Source:	Huawei, HiSilicon

Title:	Signaling of DMRS ports for SU/MU-MIMO

Document for:	Discussion and decision

# Introduction

Regarding DMRS orthogonal port number and codeword assignment, it was agreed in RAN1 #88bis meeting [1] that:

- Support at least the following design of DL DM-RS for data channels

- Support the maximal 12 orthogonal DL DMRS ports for MU-MIMO

- For 3 and 4-layer transmission, NR supports 1 codeword per PDSCH/PUSCH assignment per UE

For DMRS pattern, it was agreed in RAN1 NR Ad-Hoc #2 [2] that:

- Both in DL and UL DMRS for CP-OFDM, only PN sequence is supported

- The working assumption made in RAN1#89 for DM-RS is updated and agreed as follows for CP-OFDM:

- A UE is configured by higher layers with DMRS pattern either from the front-loaded DMRS Configuration type 1 or from the front-loaded DMRS Configuration type 2 for DL/UL:

- Configuration type 1:

- One symbol:

  - Comb 2 + 2 CS, up to 4 ports

- Two symbols:

  - Comb 2 + 2 CS + TD-OCC ({1 1} and {1 -1}), up to 8 ports

    - Note: It should be possible to schedule up to 4 ports without using both {1,1} and {1,-1}.

- Configuration type 2:

- One symbol:

  - 2-FD-OCC across adjacent REs in the frequency domain, up to 6 ports

- Two symbols:

    - 2-FD-OCC across adjacent REs in the frequency domain + TD-OCC (both {1,1} and {1,-1}) up to 12 ports

      - Note: It should be possible to schedule up to 6 ports without using both {1,1} and {1,-1}.

- From UE perspective, frequency domain CDMed DMRS ports are QCLed.

Regarding QCL, it was agreed in RAN1 NR Ad-Hoc #2 [2] that

- For QCL, NR supports:

  - At least one or two DM-RS antenna port groups per PDSCH.

In this contribution, we provide our considerations on configurations and signaling of DMRS ports for both SU and MU. Besides that, NCJT with 1 or multiple PDCCH(s) is also considered in the design of port indication.

# Indication design for NR DMRS

It has been agreed to support maximal 12 orthogonal ports and configurable patterns for DMRS in NR. Some specific considerations on DMRS indication should be considered to adapt to multiple patterns and larger orthogonal port number comparing to LTE. In the following sections, the design of NR DMRS indication scheme will be discussed.

## Hierarchical configuration of DMRS

As mentioned above, at least higher layer signalling has been suggested for the 2 DMRS pattern configurations in the working assumption [2]. That is because some of the parameters that determine DMRS pattern are depended on scenario, UE mobility, long-term or semi-static channel properties such as delay spread and so on which do not dynamically change, DMRS pattern can to some extent be configured by RRC signalling. However, there are some rapidly changing parameters such as DMRS port number, port indexes and MU-pairing information still need dynamically configuring by DCI similar as LTE, such as dynamic SU and MU. The hierarchical configuration of DMRS can greatly balance the dynamic signalling overhead and flexibility and support dynamic transmission schemes more efficiently.

Proposal 1: NR should support hierarchical configuration of DMRS

- Higher layer signaling for pattern configuration

- Dynamic signaling for DMRS port indication and MU-pairing information

## Signalling of DMRS ports

Before discussing the detailed DCI design for DL MIMO especially for MU-MIMO, NR should first identify some factors such as maximum number of DMRS ports per UE in MU-MIMO and scrambling sequence number, since these factors may have significant impact on the signaling design for MU-MIMO. In the following we present some general design principles of signaling for DMRS.

### Consider increased MU-MIMO dimension

In NR, the transmission antennas in gNB side will be more than legacy LTE’s configuration, and UEs with 4Rx antennas are more general than previous LTE releases as well. Thus, 4 layers per UE in the MU-MIMO transmission will be a general case in NR.

In the massive MIMO in NR, increase the MU pairing for the simultaneous transmission is the key point to increase the spectrum efficiency. As agreed in previous meeting in NR, up to 12 orthogonal DMRS ports have been supported in NR, which means each user can be assigned with up to 4 layers (with 4 Rx antennas), even 8 orthogonal layers (with 8 Rx antennas).

For MU-MIMO in LTE, maximum DMRS ports number per UE is 2, while total number of co-scheduled DMRS ports is limited to 4. Considering much richer transmission scenarios in NR, enhanced processing capability at UE, as well as the agreed maximal 12 orthogonal DMRS ports for MU-MIMO, the limitation on maximum DMRS ports for MU-MIMO should be relaxed because higher orthogonality can be achieved for spatial multiplexing between and within each UE. Therefore we propose to reconsider MU-MIMO dimension in NR. For example, the maximum DMRS ports number per UE should be increased and maximum 4 DMRS ports per UE is more appropriate.

Proposal 2: Support up to 4 orthogonal DMRS ports per UE for MU-MIMO.

### Consider multiple DCI tables for port indication

As agreed in RAN1 #88bis, maximal 12 orthogonal ports will be supported for MU-MIMO in NR. The support of DMRS port indication for UE from such a large set of orthogonal DMRS ports require provision in DCI of very large number bits. Obviously the number of potentially schedule DMRS ports is to some extent depends on the transmission scenario. For example, very limited DMRS port number would be considered in high frequency transmission, while more than 8 DMRS ports are more likely be scheduled in dense urban scenarios. Thus not all of the supported DCI indication combinations for DMRS port would be always used in the practice, then some DCI overhead reduction schemes may be considered based on priori knowledge of maximum DMRS port number or pattern configurations.

In the following we provide some examples of port combinations for different MU orders, e.g., 4, 6, 8 and 12. The corresponding tables are respectively provided in Appendix A. It should be noted that port combinations for different number of codeword has been reordered based on the agreements on codeword to layer mapping. Besides, maximum number of DMRS ports per UE for MU-MIMO is assumed to be 4.

As can been seen, 5 bits are needed for indicating all the port combinations for maximal 12 layers transmission. As a contrast, with the same design method, only 4 bits and 3 bits are needed for all the port combinations of up to 8 layers and 4 layers.

Besides, DMRS antenna port indication scheme may be designed differently for different pattern configurations. For example, with different lengths of code division multiplexing, different indication combinations for DMRS port should be considered to make DMRS configuration as transparent as possible to co-paired UE.

In a word, to reduce the overhead of DCI for indicating DMRS ports, different indication combinations for DMRS port should be designed and used for different scenarios or multiplexing schemes. Summarizing discussion above we have the following proposal:

Proposal 3: Multiple DCI tables of DMRS port indication based on DMRS pattern configurations or MU-orders should be supported.

### Consider non-Transparent MU-MIMO

To increase system spectral efficiency, NR should allow for multiplexing of DMRS and other signals in the DMRS symbols. For example, instead of keeping all the possible DMRS REs muted, data can be FDM-multiplexed with DMRS if the corresponding OFDM symbols have not been fully occupied by DMRS. In this case, the PDSCH mapping of a given UE should avoid the resource elements in which DMRS is transmitted for better PDSCH demodulation performance.

Regarding the UEs receiving transmission in LTE, DMRS is allocated and notified in a transparent manner. In other words, the indication scheme had been designed such the scheduled UE is only notified of information that pertains to its own transmission. Therefore, from an UE’s perspective, it has no idea whether the transmission it is receiving is a part of a MU-MIMO transmission or simply a SU-MIMO transmission. The transparency in DMRS indication for MU-MIMO was made possible by limiting total orthogonal port number to 4 and using CDM4 to multiplexing DMRS ports for MU-MIMO. However, maximal 12 orthogonal ports had been agreed to be supported for MU-MIMO in NR. With limited size of CDM, it is difficult to maintain the same level of transparency as in LTE. That is because the UE is no longer able to determine the correct number of DMRS REs in a PRB. Thus in NR a non-transparent DMRS indication with the configuration information of the co-scheduled DMRS is sometimes needed. Additional information, e.g., the total number of transmission layers or the assignment of DMRS ports to other co-scheduled UEs, should be indicated to a given UE.

Proposal 4: DMRS port indication should support Non-Transparent MU-MIMO in NR

- Information of co-scheduled DMRS ports can be added into port indication tables

For signaling of rate matching around DMRS, one straightforward method is to directly indicate UE the DMRS port indices of co-scheduled UEs, which with the large DCI overhead. To reduce the signaling of head for rate matching, two schemes are discussed in our companion contribution [4]. One of the schemes is to indicate total number of MU pairing layers, where the number of layers can be quantized based on CDM length. For example, in the case of 1 symbol, OCC length is 2, so the total layer number only needs to be 2, 4, and 6. Since the case with total layer number is 3 is the same DMRS REs with total layer number is 4, due to the introduced CDM. The detailed discussion can be found in [4]. With such scheme, the signaling overhead can be significantly reduced.

Another scheme discussed in the companion contribution [4] for rate matching is to indicate the used/occupied CDM groups. As an example, in the case of 1 symbol for DMRS, the CDM length is 2, there are 3 CDM group for DMRS configuration 2, each group with 2 DMRS ports. If the 3 layer MU pairing, at least first 2 CDM group will be occupied, so with indicating of the number of CDM group, UE can know which DMRS ports are used for other UEs. The detailed discussion also can be found in [4].

Actually, the two schemes are same for the overhead and similar in the signaling, where both of the can be indicated in DCI table for DMRS signaling.

Proposal 5: For rate matching, total number of MU layers or CDM group information should be included in DMRS DCI table design.

### Consider port indication for NCJT

It has been agreed in RAN1 NR Ad-Hoc#2 meeting that NR will support 1 or 2 DMRS port groups, in which DMRS ports within one group are QCL-ed, and DMRS ports in different groups are non-QCLed. Besides, NR also agrees that frequency domain CDM-ed DMRS ports are QCL-ed.

Therefore, when designing DCI tables, port mapping schemes and DMRS port grouping should be taken into account. For example, combinations of DMRS port from different port groups should be included in the DCI tables. For DMRS port mapping, different schemes should be considered for different DMRS configurations. To increase system spectral efficiency, NR should allow mapping DMRS port with the ordering: CDM first, then FDM. It is because that if with FDM first then CDM, which mean more REs will be occupied for DMRS transmission. But if CDM first, that mean the same REs can be reused for multiple DMRS ports, multiplexed with CDM, then the non-occupied REs can be used for data transmission to enhance the spectrum efficiency. The detailed performance evaluation of DMRS and data multiplexing can be found in [3].

Proposal 6: DMRS port grouping and port mapping schemes should be taken into account when designing DMRS port indication tables

In the following we provide two different port mapping schemes respectively for the two configurations.

Table 9: Port mapping scheme for configuration 1

| Port | Comb index | CS index | T-OCC |  |
| --- | --- | --- | --- | --- |
|  |  |  | One symbol | Two symbol |
| 0 | 0 | 0 |  |  |
| 1 | 0 | 2 |  |  |
| 2 | 1 | 0 |  |  |
| 3 | 1 | 2 |  |  |
| 4 | 0 | 0 |  |  |
| 5 | 1 | 0 |  |  |
| 6 | 0 | 2 |  |  |
| 7 | 1 | 2 |  |  |

Table 10: Port mapping scheme for configuration 2

| Antenna port | Subcarrier index | F-OCC | T-OCC |  |
| --- | --- | --- | --- | --- |
|  |  |  | One symbol | Two symbol |
| 0 | 0/1 |  |  |  |
| 1 | 0/1 |  |  |  |
| 2 | 2/3 |  |  |  |
| 3 | 2/3 |  |  |  |
| 4 | 4/5 |  |  |  |
| 5 | 4/5 |  |  |  |
| 6 | 0/1 |  |  |  |
| 7 | 2/3 |  |  |  |
| 8 | 4/5 |  |  |  |
| 9 | 0/1 |  |  |  |
| 10 | 2/3 |  |  |  |
| 11 | 4/5 |  |  |  |

As mentioned above, it has been agreed that frequency domain CDM-ed DMRS ports are QCL-ed. Therefore, DMRS ports from 2 groups (if exist) cannot be mapped in the same CDM group. Without loss of generality, we take DMRS configuration 2 as an example to introduce the solution for NCJT with 1 or 2 PDCCH(s). Based on the mapping scheme in table 10, 12 DMRS ports can be divided into 2 groups as {0,1,6,9} and {2,3,4,5,7,8,10,11}. To realize data transmission in NCJT, some other port combinations should be additionally considered in the DCI tables mentioned in Appendix A and B.

For single PDCCH case, to guarantee full scheduling flexibility of 12 DMRS ports, 23 other port combinations should be considered in Table 4. The renewed table can be found in Appendix C. For 2 PDCCHs case, 11 other port combinations should be considered in Table 4. The renewed table can be found in Appendix D.

Besides, for NCJT, since DMRS groups with different QCL assumptions may possibly be configured by RRC signaling, the number of DMRS antenna ports and DMRS port indexes in a certain DMRS group could change semi-statically. It is possible to indicate DMRS ports in DCI flexibly to support such flexible configurations. And both single and multiple NR-PDCCH scheduling based NCJT should be considered to achieve a unify DCI table. To solve this problem, another possible approach is to indicate index(s) of DMRS groups and layer numbers in DCI table. For example, based on the mapping scheme in table 10, two DMRS port groups { DMRS group 1 [0,1,6,9]; DMRS group 2 [2,3,4,5,7,8,10,11] } can be configured by RRC signaling. For single NR-PDCCH based NCJT, the total number of layers and two DMRS groups can be indicated by one DCI signaling state as follows (we just take rank=3 as an example):

DCI signaling state: {3 layer: DMRS group index 1, 1 layer; DMRS group index 2, 2 layers}

Note that there is no necessary to list DMRS group index 1 has 2 layers while DMRS group 2 has 1 layers in the DCI table as the re-configuration of QCL for each DMRS group could achieve it for multi-TRP swapping.

While for multiple NR-PDCCH based NCJT, each DCI state could indicate only one DMRS groups and its layers by introducing two DCI signaling states:

DCI signaling state 1: {3 layer, DMRS group index 1}

DCI signaling state 1: {3 layer, DMRS group index 1}

In this way, DMRS ports indication would not be affected by the re-configuration of DMRS groups and both single and multiple NR-PDCCH based NCJT could be supported.

Proposal 7: The DMRS port index(s) and the number of layers should be indicated in DCI for at least single and multiple NR-PDCCH based NCJT.

# Conclusions

This contribution discusses some general considerations on signaling design for demodulation reference signals. In summary, the following observation and proposals are made.

Proposal 1: NR should support hierarchical configuration of DMRS

- Higher layer signaling for pattern configuration

- Dynamic signaling for DMRS port indication and MU-pairing information

Proposal 2: Support up to 4 orthogonal DMRS ports per UE for MU-MIMO

Proposal 3: Multiple DCI tables of DMRS port indication based on DMRS pattern configurations or MU-orders should be supported

Proposal 4: DMRS port indication should support Non-Transparent MU-MIMO in NR

- Information of co-scheduled DMRS ports can be added into port indication tables

Proposal 5: For rate matching, total number of MU layers or CDM group information should be included in DMRS DCI table design

Proposal 6: DMRS port grouping and port mapping schemes should be taken into account when designing DMRS port indication tables

Proposal 7: The DMRS port index(s) and the number of layers should be indicated in DCI for at least single and multiple NR-PDCCH based NCJT.

# References

“RAN1 #88bis meeting Chairman’s Notes”, Spokane, USA, 3rd-7th April, 2017.

“RAN1 #89 meeting Chairman’s Notes”, Hangzhou, China, 15-19 May, 2017.

Huawei, HiSilicon, “Evaluation results of DMRS design for DL data channel”, R1-1712244, Prague, Czech Republic, 21-25 August 2017.

Huawei, HiSilicon, “Rate matching for data channels”, R1-1712233, Prague, Czech Republic, 21-25 August 2017.

# Appendix A (Tables corresponding to different pattern configurations)

- Table 1 example of port combinations for up to 4 DMRS antenna ports

| One Codeword (<=4 layers): Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords (>4 layer): Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | UE Rank | port index | Value | UE Rank | port index |
| 0 | 1 | 0 | 0 | reserve | reserve |
| 1 | 1 | 1 | 1 | reserve | reserve |
| 2 | 1 | 2 | 2 | reserve | reserve |
| 3 | 1 | 3 | 3 | reserve | reserve |
| 4 | 2 | 0-1 | 4 | reserve | reserve |
| 5 | 2 | 2-3 | 5 | reserve | reserve |
| 6 | 3 | 0-2 | 6 | reserve | reserve |
| 7 | 4 | 0-3 | 7 | reserve | reserve |

- Table 2 example of port combinations for up to 6 DMRS antenna ports

| One Codeword (<=4 layers): Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords (>4 layer): Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | UE Rank | port index | Value | UE Rank | port index |
| 0 | 1 | 0 | 0 | 5 | 0-4 (SU) |
| 1 | 1 | 1 | 1 | 6 | 0-5 (SU) |
| 2 | 1 | 2 | 2 | reserve | reserve |
| 3 | 1 | 3 | 3 | reserve | reserve |
| 4 | 1 | 4 | 4 | reserve | reserve |
| 5 | 1 | 5 | 5 | reserve | reserve |
| 6 | 2 | 0-1 | 6 | reserve | reserve |
| 7 | 2 | 2-3 | 7 | reserve | reserve |
| 8 | 2 | 4-5 | 8 | reserve | reserve |
| 9 | 3 | 0-2 | 9 | reserve | reserve |
| 10 | 3 | 3-5 | 10 | reserve | reserve |
| 11 | 4 | 0-3 | 11 | reserve | reserve |
| 12 | reserve | reserve | 12 | reserve | reserve |
| 13 | reserve | reserve | 13 | reserve | reserve |
| 14 | reserve | reserve | 14 | reserve | reserve |
| 15 | reserve | reserve | 15 | reserve | reserve |

- Table 3 example of port combinations for up to 8 DMRS antenna ports

| One Codeword (<=4 layers): Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords (>4 layer): Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | UE Rank | port index | Value | UE Rank | port index |
| 0 | 1 | 0 | 0 | 5 | 0-4 (SU) |
| 1 | 1 | 1 | 1 | 6 | 0-5 (SU) |
| 2 | 1 | 2 | 2 | 7 | 0-6 (SU) |
| 3 | 1 | 3 | 3 | 8 | 0-7 (SU) |
| 4 | 1 | 4 | 4 | reserve | reserve |
| 5 | 1 | 5 | 5 | reserve | reserve |
| 6 | 1 | 6 | 6 | reserve | reserve |
| 7 | 1 | 7 | 7 | reserve | reserve |
| 8 | 2 | 0-1 | 8 | reserve | reserve |
| 9 | 2 | 2-3 | 9 | reserve | reserve |
| 10 | 2 | 4-5 | 10 | reserve | reserve |
| 11 | 2 | 6-7 | 11 | reserve | reserve |
| 12 | 3 | 0-2 | 12 | reserve | reserve |
| 13 | 3 | 3-5 | 13 | reserve | reserve |
| 14 | 4 | 0-3 | 14 | reserve | reserve |
| 15 | 4 | 4-7 | 15 | reserve | reserve |

- Table 4 example of port combinations for up to 12 DMRS antenna ports

| One Codeword (<=4 layers): Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords (>4 layer): Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | UE Rank | port index | Value | UE Rank | port index |
| 0 | 1 | 0 | 0 | 5 | 0-4 |
| 1 | 1 | 1 | 1 | 6 | 0-5 |
| 2 | 1 | 2 | 2 | 7 | 0-6 |
| 3 | 1 | 3 | 3 | 8 | 0-7 |
| 4 | 1 | 4 | 4 | reserve | reserve |
| 5 | 1 | 5 | 5 | reserve | reserve |
| 6 | 1 | 6 | 6 | reserve | reserve |
| 7 | 1 | 7 | 7 | reserve | reserve |
| 8 | 1 | 8 | 8 | reserve | reserve |
| 9 | 1 | 9 | 9 | reserve | reserve |
| 10 | 1 | 10 | 10 | reserve | reserve |
| 11 | 1 | 11 | 11 | reserve | reserve |
| 12 | 2 | 0-1 | 12 | reserve | reserve |
| 13 | 2 | 2-3 | 13 | reserve | reserve |
| 14 | 2 | 4-5 | 14 | reserve | reserve |
| 15 | 2 | 6-7 | 15 | reserve | reserve |
| 16 | 2 | 8-9 | 16 | reserve | reserve |
| 17 | 2 | 10-11 | 17 | reserve | reserve |
| 18 | 3 | 0-2 | 18 | reserve | reserve |
| 19 | 3 | 3-5 | 19 | reserve | reserve |
| 20 | 3 | 6-8 | 20 | reserve | reserve |
| 21 | 3 | 9-11 | 21 | reserve | reserve |
| 22 | 4 | 0-3 | 22 | reserve | reserve |
| 23 | 4 | 4-7 | 23 | reserve | reserve |
| 24 | 4 | 8-11 | 24 | reserve | reserve |
| 25 | reserve | reserve | 25 | reserve | reserve |
| 26 | reserve | reserve | 26 | reserve | reserve |
| 27 | reserve | reserve | 27 | reserve | reserve |
| 28 | reserve | reserve | 28 | reserve | reserve |
| 29 | reserve | reserve | 29 | reserve | reserve |
| 30 | reserve | reserve | 30 | reserve | reserve |
| 31 | reserve | reserve | 31 | reserve | reserve |

# Appendix B (Tables considered total layer number)

- Table 5 port combinations for 1-symbol pattern in config.1

| One Codeword (<=4 layers): Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords (>4 layer): Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| value | Total layer num | UE Rank | Port index | value | Total layer num | UE Rank | Port index |
| 0 | 2 | 1 | 0 | 0 | reserve | reserve | reserve |
| 1 | 2 | 1 | 1 | 1 | reserve | reserve | reserve |
| 2 | 2 | 2 | 0-1 | 2 | reserve | reserve | reserve |
| 3 | 4 | 1 | 0 | 3 | reserve | reserve | reserve |
| 4 | 4 | 1 | 1 | 4 | reserve | reserve | reserve |
| 5 | 4 | 1 | 2 | 5 | reserve | reserve | reserve |
| 6 | 4 | 1 | 3 | 6 | reserve | reserve | reserve |
| 7 | 4 | 2 | 0-1 | 7 | reserve | reserve | reserve |
| 8 | 4 | 2 | 2-3 | 8 | reserve | reserve | reserve |
| 9 | 4 | 3 | 0-2 | 9 | reserve | reserve | reserve |
| 10 | 4 | 4 | 0-3 | 10 | reserve | reserve | reserve |
| 11 | reserve | reserve | reserve | 11 | reserve | reserve | reserve |
| 12 | reserve | reserve | reserve | 12 | reserve | reserve | reserve |
| 13 | reserve | reserve | reserve | 13 | reserve | reserve | reserve |
| 14 | reserve | reserve | reserve | 14 | reserve | reserve | reserve |
| 15 | reserve | reserve | reserve | 15 | reserve | reserve | reserve |

- Table 6 port combinations for 2-symbol pattern in config.1

| One Codeword (<=4 layers): Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords (>4 layer): Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| value | Total layer num | UE Rank | Port index | value | Total layer num | UE Rank | Port index |
| 0 | 4 | 1 | 0 | 0 | 8 | 5 | 0-4 |
| 1 | 4 | 1 | 1 | 1 | 8 | 6 | 0-5 |
| 2 | 4 | 1 | 2 | 2 | 8 | 7 | 0-6 |
| 3 | 4 | 1 | 3 | 3 | 8 | 8 | 0-7 |
| 4 | 4 | 2 | 0-1 | 4 | reserve | reserve | reserve |
| 5 | 4 | 2 | 2-3 | 5 | reserve | reserve | reserve |
| 6 | 4 | 3 | 0-2 | 6 | reserve | reserve | reserve |
| 7 | 4 | 4 | 0-3 | 7 | reserve | reserve | reserve |
| 8 | 8 | 1 | 0 | 8 | reserve | reserve | reserve |
| 9 | 8 | 1 | 1 | 9 | reserve | reserve | reserve |
| 10 | 8 | 1 | 2 | 10 | reserve | reserve | reserve |
| 11 | 8 | 1 | 3 | 11 | reserve | reserve | reserve |
| 12 | 8 | 1 | 4 | 12 | reserve | reserve | reserve |
| 13 | 8 | 1 | 5 | 13 | reserve | reserve | reserve |
| 14 | 8 | 1 | 6 | 14 | reserve | reserve | reserve |
| 15 | 8 | 1 | 7 | 15 | reserve | reserve | reserve |
| 16 | 8 | 2 | 0-1 | 16 | reserve | reserve | reserve |
| 17 | 8 | 2 | 2-3 | 17 | reserve | reserve | reserve |
| 18 | 8 | 2 | 4-5 | 18 | reserve | reserve | reserve |
| 19 | 8 | 2 | 6-7 | 19 | reserve | reserve | reserve |
| 20 | 8 | 3 | 0-2 | 20 | reserve | reserve | reserve |
| 21 | 8 | 3 | 3-5 | 21 | reserve | reserve | reserve |
| 22 | 8 | 4 | 0-3 | 22 | reserve | reserve | reserve |
| 23 | 8 | 4 | 4-7 | 23 | reserve | reserve | reserve |
| 24 | reserve | reserve | reserve | 24 | reserve | reserve | reserve |
| 25 | reserve | reserve | reserve | 25 | reserve | reserve | reserve |
| 26 | reserve | reserve | reserve | 26 | reserve | reserve | reserve |
| 27 | reserve | reserve | reserve | 27 | reserve | reserve | reserve |
| 28 | reserve | reserve | reserve | 28 | reserve | reserve | reserve |
| 29 | reserve | reserve | reserve | 29 | reserve | reserve | reserve |
| 30 | reserve | reserve | reserve | 30 | reserve | reserve | reserve |
| 31 | reserve | reserve | reserve | 31 | reserve | reserve | reserve |

- Table 7 port combinations for 1-symbol pattern in config.2

| One Codeword (<=4 layers): Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords (>4 layer): Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| value | Total layer num | UE Rank | Port index | value | Total layer num | UE Rank | Port index |
| 0 | 2 | 1 | 0 | 0 | 6 | 5 | 0-4 |
| 1 | 2 | 1 | 1 | 1 | 6 | 6 | 0-5 |
| 2 | 2 | 2 | 0-1 | 2 | 8 | 7 | 0-6 |
| 3 | 4 | 1 | 0 | 3 | 8 | 8 | 0-7 |
| 4 | 4 | 1 | 1 | 4 | reserve | reserve | reserve |
| 5 | 4 | 1 | 2 | 5 | reserve | reserve | reserve |
| 6 | 4 | 1 | 3 | 6 | reserve | reserve | reserve |
| 7 | 4 | 2 | 0-1 | 7 | reserve | reserve | reserve |
| 8 | 4 | 2 | 2-3 | 8 | reserve | reserve | reserve |
| 9 | 4 | 3 | 0-2 | 9 | reserve | reserve | reserve |
| 10 | 4 | 4 | 0-3 | 10 | reserve | reserve | reserve |
| 11 | 6 | 1 | 0 | 11 | reserve | reserve | reserve |
| 12 | 6 | 1 | 1 | 12 | reserve | reserve | reserve |
| 13 | 6 | 1 | 2 | 13 | reserve | reserve | reserve |
| 14 | 6 | 1 | 3 | 14 | reserve | reserve | reserve |
| 15 | 6 | 1 | 4 | 15 | reserve | reserve | reserve |
| 16 | 6 | 1 | 5 | 16 | reserve | reserve | reserve |
| 17 | 6 | 2 | 0-1 | 17 | reserve | reserve | reserve |
| 18 | 6 | 2 | 2-3 | 18 | reserve | reserve | reserve |
| 19 | 6 | 2 | 4-5 | 19 | reserve | reserve | reserve |
| 20 | 6 | 3 | 0-2 | 20 | reserve | reserve | reserve |
| 21 | 6 | 3 | 3-5 | 21 | reserve | reserve | reserve |
| 22 | 6 | 4 | 0-3 | 22 | reserve | reserve | reserve |
| 23 | reserve | reserve | reserve | 23 | reserve | reserve | reserve |
| 24 | reserve | reserve | reserve | 24 | reserve | reserve | reserve |
| 25 | reserve | reserve | reserve | 25 | reserve | reserve | reserve |
| 26 | reserve | reserve | reserve | 26 | reserve | reserve | reserve |
| 27 | reserve | reserve | reserve | 27 | reserve | reserve | reserve |
| 28 | reserve | reserve | reserve | 28 | reserve | reserve | reserve |
| 29 | reserve | reserve | reserve | 29 | reserve | reserve | reserve |
| 30 | reserve | reserve | reserve | 30 | reserve | reserve | reserve |
| 31 | reserve | reserve | reserve | 31 | reserve | reserve | reserve |

- Table 8 port combinations for 2-symbol pattern in config.2

| One Codeword (<=4 layers): Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords (>4 layer): Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| value | Total layer num | UE Rank | Port index | value | Total layer num | UE Rank | Port index |
| 0 | 4 | 1 | 0 | 0 | 8 | 5 | 0-4 |
| 1 | 4 | 1 | 1 | 1 | 8 | 6 | 0-5 |
| 2 | 4 | 1 | 2 | 2 | 8 | 7 | 0-6 |
| 3 | 4 | 1 | 3 | 3 | 8 | 8 | 0-7 |
| 4 | 4 | 2 | 0-1 | 4 | reserve | reserve | reserve |
| 5 | 4 | 2 | 2-3 | 5 | reserve | reserve | reserve |
| 6 | 4 | 3 | 0-2 | 6 | reserve | reserve | reserve |
| 7 | 4 | 4 | 0-3 | 7 | reserve | reserve | reserve |
| 8 | 8 | 1 | 0 | 8 | reserve | reserve | reserve |
| 9 | 8 | 1 | 1 | 9 | reserve | reserve | reserve |
| 10 | 8 | 1 | 2 | 10 | reserve | reserve | reserve |
| 11 | 8 | 1 | 3 | 11 | reserve | reserve | reserve |
| 12 | 8 | 1 | 4 | 12 | reserve | reserve | reserve |
| 13 | 8 | 1 | 5 | 13 | reserve | reserve | reserve |
| 14 | 8 | 1 | 6 | 14 | reserve | reserve | reserve |
| 15 | 8 | 1 | 7 | 15 | reserve | reserve | reserve |
| 16 | 8 | 2 | 0-1 | 16 | reserve | reserve | reserve |
| 17 | 8 | 2 | 2-3 | 17 | reserve | reserve | reserve |
| 18 | 8 | 2 | 4-5 | 18 | reserve | reserve | reserve |
| 19 | 8 | 2 | 6-7 | 19 | reserve | reserve | reserve |
| 20 | 8 | 3 | 0-2 | 20 | reserve | reserve | reserve |
| 21 | 8 | 3 | 3-5 | 21 | reserve | reserve | reserve |
| 22 | 8 | 4 | 0-3 | 22 | reserve | reserve | reserve |
| 23 | 8 | 4 | 4-7 | 23 | reserve | reserve | reserve |
| 28 | 12 | 1 | 0 | 28 | reserve | reserve | reserve |
| 29 | 12 | 1 | 1 | 29 | reserve | reserve | reserve |
| 30 | 12 | 1 | 2 | 30 | reserve | reserve | reserve |
| 31 | 12 | 1 | 3 | 31 | reserve | reserve | reserve |
| 32 | 12 | 1 | 4 | 32 | reserve | reserve | reserve |
| 33 | 12 | 1 | 5 | 33 | reserve | reserve | reserve |
| 34 | 12 | 1 | 6 | 34 | reserve | reserve | reserve |
| 35 | 12 | 1 | 7 | 35 | reserve | reserve | reserve |
| 36 | 12 | 1 | 8 | 36 | reserve | reserve | reserve |
| 37 | 12 | 1 | 9 | 37 | reserve | reserve | reserve |
| 38 | 12 | 1 | 10 | 38 | reserve | reserve | reserve |
| 39 | 12 | 1 | 11 | 39 | reserve | reserve | reserve |
| 40 | 12 | 2 | 0-1 | 40 | reserve | reserve | reserve |
| 41 | 12 | 2 | 2-3 | 41 | reserve | reserve | reserve |
| 42 | 12 | 2 | 4-5 | 42 | reserve | reserve | reserve |
| 43 | 12 | 2 | 6-7 | 43 | reserve | reserve | reserve |
| 44 | 12 | 2 | 8-9 | 44 | reserve | reserve | reserve |
| 45 | 12 | 2 | 10-11 | 45 | reserve | reserve | reserve |
| 46 | 12 | 3 | 0-2 | 46 | reserve | reserve | reserve |
| 47 | 12 | 3 | 3-5 | 47 | reserve | reserve | reserve |
| 48 | 12 | 3 | 6-8 | 48 | reserve | reserve | reserve |
| 49 | 12 | 3 | 9-11 | 49 | reserve | reserve | reserve |
| 50 | 12 | 4 | 0-3 | 50 | reserve | reserve | reserve |
| 51 | 12 | 4 | 4-7 | 51 | reserve | reserve | reserve |
| 52 | 12 | 4 | 8-11 | 52 | reserve | reserve | reserve |
| 53 | reserve | reserve | reserve | 53 | reserve | reserve | reserve |
| 54 | reserve | reserve | reserve | 54 | reserve | reserve | reserve |
| 55 | reserve | reserve | reserve | 55 | reserve | reserve | reserve |
| 56 | reserve | reserve | reserve | 56 | reserve | reserve | reserve |
| 57 | reserve | reserve | reserve | 57 | reserve | reserve | reserve |
| 58 | reserve | reserve | reserve | 58 | reserve | reserve | reserve |
| 59 | reserve | reserve | reserve | 59 | reserve | reserve | reserve |
| 60 | reserve | reserve | reserve | 60 | reserve | reserve | reserve |
| 61 | reserve | reserve | reserve | 61 | reserve | reserve | reserve |
| 62 | reserve | reserve | reserve | 62 | reserve | reserve | reserve |
| 63 | reserve | reserve | reserve | 63 | reserve | reserve | reserve |

# Appendix C (Tables considered NCJT with single PDCCH)

- Table 11: port combinations for NCJT (single PDCCH)

| One Codeword (<=4 layers): Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords (>4 layer): Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | UE Rank | port index | Value | UE Rank | port index |
| 0 | 1 | 0 | 0 | 5 | 0-4 |
| 1 | 1 | 1 | 1 | 6 | 0-5 |
| 2 | 1 | 2 | 2 | 7 | 0-6 |
| 3 | 1 | 3 | 3 | 8 | 0-7 |
| 4 | 1 | 4 | 4 | 5 | 1,2-5 |
| 5 | 1 | 5 | 5 | 5 | 7-11 |
| 6 | 1 | 6 | 6 | 5 | 5-9 |
| 7 | 1 | 7 | 7 | 5 | 0-3,6 |
| 8 | 1 | 8 | 8 | 5 | 0-2,6,9 |
| 9 | 1 | 9 | 9 | 6 | 3-8 |
| 10 | 1 | 10 | 10 | 6 | 6-11 |
| 11 | 1 | 11 | 11 | 6 | 0-4,6 |
| 12 | 2 | 0-1 | 12 | 6 | 0-3,6,9 |
| 13 | 2 | 2-3 | 13 | 7 | 2-8 |
| 14 | 2 | 4-5 | 14 | 7 | 1-7 |
| 15 | 2 | 6-7 | 15 | 7 | 0-4,6,9 |
| 16 | 2 | 8-9 | 16 | 8 | 2-8,10 |
| 17 | 2 | 10-11 | 17 | 8 | 0-5,7-8 |
| 18 | 3 | 0-2 | 18 | 8 | 0-6,9 |
| 19 | 3 | 3-5 | 19 | reserve | reserve |
| 20 | 3 | 6-8 | 20 | reserve | reserve |
| 21 | 3 | 9-11 | 21 | reserve | reserve |
| 22 | 4 | 0-3 | 22 | reserve | reserve |
| 23 | 4 | 4-7 | 23 | reserve | reserve |
| 24 | 4 | 8-11 | 24 | reserve | reserve |
| 25 | 2 | 0,2 | 25 | reserve | reserve |
| 26 | 2 | 1,3 | 26 | reserve | reserve |
| 27 | 3 | 6-7,9 | 27 | reserve | reserve |
| 28 | 3 | 8,10-11 | 28 | reserve | reserve |
| 29 | 3 | 0,2-3 | 29 | reserve | reserve |
| 30 | 3 | 1,4-5 | 30 | reserve | reserve |
| 31 | 4 | 6-9 | 31 | reserve | reserve |
| 32 | 4 | 0-1,2,6 | 32 | reserve | reserve |
| 33 | reserve | reserve | 33 | reserve | reserve |
| 34 | reserve | reserve | 34 | reserve | reserve |
| 35 | reserve | reserve | 35 | reserve | reserve |
| 36 | reserve | reserve | 36 | reserve | reserve |
| 37 | reserve | reserve | 37 | reserve | reserve |
| 38 | reserve | reserve | 38 | reserve | reserve |
| 39 | reserve | reserve | 39 | reserve | reserve |
| 40 | reserve | reserve | 40 | reserve | reserve |
| 41 | reserve | reserve | 41 | reserve | reserve |
| 42 | reserve | reserve | 42 | reserve | reserve |
| 43 | reserve | reserve | 43 | reserve | reserve |
| 44 | reserve | reserve | 44 | reserve | reserve |
| 45 | reserve | reserve | 45 | reserve | reserve |
| 46 | reserve | reserve | 46 | reserve | reserve |
| 47 | reserve | reserve | 47 | reserve | reserve |
| 48 | reserve | reserve | 48 | reserve | reserve |
| 49 | reserve | reserve | 49 | reserve | reserve |
| 50 | reserve | reserve | 50 | reserve | reserve |
| 51 | reserve | reserve | 51 | reserve | reserve |
| 52 | reserve | reserve | 52 | reserve | reserve |
| 53 | reserve | reserve | 53 | reserve | reserve |
| 54 | reserve | reserve | 54 | reserve | reserve |
| 55 | reserve | reserve | 55 | reserve | reserve |
| 56 | reserve | reserve | 56 | reserve | reserve |
| 57 | reserve | reserve | 57 | reserve | reserve |
| 58 | reserve | reserve | 58 | reserve | reserve |
| 59 | reserve | reserve | 59 | reserve | reserve |
| 60 | reserve | reserve | 60 | reserve | reserve |
| 61 | reserve | reserve | 61 | reserve | reserve |
| 62 | reserve | reserve | 62 | reserve | reserve |
| 63 | reserve | reserve | 63 | reserve | reserve |

# Appendix D (Tables considered NCJT with multiple PDCCHs)

- Table 12: port combinations for NCJT (2 PDCCHs)

| One Codeword (<=4 layers): Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords (>4 layer): Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | UE Rank | port index | Value | UE Rank | port index |
| 0 | 1 | 0 | 0 | 5 | 0-4 |
| 1 | 1 | 1 | 1 | 6 | 0-5 |
| 2 | 1 | 2 | 2 | 7 | 0-6 |
| 3 | 1 | 3 | 3 | 8 | 0-7 |
| 4 | 1 | 4 | 4 | 6 | 2-7 |
| 5 | 1 | 5 | 5 | 6 | 2-5,7-8 |
| 6 | 1 | 6 | 6 | 5 | 2-5,7 |
| 7 | 1 | 7 | 7 | 7 | 2-5,7-8,10 |
| 8 | 1 | 8 | 8 | reserve | reserve |
| 9 | 1 | 9 | 9 | reserve | reserve |
| 10 | 1 | 10 | 10 | reserve | reserve |
| 11 | 1 | 11 | 11 | reserve | reserve |
| 12 | 2 | 0-1 | 12 | reserve | reserve |
| 13 | 2 | 2-3 | 13 | reserve | reserve |
| 14 | 2 | 4-5 | 14 | reserve | reserve |
| 15 | 2 | 6-7 | 15 | reserve | reserve |
| 16 | 2 | 8-9 | 16 | reserve | reserve |
| 17 | 2 | 10-11 | 17 | reserve | reserve |
| 18 | 3 | 0-2 | 18 | reserve | reserve |
| 19 | 3 | 3-5 | 19 | reserve | reserve |
| 20 | 3 | 6-8 | 20 | reserve | reserve |
| 21 | 3 | 9-11 | 21 | reserve | reserve |
| 22 | 4 | 0-3 | 22 | reserve | reserve |
| 23 | 4 | 4-7 | 23 | reserve | reserve |
| 24 | 4 | 8-11 | 24 | reserve | reserve |
| 25 | 2 | 7-8 | 25 | reserve | reserve |
| 26 | 2 | 6,9 | 26 | reserve | reserve |
| 27 | 3 | 7-8,10 | 27 | reserve | reserve |
| 28 | 3 | 0-1,6 | 28 | reserve | reserve |
| 29 | 4 | 2-5 | 29 | reserve | reserve |
| 30 | 4 | 7-8,10-11 | 30 | reserve | reserve |
| 31 | 4 | 0,1,6,9 | 31 | reserve | reserve |