3GPP TSG RAN WG1 Meeting NR#3			          R1-1715768

Nagoya, Japan, 18th – 21st, September 2017

Source: 	ETRI

Title: 	UCI multiplexing of different usage scenario

Agenda Item:	6.3.2.4	Other aspects for PUCCH

Document for:	Discussion

- Introduction

In previous meetings, RAN1 agreed the following. The two PUCCHs are allowed to be multiplexed in the same slot. In this contribution, we discuss the possible scenario that one UE should transmit two PUCCHs in the same slot. This is the revision of previous contribution R1-1713818.

Agreements: (#88) - Both TDM and FDM between short duration PUCCH and long duration PUCCH are supported at least for different UEs in one slot Agreements: (#88bis) - Two NR-PUCCHs can be transmitted from one UE on the same slot in TDM manner. - The two NR-PUCCHs can be short-PUCCH. - The two NR-PUCCHs can be long-PUCCH + short-PUCCH. - FFS whether or not to have the two NR-PUCCHs as long-PUCCH + long-PUCCH - FFS: other multiplexing scheme(s) between the two NR-PUCCHs - FFS the case of more than 2 NR-PUCCHs in one slot from a UE (if more than 2, only short-PUCCHs) Agreements: (#90) - It is up to RAN2 how many SR configurations the UE can be configured with. - In case of SR only, the physical layer can only transmit one SR at any given time - If multiple SR are triggered prioritization of which SR should be transmitted is decided by RAN2 - Multiplexing of SR and HARQ feedback is supported on short-PUCCH - Multiplexing of SR and HARQ feedback is supported on long-PUCCH - An SR can be configured with a periodicity of at least equal to X OFDM symbol(s) (at least for short-PUCCH), and with up to the largest periodicity supported in LTE (i.e. 80 ms) - Working assumptions: - X=1, which implies short-PUCCH could be located at any OFDM symbol of a slot - FFS: Supported periodicity values - FFS: Possible limitations due to other factors - One configured SR can be associated with either short or long PUCCH

- Discussion

RAN1 consider UCI from different usage scenario such as eMBB and LLC because a single UE can serve both eMBB and LLC traffic. This usage scenario may happen in 5G, e.g., a vehicle can enjoy streaming applications while communicating with other vehicles to keep safety distance. Since a UE may receive and/or transmit data for different usage scenario, collisions between PDSCHs and between PUSCHs would occur. However, the solution to the data collision seems clear because the priority from latency requirements is obvious. The LLC data as an aggressor precedes the eMBB data as a victim. This applies to both DL and UL, i.e., a UE is monitoring every PDCCH occasion for LLC in a slot to detect any LLC scheduling.

The gNB can give a single UE an eMBB PDSCH in a slot (i.e., K1>0) and an LLC PDSCH in a mini-slot (K1=0). Depending on K1, UE should handle PUCCH for both eMBB and LLC to carry UL HARQ-ACK bits. The Figure 1 illustrates an example for K1=4 for eMBB and K1=1 for LLC. A UE receives eMBB PDSCH at slot 0 and LLC PDSCH at mini-slots 1 and 3 in slot 3. Assuming that the UE attempts to transmit eMBB PUCCH at the slot 4, and LLC PUCCH at the mini-slot 5 and 7 in slot 4. In this case, the transmission of PUCCH at slot 4 should be defined.

Observation 1: The HARQ-ACK at least for LLC can be generated at any mini-slot.

Figure 1 HARQ-ACK timing example for eMBB and LLC

If the LLC PUCCH should be reported in the same UL slot, then there are at least two basic approaches regardless of whether UE may be aware of the presence of UL LLC transmission or not. The first approach is to transmit two PUCCHs independently in the same slot. It is applicable only for UEs in a good UL coverage.

The second approach is to merge/select HARQ-ACK into one PUCCH in the same UL slot, and is more applicable for UEs in a wide range of UL coverage. If UE is aware of the presence of all UL HARQ-ACK bits for LLC before encoding eMBB PUCCH in a given UL slot (i.e., slot 4), then UE can utilize a single format such as LTE PUCCH format 3/4/5 that conveys all HARQ-ACK bits including eMBB and LLC. Otherwise, which can be most cases, possibly LLC HARQ-ACK bits (i.e., mini-slot 3 in slot 3) are generated after encoding or even during transmitting eMBB PUCCH. The desirable UE behaviour is worth to study because the UL delay to wait for UL slot boundary will harm LLC latency performance.

As an alternative for the second above approach, we can delay to transmit eMBB PUCCH after LLC PUCCH, i.e., from K1 into K1+1. However, this approach does not work if consecutive LLC PUCCH can delay eMBB PUCCH indefinitely. The serving gNB should trigger retransmission of eMBB PUCCH. As another alternative, we can apply the UL puncturing. This UL puncturing means that eMBB PUCCH is not transmitted in symbol(s)/ mini-slot(s) in which LLC PUCCH is transmitted (TDM). Referring to Figure 1, eMBB PUCCH is said not to transmit at mini-slot 4 and 6 in slot 4. Instead, additional LLC PUCCH is transmitted at those mini-slot. By doing so, LLC feedback latency requirements can be met by sacrificing eMBB feedback latency. For the above two alternatives, the remedy for eMBB PUCCH needs further study.

Proposal 1: The short PUCCH carrying HARQ-ACK can be transmitted in any part of a slot.

Figure 2 TDM of PUCCHs based on puncturing

If punctured eMBB PUCCH and the puncturing LLC PUCCH do have different frequency resource, as shown in Figure 2, then each PUCCH should have own DM-RS resources. The DM-RS resource for puncturing PUCCH is an unexpected overhead compared to the case where punctured PUCCH and puncturing PUCCH share the same frequency resource. Since the front-loaded DM-RS resource will be applied, the eMBB PUCCH can transmit DM-RS resource before it is punctured. If the puncturing LLC PUCCH could exploit those transmitted DM-RS resource, then it will be helpful to decoding at the serving gNB. For example, LLC PUCCH can have less additional DM-RS overhead if two PUCCHs share the same frequency resource, which can decrease the amount of punctured REs in the eMBB PUCCH.

Observation 2: Additional DM-RS overhead should be minimized if UL LLC control channel and UL eMBB control channel are multiplexed.

Other possible alternative using reduced DM-RS overhead can be a UL resource selection which is adopted from the LTE CA. This approach is to transmit LLC PUCCH instead of eMBB PUCCH, or vice versa. The previous paragraph describes transmitting two PUCCHs based on puncturing, and this alternative transmits one merged PUCCH. Thus, this alternative does not require additional DM-RS overhead. The transmission resource is determined by the HARQ-ACK bits among the configured PUCCH resource pool. We can consider transmitting LLC PUCCH whose resource is chosen by the eMBB HARQ-ACK bits. The serving gNB should configure LLC PUCCH resources to the UE. If UE choose one of configured resource with respect to LLC HARQ-ACK bits, then it is the information to the serving gNB to determine eMBB HARQ-ACK bits.

Proposal 2: PUCCH multiplexing based on resource selection should be further studied.

- Conclusion

In this contribution, we observe the followings:

Observation 1: The HARQ-ACK at least for LLC can be generated at any mini-slot.

Observation 2: Additional DM-RS overhead should be minimized if UL LLC control channel and UL eMBB control channel are multiplexed.

Based on those observations, we made the following proposals:

Proposal 1: The short PUCCH carrying HARQ-ACK can be transmitted in any part of a slot.

Proposal 2: PUCCH multiplexing based on resource selection should be further studied.