3GPP TSG RAN WG1 NR Ad-Hoc#3                                      	R1-1716149

Nagoya, Japan, 18th – 21st, September 2017

Agenda Item:	6.1.1

Source:	Ericsson

Title:	Remaining details on synchronization signal design

Document for:	Discussion, decision

# Introduction

For the synchronization signal design, the following topics and working assumptions were agreed and/or FFS in RAN1#90:

Agreements:

- At least for rate matching purpose for serving cell, for below 6GHz, full bitmap (8bits) is used for indication of actual SS/PBCH block transmission

  - FFS: whether to support indication of actual SS/PBCH block transmission for mobility measurement at least for neighbour cells, how to indicate it, and whether this indication is applied to mobility measurement for serving cell

  - FFS: whether the indication can be used for the RACH preamble configurations and associations

Working assumptions:

- UE-specific RRC signaling with full bitmap can be used for indicating the actually transmitted SS blocks for both sub6GHz and over6GHz cases

- The actually transmitted SS blocks is indicated in RMSI for both sub6GHz and over6GHz cases

  - Indication is in compressed form in above 6 GHz case, and an indication method is down-selected from following alternatives

    - Alt.1: Group-Bitmap + Bitmap in Group

    - Alt.2: Group-Bitmap + The number of actually transmitted SS/PBCH block in Group (with fixed starting index of SS/PBCH block)

    - Alt.3: Bitmap in Group + The number of actually transmitted Groups (with fixed starting index of Group)

    - Alt.4: Group-Bitmap + The number of actually transmitted SS/PBCH block in each Group

    - Alt.5: The number of actually transmitted SS/PBCH blocks + starting index + gap between two consecutive SS/PBCH blocks

    - Alt.6: Group-Bitmap

    - Other alternatives are not precluded

- Indicated resources are reserved for actually transmitted SS blocks

  - Data channels are rate matched around actually transmitted SS blocks

A few remaining details for the synchronization signal design will be discussed below, including the actual transmitted SS blocks indication and multiple SS blocks transmission in the wideband CC.

# Discussion

## Actual transmitted SS blocks indication

If the indication of the actual transmitted SS blocks is delivered in RMSIError! Reference source not found. in above 6GHz case, it is necessary to consider the compression form. In RAN1#90 meeting, several alternatives have been proposed as the working assumptions, as illustrated in Figure 1.

(a) Alt.1: bit-mapping of groups and bit-mapping of blocks in a group

(b) Alt.2: bit-mapping of groups and consecutive SS blocks in a group

(c) Alt.3: Consecutive groups and bit-mapping of SS blocks in a group

(d) Alt.4: bit-mapping of groups and a number of consecutive SS blocks in a group

(e) Alt.5: Combination of starting point, gap and number indications

(f) Alt.6: bit-mapping of groups and all SS blocks used in a group

Figure 1 Illustration of the alternatives working assumptions of RAN1#90

In general, the more bits used would mean the more flexible selection on the SS block positions in a SS burst set. All alternatives, exclude Alt.5, define two-level indication, one for the group indication, the other for the block indication in a group. For Alt.1, the optimal number of groups and blocks in a group for the two-level indication to indicate 64 positions is 8, i.e., 8 groups and 8 blocks in a group, which needs total 16 bits. In this case, arbitrary SS block positions in a group can be indicated with full flexibility, and each group has the same transmission pattern.

Note that it maybe not necessary to have such flexible selection, i.e., bit-mapping, in either level, so Alt.2 and Alt.3 use consecutive SS blocks and groups, respectively, to save the bits in one level. If only one group is defined, the number of bits for Alt.2 is 6, and if using the same group size as Alt.1, the number is 11, which is less than Alt.1, but the cost is the restriction on the SS blocks position candidates and total numbers. To possibly support different number of consecutive SS blocks in different group, Alt.4 with more flexible selection in each group than Alt.2 is proposed, where the actual number and position candidates of the selected SS blocks in a SS burst set is larger than Alt.2, and the cost is the more bits, e.g., 32 bits.

As an extreme case of Alt.1, Alt.6 uses all SS blocks in a group, and only the bit-mapping of groups are indicated. In this case, the granularity of the punctured resource grid for the rate matching is slots-level, which may much reduce the flexibility within a group.

Alt.5 uses a combination of starting index, gap between two consecutive blocks and number of actually transmitted blocks to cover the all possible 64 positions in a SS burst set, which is much similar the resource block (RB) allocation indication in LTE DCI. Thus, we propose to re-use the same concept, e.g., SS block resource indication value (denoted as RIVSSB in following), to indicate the actual transmitted SS blocks from all 64 available SS block positions in the SS burst set as illustrated in Figure 2.

Figure 2 Proposed alternative with starting position and length indication

Assume the number of all possible candidates and selected SS block locations are L and LSS, respectively. The time location index for the first actual transmit SS blocks is tstart, the SSB-RIV can be derived as:

if (LSS-1)≤L/2, then

RIVSSB=LLSS-1+tstart

else

RIVSSB=LL-LSS-1+(L-1-tstart)

where LSS≥1, and shall not exceed (L-tstart).

In the receiver side, the start position and the length of the allocation can be derived from RIVSSB as:

a=RIVSSBL+1; b=RIVSSBmodL

if a+b>L, then

LSS=L+2-a; tstart=L-1-b;

else

LSS=a; tstart=b;

With the above method, log2L(L+1)2 bits are needed for any consecutive actual SS block transmissions with a given starting position. With one group, i.e., L=64, 12bits are needed. Note that, the distributed allocation with some gap between two consecutive SS block positions, like Alt.5, can also be supported.

Consider using the same the method as for indicating resource block allocation in LTE, e.g., resource allocation type 2, to indicate the actual transmitted SS block positions in RMSI in case of above 6GHz.

## Multiple SS blocks in wideband CC

At RAN1 #89 it was decided that NR should support single and multiple SS block transmissions in wideband CC in the frequency domain. The scenario of multiple SS blocks is illustrated in Figure 3.

Figure 3 Multiple frequency-separated SS blocks within a wideband carrier

At RAN1 #90 there were further discussions on multiple SS blocks in the frequency domain. This included a discussion on the relation between SS Blocks and the concept of a “cell”.

In LTE, a cell is associated with a PSS/SSS/PBCH carrying the physical cell identity (PCI). It is reasonable to assume something similar for NR. Especially, as also for NR the PSS/SSS is assumed to represent the physical cell identity, it seems highly reasonable to assume that SS blocks carrying different PCI (i.e. different physical cell identities) correspond to different cells.

SS blocks carrying different PCI correspond to different cells.

Note that this does not prevent multiple SS Blocks carrying different PCI within the bandwidth of a wideband carrier. However, those SS blocks would then correspond to different cells with overlapping carriers. In other words, the scenario of Figure 3 should, in this case, rather be illustrated as in Figure 4.

Figure 4 Multiple SS blocks within a wideband carrier bandwidth

What remains is then the question of multiple frequency-separated SS blocks carrying the same PCI and the relation to a cell.

Clearly different frequency-separated SS blocks carrying the same PCI may correspond to different cells.

- The different SS blocks may correspond to different frequency-separated cells as illustrated in the upper part of Figure 5.

- At least in principle, the different SS blocks may also correspond to different cells with wideband carriers that overlap in the frequency domain (lower part of Figure 5). Although this may seem like a somewhat strange scenario, it is clearly a possible scenario.

Figure 5

The question is then if multiple frequency-separated SS blocks carrying the same PC may also correspond to the same cell, something which may be indicated by the multiple-SSB scenario of Figure 3

However, this would imply that identical SS block arrangements (multiple frequency-separated SS blocks carrying the same PCI) could, in some cases, correspond to a single cell (like in Figure 3) and, in other cases, correspond to multiple cells (like in Figure 5), a situation that should preferable be avoided.

One way to avoid this ambiguity and still allow for multiple “SS blocks” within a wideband carrier would be to have two types of SS blocks, for simplicity here referred to as primary SS blocks and secondary SS blocks

- Within a cell there can be only one primary SS block in the frequency domain

- Within a cell there can be zero, one or multiple secondary SS blocks frequency multiplexed with the primary SS block

The secondary SS blocks are distinguishable from the primary SS Block either by using a different set of PSS/SSS sequences or by not being located on the same grid as the primary SS blocks. In this way the secondary SS block(s) cannot be found at initial cell search.

In case of frequency-multiplexed SS Blocks within a cell, only one SS block (the “primary SS block”) can be found at initial cell search.

In case of frequency-multiplexed SS Blocks within a cell, the remaining (“secondary”) SS Blocks cannot be found at initial cell search. This can be achieved either by using different PSS/SSS sequences or by the SS block being located outside of the defined sync grid.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Consider using the same the method as for indicating resource block allocation in LTE, e.g., resource allocation type 2, to indicate the actual transmitted SS block positions in RMSI in case of above 6GHz.

Proposal 2	SS blocks carrying different PCI correspond to different cells.

Proposal 3	In case of frequency-multiplexed SS Blocks within a cell, only one SS block (the “primary SS block”) can be found at initial cell search.

Proposal 4	In case of frequency-multiplexed SS Blocks within a cell, the remaining (“secondary”) SS Blocks cannot be found at initial cell search. This can be achieved either by using different PSS/SSS sequences or by the SS block being located outside of the defined sync grid.

# References

R1-1716151, “NR delivery of remaining minimum system information”, Ericsson, RAN1 NR Ad Hoc #3, Nagoya, Japan, September 18th – 21st, 2017

The time-multiplexed SS blocks within an SS burst set (which always has the same PCI), clearly correspond to the same cell.