3GPP TSG RAN WG1 Meeting AH 1801     	                                                R1-1800230

Vancouver, Canada, January 22nd – 26th, 2018

Source:	CATT

Title:	Summary of Offline Discussion on RMSI

Agenda Item:	7.1.2.2

Document for:	Discussion and Decision

# Introduction

This purpose of the document is to provide a summary on the remaining key issues and the proposed solutions on AI 7.1.2.2 “Remaining minimum system information” for facilitating the continue discussion in RAN1#AH1801 based on the contributions submitted under this AI [2-17].

In this document, the discussion of the remaining issues is organized as follows:

- Section 2: Text proposal for the corrections/clarification of the RMSI CORESET configuration based on existing agreements (no change on the existing agreements)

- Section 3: Proposals for corrections/clarification of the RMSI CORESET configuration that may require changing the existing agreements or adding new requirements;

- Section 4: Proposals specific on the design of RMSI PDCCH

- Section 5: Proposals specific on RMSI PDSCH

- Appendix A: Suggested offline agreements for Text Proposals for Section 2 (an separate attachment)

# Text corrections based on existing agreements

## Text Correction 1: PRB grid offset for RMSI CORESET

Background

Agreements (RAN1#91)

From RAN1, RMSI is assumed to be always PRB-aligned with PRB grid. However, the current 4-bit PRB grid offset in PBCH with 15kHz SCS can’t ensure the above assumption when RMSI has 30kHz SCS. Therefore, for FR1, RAN1 agrees to increase from 4-bit PRB grid offset to 5-bit PRB grid offset in PBCH where the 5-bit PRB gird offset in PBCH is in unit of subcarrier based on 15kHz SCS, while for FR2, there is still 4-bit PRB grid offset and RAN1 assumes the 4-bit PRB grid offset in PBCH is in unit of subcarrier based on RMSI numerology.

Submitted Proposals:

========= (R1-1800030) Text Proposal (TS 38.211) =================

7.4.3	SS/PBCH block

7.4.3.1	Time-frequency structure of an SS/PBCH block

In the time domain, an SS/PBCH block consists of 4 OFDM symbols, numbered in increasing order from 0 to 3 within the SS/PBCH block, where PSS, SSS, and PBCH with associated DM-RS occupy different symbols as given by Table 7.4.3.1-1.

In the frequency domain, an SS/PBCH block consists of 240 contiguous subcarriers with the subcarriers numbered in increasing order from 0 to 239 within the SS/PBCH block. The quantities  and  represent the frequency and time indices, respectively, within one SS/PBCH block. The UE may assume resource elements denoted as ‘Set to 0’  in Table 7.4.3.1-1 are set to zero.  is the subcarrier offset from subcarrier 0 in an SS/PBCH block to the subcarrier 0 in common resource block  , where  is obtained from the higher-layer parameter offset-ref-low-scs-ref-PRB.

Any common resource block partially or fully overlapping with an SS/PBCH block shall be viewed as occupied and not used for transmission of PDSCH or PDCCH. The UE may assume resource elements not used for SS/PBCH transmission but part of a partially overlapping common resource to be set to zero.

For an SS/PBCH block, the UE shall assume

-	antenna port ,

-	the same cyclic prefix length and subcarrier spacing for the PSS, SSS, and PBCH,

-	for SS/PBCH block type A, , , and  is expressed in terms of 15 kHz subcarrier spacing, the subcarrier 0 in an SS/PBCH block corresponds to subcarrier  in common resource block in terms of 15kHz, and subcarrier 0 in common resource block coincides with subcarrier 0 in common resource block  of subcarrier spacing  where  is indicated by higher layer parameter RMSI-scs, and

-	for SS/PBCH block type B, ,  is expressed in terms of the subcarrier spacing  indicated by higher layer parameter RMSI-scs, and  is expressed in terms of 60 kHz subcarrier spacing. The subcarrier 0 in an SS/PBCH block corresponds to subcarrier  in common resource block in terms of 60kHz.

The UE may assume that SS/PBCH blocks transmitted with the same block index are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters. The UE shall not assume quasi co-location for any other SS/PBCH block transmissions.

========= (R1-1800414) Text Proposal (TS 38.211) =================

7.4.3	SS/PBCH block

7.4.3.1	Time-frequency structure of an SS/PBCH block

In the time domain, an SS/PBCH block consists of 4 OFDM symbols, numbered in increasing order from 0 to 3 within the SS/PBCH block, where PSS, SSS, and PBCH with associated DM-RS occupy different symbols as given by Table 7.4.3.1-1.

In the frequency domain, an SS/PBCH block consists of 240 contiguous subcarriers with the subcarriers numbered in increasing order from 0 to 239 within the SS/PBCH block. The quantities  and  represent the frequency and time indices, respectively, within one SS/PBCH block. The UE may assume resource elements denoted as 'Set to 0' in Table 7.4.3.1-1 are set to zero. Subcarrier 0 in an SS/PBCH block corresponds to subcarrier  in common resource block , where  is obtained from the higher-layer parameter offset-ref-low-scs-ref-PRB, and is obtained from the higher-layer parameter SSB-subcarrier-offset.

========= (R1-1800414) Text Proposal (TS 38.213) =================

13 UE procedure for monitoring Type0-PDCCH common search space

== UNCHANGED TEXT OMMITED ==

The offset in Tables 13-1 through 13-8 is defined with respect to the subcarrier spacing of the control resource set and is the difference between the smallest RB index of the SS/PBCH block and the smallest RB index of the control resource set for Type0-PDCCH common search space. Condition A or condition B in Tables 13-1 through 13-8 corresponds to the case of PRG [6, TS 38.214] alignment or non-alignment, respectively, between SS/PBCH block RBs and RBs of the control resource set for Type0-PDCCH common search space the case of the higher-layer parameter SSB-subcarrier-offset [4, TS 38.211] equal to 0 or not, respectively.

========= (R1-1800848) Text Proposal (TS 38.213) =================

13 UE procedure for monitoring Type0-PDCCH common search space

== UNCHANGED TEXT OMMITED ==

The offset in Tables 13-1 through 13-8 is defined with respect to the subcarrier spacing of the control resource set and is the difference between the smallest RB index of the SS/PBCH block and the smallest RB index of the control resource set for Type0-PDCCH common search space. Condition A or condition B in Tables 13-1 through 13-8 corresponds to the case of PRB [6, TS 38.214] alignment or non-alignment, respectively, between SS/PBCH block RBs and RBs of the control resource set for Type0-PDCCH common search space.

========= (R1-1800275)  Text Proposal (TS 38.213) =================

13 UE procedure for monitoring Type0-PDCCH common search space

== UNCHANGED TEXT OMMITED ==

The offset in Tables 13-1 through 13-8 is defined with respect to the subcarrier spacing of the control resource set and is the difference from the smallest RB index of the control resource set for Type0-PDCCH common search space to the smallest RB index of the SS/PBCH block. Condition A or condition B in Tables 13-1 through 13-8 corresponds to the case of PRG [6, TS 38.214] alignment or non-alignment, respectively, between SS/PBCH block RBs and RBs of the control resource set for Type0-PDCCH common search space.

========= (R1-1800174)  Text Proposal (TS 38.213) =================

13 UE procedure for monitoring Type0-PDCCH common search space

A UE determines a number of consecutive resource blocks and a number of consecutive symbols for the control resource set of Type0-PDCCH common search space from the first four bits of RMSI-PDCCH-Config as described in Tables 13-1 through 13-8 and determines PDCCH monitoring occasions from the second four bits of RMSI-PDCCH-Config as described in Tables 13-9 through 13-13 where  and  are the SFN and slot of the control resource set and  and  are the SFN and slot of the SS/PBCH block, respectively.   Both  and  are based on the same subcarrier spacing and same cyclic shift as for the reception of PDCCH Type0-PDCCH common search space.

The offset in Tables 13-1 through 13-8 is defined with respect to the subcarrier spacing of the control resource set and is the difference between the smallest RB index of the SS/PBCH block and the smallest RB index of the control resource set for Type0-PDCCH common search space. Condition A or condition B in Tables 13-1 through 13-8 corresponds to the case of PRG [6, TS 38.214] alignment or non-alignment, respectively, between SS/PBCH block RBs and RBs of the control resource set for Type0-PDCCH common search space.

For a first SS/PBCH block and control resource set (CORESET) multiplexing pattern, a UE monitors PDCCH in the Type0-PDCCH common search space over two consecutive slots . For SS/PBCH block with index , the UE determines an index of the first slot  as  located in a frame with system frame number (SFN) satisfying  if  or in a frame with SFN satisfying  if .

For a second SS/PBCH block and control resource set (CORESET) multiplexing pattern, a UE monitors PDCCH in the Type0-PDCCH common search space over slot which is given by Table 13-11 and 13-12.

For a third SS/PBCH block and control resource set (CORESET) multiplexing pattern, a UE monitors PDCCH in the Type0-PDCCH common search space over slot which is given by Table 13-13

========= (R1-1800298)  Text Proposal (TS 38.213) =================

13 UE procedure for monitoring Type0-PDCCH common search space

A UE determines a number of consecutive resource blocks and a number of consecutive symbols for the control resource set of Type0-PDCCH common search space from the first four bits of RMSI-PDCCH-Config as described in Tables 13-1 through 13-8 and determines PDCCH monitoring occasions from the second four bits of RMSI-PDCCH-Config as described in Tables 13-9 through 13-13 where  and  are the SFN and slot of the control resource set based on subcarrier spacing of the control resource set and  and  are the SFN and slot of the SS/PBCH block with index i based on subcarrier spacing of SS/PBCH block, respectively.

The offset in Tables 13-1 through 13-8 is defined with respect to the subcarrier spacing of the control resource set and is the difference between the smallest RB index of the SS/PBCH block and the smallest RB index of the control resource set for Type0-PDCCH common search space. Condition A or condition B in Tables 13-1 through 13-8 corresponds to the case of PRG [6, TS 38.214] alignment or non-alignment, respectively, between SS/PBCH block RBs and RBs of the control resource set for Type0-PDCCH common search space.

For the SS/PBCH block and control resource set (CORESET) multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH common search space over two consecutive slots  with PDCCH monitoring periodicity of  slots for a SS/PBCH block with index i.  For SS/PBCH block with index , the UE determines an index of the first slot , which corresponds to the PDCCH monitoring offset of , as  located in a frame with system frame number, (SFN), , satisfying  if , or in a frame with satisfying  if , where parameter M, O are provided by Tables 13-9 and 13-10. The UE also determines the an index of the first symbol(s) of the control resource set within slot , by the first symbol index provided by Table Tables 13-9 and 13-10.

For the SS/PBCH block and control resource set (CORESET) multiplexing pattern 2 and 3, a UE monitors PDCCH in the Type0-PDCCH common search space over one slot, , with PDCCH monitoring periodicity of  slots equal to the periodicity of SS/PBCH block, for a SS/PBCH block with index i.  For SS/PBCH block with index , the UE determines an index of the slot  and SFN  based on parameter provided by Tables 13-11 through 13-13.

========= (R1-1800894)  Text Proposal (TS 38.213) =================

# 13	UE procedure for monitoring Type0-PDCCH common search space

A UE determines a number of consecutive resource blocks and a number of consecutive symbols for the control resource set of Type0-PDCCH common search space from the first four bits of RMSI-PDCCH-Config as described in Tables 13-1 through 13-8 and determines PDCCH monitoring occasions from the second four bits of RMSI-PDCCH-Config as described in Tables 13-9 through 13-13 where  and  are the SFN and slot of the control resource set and  and  are the SFN and slot of the SS/PBCH block, respectively.

The offset in Tables 13-1 through 13-8 is defined with respect to the subcarrier spacing of the control resource set and is the difference between the smallest RB index of the SS/PBCH block and the smallest RB index of the control resource set for Type0-PDCCH common search space. Condition A or condition B in Tables 13-1 through 13-8 corresponds to the case of PRG [6, TS 38.214] alignment or non-alignment, respectively, between SS/PBCH block RBs and RBs of the control resource set for Type0-PDCCH common search space.

For a first SS/PBCH block and control resource set (CORESET) multiplexing pattern, a UE monitors PDCCH in the Type0-PDCCH common search space over two consecutive slots . For SS/PBCH block with index , the UE determines an index of the first slot  as  located in a frame with system frame number (SFN) satisfying  if  or in a frame with SFN satisfying  if where  depending on the numerology of RMSI and defined in table 4.2-1 in [4, TS 38.211].

Suggested Offline Agreements:

- See the attached Appendix A.

## Text Correction 2: The first symbol index in Table 13-12

Background

There are some simple mistakes in Table 13-12 for the indication of the first symbol index.

Submitted Proposals:

A number of contributions have included text proposals for the corrections (e.g., R1-1800080, R1-1800174, R1-1800229, R1-1800298, R1-1800399).

Suggested Offline Agreements:

- See the attached Appendix A.

## Text Correction 3: Editorial mistakes in Table 13-6

Background

There some editorial mistakes in Table 13-6 in TS 38.213 when the table was ported from Chairman’s Note into the specification.

Submitted Proposals:

A number of contributions have included the text proposals for the corrections (e.g.,  R1-1800150, R1-1800174, R1-1800229, R1-1800298, R1-1800848, ).

Suggested Offline Agreements:

- See the attached Appendix A.

## Add missing configuration combination {120kHz, 120kHz} for Pattern 2:

Background

For Pattern 2, the combination {120kHz, 120kHz} is missing.

Submitted Proposals:

Proposal from R1-1800298

- Modify Table 13-6 and Add new table 13-14.

Table 13-6: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {120, 120} kHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 2 | 14 |
| 4 | 3 | 48 | 2 | -20 if condition A -21 if condition B |
| 5 | 3 | 48 | 2 | 24 |
| 6 | 3 | 96 | 2 | -20 if condition A -21 if condition B |
| 7 | 3 | 96 | 2 | 48 |
| 8 | 2 | 48 | 2 | -20 if condition A -21 if condition B |
| 9 | 2 | 48 | 2 | 24 |
| 10 | 2 | 96 | 2 | -20 if condition A -21 if condition B |
| 11 | 2 | 96 | 2 | 48 |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-14: PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing type 2 and {SS/PBCH block, PDCCH} subcarrier spacing {120, 120} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, … 15) |
| --- | --- | --- |
| 0 | or | 0, 2 in , , (with) 12, in (with ) 0 in (with ) |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

Suggested Offline Agreements:

- See the attached Appendix A.

## Other Text Proposals for editorial corrections for TS 38.213

Submitted Proposals:

============== (R1-1800150)  Text Proposal (TS 38.213) =================

## 10.1	UE procedure for determining physical downlink control channel assignment

…

A UE is provided a configuration for a control resource set for Type0-PDCCH common search space by higher layer parameter RMSI-PDCCH-Config and a subcarrier spacing by higher layer parameter RMSI-scs for PDCCH reception. The UE determines the control resource set and the monitoring occasions for Type0-PDCCH common search space as described in Subclause 13. The Type0-PDCCH common search space is defined by the CCE aggregation levels and the number of candidates per CCE aggregation level given in Table 10.1-1.

…

For each control resource set in a DL BWP of a serving cell, a respective higher layer parameter CORESET-freq-dom provides a bitmap. The bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  PRBs with the starting position  where the first PRB of the first group of 6 PRBs has index .

============== END of Text Proposal (TS 38.213) =================

============== (R1-1800174) Text Proposal (TS 38.213) =================

Table 13-10: Parameters for PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing type 1 and carrier frequencies above 6 GHz

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 2 | 2.5 | 1 | 1 | 0 |
| 3 | 2.5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 6 | 0 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 7 | 2.5 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 8 | 5 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 9 | 7.5 | 1 | 1 | 0 |
| 10 | 7.5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 11 | 7.5 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 12 | 0 | 1 | 2 | 0 |
| 13 | 5 | 1 | 2 | 0 |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

============== END of Text Proposal (TS 38.213) =================

============== (R1-1800080) Text Proposal (TS 38.213) =================

11.1	Slot configuration

......

For a set of symbols of a slot that are indicated as uplink by higher layer parameter UL-DL-configuration-common or, when provided, by higher layer parameter UL-DL-configuration-dedicated, the UE is not expected to be indicated by a DCI format with CRC scrambled by C-RNTI or be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS or SS/PBCH block in the set of symbols of the slot.

============== END of Text Proposal (TS 38.213) =================

============== (R1-1800298) Text Proposal (TS 38.213) =================

Table 13-11: PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing type 2 and {SS/PBCH block, PDCCH} subcarrier spacing {120, 60} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, … 15) |
| --- | --- | --- |
| 0 |  | 0, 1, 6, 7 for , , , |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

Table 13-12: PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing type 2 and {SS/PBCH block, PDCCH} subcarrier spacing {240, 120} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, … 15) |
| --- | --- | --- |
| 0 | or | 0, 1, 2, 3 in , , , (with) 12, 13 in, , (with ) 0, 1 in , (with ) |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

Table 13-13: PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing type 3 and {SS/PBCH block, PDCCH} subcarrier spacing {120, 120} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, … 15) |
| --- | --- | --- |
| 0 |  | 4, 8, 2, 6 in , , , |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

Suggested Offline Agreements:

- See the attached Appendix A.

## Effort in reducing the number of the tables in TS 38.213

Background

In TS 38.213, there are 13 tables for the RMSI CORESET configurations.

Submitted Proposals:

Proposal from R1-1800174

- It was proposed to reduce the number of RMSI CORESET configuration tables by integrating them in to 3 (See R1-1800174 for details of the TPs.

Suggested Offline Conclusion:

- It may be too early to consolidate the tables.

Corrections on current agreements

## Pattern 1: SFN Index for Type0-PDCCH common search space

Background

For the following working assumption was made in RAN1#91,

Working assumption:

- When the SS/PBCH blocks and corresponding RMSI search space sets occur for pattern 1 NR supports the configuration where the RMSI CORESET monitoring window associated with the SS/PBCH block SSB_i in a burst set is defined as follows:

  - The RMSI CORESET monitoring window duration with RMSI numerology is 2 slots;

  - The first slot index of the monitoring window of SSB_i

    - The first slot index S with RMSI numerology for a monitoring window of SSB_i, with respect to the SFN boundary, is determined by

      - S = mod(X*n +  f(i), number of slots per frame in RMSI numerology)

  - SFN index:

    - When floor((X*n +  f(i))/(number of slots per frame)) = 0, the SFN to carry the RMSI monitoring window is determined by mod(SFN,2)=0

    - When floor((X*n +  f(i))/(number of slots per frame)) = 1, the SFN to carry the RMSI monitoring window is determined by mod(SFN-1,2)=0

    - Note that this is an update of the previous agreements: mod(SFN,2)=0 applies to all the CORESETs

The above WA was captured in TS 38.213 as “the Type0-PDCCH common search space has two consecutive slots . For SS/PBCH block with index , the UE determines an index of the first slot  as  located in a frame with system frame number (SFN) satisfying  if  or in a frame with SFN satisfying if .

The issues with above definition are that

- It defines the is located in a frame with SFN satisfying or  But, there are SFNs that satisfying SFN mod 2 =0 or 1 for every two frames. It still needs to clarify what there the frame is located by referring to the SFN of the SSB;

- The definition only considers the situation when  ={0, 1}. It needs to consider the situation when   > 1;

Submitted Proposals:

========= (R1-1800030) Text Proposal (TS 38.213) =================

For a first SS/PBCH block and control resource set (CORESET) multiplexing pattern, a UE monitors PDCCH in the Type0-PDCCH common search space over two consecutive slots . For SS/PBCH block with index , the UE determines an index of the first slot  as  located in a frame with system frame number (SFN) satisfying  if , where

- , if

- , if

- is the maximum number of SSB/PBCH blocks

========= (R1-1800407) Text Proposal 1   (TS 38.213) =================

For a first SS/PBCH block and control resource set (CORESET) multiplexing pattern, a UE monitors PDCCH in the Type0-PDCCH common search space over two consecutive slots nj, j=0, 1n0, n0+1. For SS/PBCH block with index , the UE determines indices of the slots nj, j=0, 1 n0 as nj=O∙2μ+i∙M+jmodNslotframe, μ, j=0, 1 n0=O∙2μ+i∙Mmod Nslotframe, μ located in a frame with system frame number (SFN) satisfying SFNC=SFNSSB+O∙2μ+i∙M+jNslotframe, μ, j=0, 1 SFNmod 2=0O∙2μ+i∙MNslotframe, μ=0SFNmod 2=1 O∙2μ+i∙MNslotframe, μ=1.

========= (R1-1800407) Text Proposal 2 with the consideration of load balancing  (TS 38.213) =================

For a first SS/PBCH block and control resource set (CORESET) multiplexing pattern, a UE monitors PDCCH in the Type0-PDCCH common search space over two consecutive slots nj, j=0, 1n0, n0+1. For SS/PBCH block with index , the UE determines indices of the slots nj, j=0, 1n0 as nj=O∙2μ+i2∙M+jmodNslotframe, μ, j=0, 1n0=O∙2μ+i∙Mmod Nslotframe, μ located in a frame with system frame number (SFN) satisfying SFNC=SFNSSB+O∙2μ+i2∙M+jNslotframe, μ, j=0, 1SFNmod 2=0 if i mod 2=0O∙2μ+i∙MNslotframe, μ=0 or in a frame with SFN satisfying SFNC=SFNSSB+2+O∙2μ+i2∙M+jNslotframe, μ, j=0, 1SFNmod 2=1 if i mod 2=1O∙2μ+i∙MNslotframe, μ=1.

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | It seems much clean if is defined relative to SFNSSB as suggested in R1-1800407. For the two PDCCH slots, we may change n0, n0+1 to. It should be clear enough by just saying are two consecutive slots, and no need to provide the exact slot number for . |

Proposals:

For the SS/PBCH block and control resource set (CORESET) multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH common search space over two consecutive slots  based on subcarrier spacing of the control resource set. For SS/PBCH block with index  in a frame with system frame number SFNSSB, the UE determines an index of the first slot  as  located in a frame with system frame number  satisfying SFNc=SFNSSB+(O ∙2μ+i∙M)/Nslotframe,μwhere parameter M, O are provided by Tables 13-9 and 13-10, depending on the numerology of RMSI and defined in table 4.2-1 in [4, TS 38.211]. The UE also determines the index of the first symbol(s) of the control resource set within slot , by the first symbol index provided by Table Tables 13-9 and 13-10.

Suggested Offline Agreements:

## Collision between RMSI CORESET and the SSB: Pattern 1

Background

For SS/PBCH block and RMSI CORESET multiplexing pattern 1, there are collisions between RMSI CORESET and the SSBs as pointed out in  the contributions, e.g., in R1-1800080, R1-1800174, R1-1800351, R1-1800651 and R1-1800804.

Submitted Proposals:

- Proposal from R1-1800080:

  - Proposal 1: The group offset ‘O’ is not explicitly indicated in the PDCCH-Config index. Instead, the offset is derived from the RMSI-PDCCH-Config and the half-frame (within 20 ms) in which the UE has detected an SSB.

  - Proposal 2: The RMSI PDCCH rate matching assumption around potential SS/PBCH blocks should depend on half-frame(s) in which the UE has detected SSB(s) and the value of RMSI-PDCCH-Config

- Proposal from R1-1800174:

  - Proposal 1: For RMSI PDCCH, UE assumes resources within RMSI CORESET that partially or fully overlapped with candidate SS block resources are not used for RMSI PDCCH transmission.

- Proposal from R1-1800351:

The parameters in TS 38.213 V1.3.0 Table 13-9, 13-10, 13-11, 13-12, and 13-13 should be changed.

Two values of M (i.e. 1/2 and 1) are supported. It is not support that M is 2.

The set of value of O should be changed to {0, 2, 10, 12} for FR1 and {0, 2.5, 10, 12.5} for FR2.

It should be supported that RMSI CORESET is transmitted by alternating even index beam and odd index beam every 20ms.

- Proposal from R1-1800407:

Proposal 1: A part of PDCCH for RMSI should be initialized by SS/PBCH block index.

- Proposal from R1-1800651:

  - Proposal 1: For NW scheduling flexibility, the configuration in which RBs for Type0-PDCCH indicated in NR-PBCH are collided with the possible time/frequency resource location of SSB(s) should be allowed. In such case, UE should assume following aspects.

  - When there is at least one symbol non-overlapped with SS/PBCH block(s) in a CORESET for Type0-PDCCH CSS and/or the total number of RBs non-overlapped with SS/PBCH block(s) per symbol for Type0-PDCCH CSS is equal to or larger than X RBs, UE assumes that Type0-PDCCH CSS is transmitted on RBs non-overlapped with SS/PBCH block.

  - When all symbols in a CORESET for Type0-PDCCH CSS are overlapped with SS/PBCH block(s) and the total number of RBs non-overlapped with SS/PBCH block(s) per symbol for Type0-PDCCH CSS is smaller than X RBs, UE assumes that SS/PBCH block(s) are not transmitted in the certain symbol.

- Proposals from R1-1800804

  - Proposal: For a scheduled PDSCH carrying RMSI that overlaps SS block positions in the cell, similarly as for forward compatible resources, L1 signaling indicates whether the scheduled PDSCH is rate-matched around the SS block positions or not. For initial access purposes, UE would treat the SS block locations as forward compatible resources in terms of PDSCH rate matching.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

Proposals:

TBD

Suggested Offline Agreements:

- TBD

## Collision between RMSI CORESET and the SSB: Pattern 2

Background

It was agreed in RAN1#91 as working assumption to support the CORESET configuration with the duration of 2 OFDM symbols for Patter 2, as shown in Table 13-5 and Table 13-8 of TS 38.213. However, for Patter 2 only two OFDM symbols are available in front of every two consecutive SS/PBCH blocks. Thus, there will be overlapping of the RMSI OCRESETs for the two consecutive SS/PBCH blocks.

Figure 2: Illustration of the RMSI CORESET monitoring window for Pattern 2

Submitted Proposals:

- Proposal from R1-1800030:

  - For SSB and RMSI CORESET multiplexing pattern 2, the configurations with =2 in Table 13-5 and Table 13-8 of TS 38.213 should be removed to avoid the collision of RMSI CORESETs associated with the adjacent SSBs. And 24PRB configuration with =1 should be added in Table 13-5 of TS 38.213.

- Proposal from R1-1800080:

  - NR should support RMSI CORESETs for two continuous SS/PBCH blocks overlapping with each other.

- Proposal from R1-1800651:

  - Proposal 1: For NW scheduling flexibility, the configuration in which RBs for Type0-PDCCH indicated in NR-PBCH are collided with the possible time/frequency resource location of SSB(s) should be allowed. In such case, UE should assume following aspects.

  - When there is at least one symbol non-overlapped with SS/PBCH block(s) in a CORESET for Type0-PDCCH CSS and/or the total number of RBs non-overlapped with SS/PBCH block(s) per symbol for Type0-PDCCH CSS is equal to or larger than X RBs, UE assumes that Type0-PDCCH CSS is transmitted on RBs non-overlapped with SS/PBCH block.

  - When all symbols in a CORESET for Type0-PDCCH CSS are overlapped with SS/PBCH block(s) and the total number of RBs non-overlapped with SS/PBCH block(s) per symbol for Type0-PDCCH CSS is smaller than X RBs, UE assumes that SS/PBCH block(s) are not transmitted in the certain symbol.

- Proposal from R1-1800174:

  - It is usually up to gNB'ss implementation how PDCCH resource of these two CORESETs are mapped on the overlapped symbols, and no extra UE behavior need to be specified.

- Text from R1-1800298:

In case of SSB and RMSI SCS of {240kHz, 120kHz}, it would not be possible to support 2 OFDM symbol duration CORESET configuration. Therefore, entries need to be removed.

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | While the configurations with =2 in Table 13-5 and Table 13-8 of TS 38.213 may cause the collision of RMSI CORESETs with the adjacent SSBs if the adjacent SSBs are transmitted, the network may avoid the collision by not transmitting the SSBs at the locations that can create collision. So, I assume we may take one of the two options: a) remove the configuration; b) keep the configuration, and it is up to the network to avoid the collision. From UE’s point of view, the UE will assume no collision at the RMSI CORESET locations associated with a detected CD-SSB. |

Proposals:

- Down-select one of the following options for dealing with the potential collision between an SSB and the RMSI CORESET in multiplexing pattern 2:

  - Option 1: Remove the configurations with =2 in Table 13-5 and Table 13-8 of TS 38.213;

- Option 2: Keep the configurations with =2 in Table 13-5 and Table 13-8 of TS 38.213.  It will be up to the network’s implementation on how to avoid the collision. From UE’s perspective, it is assumed no collision between an SSB and the RMSI CORESET of a CD-SSB.

Suggested Offline Agreements:

- TBD

## Modification of the configuration for RMSI CORESET based on new SS Raster definitions

Background

In RNA1#91, the configuration for the control resource set of Type0-PDCCH common search space was developed assuming the same SS Raster definition for all bands under 6GHz. The configuration was first developed for the minimum channel bandwidth of 5MHz, and then directly extended to 10MHz and 20MHz channel bandwidth without the consideration of different SS Raster definitions with different minimum channel bandwidths and different frequency ranges under 6GHz. According the RAN4’LS [R1-1800008], however, different SS Raster definitions are used for frequency ranges [0 – 2.65GHz] and [2.4 – 6GHz]. Also, the SS Raster intervals are different for the bands with different minimum channel bandwidth of 5MHz and 10MHz due to the down-selection factors for minimum channel bandwidth of 10MHz as shown in the following table.

Table 3-2 Parameter settings related to minimum bandwidth and SS Raster Interval [R1-1800229]

| Frequency Range | SSB SCS (kHz) | Minimum channel bandwidth (MHz) | SS Raster interval (MHz) | SS Raster interval (SSB RBs) |
| --- | --- | --- | --- | --- |
| [0 – 2.65GHz] | 15 | 5 | 0.900 | 5 |
|  |  | 10 | 1.8 | 10 |
|  | 30 | 10 | 1.8 (Note 1) | 5 (Note 1) |
| [2.4 – 6GHz] | 15 | 5 | 1.44 | 8 |
|  |  | 10 | 4.32 | 24 |
|  |  | 40 | 4.32 (Note 2) | 12 (Note 2) |
|  | 30 | 10 | 4.32 (Note 1) | 12 (Note 1) |
|  |  | 40 | 30.24 | 84 |
| Above-24.25GHz | 120 | 50 | 17.28 | 12 |
|  | 240 | 100 | 34.56 | 12 |

Submitted Proposals:

The following proposal was made in R1-1800351

The current RMSI CORESET configuration tables should be modified.

Separate configuration table is necessary to configure frequency position of RMSI CORESET when sync raster is large.

The configuration table for the offset for the band which has large sync raster is proposed as following Table 2:

Table 2. Candidate offset for the band which has large sync raster

| Frequency Range | SCS of SS/PBCH block | min CH BW | Sync Raster | Configuration |
| --- | --- | --- | --- | --- |
| 0~2.65 GHz | 15 kHz | 10 MHz | 5400 kHz (30 RBs) | Depicted in Table 3 and Table 4 |
| 2.4~24.5 GHz | 15 kHz | 10 MHz | 4320 kHz (24 RBs) | Depicted in Table 3 and Table 4 |
|  | 30 kHz | 40 MHz | 30240 kHz (84 RBs) | Depicted in Table 5 and Table 6 |

The following proposal was made in R1-1800229

Separate configuration tables are developed for frequency ranges [0 – 2.65GHz] and [2.4 – 6GHz] due to different SS Raster definitions;

- Propose modifying Tables 13-1, 13-2, 13-3, 13-4  in TS 38.213 to support frequency offset configurations for both minimum channel bandwidth of 5MHz and 10MHz in frequency range [0 – 2.65GHz]

- Propose adding new Tables 13-x1, 13-x2, 13-x3, 13-x4 to support frequency offset configurations for both minimum channel bandwidth of 5MHz and 10MHz in frequency range [2.4 – 6GHz] except minimum channel bandwidth 40MHz.

- Propose adding separate table specific for except the bands with {SSB SCS, PDCCH SCS}={30, 30} kHz and minimum channel bandwidth 40MHz due to extremely large SS Raster interval of 30.24 MHz.

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | Obviously, the issues introduced by different SS Raster definitions for different frequency ranges and different minimum channel bandwidth need to be addressed. There are different ways to handle the problem. To minimize the impact on existing agreements, our opinion is to have separate configuration tables. |

Proposals:

Separate configuration table is necessary to configure frequency position of RMSI CORESET.  For example, separate configuration tables are developed for frequency ranges [0 – 2.65GHz] and [2.4 – 6GHz] due to different SS Raster definitions;

- The current RMSI CORESET configuration Tables 13-1, 13-2, 13-3, 13-4  are modified support frequency offset configurations for frequency range [0 – 2.65GHz]

- Propose adding new Tables 13-x1, 13-x2, 13-x3, 13-x4 to support frequency offset configurations for frequency range [2.4 – 6GHz] except minimum channel bandwidth 40MHz.

- Propose adding separate table specific for except the bands with {SSB SCS, PDCCH SCS}={30, 30} kHz and minimum channel bandwidth 40MHz due to extremely large SS Raster interval of 30.24 MHz.

Suggested Offline Agreements:

- TBD

## Method for determining the RMSI CORESET frequency offset

Background

In RNA1#91, significant effort was spent on determining the RMSI CORESET frequency offsets for different RMSI COORESET configurations by providing different examples for different configurations (see e.g., R1-1721629). Given that different SS Raster definitions are used for different frequency ranges with different down-selection factors, it would be desirable to define clearly how the RMSI CORESET frequency offsets are derived for consistency and avoiding mistakes based on the information of the SSB SCS, RMSI SCS and bandwidth, SS Raster intervals, and the channel bandwidth etc.

Submitted Proposals:

The following method was proposed in R1-1800229 for the determination of the RMSI CORESET frequency offsets

- In RMSI CORESET configuration for a given {SSB SCS, RMSI CORESET SCS} combination, the frequency offset O={O0,…,OP-1} , where P  is the minimum number of the frequency offsets, between the SSB and the associated RMSI CORESET for a particular bandwidth, can be determined as follows:

ΔSSB=floor(NBW-NRMSI+1*2-μ)

N=min⁡(NSync, NSSB-19)

P=ceiling (NΔSSB)

ΔRMSI=ΔSSB*2μ，          if ΔSSB≤NRMSI*2-μ-20NRMSI-20*2μ,    if ΔSSB>NRMSI*2-μ-20

O=O0, O1, .., OP-1={O0, O0+ΔRMSI,…, O0+P-1*ΔRMSI}

O0=floorNRMSI-20*2μ-P-1*ΔRMSI2

Where

- SCSSSB: the SSB SCS

- SCSRMSI: the RMSI CORESET SCS

- μ: the scale factor between SCSSSB and SCSRMSI, i.e., SCSSSB=2μ*SCSRMSI, (μ=-1, 0, 1, 2)

- NBW: the total number of RBs (in RMSI SCS) within the considered channel bandwidth

- NRMSI: the number of RBs of the RMSI CORESET (in RMSI SCS) under consideration

- NSSB: the total number of RBs (in SSB SCS) within the considered channel bandwidth

- NSync: the (effective) interval of the SS Raster in terms of RBs (in SSB SCS)

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | - Our suggestion is to adopt the method, so that we can efficiently and consistently determine the RMSI CORESET frequency offsets for various scenarios. |

Proposals:

Adopt the method proposed in R1-1800229 for the determination of the RMSI CORESET frequency offsets.

Suggested Offline Agreements:

- TBD

## Increase the number of bits for RMSI CORESET configuration

Background

In RAN#78, about UE Mandatory Channel BW, it was agreed that [RP-172759]

- “For all NR bands below 6 GHz, all bandwidths listed in TS 38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC.”

Submitted Proposals:

The following proposal was made in R1-1800351

- Wider bandwidth (i.e. up to 100MHz) should be supported for RMSI CORESET configuration.

- One more RB number (i.e. 196RBs) for CORESET configuration is introduced.

- 5 bits is used for RMSI CORESET configuration. Also, TS 38.213 V1.3.0 Table 13-1, 13-2, 13-3, and 13-4 can be revised as highlight part of below table.

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | It would be desirable to be able to support wider RMSI CORESET bandwidth. But, increasing the number of bits for RMSI CORESET configuration would have impact on PBCH content, and other existing requirements. Thus, more discussion is needed on the benefit and impact. |

Proposals:

Discuss the benefits and the impact to add one more bit for RMSI CORESET configuration, and make the decision/conclusion  in early this week.

Suggested Offline Agreements:

TBD

## Pattern 2: RMSI CORESET with subcarrier spacing {240, 60} kHz

Background

For Pattern 2, it was agreed to support the combinations of {SS/PBCH block, PDCCH}SCS = {120, 60}kHz and {240, 120}kHz. But, {SS/PBCH block, PDCCH}SCS = {240, 60}kHz is not supported due to the limitation of available OFDM symbols before the SS/PBCH blocks.

Submitted Proposals:

Proposals from R1-1800351:

- Pattern 2 should be supported for RMSI CORESET transmission of {SS/PBCH block, PDCCH} subcarrier spacing {240, 60} kHz case.

- In case of Pattern 2 for subcarrier spacing {240, 60} kHz, RMSI CORESET is transmitted by means of alternating even index beam and odd index beam every 20ms.

- The Table 13-7 in TS 38.213 is revised to include RMSI CORESET configurations {48 RBs, 1 OS} and {96 RBs, 1 OS}

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | It might be beneficial to add the support of the combination of {240, 60} kHz. How to transmit RMSI can be up to the network implementation. Thus, no need to limit it to alternating even index beam and odd index beam every 20ms. |

Proposals:

For Pattern 2, add the support of  RMSI CORESET configuration with {SS/PBCH block, PDCCH} SCS = {240, 60} kHz.

Suggested Offline Agreements:

- TBD

RMSI PDCCH

The following issues associated with RMSI PDCCH are discussed in some contributions:

The number of candidates per CCE AL

RMSI PDCCH DCI format, contents and parameters

CCE-to-REG mapping

CORESET-precoder-granularity

PDCCH parameters

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | Our preference is that the details on RMSI PDCCH should be covered by Control Channel Session. |

## CCE ALs and number of candidates per CCE AL

Background

For Type0-PDCCH common search space, the combinations of the supported resource configurations {BW (RBs), durations (Number of Symbols) were agreed in previous meetings as follows:  {24, 2}, {24, 3}, {48,1}, {48,2}, {48,3}, {96,1}, {96,2}, {96,3}. However, the number of candidates per CCE aggregation level is undecided.

Submitted Proposals:

Proposals from R1-1800539:

Proposal: For blind decoding of Type0-PDCCH common search space, design different number of candidates for AL=4,8,16 for different CORESET resources as shown in Table 1.

Table 1 Number of candidates of different aggregation levels in RMSI CORESET

| CORESET BW in PRB | Number of Symbols for CORESET | Number of Candidates for aggregation level 4 | Number of Candidates for aggregation level 8 | Number of Candidates for aggregation level 16 |
| --- | --- | --- | --- | --- |
| 24 | 2 | 2 | 1 | 0 |
| 24 | 3 | 3 | 1 | 0 |
| 48 | 1 | 2 | 1 | 0 |
| 48 | 2 | 3 | 2 | 1 |
| 48 | 3 | 3 | 2 | 1 |
| 96 | 1 | 3 | 2 | 1 |
| 96 | 2 | 2 | 2 | 2 |
| 96 | 3 | 2 | 2 | 2 |

Proposals from R1-1800804

Proposal: Number of PDCCH candidates for ALs {4, 8, 16} are {3, 2, 1} when CORESET size supports AL=16. Otherwise, number of PDCCH candidates for ALs {4, 8, 16} are {4, 2, 0}.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

## DCI Format and contents

Background

In initial access, the transmission of PDSCH of RMSI is broadcasted. The DCI Format for RMSI may be different from the general DCI format.

Submitted Proposals:

Proposals from R1-1800414:

The DCI contents of the compact DCI format for RMSI are determined per SS/PBCH block and CORESET multiplexing pattern as in the table in Proposal 4.

Proposal from R1-1800030

- Define DCI format for RMSI scheduling by reusing the size of Format 1_0 and re-interpret some of the fields for rate matching purposes (two bits would be enough).

Proposals from R1-1800298

- Agree to following default values for PDCCH related parameters for RMSI.

| Parameter | Proposed values for RMSI PDCCH | Notes |
| --- | --- | --- |
| CORESET-ID | Set to 0 |  |
| PDCCH-DMRS-Scrambling-ID | Set to nIDcell (i.e. cell ID) |  |
| CORESET-time-duration | Signaled in PBCH | Already captured in spec |
| CORESET-freq-dom | Signaled in PBCH | Already captured in spec |
| CORESET-CCE-to-REG-mapping-type | Set to interleaved |  |
| CORESET-REG-bundle-size | Set to 6 | Already captured in spec |
| CORESET-shift-index | Set to nIDcell | Already captured in spec |
| TCI-StatesPDCCH | Set to not indicated (empty list) |  |
| TCI-PresentInDCI | Set to not-present |  |
| CORESET-precoder-granularity | Set to CORESET-REG-bundle-size |  |
| Common-search-space-flag | Set to common-search-space |  |
| Aggregation-level-1 | Set to 0 | Already captured in spec |
| Aggregation-level-2 | Set to 0 | Already captured in spec |
| Aggregation-level-4 | Set to 4 | Already captured in spec |
| Aggregation-level-8 | Set to 2 | Already captured in spec |
| Aggregation-level-16 | Set to 1 |  |
| Monitoring-periodicity-PDCCH-slot | Defined in 38.213 | Already captured in spec |
| Monitoring-offset-PDCCH-slot | Signaled in PBCH | Already captured in spec |
| Monitoring-symbols-PDCCH-within-slot | Signaled in PBCH | Already captured in spec |

Comments

| Company Name |  | Comments |
| --- | --- | --- |

## CCE-to-REG mapping

Background

For RMSI CORESET configuration, CCE to REG mapping mode have not been agreed yet.

Submitted Proposals:

Proposals from R1-1800174

- For RMSI CORESET, interleaved CCE-to-REG mapping mode is adopted.

Proposals from R1-1800804

Adopt interleaved CCE-to-REG mapping for RMSI CORESET.

Use 3-row interleaver for interleaved CCE-to-REG mapping for RMSI CORESET

Proposals from R1-1800894

Propose to define the RMSI CORESET CCE to REG mapping type to a fixed interleaved type with all the necessary parameters required from higher layers in current specification to be also fixed.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

## Precoding granularity

Background

For RMSI CORESET configuration, CORESET-precoder-granularity is not decided yet.

Submitted Proposals:

Proposals from R1-1800174

- For CORESET configured by PBCH, the same precoding in the frequency domain is applied within a REG bundle, the size of which is 6 REG.

Proposals from R1-1800804

When trying to decode PDCCH configured by PBCH or RMSI, UE shall assume that DMRS are only present within the resource element groups constituting the PDCCH.

For CORESET configured by PBCH, UE shall assume that CORESET-precoder-granularity equals to CORESET-REG-bundle-size.

Proposals from R1-1800894

- Proposal 2	Propose to define the RMSI CORESET precoder granularity to a fixed value or to be tied to other know parameters.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

RMSI PDSCH

## RMSI Contents and Size

Background

The issue on whether RMSI should be transmitted in one package or split into two parts, and which WG should make the decision was discussed in RAN1#91 without conclusion.

Submitted Proposals:

Proposal from R1-1800414

- Proposal 5: Take the information in TABLE 2 as starting point, and collect further decisions during RAN1#AH 1801 to determine final number of bits for the RMSI; then make a decision whether to split the transmissions of RMSI into two, i.e., SIB1 and SIB2.

  - If a decision is made to split the RMSI into to chunks, discuss further during RAN1#AH 1801 which parameters should be included in the SIB1 and SIB2.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

## Max TBS for RMSI

Background

The issue on whether RMSI should be transmitted in one package or split into two parts, and which WG should make the decision was discussed in RAN1#91 without conclusion.

Submitted Proposals:

Proposal from R1-1800150

- The max TBS for RMSI is not larger than 600 bits. Send LS to RAN2 for confirmation.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

Proposal:

- The max TBS for RMSI is not larger than 600 bits. Send LS to RAN2 for confirmation.

Suggested Offline Agreements:

- TBD

## Modulation scheme for RMSI

Background

Modulation scheme for RMSI and other DL broadcast messages are so far not discussed in initial access session. It was proposed in R1-1800030 For the common downlink message in PDSCH, e.g. RMSI, broadcast OSI, paging, RACH response, the modulation scheme should be limited to consider the cell edge coverage requirement.

Submitted Proposals:

Proposal from R1-1800030

============== (R1-1800030) Text Proposal (TS 38.214) =================

#### 5.1.3.1	Modulation order and target code rate determination

The UE shall use = 2 if the DCI CRC is scrambled by P-RNTI, RA-RNTI, or SI-RNTI, otherwise, the UE shall useand Table 5.1.3.1-1 to determine the modulation order () used in the physical downlink shared channel.

For the PDSCH assigned by a PDCCH with DCI format 1_0/1_1 with CRC scrambled by C-RNTI,

if the higher layer parameter MCS-Table-PDSCH is not set to ‘256QAM’,

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

else

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

References

- R1-1800001	Draft Report of 3GPP TSG RAN WG1 #91 	ETSI

- R1-1800030	Correction on RMSI configurations	Huawei, HiSilicon

- R1-1800080	Remaining details of RMSI	ZTE, Sanechips

- R1-1800150	Remaining issues for RMSI	MediaTek Inc.

- R1-1800174	Discussion on Remaining Minimum System Information	vivo

- R1-1800229	On Remaining Issues on RMSI	CATT

- R1-1800275	Remaining issues of Remaining minimum system information	Spreadtrum Communications

- R1-1800298	Remaining details of RMSI design	Intel Corporation

- R1-1800351	RMSI delivery and CORESET configuration	LG Electronics

- R1-1800399	Correction on RMSI CORESET configuration	Lenovo, Motorola Mobility

- R1-1800407	Correction on RMSI CORESET configuration	ETRI

- R1-1800414	Corrections on Remaining Minimum System Information	Samsung

- R1-1800539	Discussion on search space design for RMSI CORESET	CMCC

- R1-1800651	Remaining issues on Remaining minimum system information	NTT DOCOMO, INC.

- R1-1800804	On Remaining System Information Delivery	Nokia, Nokia Shanghai Bell

- R1-1800848	Remaining system information delivery consideration	Qualcomm Incorporated

- R1-1800894	Remaining details on remaining minimum system information	Ericsson

- R1-1721643	Reply LS on Minimum Bandwidth		RAN4

- R1-1800008	LS on channel raster and SS raster 		RAN4

- 3GPP 38.101-1

- 3GPP 38.101-2

Agreements on RMSI CORSET configuration in RAN1#91

Agreements: - NR supports RMSI CORESET configuration such that the total bandwidth covering that of the SS/PBCH blocks and the initial active DL BWP containing RMSI CORESET when they occur in different time instances is confined within minimum carrier bandwidth - NR supports RMSI CORESET configuration such that the total bandwidth covering that of the SS/PBCH blocks and the initial active DL BWP containing RMSI CORESET when they occur in the same time instance or different time instances is confined within UE minimum bandwidth - Note: the above doesn’t prohibit the case when the SS/PBCH blocks are fully contained within the initial active DL BWP containing RMSI CORESET Agreements: - RMSI CORESET configuration supports one-to-one association between one SS/PBCH block and one RMSI in wideband operation - RAN1 makes decision in this week on whether to support RMSI CORESET configuration with many-to-one association between multiple SS/PBCH blocks and one RMSI in wideband operation - No RRC impact is expected Agreements: - For RMSI CORESET configuration, support the following combinations of different numerologies for SS/PBCH blocks and the RMSI CORESETs in different time instances: - {SSB SCS, RMSI SCS} = {{15, 30}, {30, 15}, {120, 60}, {240, 60}, {240, 120}} kHz Agreements: - Confirm the following working assumption - (Working assumption) PBCH contents, except the SSB index, should be the same for all SS/PBCH blocks within an SSB burst set for the same centre frequency - All of the RMSI CORESETs associated the SS/PBCH blocks within an SSB burst set should have the same settings (including time duration) except time-domain location related properties - FFS details of the time-domain related properties Agreements: - RMSI TTI is 160ms from RAN1 perspective Agreement: RMSI PDCCH REG bundle size is 6 PRBs Agreements: - When the SS/PBCH blocks and corresponding RMSI CORESETs occur in different time instances, - The UE assumes that the RMSI CORESET monitoring window corresponding to an SS/PBCH block in the radio frame satisifies the condition mod(SFN,2)=0 - Note: RMSI scheduling periodicity is up to gNB implementation Agreements: - When the SS/PBCH blocks and corresponding RMSI CORESETs occur in different time instances, - Configuration of the RMSI CORESET monitoring window - The monitoring window is associated with one SS/PBCH block in a burst set - monitoring window position, FFS details - monitoring window duration 2 slots - FFS The candidate symbol position(s) Agreements: - NR does not support RMSI CORESET configuration with many-to-one association between multiple SS/PBCH blocks and one RMSI in wideband operation. - In Rel-15, NR does not support RMSI search space sets configuration where the SS/PBCH blocks and corresponding RMSI search space sets occur in the same time instances if the carrier frequency is lower than 6GHz. Working assumption: - NR supports RMSI search space sets configuration for the following combinations with the same numerologies of the SS/PBCH block, the RMSI search space sets, and PDSCH, where the SS/PBCH blocks, and corresponding RMSI search space sets and PDSCH occur in the same time instances. - {SSB SCS, RMSI PDSCH SCS} = {120, 120} kHz - Two symbols for PDCCH and two symbols for PDSCH. - Note: This is pattern 3 in the RMSI search space sets configuration table. - NR supports RMSI search space sets configuration for the following combinations with different numerologies of the SS/PBCH block and the RMSI PDSCH, when the SS/PBCH blocks and corresponding RMSI PDSCH occur in the same time instances. - {SSB SCS, RMSI PDSCH SCS} = {120, 60}, {240, 120} kHz - Note: This is pattern 2 in the RMSI search space sets configuration table. Working assumption: - For pattern 3, the RMSI CORESET monitoring window associated with the SS/PBCH block SSB_i in a burst set, is defined as follows: - The starting symbol index R for the RMSI CORESET monitoring window is the same as the starting symbol of the SSB; - The duration of the monitoring window is 1; - For pattern 2, the RMSI CORESET monitoring window associated with the SS/PBCH block SSB_i in a burst set, is defined as follows: - The starting symbol index R for the RMSI CORESET monitoring window occurs earlier than the SSB symbols in the same slot or one slot before; - The duration of the monitoring window is 1; Agreement: - NR supports RMSI PDCCH aggregation levels of 4 CCEs, 8 CCEs, 16 CCEs. Working assumption: - The frequency offset in PRB level between RMSI CORESET and SS/PBCH block in the table is define as the frequency difference from the lowest PRB of RMSI to the lowest PRB of SS/PBCH block - Note: The offset in subcarrier level between the edge of SS/PBCH block PRB and RMSI CORESET PRB grid is indicated by PRB grid offset (SSB-subcarrier-offset) in PBCH (5 bits for below-6GHz and 4 bits for above-6GHz) - For each of the following combination of SS/PBCH SCS and RMSI CORESET SCS, the multiplexing patterns between SS/PBCH block and RMSI (CORESET and PDSCH), RMSI CORESET BW, RMSI CORESET duration, and PRB-level offset are jointly coded in a table using 4 bits of RMSI configuration in NR-PBCH. - {SSB SCS, RMSI SCS} = {15, 15}, {15, 30}, {30, 15}, {30, 30}, {120, 60}, {120,120},{240, 60}, {240, 120}} kHz - Definition for the multiplexing pattern tables SS/PBCH SCS and RMSI CORESET SCS - “Pattern 1” refers to the multiplexing pattern that SS/PBCH block and RMSI CORESET occur in different time instances, and SS/PBCH block TX BW and the initial active DL BWP containing RMSI CORESET overlap - “Pattern 2” refers to the multiplexing pattern that SS/PBCH block and RMSI CORESET occur in different time instances, and SS/PBCH block TX BW and the initial active DL BWP containing RMSI CORESET do not overlap - “Pattern 3” refers to the multiplexing pattern that SS/PBCH block and RMSI CORESET occur in the same time instance, and SS/PBCH block TX BW and the initial active DL BWP containing RMSI CORESET do not overlap - Note: The following figure is for information purpose only. Table 1 {SSB SCS, RMSI SCS} = {15, 15}kHz Configuration Index; Multiplexing Pattern; CORESET BW in PRB; Number of Symbols for CORESET; Frequency Offset in PRB of RMSI CORESET numerology. 1; Pattern 1; 24; 2; 0. 2; Pattern 1; 24; 2; 2. 3; Pattern 1; 24; 2; 4. 4; Pattern 1; 24; 3; 0. 5; Pattern 1; 24; 3; 2. 6; Pattern 1; 24; 3; 4 (24 RBs). 7; Pattern 1; 48; 1; 12 = 0 + 12 (51 RBs). 8; Pattern 1; 48; 1; 16 = 4 + 12. 9; Pattern 1; 48; 2; 12. 10; Pattern 1; 48; 2; 16. 11; Pattern 1; 48; 3; 12. 12; Pattern 1; 48; 3; 16. 13; Pattern 1; 96; 1; 38 = 0 + 38. 14; Pattern 1; 96; 2; 38. 15; Pattern 1; 96; 3; 38. 16; Reserved. Table 2 {SSB SCS, RMSI SCS} = {15, 30}kHz Configuration Index; Multiplexing Pattern; CORESET BW in PRB; Number of Symbols for CORESET; Frequency Offset in PRB of RMSI CORESET numerology. 1; Pattern 1; 24; 2; 0. 2; Pattern 1; 24; 2; 1. 3; Pattern 1; 24; 2; 2. 4; Pattern 1; 24; 3; 0. 5; Pattern 1; 24; 3; 1. 6; Pattern 1; 24; 3; 2. 7; Pattern 1; 48; 1; 12. 8; Pattern 1; 48; 1; 13. 9; Pattern 1; 48; 2; 12. 10; Pattern 1; 48; 2; 13. 11; Pattern 1; 48; 3; 12. 12; Pattern 1; 48; 3; 13. 13; Reserved. 14; Reserved. 15; Reserved. 16; Reserved. Table 3 {SSB SCS, RMSI SCS} = {30, 15}kHz Configuration Index; Multiplexing Pattern; CORESET BW in PRB; Number of Symbols for CORESET; Frequency Offset in PRB of RMSI CORESET numerology. 1; Pattern 1; 48; 1; 2. 2; Pattern 1; 48; 1; 6. 3; Pattern 1; 48; 2; 2. 4; Pattern 1; 48; 2; 6. 5; Pattern 1; 48; 3; 2. 6; Pattern 1; 48; 3; 6. 7; Pattern 1; 96; 1; 28. 8; Pattern 1; 96; 2; 28. 9; Pattern 1; 96; 3; 28. 10; Reserved. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. 16; Reserved. Table 4 {SSB SCS, RMSI SCS} = {30, 30}kHz Configuration Index; Multiplexing Pattern; CORESET BW in PRB; Number of Symbols for CORESET; Frequency Offset in PRB of RMSI CORESET numerology. 1; Pattern 1; 24; 2; 0. 2; Pattern 1; 24; 2; 1. 3; Pattern 1; 24; 2; 2. 4; Pattern 1; 24; 2; 3. 5; Pattern 1; 24; 2; 4. 6; Pattern 1; 24; 3; 0. 7; Pattern 1; 24; 3; 1. 8; Pattern 1; 24; 3; 2. 9; Pattern 1; 24; 3; 3. 10; Pattern 1; 24; 3; 4. 11; Pattern 1; 48; 1; 12. 12; Pattern 1; 48; 1; 14. 13; Pattern 1; 48; 1; 16. 14; Pattern 1; 48; 2; 12. 15; Pattern 1; 48; 2; 14. 16; Pattern 1; 48; 2; 16. Table 5 {SSB SCS, RMSI SCS} = {120, 60}kHz Configuration Index; Multiplexing Pattern; CORESET BW in PRB; Number of Symbols for CORESET; Frequency Offset in PRB of RMSI CORESET numerology. 1; Pattern 1; 48; 1; 0. 2; Pattern 1; 48; 1; 8. 3; Pattern 1; 48; 2; 0. 4; Pattern 1; 48; 2; 8. 5; Pattern 1; 48; 3; 0. 6; Pattern 1; 48; 3; 8. 7; Pattern 1; 96; 1; 28. 8; Pattern 1; 96; 2; 28. 9; Pattern 2; 48; 1; [-42] if PRG grid not aligned and [-41] if PRG grid aligned. 10; Pattern 2; 48; 1; [49]. 11; Pattern 2; 48; [2]; [-42] if PRG grid not aligned and [-41] if PRG grid aligned. 12; Pattern 2; 48; [2]; [49]. 13; Pattern 2; [96]; 1; [-42] if PRG grid not aligned and [-41] if PRG grid aligned. 14; Pattern 2; [96]; 1; [97]. 15; Pattern 2; [96]; [2]; [-42] if PRG grid not aligned and [-41] if PRG grid aligned. 16; Pattern 2; [96]; [2]; [97]. Note: Configurations 13, 14, 15, and 16 are supported only when carrier bandwidth is larger than 100MHz Table 6 {SSB SCS, RMSI SCS} = {120, 120}kHz Configuration Index; Multiplexing Pattern; CORESET BW in PRB; Number of Symbols for CORESET; Frequency Offset in PRB of RMSI CORESET numerology. 1; Pattern 1; 24; 2; 0. 2; Pattern 1; 24; 2; 4. 3; Pattern 1; 48; 1; 14. 4; Pattern 1; 48; 2; 14. 5; Pattern 3; 24; 2; [-21] if PRG grid not aligned and [-20] if PRG grid aligned. 6; Pattern 3; 24; 2; 24. 7; Pattern 3; [48]; 2; [-21] if PRG grid not aligned and [-20] if PRG grid aligned. 8; Pattern 3; [48]; 2; [48]. 9; Reserved. 10; Reserved. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. 16; Reserved. Note: Configurations 7 and 8 are supported only when carrier bandwidth is larger than 100MHz Table 7 {SSB SCS, RMSI SCS} = {240, 60}kHz Configuration Index; Multiplexing Pattern; CORESET BW in PRB; Number of Symbols for CORESET; Frequency Offset in PRB of RMSI CORESET numerology. 1; Pattern 1; 96; 1; 0. 2; Pattern 1; 96; 1; 16. 3; Pattern 1; 96; 2; 0. 4; Pattern 1; 96; 2; 16. 5; Reserved. 6; Reserved. 7; Reserved. 8; Reserved. 9; Reserved. 10; Reserved. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. 16; Reserved. Table 8 {SSB SCS, RMSI SCS} = {240, 120}kHz Configuration Index; Multiplexing Pattern; CORESET BW in PRB; Number of Symbols for CORESET; Frequency Offset in PRB of RMSI CORESET numerology. 1; Pattern 1; 48; 1; 0. 2; Pattern 1; 48; 1; 8. 3; Pattern 1; 48; 2; 0. 4; Pattern 1; 48; 2; 8. 5; Pattern 2; 24; 1; [-42] if PRG grid not aligned and [-41] if PRG grid aligned. 6; Pattern 2; 24; 1; [25]. 7; Pattern 2; 24; [2]; [-42] if PRG grid not aligned and [-41] if PRG grid aligned. 8; Pattern 2; 24; [2]; [25]. 9; Pattern 2; [48]; 1; [-42] if PRG grid not aligned and [-41] if PRG grid aligned. 10; Pattern 2; [48]; 1; [49]. 11; Pattern 2; [48]; [2]; [-42] if PRG grid not aligned and [-41] if PRG grid aligned. 12; Pattern 2; [48]; [2]; [49]. 13; Reserved. 14; Reserved. 15; Reserved. 16; Reserved. Note: Configurations 9, 10, 11, and 12 are supported only when carrier bandwidth is larger than 100MHz Working assumption: - RMSI CORESET starting OFDM symbol and RMSI monitoring window timing Tables Table 1 For Pattern 1 and sub6 GHz Configuration Index; Group offset X (msec); Number of search space sets per slot N; M; Starting OFDM symbol index (note: l is CORESET duration). 1; 0; 1; 1; 0. 2; 0; 2; 1; {0, l}. 3; 2; 1; 1; 0. 4; 2; 2; 1; {0, l}. 5; 5; 1; 1; 0. 6; 5; 2; 1; {0, l}. 7; 7; 1; 1; 0. 8; 7; 2; 1; {0, l}. 9; 0; 1; 2; 0. 10; 5; 1; 2; 0. 11; 0; 1; 1; 1. 12; 0; 1; 1; 2. 13; 2; 1; 1; 1. 14; 2; 1; 1; 2. 15; 5; 1; 1; 1. 16; 5; 1; 1; 2. Table 2 For Pattern 1 and sub6 GHz Configuration Index; Group offset X (msec); Number of search space sets per slot N; M; Starting OFDM symbol index (note: l is CORESET duration). 1; 0; 1; 1; 0. 2; 0; 2; 1; {0, 7}. 3; 2.5; 1; 1; 0. 4; 2.5; 2; 1; {0, 7}. 5; 5; 1; 1; 0. 6; 5; 2; 1; {0, 7}. 7; 0; 2; 1; {0, l}. 8; 2.5; 2; 1; {0, l}. 9; 5; 2; 1; {0, l}. 10; 7.5; 1; 1; 0. 11; 7.5; 2; 1; {0, 7}. 12; 7.5; 2; 1; {0, l}. 13; 0; 1; 2; 0. 14; 5; 1; 2; 0. 15; Reserved. 16; Reserved. Table 3 For Pattern 2, and the combination of SS SCS = 120 kHz and RMSI CORESET SCS = 60 kHz Configuration Index; RMSI CORESET monitoring window timing (SFN and slot number); Starting OFDM symbol index (note: l is CORESET duration, i is SSB index, k = 0, 1, … 15). 1; The SFN of the RMSI CORESET monitoring window is the same as the corresponding SSB. The slot number is the same as the one for the corresponding SSB.; 0, 1, 6, 7 for i = 4k, 4k+1, 4k+2, 4k+3. 2; Reserved. 3; Reserved. 4; Reserved. 5; Reserved. 6; Reserved. 7; Reserved. 8; Reserved. 9; Reserved. 10; Reserved. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. 16; Reserved. Table 4 For Pattern 2, and the combination of SS SCS = 240 kHz and RMSI CORESET SCS = 120 kHz Configuration Index; RMSI CORESET monitoring window timing (SFN and slot number); Starting OFDM symbol index (note: l is CORESET duration, i is SSB index, k = 0, 1, … 15). 1; The SFN of the RMSI CORESET monitoring window is the same as the corresponding SSB. The slot number is the same or one smaller than the one for the corresponding SSB according to the right entry.; 0, 1, 2 in the same slot as the SSB is mapped for i = 8k, 8k+1, 8k+2; 3,12,13,0,1 in in the previous slot where the SSB is mapped for 8k+3, 8k+4,8k+5,8k+6,8k+7. 2; Reserved. 3; Reserved. 4; Reserved. 5; Reserved. 6; Reserved. 7; Reserved. 8; Reserved. 9; Reserved. 10; Reserved. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. 16; Reserved. Table 5 For Pattern 3, and the combination of SS SCS = 120 kHz and RMSI CORESET SCS = 120 kHz Configuration Index; RMSI CORESET monitoring window timing (SFN and slot number); Starting OFDM symbol index (note: l is CORESET duration, i is SSB index, k = 0, 1, … 15). 1; The SFN of the RMSI CORESET monitoring window is the same as the corresponding SSB. The slot number is the same as the one for the corresponding SSB.; 4, 8, 2, 6 for i = 4k, 4k+1, 4k+2, 4k+3. 2; Reserved. 3; Reserved. 4; Reserved. 5; Reserved. 6; Reserved. 7; Reserved. 8; Reserved. 9; Reserved. 10; Reserved. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. 16; Reserved. Working assumption: - When the SS/PBCH blocks and corresponding RMSI search space sets occur for pattern 1 NR supports the configuration where the RMSI CORESET monitoring window associated with the SS/PBCH block SSB_i in a burst set is defined as follows: - The RMSI CORESET monitoring window duration with RMSI numerology is 2 slots; - The first slot index of the monitoring window of SSB_i - The first slot index S with RMSI numerology for a monitoring window of SSB_i, with respect to the SFN boundary, is determined by - S = mod(X*n + f(i), number of slots per frame in RMSI numerology) - SFN index: - When floor((X*n + f(i))/(number of slots per frame)) = 0, the SFN to carry the RMSI monitoring window is determined by mod(SFN,2)=0 - When floor((X*n + f(i))/(number of slots per frame)) = 1, the SFN to carry the RMSI monitoring window is determined by mod(SFN-1,2)=0 - Note that this is an update of the previous agreements: mod(SFN,2)=0 applies to all the CORESETs - Here: - n = (RMSI SCS)/(15 kHz) - X can be configured as {0, 2, 5, 7} when SSB SCS is 15kHz or 30kHz; and configured as {0, 2.5, 5, 7.5} when SSB SCS is 120kHz or 240kHz for pattern 1 - f(i) = floor(i*M) - M equals 1/2, if N=2 and X belongs to {0,2,5,7} - M = 1, if N=1 and X belongs to {0,2,5,7} - M = 2, if N=1 and X belongs to {0,5} - i is the SSB index of SSB_i - N is the number of search space sets per slot, which can be configured as {1, 2} - The starting symbol index R for the RMSI CORESET monitoring window is determined as follows - when N = 1, R = 0, l={1,2,3}, where l is the CORESET duration - when N = 2, R is obtained with one of the following options, which will be selected in different use cases: - R={0,l} for sub6GHz where l={1,[2],[3]} is the CORESET duration - If SSB index is even, 0; if odd, l - R={0,7} & {0,l} for over6GHz - If SSB index is even, 0; if odd, 7 - At least for the 30kHz SSB SCS/15kHz RMSI SCS LTE-NR coexistence case, R = 1, or 2 should be supported - The number of RMSI search space sets to monitor is at most one per SSB within duration of 14 symbols with RMSI numerology.

## Maximum transmission bandwidth configuration

For FR1, the maximum transmission bandwidth configuration NRB for each channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1 (TS 38.101-1)

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | [TBD] | 216 | 270 | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | [TBD] | 106 | 133 | 162 | 217 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | [TBD] | 51 | 65 | 79 | 107 | 135 |

For FR2, The maximum transmission bandwidth configuration NRB for each channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |