3GPP TSG RAN WG1 Meeting AH 1801                                                                          R1-1800269

Vancouver, Canada, January 22nd – 26th, 2018

Source:	CATT

Title:	Initial self evaluation for Dense Urban-eMBB

Agenda Item:	7.9

Document for:	Discussion and Decision

Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1], the self evaluation will provide the performance towards all the ITU-R IMT-2020 requirements. The key minimum technical performance requirements are provide in Report ITU-R M.2410 [2] for the test environment of Dense Urban-eMBB.

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Dense Urban – eMBB | 0.225 | 0.15 |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Dense Urban – eMBB (Note 1) | 7.8 | 5.4 |

NOTE 1 – This requirement applies to Macro TRxP layer of the Dense Urban – eMBB test environment as described in Report ITU-R M.2412-0.

According to the evaluation methodology of Report ITU-R M.2412 [3], the average spectral efficiency and the 5th percentile user spectral efficiency need system-level simulation evaluation to demonstrate the achievement for the ITU-R technical performance requirements, while user experienced data rate can be calculated by 5th percentile user spectral efficiency SEuser through equation in [2] (this KPI is only applied to Dense Urban-eMBB).

Ruser = W × SEuser

In this document, we focus on the Dense Urban-eMBB to demonstrate the technical performance requirements, including the DL average spectral efficiency, the DL 5th percentile user spectral efficiency and the DL user experienced data rate.

Evaluation methodology

System-level simulation with proper link to system interface based on link-level simulation results could be used to evaluate average spectral efficiency and 5th percentile user spectral efficiency.

The user experienced data rate can be calculated based on proper bandwidth to demonstrate the achievement for the requirement of the ITU-R.

Evaluation assumption

In [3], a number of evaluation parameters are defined for the detailed evaluations. Some of them need further decision from IMT-2020 technology proponent, e.g. the exact number of antenna elements applied and it is also necessary to discuss the parameters which were not detailed (“reported by proponent”) in ITU report, e.g. TxRU to antenna element mapping. Before future discussion on those parameters, the evaluation assumptions used in this document are summarized in Annex, which will be updated when 3GPP has further discussions and/or agreements.

Initial downlink evaluation results

Based on the above information, the initial evaluation results are provided for DL average spectral efficiency and DL 5th percentile spectral efficiency under Dense Urban-eMBB in Table 1 and Table 2. The user experienced data rates results are provided in Table 3 which are calculated based on initial results of the 5th percentile spectral efficiency. Evaluation assumptions can be found in Annex.

Note: antenna configuration applied in this document follows the ones in calibration for DL geometry [4]. More antenna configurations will be further evaluated to observe which configurations are more proper.

Spectral efficiency:

The evaluation results mainly focus on those at 4GHz. For the evaluation with above 6GHz, the strategies of transmission (resource schedule, analogy beam management, .etc) and reception with multiple panels in UEs side need more investigation/discussion and will be updated.

Note: non-ideal evaluation assumptions and overhead will be updated in future. Overhead for TDD and FDD are assumed same in this document for simplicity.

Table 1 Initial DL average spectral efficiency evaluation (Dense Urban-eMBB at 4GHz)

| Test environment | Duplex scheme | SEavg (bit/s/Hz/TRxP) | Requirement (bit/s/Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A (4 GHz, 32TxRu/128T4R) | TDD | 14.93 | 7.8 | +91.41% |
|  | FDD | 11.90 | 7.8 | +52.56% |

Table 2 Initial DL 5th percentile spectral efficiency evaluation (Dense Urban-eMBB at 4GHz)

| Test environment | Duplex scheme | SEuser (bit/s/Hz) | Requirement (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A (4 GHz, 32TxRu/128T4R) | TDD | 0.51 | 0.225 | +126.67% |
|  | FDD | 0.37 | 0.225 | +64.44% |

Observation 1: For sub 6 GHz, NR can fulfill the DL spectral efficiency requirements in the Dense Urban-eMBB test environment in both TDD and FDD mode.

- The 128T4R antenna element configuration with 32TxRu at transmitter can fulfil the requirement

User experienced data rate:

Considering various bandwidth at sub 6GHz, initial DL user experienced data rates are provided in Table 3.

Table 3 Initial DL user experienced data rate evaluation at 4GHz

| Test environment | Duplex mode | Bandwidth W (MHz) | Ruser (Mbit/s) | Requirement (Mbit/s) |
| --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A (4 GHz, 32TxRu/128T4R) | TDD | 100 | 51.00 | 100 |
|  |  | 150 | 76.50 | 100 |
|  |  | 200 | 102.00 | 100 |
|  |  | 250 | 127.50 | 100 |
|  | FDD | 200 | 74.00 | 100 |
|  |  | 250 | 92.50 | 100 |
|  |  | 300 | 111.00 | 100 |
|  |  | 350 | 129.50 | 100 |

Observation 2: The ITU requirement on DL user experienced data rate can be met by NR in sub 6GHz.

- For TDD mode, the bandwidth should be not smaller than 200MHz

- For FDD mode, the bandwidth should be not smaller than 300MHz

Note that the above results will be updated when evaluation assumptions and spectral efficiency are further updated.

Conclusion

In this contribution, we provide our preliminary evaluation results of Dense Urban -eMBB, we have following observations:

Observation 1: For sub 6 GHz, NR can fulfill the DL spectral efficiency requirements in the Dense Urban-eMBB test environment in both TDD and FDD mode.

- The 128T4R antenna element configuration with 32TxRu at transmitter can fulfil the requirement

Observation 2: The ITU requirement on DL user experienced data rate can be met by NR in sub 6GHz.

- For TDD mode, the bandwidth should be not smaller than 200MHz

- For FDD mode, the bandwidth should be not smaller than 300MHz

- References

  - RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - RT-170019, “Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation”， Huawei.

- Annex: Evaluation assumptions

In this contribution, the evaluation assumptions for Dense Urban-eMBB are provided in the following table.

| Dense Urban - eMBB | Config. A |
| --- | --- |
| Carrier frequency for evaluation | 1 layer (Macro) with 4 GHz |
| Simulation bandwidth | FDD: 10M+10M TDD: 20M |
| Subcarrier spacing | 15K |
| Symbols number per slot | 14 |
| BS antenna height | 25 m |
| Total transmit power per TRxP | FDD: 41 dBm TDD: 44 dBm |
| UE power class | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss (applies to Channel model B) |
| Inter-site distance | 200 m |
| Number of antenna elements per TRxP | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 32TXRU, (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| Number of TXRU per UE | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) |
| Device deployment | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer |
| UE speeds of interest | Indoor users: 3km/h Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| UE noise figure | 7 dB |
| BS antenna element gain | 8 dBi |
| BS antenna element pattern | See Table 9 in Report ITU-R M.2412 |
| UE antenna element gain | 0 dBi |
| UE antenna element pattern | Omni-directional |
| Thermal noise level | -174 dBm/Hz |
| Traffic model | Full buffer |
| UE density | 10 UEs per TRxP |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Channel model | Channel model B |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS |
| Electronic tilt | 102° in LCS |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Polarized antenna model | Model-2 in TR36.873 |
| Receiver | MMSE-IRC |
| CSI feedback | 10ms period based on CSI-RS |
| CSI method | FDD: CSI TypeII codebook based TDD: full reciprocity based |
| Channel estimation | ideal |
| Interference estimation | ideal |