3GPP TSG RAN WG1 Meeting AH 1801				      R1-1800567

Vancouver, Canada, January 22nd – 26th, 2018

Agenda item:	7.2.2.2

Source: 	AT&T

Title: 	Remaining issues and text proposal in CSI reporting

Document for:	Discussion/Decision

# Introduction

In RAN #71, a new study item New Radio (NR) Access Technology was approved.  However, we observed some discrepancy in the RAN plenary agreed specifications for [1] and [2].

In this contribution, we provide corrections to the issues we observed.

# Text Proposal Regarding Strongest Layer Indicator in TS 38.214

In [1], strongest layer indicator (SLI) was defined. However, in [2], it is referred as layer indicator (LI).  In our view both the specifications needs to use same term.  Hence we propose

Proposal 1:   To have consistency with TS 38.212, replace strongest layer Indicator (SLI) with Layer Indicator (LI) in 38.214

# Text Proposal on Table 6.3.1.1.2-9 in TS 38.212

In [2], the following table indicates mapping order for CSI fields of CSI part.

Table 6.3.1.1.2-9: Mapping order of CSI fields of one CSI report, CSI part 1, PMI-FormatIndicator= subbandPMI or CQI-FormatIndicator=subbandCQI CSI report number; CSI fields. CSI report #n CSI part 1; CRI as in Tables 6.3.1.1.2-3/4/5, if reported. Rank Indicator as in Tables 6.3.1.1.2-3/4/5, if reported. Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported. Wideband CQI as in Tables 6.3.1.1.2-3/4/5, if reported. Subband differential CQI for the first TB as in Tables 6.3.1.1.2-3/4/5, if reported. Indicator of the number of non-zero wideband amplitude coefficients for layer as in Table 6.3.1.1.2-5, if reported.

However, the wideband CQI term needs some clarification in a multi codeword transmission.  In our view, it should be replaced with Wideband CQI for the first TB

Proposal 2:  To avoid the confusion regarding wideband CQI, the following table should be agreed in TS 38.212

Table 6.3.1.1.2-9: Mapping order of CSI fields of one CSI report, CSI part 1, PMI-FormatIndicator= subbandPMI or CQI-FormatIndicator=subbandCQI CSI report number; CSI fields. CSI report #n CSI part 1; CRI as in Tables 6.3.1.1.2-3/4/5, if reported. Rank Indicator as in Tables 6.3.1.1.2-3/4/5, if reported. Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported. Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5, if reported. Subband differential CQI for the first TB as in Tables 6.3.1.1.2-3/4/5, if reported. Indicator of the number of non-zero wideband amplitude coefficients for layer as in Table 6.3.1.1.2-5, if reported.

# Text Proposal on Table 6.3.1.1.2-11 in in TS 38.212

The following CSI report table was specified in [2]. However, the heading are missing in the table.

Table 6.3.1.1.2-11: Mapping order of CSI fields of one CSI report, CSI part 2 subband, PMI-FormatIndicator= subbandPMI or CQI-FormatIndicator=subbandCQI CSI report #n Part 2 subband; Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if CQI-FormatIndicator=subbandCQI and if reported. PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, if PMI-FormatIndicator= subbandPMI and if reported. Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if CQI-FormatIndicator=subbandCQI and if reported. PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, if PMI-FormatIndicator= subbandPMI and if reported.

We propose to add header for the table to avoid confusion.

CSI report number; CSI fields. CSI report #n Part 2 subband; Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if CQI-FormatIndicator=subbandCQI and if reported. PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, if PMI-FormatIndicator= subbandPMI and if reported. Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if CQI-FormatIndicator=subbandCQI and if reported.

Proposal 3:  To avoid the confusion regarding CSI report, the above table should be agreed in TS 38.212

# Conclusions

In this contribution we provide some corrections to the agreed specifications.

Based on our observations, we have the following proposal:

Proposal 1:   To have consistency with TS 38.212, replace strongest layer Indicator (SLI) with Layer Indicator (LI) in 38.214

Proposal 2:  To avoid the confusion regarding wideband CQI, the following table should be agreed in TS 38.212

Table 6.3.1.1.2-9: Mapping order of CSI fields of one CSI report, CSI part 1, PMI-FormatIndicator= subbandPMI or CQI-FormatIndicator=subbandCQI CSI report number; CSI fields. CSI report #n CSI part 1; CRI as in Tables 6.3.1.1.2-3/4/5, if reported. Rank Indicator as in Tables 6.3.1.1.2-3/4/5, if reported. Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported. Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5, if reported. Subband differential CQI for the first TB as in Tables 6.3.1.1.2-3/4/5, if reported. Indicator of the number of non-zero wideband amplitude coefficients for layer as in Table 6.3.1.1.2-5, if reported.

Proposal 3:  To avoid the confusion regarding CSI report, the following table should be agreed in TS 38.212

CSI report number; CSI fields. CSI report #n Part 2 subband; Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if CQI-FormatIndicator=subbandCQI and if reported. PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, if PMI-FormatIndicator= subbandPMI and if reported. Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if CQI-FormatIndicator=subbandCQI and if reported.

# References

[1]. 38.214_v2.0.0, NR, Physical layer procedures for data.

[2]. 38.212_v2.0.0, NR, Multiplexing and channel coding.