3GPP TSG RAN WG1 Meeting AH 1801			R1-1800948

Vancouver, Canada, January 22nd – 26th, 2018

Agenda Item:	7.3.2.2

Source: 	Ericsson

Title:	On Long PUCCH Repetition and Other Issues

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss the encoding issues related to multiplexing of CSI and HARQ-ACK/SR on PUCCH. Also, some updates for improved clarity of the UE behaviour when transmission long PUCCH over multiple slots is provided.

# Discussion

We discussed in our companion contribution [1] that for encoding purposes of the AN/SR and CSI, since CSI Part 1 and Part 2 are differently encoded depending on the PUCCH format used in the CSI reporting configuration proper considerations should be taken into account to avoid additional complexity within a unified framework based on the previous agreements.

We also discussed that for multiplexing of the CSI and HARQ-ACK/SR, the total number of UCI bits should be used to determine a PUCCH resource set where a PUCCH configuration is determined by PUCCH resource indicator field in DCI.

- If CSI is configured with PUCCH format 2, it implies that CSI Part 1 and Part 2 are jointly encoded. Furthermore, we have already agreed that if CSI and AN/SR are simultaneously transmitted using PUCCH format 2, AN/SR and CSI are jointly encoded using the maximum configured code rate.  To have a unified frame work, we extend this procedure and propose that if CSI is configured with PUCCH format 2, and if the UE uses PUCCH format 3 (or 4) resource for simultaneous transmission of AN/SR and CSI as described previously, the CSI and AN/SR are jointly encoded based on the max code rate for PUCCH F3 (or F4) and the same RE mapping rule as for only AN transmission on a PUCCH format 3 (or 4) is applied.

- If CSI is configured with a PUCCH format 3 (or 4), it implies that CSI Part 1 and Part 2 are separately encoded and mapped separately to REs in a PUCCH resource. Furthermore, we have already agreed that if CSI and AN/SR are simultaneously transmitted on a PUCCH format 3 (or 4), AN/SR and CSI part 1 are jointly encoded using the maximum configured code rate. The remaining resources are used for encoding of CSI Part 2. To have a unified frame work and avoid introducing a new mapping scheme, we extend this procedure and propose that if CSI is configured with a PUCCH format 3 (or 4), and if the UE uses PUCCH format 2 resource for simultaneous transmission of AN/SR and CSI as described previously, the AN/SR and CSI part 1 are jointly encoded using the max code rate of PUCCH F2 and CSI part 2 is dropped.

We summarize the above UE behavior with respect to encoding of the UCI bits in Table 1 based on the previous agreements and the following proposals:

Proposal:

- If the parameter PUCCH-F3/4-simultanoeus-HARQ-ACK-CSI is set to TRUE, and if CSI is configured with PUCCH format 2, and if the UE determines a PUCCH format 3/4 resource for simultaneous transmission of AN/SR and CSI, the CSI and AN/SR are jointly encoded based on the max code rate for PUCCH F3 (or F4) and the same RE mapping rule as for only AN transmission on a PUCCH format 3/4 is applied.

- If the parameter PUCCH-F2-simultanoeus-HARQ-ACK-CSI is set to TRUE, and if CSI is configured with PUCCH format 3/4, and if a UE determines a PUCCH format 2 resource for simultaneous transmission of AN/SR and CSI, CSI Part 2 is dropped and CSI Part 1 is jointly encoded with AN/SR using the configured max code rate for PUCCH format 2 and the same RE mapping rule as for only AN transmission on a PUCCH format 2 is applied.

Table 1: UCI encoding rules when AN/SR and CSI PUCCH resources collide

|  | PUCCH-F2-simultaneous-HARQ-ACK-CSI = TRUE Determined resource with AN/SR+CSI is F2 | PUCCH-F3-simultaneous-HARQ-ACK-CSI = TRUE Determined resource with AN/SR+CSI is F3 | PUCCH-F4-simultaneous-HARQ-ACK-CSI = TRUE Determined resource with AN/SR+CSI is F4 |
| --- | --- | --- | --- |
| CSI configured with F2 | Jointly encode A/N and CSI report at the configured max code rate using the A/N+CSI PUCCH resource |  |  |
| CSI configured with F3 | Jointly encode A/N and CSI Part 1 report at the configured max code rate and drop CSI Part 2 | Jointly encode A/N and CSI Part 1 at the configured max code rate. Separately encode CSI Part 2 using remaining resources (if any) of the PUCCH resource |  |
| CSI configured with F4 |  |  |  |

# Text proposal

- Text proposal Subclause 6.3.1.4 in TS 38.212

-----------------------------------------<Start of Text Proposal>-------------------------------------------------------------

#### 6.3.1.4	Rate matching

For PUCCH formats 2/3/4, the total rate matching output sequence length  is given by Table 6.3.2.4-1, where  , , and  are the number of symbols carrying UCI for PUCCH formats 2/3/4 respectively;  and  are the number of PRBs that are determined by the UE for PUCCH formats 2/3 transmission respectively according to section x.x of [5, TS38.213]; and  is the spreading factor for PUCCH format 4.

- Table 6.3.1.4-1: Total rate matching output sequence length

| PUCCH format | Modulation order |  |
| --- | --- | --- |
|  | QPSK | π/2-BPSK |
| PUCCH format 2 |  | N/A |
| PUCCH format 3 |  |  |
| PUCCH format 4 |  |  |

##### 6.3.1.4.1	UCI encoded by Polar code

The input bit sequence to rate matching is  where  is the code block number, and  is the number of coded bits in code block number .

- Table 6.3.1.4.1-1: Rate matching output sequence length

| UCI(s) for transmission on a PUCCH | UCI for encoding | Value of |
| --- | --- | --- |
| HARQ-ACK | HARQ-ACK |  |
| HARQ-ACK, SR | HARQ-ACK, SR |  |
| CSI (CSI not of two parts) | CSI |  |
| SR, CSI (CSI not of two parts) | SR, CSI |  |
| HARQ-ACK, CSI (CSI not of two parts) | HARQ-ACK, CSI |  |
| HARQ-ACK, SR, CSI (CSI not of two parts) | HARQ-ACK, SR, CSI |  |
| CSI (CSI of two parts) | CSI part 1 |  |
|  | CSI part 2 |  |
| SR, CSI (CSI of two parts) | SR, CSI part 1 |  |
|  | CSI part 2 |  |
| HARQ-ACK, CSI (CSI of two parts) PUCCH format 3/4 for UCI | HARQ-ACK, CSI part 1 |  |
|  | CSI part 2 |  |
| HARQ-ACK, CSI (CSI of two parts) PUCCH format 2 for UCI | HARQ-ACK, CSI part 1 |  |
| HARQ-ACK, SR, CSI (CSI of two parts) PUCCH format 3/4 for UCI | HARQ-ACK, SR, CSI part 1 |  |
|  | CSI part 2 |  |
| HARQ-ACK, SR, CSI (CSI of two parts) PUCCH format 2 for UCI | HARQ-ACK, SR, CSI part 1 |  |

Rate matching is performed according to Section 5.4.1 by setting  and the rate matching output sequence length to , where  is the number of code blocks for UCI determined according to Section 5.2.1 and the value of  is given by Table 6.3.1.4.1-1:

-	 is the number of bits for HARQ-ACK for transmission on the current PUCCH;

-	 is the number of bits for SR for transmission on the current PUCCH;

-	 is the number of bits for CSI part 1 for transmission on the current PUCCH;

-	 is the number of bits for CSI part 2 for transmission on the current PUCCH;

-	 is the number of CRC bits;

-	 is the configured maximum PUCCH coding rate;

-	 is given by Table 6.3.1.4-1.

The output bit sequence after rate matching is denoted as  where  is the length of rate matching output sequence in code block number .

##### 6.3.1.4.2	UCI encoded by channel coding of small block lengths

The input bit sequence to rate matching is .

The value of  is determined according to Table 6.3.1.4.1-1 by setting .

Rate matching is performed according to Section 5.4.3 by setting the rate matching output sequence length .

The output bit sequence after rate matching is denoted as .

--------------------------------<Start of Text Proposal>-------------------------------------------------------------

Text proposal for Subclause 9.2.6 TS 38.213

-----------------------------------------<Start of Text Proposal>-------------------------------------------------------------

### 9.2.6	UCI repetition procedure

For PUCCH formats 1, 3, or 4, a UE can be configured a number of slots, , for a PUCCH transmission by respective higher layer parameters PUCCH-F1-number-of-slots , PUCCH-F3-number-of-slots, or PUCCH-F4-number-of-slots.

For ,

- the UE transmit the UCI as described in Subclause 9.2.1 in the first slot of the slots.

- the UE repeats the UCI in the PUCCH transmission in the first slot of the  slots in the PUCCH transmission in each of the remaining  slots;

- a PUCCH transmission has the same number of consecutive symbols, as provided by higher layer parameter PUCCH-F1-F3-F4-number-of-symbols, in each of the  slots;

- a PUCCH transmission has a same first symbol, as provided by higher layer parameter PUCCH-F1-F3-F4-starting-symbol, in each of the  slots;

- the UE is configured by higher layer parameter PUCCH-F1-F3-F4-interslot-FH whether or not to perform frequency hopping for PUCCH transmissions in different slots. If PUCCH-F1-F3-F4-interslot-FH = ON, a first PRB for PUCCH transmission is provided by higher layer parameter PUCCH-starting-PRB and a second PRB for PUCCH transmission is provided by higher layer parameter PUCCH-2nd-hop-PRB.

  - If the UE is configured to perform frequency hopping for PUCCH transmissions in different slots,

    - the UE performs frequency hopping per slot;

    - the UE is expected to repeat the frequency resources used in the first and second slot, in the same order, until all repetitions are complete;

    - the UE is not expected to be configured to perform frequency hopping for a PUCCH transmission within a slot.

If a UE is provided higher layer parameter UL-DL-configuration-common or is additionally provided higher layer parameter UL-DL-configuration-dedicated for the slot format per slot over the number of slots, as described in Subclause 11.1, the UE determines the  slots for a PUCCH transmission as the first slots starting from a slot indicated to the UE as described in Subclause 9.2.3 and having

- an UL symbol for a symbol provided by higher layer parameter PUCCH-F1-F3-F4-starting-symbol is an UL symbol, and

- consecutive UL symbols, starting from the symbol, equal to the number of symbols provided by higher layer parameter PUCCH-F1-F3-F4-number-of-symbols.

If a UE is not provided higher layer parameter UL-DL-configuration-common, the UE determines the  slots for a PUCCH transmission as the  consecutive slots starting from a slot indicated to the UE as described in Subclause 9.2.3.

--------------------------------<Start of Text Proposal>-------------------------------------------------------------

# Conclusion

In this contribution two text proposals resolving some issues for long PUCCH are provided. Hence, we propose:

Proposal:

- Endorse the text proposal in Section 3 in R1-1800948 for Subclauses 6.3.1.4 of TS 38.212 V2.0.0.

- Endorse the text proposal in Section 3 in R1-1800948 for Subclauses 9.2.6 of TS 38.213 V2.0.0.

# References

R1-1800947		On UE Behavior for UCI Reporting and Other Issues, Ericsson

TS 38.212 V2.0.0

TS 38.213 V2.0.0