3GPP TSG RAN WG1 Meeting AH 1801		R1-1801092

Vancouver, Canada, January 22nd – 26th, 2018

Agenda item:	7.3.2.3

Source: 	Qualcomm Incorporated

Title: 	Summary of remaining issues for UCI piggyback on PUSCH

Document for:	Discussion/Decision

# Introduction

In previous RAN1 meetings, the following agreements for UCI piggyback on PUSCH are made.

Agreements:

- For frequency first mapping, UCI resource mapping principles (e.g., around RS) are common for PUSCH with DFT-s-OFDM waveform and CP-OFDM waveform

Agreements:

- Detail UCI mapping rule on PUSCH is as follows:

  - Map HARQ-ACK to REs around DMRS symbol(s)

  - If PUSCH punctured by HARQ-ACK,

    - Map CSI part 1 starting after certain amount of reserved HARQ-ACK REs.

      - FFS reserved HARQ-ACK REs

      - PUSCH can be mapped to reserved REs

  - If PUSCH rate matched by HARQ-ACK,

    - map HARQ-ACK first, followed by CSI part1.

  - FFS: how to map CSI part 2, e.g.,

    - Map CSI part 2 after CSI part 1

    - Map CSI part 2 after UL_SCH

Agreements:

- Modulated HARQ-ACK symbols are mapped starting on the first available non-DMRS symbol after the first DMRS symbol(s), regardless of number of DMRS symbols in PUSCH transmission.

- Modulated CSI part 1 symbols are mapped starting on the first available non-DMRS symbol, regardless of number of DMRS symbols in PUSCH transmission.

  - CSI part 1 is not mapped on the reserved HARQ-ACK REs in case of HARQ-ACK puncturing PUSCH

  - CSI part 1 is not mapped on the HARQ-ACK REs in case of HARQ-ACK rate-matching PUSCH.

- Modulated CSI part 2 symbols are mapped starting on the first available non-DMRS symbol, regardless of number of DMRS symbols in PUSCH transmission.

  - CSI part 2 can be mapped on the reserved HARQ-ACK REs in case of HARQ-ACK puncturing PUSCH.

  - CSI part 2 is not mapped on the HARQ-ACK REs in case of HARQ-ACK rate-matching PUSCH.

  - CSI part 2 is not mapped on the CSI part 1 REs.

In this contribution, a summary of companies’ view on the remaining key issues for UCI multiplexing on PUSCH is provided.

# Summary of key proposals

## FDM UCI with DMRS or not

Proposal 1a: It is clarified that based on previous agreements, when UCI is piggybacked on PUSCH, UCI is not FDMed with DMRS.

Supportive companies: QC, MTK, Intel, LGE, Samsung, Nokia, E///

Proposal 1b: When UCI is piggybacked on PUSCH, support UCI FDMed with DMRS.

Supportive companies: HW

## Reserved REs when HARQ-ACK puncture PUSCH

Proposal 2: When UE determines to transmit 0, 1, or 2 HARQ-ACK bits, the amount of reserved REs for HARQ-ACK is calculated assuming 2-bits HARQ-ACK. (Supportive companies: QC, HW, MTK, CATT, E///, Nokia)

- In case the number of HARQ-ACK bits determined at UE is less than 2, the modulated HARQ-ACK symbols are mapped to a subset of the reserved REs.

- FFS how to determine the subset

Proposal 3a: Locations of the reserved REs for HARQ-ACK are determined following the same rule defined (in RAN1 #91) for mapping modulated HARQ-ACK symbols to REs. (Supportive companies: QC, VIVO, Intel, LGE)

Proposal 3b: The mapping of reserved REs is always continuous in a frequency first method. (supportive companies: HW)

## Bug fix for amount of RE calculation for UCI

Proposal 5: It is clarified that modulated HARQ-ACK symbols are not mapped to non-DMRS symbol before the first DMRS symbol(s) (supportive companies: MTK, QC)

Proposal 6a:

- When piggybacked on PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK denoted as , CSI part 1 transmission denoted as , and CSI part 2 transmission denoted as , are determined as follows:

- When piggybacked on PUSCH with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK denoted as , CSI part 1 transmission denoted as , CSI part 2 transmission denoted as , and UL-SCH denoted as are determined as follows:

where

-	 is the number of HARQ-ACK bits;

-	is the number of CRC bits for HARQ-ACK;  if is less than or equal to 11 bits.

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	 is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the number of bits for CSI part 1;

-	 is the number of bits for CSI part 2 assuming rank1;

-	 is the number of CRC bits for CSI part 1;  if is less than or equal to 11 bits.

-         is the number of CRC bits for CSI part 2 assuming rank 1;  if is less than or equal to 11 bits.

-	 is the number of bits for CSI part 2 calculated with actual rank for CSI feedback;

-	is the number of CRC bits for .  if is less than or equal to 11 bits.

-	 is the number of elements in set , where  is the set of resource elements available for transmission of UCI in OFDM symbol , for . is an empty set and for DMRS symbol l.  is the total number of OFDM symbols of the PUSCH. REs occupied by PTRS are excluded in set .

-	is the symbol index of first available non-DMRS symbol after the first DMRS symbol(s).

Proposal 6b: A threshold parameter can be used for determining an upper bound for UCI resources when multiplexed with UL-SCH data on PUSCH or HARQ-ACK resources for HARQ-ACK+CSI on PUSCH without UL-SCH data,

- The threshold parameter is predefined in specification or can configured by RRC if feasible within the Rel-15 time frame;

  - Different values may apply for different UCI types or number of UCI bits;

  - A suitable value range is {0.6, 0.7, 0.8, 0.9}.

Eg. For HARQ-ACK piggyback on PUSCH with UL-SCH

Supportive companies: CATT

Proposal 7: When HARQ-ACK is multiplexed with a single-part CSI report on PUSCH without UL-SCH data, the CSI is not mapped to reserved HARQ-ACK REs on PUSCH. (supportive company: CATT)

Proposal 8: In step 5 in the pseudo codes to do “Data and control multiplexing” in Section 6.2.7 of 38.212, ACK puncture REs over set  instead of . (supportive company: QC)

Proposal 9: Clarify that set  is defined as a set of “physical” resource elements in PRB domain not in VRB domain. (supportive company: QC)

## Partially overlapped PUCCH and PUSCH

Proposal 10a: When PUCCH partially overlap with PUSCH, drop either PUSCH or PUCCH depends on priority ordering between PUCCH and PUSCH. (Supportive companies: QC)

- FFS: priority ordering between different UCI type carried on PUCCH and the payload type (e.g. URLLC or eMBB) carried by PUSCH.

Proposal 10b: UE maps UCI intended on PUCCH on PUSCH according to following conditions: (Supportive companies: Nokia)

- If a UE would transmit UCI on a PUCCH format 0 or 2 that has a same first symbol and duration with a PUSCH transmission, the UE multiplexes the UCI in the PUSCH transmission and does not transmit the PUCCH.

- If a UE would transmit UCI on a PUCCH format 1, 3 or 4 that has a first symbol after or at most [3] symbols before the first symbol of a PUSCH transmission, the UE multiplexes the UCI in the PUSCH transmission and does not transmit the PUCCH.

## UCI piggyback on PUSCH in UL CA

Proposal 11a: In scenario of CA, UCI is piggybacked on a single CC based on the follow rule (same as LTE). (supportive company: QC)

- In case of no aperiodic CSI triggering

  - If there is PUSCH on the UL PCC

    - UCI is mapped onto the UL PCC

  - If there is no PUSCH on the UL PCC

    - UCI is mapped onto the UL SCC with the smallest value of carrier index, i.e. SCellIndex-r10, of the associated PUCCH group

- In case of aperiodic CSI triggering

  - UCI is mapped onto the PUSCH on the CC indicated by the UL grant containing the aperiodic CSI trigger

Note: UE is not expected to receive more than one positive aperiodic CSI trigger for a given subframe

Proposal 11b: If a UE is configured for UL CA and dynamic HARQ-ACK codebook determination, a DCI format scheduling PUSCH includes a ‘HARQ-ACK multiplexing’ field of 1 bit. (supportive company: Samsung)

## Mod order determination for UCI-only PUSCH

Proposal 12a: A DCI format scheduling PUSCH includes an ‘UL-SCH indicator’ field of 1 bit. (Supportive companies: Samsung)

Proposal 12b: Reuse the NDI, RV and MCS fields in DCI to trigger CSI on PUSCH without UL-SCH data (Supportive companies: CATT)

- NDI bit is toggled compared to the previous transmission for the same HARQ process and RV is set to 0 combined with IMCS = {[28], 29, 30, 31} to indicate the modulation order for CSI on PUSCH without UL-SCH data is QPSK, 16QAM, 64QAM and [256QAM] respectively.

Proposal 12c: When piggybacked on PUSCH without UL-SCH, the modulation order of UCI is derived at UE based on coding rate. (Supportive companies: QC)

## Other issues/CRs