3GPP TSG RAN WG1 Meeting AH 1801			R1- 1801151

Vancouver, Canada, January 22nd – 26th, 2018

Agenda item:	7.2.3.3

Source: 	Qualcomm Incorporated

Title: 	Remaining details on MCS

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide a text proposal for the CQI reference section of 38.214 [1].

# Discussion

The following agreement was reached in RAN1 #91 as the UE’s assumption for CQI computation:

Agreement

UE can assume the following for CSI computation for eMBB:

Control symbols equal to 2 OFDM symbols

Number of PDSCH symbols is equal to 12

BWP PDSCH numerology

UE assumes the same BW as the BW that it reports CQI for

RV equal to 0

Configured DMRS including type and symbol

The number of front loaded DMRS symbols assumed to be same as the maximum front-loaded symbols configured by RRC

The number of additional DMRS symbols assumed to be same as the additional symbols configured by RRC

Assume TDM of DMRS and PDSCH

The above agreement was captured as follows in the current 38.214 text:

In the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, PMI and RI: - The first 2 OFDM symbols are occupied by control signaling - The number of PDSCH symbols is equal to 12. - The bandwidth part subcarrier spacing configured for the PDSCH reception - The bandwidth as configured for the PDSCH reception - The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception - No resource elements used by primary or secondary synchronization signals or PBCH. - Redundancy Version 0 - The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Subclause 4.1. - Assume no REs allocated for CSI-RS and zero-power CSI-RS - Assume the same number of front loaded DM-RS symbols as the maximum front-loaded symbols configured by the higher layer parameter DL-DMRS-max-len. - Assume the same number of additional DM-RS symbols as the additional symbols configured by the higher layer parameter DL-DMRS-add-pos. - Assume the PDSCH symbols are not containing DM-RS. - The PDSCH transmission scheme where the UE may assume that the gNB transmission on the PDSCH would be performed with up to 8 transmission layers on antenna ports [1000-1011] as defined in Subclause 7.3.1.4 of [4, TS 38.211].

Comparing the above agreement and wording in the spec we make the following observations:

| Current text | Agreement text | Proposal / Comment |
| --- | --- | --- |
| The bandwidth as configured for the PDSCH reception | UE assumes the same BW as the BW that it reports CQI for | Text proposal: “The bandwidth as configured for the corresponding CQI report” |
| No resource elements used by primary or secondary synchronization signals or PBCH. | Not in the agreement | Comment: Even though this is not in the agreement, this is reasonable to keep, similar to LTE. |
| The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Subclause 4.1. | Not in the agreement | Comment: even though this is not formally agreed, it is reasonable to keep in the 214 spec. |

Proposal 1: In the CQI assumption definition, update the wording: “The bandwidth as configured for the PDSCH reception the corresponding CQI report”.

# Performance using CP-OFDM

In our earlier contribution [3], we tested the performance of different modulation and code rate combinations and proposed a new MCS table for NR. In that contribution we reported simulation results showing that, over the EPA fading channel, better performance was obtained by lowering the code rate at which the modulation order is increase. The results are listed in Table 1. The simulations used base-graph 1 with the systematic-bit priority mapping interleaver.

Observation 1: Using CP-OFDM, performance over fading is degraded if the modulation order transition code rate is high.

Table 1 Performance of the NR LDPC coding chain over the EPA channel

# Conclusions

Proposal 1: In the CQI assumption definition, update the wording: “The bandwidth as configured for the PDSCH reception the corresponding CQI report”.

Observation 1: Using CP-OFDM, performance over fading is degraded if the modulation order transition code rate is high.

# References

- 3GPP TS 38.214 V15.0.0, NR Physical Layer Procedures for Data (Release 15).

- 3GPP TS 38.331 V1.0.1, NR Radio Resource Control (RRC) Protocol Specification (Release 15).

- R1-1720664, “Remaining details on MCS,” Qualcomm Incorporated, RAN1 91, Nov. 2017, Reno, USA.

- RAN1 91, Chairman’s notes, Nov. 2017, Reno, USA.