3GPP TSG-RAN2 Meeting #100	R2-1712236

# Reno, Nevada, United States, 27th Nov 2017 - 1st Dec 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1514 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE capability for support of SRS enhancements without support of comb 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  | Date: |  |  | 2017-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN1 has made following agreements and sent LS to RAN2 (R2-1712112, R1-1719093) to implement the signalling changes required. - RAN1 concluded that the support of comb 4 should be optional, and agreed on introducing a new optional UE capability for support of Rel-14 SRS enhancements (currently signalled by ce-SRS-Enhancement-r14) without support of comb 4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduced ce-SRS-EnhancementWithoutComb4 to indicate optional capability for support of Rel-14 SRS enhancements without support of SRS comb 4. - Clarified existing ce-SRS-Enhancement that this capability means support of SRS combs 2 and 4. - Clarified that only the existing or the new capability can be included at a time. Impact Analysis: Impacted functionality: TDD SRS coverage enhancement capability signalling Interoperability issue: If the eNB implements the CR but UE does not: No interoperability issue is foreseen as the UE can only include legacy capability bit, and thus the eNB behaviour should be based on legacy capability indication from the UE. If the eNB does not implement the CR but UE does: UE that supports other SRS coverage enhancement features but does not support SRS comb 4 includes the new capability only which the eNB cannot understand. In that case, SRS coverage enhancements cannot be applied to this UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot signal its optional capability for support of SRS enhancements without support of SRS comb 4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.70, 4.3.70x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 3127 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st change

### 4.3.4	Physical layer parameters

#### <<unchanged sections skipped>>

#### 4.3.4.70	ce-SRS-Enhancement-r14

This field indicates whether the UE supports SRS coverage enhancement with support of SRS combs 2 and 4, as specified in TS 36.213 [22]. A UE indicating support of ce-SRS-Enhancement-r14 shall also indicate support of ce-ModeA-r13 and shall not indicate support of ce-SRS-EnhancementWithoutComb4-r14.

#### 4.3.4.70x	ce-SRS-EnhancementWithoutComb4-r14

This field indicates whether the UE supports SRS coverage enhancement with support of SRS comb 2 but without support of SRS comb 4, as specified in TS 36.213 [22]. A UE indicating support of ce-SRS-EnhancementWithoutComb4-r14 shall also indicate support of ce-ModeA-r13 and shall not indicate support of ce-SRS-Enhancement-r14.

#### <<unchanged sections skipped>>

End of changes