3GPP TSG-RAN WG2 #100	Tdoc R2-1712346

Reno, Nevada, 27th November – 1st December 2017	(update of R2-1711353)

Agenda Item:	10.4.1.7.4

Source:	Ericsson

Title:	Additional SIB in EUTRAN for supporting NR SA deployments

Document for:	Discussion, Decision

# Introduction

In this contribution, we review the contents of the additional LTE SIB in order to support the UE with the cell reselection of NR in a NR SA deployment.

# Discussion

The list of LTE SIBs and their brief description is provided in Table 1.

Table 1: List of LTE SIBs

| SIB | Contents |
| --- | --- |
| SIB1 | Information needed to access the cell and scheduling information for other SIBs |
| SIB2 | Radio resource configuration common for all UEs |
| SIB3 | Common cell re-selection information |
| SIB4 | Neighbouring cell information for intra-frequency cell re-selection |
| SIB5 | Information relevant for inter-frequency cell re-selection |
| SIB6 | Information relevant for inter-RAT cell re-selection to UTRA |
| SIB7 | Information relevant for inter-RAT cell re-selection to GERAN |
| SIB8 | Information relevant for inter-RAT cell re-selection to CDMA2000 |
| SIB9 | Home eNB name |
| SIB10 | ETWS primary notification |
| SIB11 | ETWS secondary notification |
| SIB12 | CMAS notification |
| SIB13 | MBMS control information |
| SIB14 | EAB parameters |
| SIB15 | MBMS service area identities |
| SIB16 | GPS and UTC |
| SIB17 | Traffic steering between E-UTRAN and WLAN |
| SIB18 | Sidelink communication related information |
| SIB19 | Sidelink discovery related information |
| SIB20 | MBMS transmission using SC-PTM |
| SIB21 | V2X sidelink related information |

In LTE, the SIB4 provides the intra-frequency cell reselection parameters related to the neighbour cell and SIB5 provides the inter-frequency cell reselection parameters. The SIB6, SIB7, SIB8 provide the cell re-selection parameters for the UTRA, GERAN and CDMA2000 RATs. In the same way, in order to support the UE with the cell reselection towards NR, the LTE cells need to broadcast system information related to the NR cell reselection.

Adopt the additional SIB (SIB22) that contains the inter-RAT cell re-selection to NR.

- As mentioned before, the contents of the SIB5 in LTE are related to the inter frequency cell reselection within EUTRA. The parameters provided in this SIB includes a list of carriers and for each of the carrier, the SIB5 includes information related to, amongst other things,  ARFCN corresponding to the carrier, minimum required rx level (q-RxLevMin), cell reselection timer (TreselectionEutra) etc.

- In the NR specifications, there will be system information blocks corresponding to the inter frequency NR cell reselection parameters similar to SIB5 in LTE. The contents of such a system information block in NR will include:

- List of frequencies to be measured in NR carrier, their priorities and the related measurement configurations

  - Pointer to these carriers (based on the corresponding measurement objects for those carriers. How to provide this is still under discussion for the EN-DC specification finalization)

  - SMTC (SS Measurement Timing Configuration, smtc1) window as discussed for SSB measurements including the ssb-Timing and ssb-Duration related to the SS Blocks transmitted in NR frequencies.

  - Priorities for different frequencies if there are more than one NR frequencies as the neighbouring NR cell.

  - Cell quality derivation parameters like nroSS-BlocksToAverage (number of beams to be considered for cell quality derivation) and absThreshSS-BlocksConsolidation (threshold used for evaluating beam selection criterion).

- BlackListedCells per frequencies  the list of cells which cannot be used for cell re-selection on those frequencies, therefore the UE need not perform measurements. It has already been agreed that the blackListedCells is supported in NR. From RAN2#AdHoc Qingdao, we have the following agreement:

…

2:	In NR, a list of 'blacklisted' cells can be configured as part of the measurement object.

- Yet it needs to be agreed whether this list is also present in the cell re-selection related SIB in NR.

- the frequency specific offsets (agreed as part of connected mode mobility parameters and this is expected to be reused for cell reselection aspects as well)

- measurement quality evaluation metrics like QrxLevMin (yet to be discussed in NR SA discussions).

- cell reselection related parameters threshX-High, threshX-Low, reselection priorities, PMax (yet to be discussed in NR SA discussions)

- However, there are still many open issues in NR related to how to indicate a cell in a carrier and which cell reselection parameters to be borrowed from LTE as is, which parameters are removed and which additional parameters are introduced newly in NR. Therefore, one could say for the time being that the contents of the SIB22 could be filled in once the inter frequency cell reselection parameters are settled for the stand alone NR cells.

Contents of the SIB22 is based on the inter-frequency cell reselection related system information block in NR.

The SIB22 should include, at least the following parameters that have already been agreed to be included in NR RRC specification;

  - List of frequencies supported in NR and their priorities

  - SMTC window (smtc1) per each frequency as has been agreed for SSB measurements including the ssb-Timing and ssb-Duration.

  - Cell measurement parameters like nroSS-BlocksToAverage and absThreshSS-BlocksConsolidation

  - BlackCellsList

  - Frequency specific offsets

# Conclusion

Based on the discussion in section 2 we propose the following:

- Adopt the additional SIB (SIB22) that contains the inter-RAT cell re-selection to NR.

- Contents of the SIB22 is based on the inter-frequency cell reselection related system information block in NR.

- The SIB22 should include, at least the following parameters that have already been agreed to be included in NR RRC specification;

  - List of frequencies supported in NR and their priorities

  - SMTC window (smtc1) per each frequency as has been agreed for SSB measurements including the ssb-Timing and ssb-Duration.

  - Cell measurement parameters like nroSS-BlocksToAverage and absThreshSS-BlocksConsolidation

  - BlackCellsList

  - Frequency specific offsets

# References

RP-171485, “Work Item on New Radio (NR) Access Technology”

R2-1708162” System information structure and contents”, Ericsson

R2-1708157, “Public Warning System in NR”, Ericsson, NTT DOCOMO INC., SoftBank