3GPP TSG-RAN WG2 #100	R2-1712354

Reno, USA, November 27th - December 1st 2017

Agenda item:	10.4.1.8

Source:	Nokia, Nokia Shanghai Bell

Title:	Access Barring Control for RRC_INACTIVE

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In the light of RAN2 NR #2 agreements, unified access control requires detailed considerations on applicability to different UE states:

Agreements

1	RAN2 aims that the 5G AC mechanism for a UE in RRC_IDLE is applicable to a UE in RRC_INACTIVE.

FFS if any aspects may not be applicable or may need to be changed for RRC_INACTIVE relative to RRC_IDLE (to be addressed by both CT1 and RAN2).

2	RAN2 aims to define the 5G AC mechanism for a UE in RRC_CONNECTED. Details FFS

3	UE NAS provides the access category information to UE RRC at least for RRC_IDLE

FFS for RRC_INACTIVE

This contribution discusses applicability of unified access control to RRC_INACTIVE UE state and elaborates differentiation in RRC role for NR access control for the state.

# 2	Discussion

## 2.1	Access Barring for RRC_INACTIVE

Given similar characteristics of RRC_INACTIVE with RRC_IDLE and contents of an RRC connection request message handling in terms of Access Barring handling could be a reasonable commonality. I.e. Access Barring procedures would have to precede initiation of RRC procedures that are triggered for INACTIVE to CONNECTED transition.

The access barring configuration per Access Category is provided with System Information. The UE acquires and have this information (i.e. configuration) available at Access Stratum layer. An access attempt is categorized into an Access Category. If the Access Category specific broadcast configuration allowed the Access Category to be served, it results in successful connection establishment (or resume type). Otherwise, the UE’s request is barred. Overall, the UE’s AS layer performs the barring check.

Proposal 1: Access Barring check for RRC_IDLE is a baseline for Access Barring check design for RRC_INACTIVE

## 2.2	Access Category for RRC_INACTIVE

However, as discussed in [2] handling of a suspended connection of UE in INACTIVE would require access decision for a connection that had assigned certain Access Category, but this Access Category is not refreshed. Access Category determination for that connection resume may be realized by two different approaches:

- certain coordination between UE’s NAS layer and AS layer for provision of the Access Category

- E.g. when initiating the RRC Connection Resume type procedure, the AS layer will coordinate with NAS the current/refreshed assessment for the access type

- RRC layer internal action for provision of the Access Category

- E.g. When initiating the RRC Connection Resume type procedure, the AS layer will apply:

  - either the stored the stored Access Category associated with the previously established connection (but suspended), or

  - Access Category assessment for the session to be resumed, the AS layer may request NAS layer to provide assistance information.

Regardless of the undertaken approach, the RRC layer needs to take an action to trigger categorization of the resumed RRC connection. By this mean, Access Category determination applies differently to RRC Inactive UE state:

Proposal 2: UE AS initiates provision of Access Category RRC Inactive UE state.

# 3	Conclusions

This contribution has discussed access barring control for RRC_INACTIVE and has made the following proposals:

Proposal 1: Access Barring check for RRC_IDLE is a baseline for Access Barring check design for RRC_INACTIVE

Proposal 2: UE AS initiates provision of Access Category RRC Inactive UE state.

Proposal 3: Agree Text Proposal provided below.

# Text Proposal to 38.300

Modified Subclause

## 7.4	Access Control

NG-RAN should support overload and access control functionality such as RACH back off, RRC Connection Reject, RRC Connection Release and UE based access barring mechanisms.

One unified access barring mechanism for NR should be introduced to address all the use cases and scenarios that E-UTRA addressed with different specialized mechanisms. The unified access barring mechanism should be forward compatible in order to cope with future use cases/scenarios.

In RRC_IDLE, the UE NAS informs RRC of the access category and the Connection Request includes some information to enable the gNB to decide whether to reject the request.

In RRC_INACTIVE, the UE RRC triggers provision of Access Category.

End of Modified Subclause

# References

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage 2 (Release 15), v0.2.1, RAN2

- R2-1712352, Access Control applicability to different RRC states, Nokia, Nokia Shanghai Bell