3GPP TSG-RAN WG2 #100                                                                                   R2-1712428

Reno, USA, 27th November – 1st December 2017	Resubmission of R2-1707911

Source:	CATT

Title:	Service types in Inactive Mode

Agenda Item:	10.4.4.5

Document for:	Discussion and Decision

Introduction

According to [1], three levels of services are provided while a UE is in RRC_IDLE, i.e. limited service, normal service and operator service. Cells are categorized into acceptable cell and suitable cell. We also agreed that RAN2 assume that the UE in RRC_Inactive can perform RAN paging area updates and answer to RAN paging in a “non allowed area”. However, the corresponding section on service types in inactive mode is still FFS. In this contribution, we would like to discuss this open issue.

Discussion

When a UE is in inactive mode, it is unavoidable that the UE has to select an acceptable cell, i.e. the cell selection criteria is fulfilled and the cell is not barred, but the cell is not served by the selected PLMN, registered PLMN, or a PLMN of the equivalent PLMN list. The first thing we need to discuss is the behavior when a UE select an acceptable cell.

When the inactive UE selects on an acceptable cell, there are two options for the UE:

Option 1: When the inactive UE selects on an acceptable cell, the UE would keep in inactive state and try to find a suitable cell.

Option 2: When the inactive UE selects on an acceptable cell, the UE would switch to idle state and try to find a suitable cell.

Camped on any cell state is temporary and having limited service (e.g. emergency calls) in the state is not a common case. With option 1, the UE can stay in inactive state when it finds a suitable cell later. The signaling overhead can be saved if it communicates with the network after returning back to a suitable cell. Option 1 is preferred.

Proposal 1: Camping on an acceptable cell is supported for UE in RRC inactive state.

Proposal 2: When the inactive UE selects on an acceptable cell, the UE would keep in inactive state and try to find a suitable cell

However, if the UE camps on an acceptable cell and keeps in inactive state, RNA of the acceptable cell is different from that stored in the inactive UE in most cases. Is an RNA update procedure needed? In our understanding, when a UE camping on an acceptable cell, only limited services (e.g. originate emergency calls, and receiving ETWS and CMAS notifications) are allowed. Therefore, RNA update procedure is not allowed when an inactive UE camping on an acceptable cell. How to enable ETWS and CMAS notifications in NR is FFS.

Proposal 3: When the inactive UE is camped on an acceptable cell, only limited services are allowed and RNA update is not allowed.

Based on the above analyze, we proposed that service types in inactive mode are same as that in idle mode.

Proposal 4: Service types in inactive mode are same as that in idle mode.

Conclusion

In this contribution, we discuss some details about cell selection and cell reselection in NR and propose:

Proposal 1: Camping on an acceptable cell is supported for UE in RRC inactive state.

Proposal 2: When the inactive UE selects on an acceptable cell, the UE would keep in inactive state and try to find a suitable cell.

Proposal 3: When the inactive UE is camped on an acceptable cell, only limited service is allowed and RNA update is not allowed.

Proposal 4: Service types in inactive mode are same as that in idle mode.

Reference

[1]  TS 38.304 v0.2.0