3GPP TSG-RAN WG2 #100	Tdoc R2-1712460

Reno, Nevada, USA, 27th November – 1st December 2017

Agenda Item:	9.15  Highly Reliable Low Latency Communication for LTE

Source:	Ericsson

Title:	Reliable HARQ feedback for PHICH-less UL SPS

Document for:	Discussion, Decision

# Introduction

In the LS from RAN1 [1], for URLLC, “Reliability” is defined as the success probability R of transmitting X bits within L seconds, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface, at a certain channel quality Q (e.g., coverage-edge). Moreover, the latency bound L includes transmission latency, processing latency, retransmission latency and queuing/scheduling latency (including scheduling request and grant reception if any). In addition, two targeted reliability and latency requirements are agreed be evaluated:

- URLLC for LTE should target the requirement defined by ITU, i.e., 10-5 error probability in transmitting a layer 2 PDU of 32 bytes within 1 ms. Additional less stringent requirements can be considered. - In addition to (10-5, 1ms, 32 bytes packet), URLLC for LTE should target the requirement of 10-4 error probability in transmitting a layer 2 PDU of 32 bytes within 10 ms.

In this paper, we discuss an error case due to the lack of PHICH in UL SPS. This error may lead to that the URLLC requirement is not satisfied and we then propose a reliable HARQ feedback mode.

# Discussion

For UL SPS sTTI, it has been agreed that “PHICH-less asynchronous UL HARQ for sPUSCH is supported” and “RAN2 sees benefits to support SPS for short TTI”. In addition, RAN1 agreed that “PHICH-less asynchronous HARQ for UL is used for 1ms TTI with shortened processing time”. These latency-reduction techniques are essential tools and building blocks to further increase the reliability for URLLC based data transmissions since more retransmissions can possibly happen within a restricted latency-budget.

PHICH-less UL SPS for short TTI and 1ms TTI with short processing time can be used for URLLC.

In UL SPS transmission with SkipUplink being configured in LTE, eNB does not expect a transmission in every resource specified by the configured grant, and thus does not react if a transmission is not detected at eNB. In order to cover the potential error case that the not-detected transmission attempt was actually a non-decodable transmission, LTE rel-14 assumes that the eNB reacts to that with sending a NACK on PHICH, and only sends an ACK on PHICH if an UL MAC PDU is correctly received.  In other words, an eNB can be expected to always sends NACK on PHICH unless an MAC PDU is successfully received. On the UE side, UE only inspects PHICH channel after it initiates an UL transmission, and acts accordingly.

Considering the high reliability requirements of URLLC data transmissions and noting that the PHICH channel will not be supported for sTTI based uplink transmissions, we need to investigate how to support asynchronous UL HARQ without PHICH for the case of SPS for short TTI and 1ms TTI with short processing time. If SkipUplink is configured and there is no PHICH channel, then eNB does not react if a transmission is not detected. Thus, UE cannot be certain whether the initial UL transmission has been correctly detected or not.

For an UL SPS without PHICH and configured with SkipUplink, UE cannot be certain whether the UL transmission has been correctly detected.

For an UL SPS without PHICH and configured with SkipUplink, RAN2 should ensure the reliability performance considering that HARQ feedback can be lost.

One solution is to perform an autonomous retransmission if no feedback is received from the eNB within a pre-determined time window.

The concept of an additional timer for this pre-determined time window was proposed for LTE LAA AUL and for NR UL SPS. But this is not strictly necessary in LTE UL SPS. There is already a formula to derive the HARQ ID for UL SPS. The autonomous retransmission can be on the next time occasion when the same HARQ ID is configured to transmit. Note that the HARQ ID formula has already considered the eNB processing time.

The error case to cover is that the initial transmission is miss-detected at the eNB. Incremental-combining scheme with a different redundant version does not help since the later received RV 2, 3 and 1 are not self-decodable, and we should re-transmit with the same RV 0.

After a transmission of a HARQ process in UL SPS, the eNB can configure the UE to, if no feedback has been received until the time when the same HARQ ID is configured to transmit, then UE re-transmits the MAC PDU on the configured UL grant with the same RV 0.

# Conclusion

In section 2 we made the following observations:

Observation 1	PHICH-less UL SPS for short TTI and 1ms TTI with short processing time can be used for URLLC.

Observation 2	For an UL SPS without PHICH and configured with SkipUplink, UE cannot be certain whether the UL transmission has been correctly detected.

Based on the discussion in section 2 we propose the following:

Proposal 1	For an UL SPS without PHICH and configured with SkipUplink, RAN2 should ensure the reliability performance considering that HARQ feedback can be lost.

Proposal 2	After a transmission of a HARQ process in UL SPS, the eNB can configure the UE to, if no feedback has been received until the time when the same HARQ ID is configured to transmit, then UE re-transmits the MAC PDU on the configured UL grant with the same RV 0.

# References

R1-1719210, LS on Ultra Reliable Low Latency Communication for LTE