3GPP TSG-RAN WG2 #100	Tdoc R2-1712521

Reno, Nevada, USA, 27th Nov – 1st Dec, 2017

Agenda Item:	10.2.20

Source:	Ericsson

Title:	TP on 38.300 for RNA Configuration

Document for:	Decision

# Introduction

In RAN2 #99bis a response LS to a RAN3 LS on definition of RAN Notification Area in inactive state was agreed ([1],[2]). In the response, RAN2 indicated endorsement of that RNA should be possible to be defined as either a list of cells, either through RAN Area Codes or via a list of TAI’s.

This contribution include a text proposal to 3GPP TS 38.300 to reflect these three methods.

# Discussion

Adopt the text proposal for inclusion in 3GPP TS 38.300

# Text Proposal

#### 9.2.2.3	RAN-Based Notification Area

The RNA for a UE is configured by NG-RAN according to one of the following:

- A list of cells

- A list of RAN Area ID's, defined as TAI + a RAN Area Code (RANAC)

- List of TAI

The RNA for a UE shall only include cells that are also part of the UE CN Registration Area.

If RNA is identified through a list or RAN Area ID’s, TAI + RANAC shall be broadcast in cells belonging to the RNA. Only one RANAC is broadcast in a cell.

NG-RAN may provide different RNA definitions to different UE’s and UE’s shall support all options

FFS – Size of RANAC

# Conclusion

Based on the discussion in section 2 and 3 we propose the following:

Proposal 1	Adopt the text proposal for inclusion in 3GPP TS 38.300

# References

R2-1712006, Reply LS on definition of RAN Notification Area in inactive state, RAN2, 3GPP TSG RAN WG2#99bis, Prague, Czech Republic, 9 - 13 October, 2017

R2-1710036, LS on definition of RAN Notification Area in inactive state (R3-173427), RAN3, 3GPP TSG RAN WG2#99bis, Prague, Czech Republic, 9 - 13 October, 2017