3GPP TSG-RAN WG2 #100	Tdoc R2-1712531

Reno, Nevada, USA, Nov 27th – Dec 1st 2017	(Revision of R2-1710478)

Agenda Item:	10.4.1.8

Source:	Ericsson

Title:	Access Control for NR

Document for:	Discussion, Decision

# Introduction

This contribution is an update of R2-1710478 [1]. The main changes are:

- More details on the Access category selection assistance information

At the SA1#79 meeting, a CR [3] on stage-1 requirements for unified access control for 5G to TS 22.261 [4] was agreed and SA1 sent a reply LS [2] to e.g. RAN2 to take appropriate actions. In this paper, we, based on these requirements, propose a functional division between AS and NAS for access control.

Further, at the CT1#106 meeting, CT1 sent an LS [7] to SA1 and RAN2, with questions about requirements on unified access control and their implications.

# Discussion

## Analysis of stage-1 requirements

The stage-1 requirements for unified access control now specifies the following access categories (Table 1):

Table 1: Access Categories, from [3]

| Access category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 (NOTE 1) | All | MO signalling resulting from paging |
| 1 (NOTE 2) | One or some of Access Classes 11-15 are set. At least one of them is valid in the registered PLMN and justified its priority handling by the registered PLMN with regards to access control. | All |
| 2 (NOTE 3) | UE is configured for delay tolerant service and subject to access control for access category 2, which is judged based on relation of UE’s HPLMN and the registred PLMN. | All |
| 3 | All except for the cases of access categories 1-2. | Emergency |
| 4 | All except for the cases of access categories 1-2. | MO signalling |
| 5 | All except for the cases of access categories 1-2. | MMTEL voice |
| 6 | All except for the cases of access categories 1-2. | MMTEL video |
| 7 | All except for the cases of access categories 1-2. | SMS |
| 8 | All except for the cases of access categories 1-2. | MO data that do not belong to any other access categories |
| 9-31 |  | Reserved standardized access categories |
| 32-63 | All except for the cases of access categories 1-2 and except for roaming-UEs | Based on operator classification |
| NOTE 1: Access category 0 is not barred. NOTE 2: Access Classes 11 and 15 are valid in Home PLMN only if the EHPLMN list is not present or in any EHPLMN. Access Classes 12, 13 and 14 are valid in Home PLMN and visited PLMNs of home country only. For this purpose the home country is defined as the country of the MCC part of the IMSI. If the barring control information contains flag for “unbarred” for at least one of these valid Access Classes, all access attempts from the UE require priority handling and fall into access category 1. Otherwise the UE does not require priority handling with regards to access control and other access categories apply. Access category 1 is not barred. NOTE 3: The barring parameter for access category 2 is accompanied with information on whether the access control applies to UEs registered in UE’s HPLMN/EHPLMN, the most preferred VPLMN, or other PLMNs. |  |  |

From analyzing the stage-1 requirements, we observe:

Access control is applied in all RRC states (RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED) at the time of initiating a new access attempt (e.g. new session request).

The stage-1 requirements for the unified access control expect that one access attempt is assigned one access category only.

A given access attempt will be detected by the layer having the information about the classification of the access attempt (e.g. 5GMM, 5GSM and MMTEL).

The access category is determined by the layer/entity having the information about the classification of the access attempt which identified the access attempt.

A given access category may be used for access attempts of more than one layer: e.g. SMS, which may be SMS over IP or SMS over NAS – which are different layers.

In order to enable determination of applicability of access category 1, “Access category selection assistance information” is needed to indicate whether the access category 1 is applicable to a UE and for each of the access classes 11-15 configured in the UICC.

In order to enable determination of applicability of access category 2, “Access category selection assistance information” is needed to indicate whether the access category 2 is applicable to a UE configured for delay tolerant service and registered in UE’s HPLMN/EHPLMN, the most preferred VPLMN, and/or other PLMNs.

## Access category selection assistance information

For the “Access category selection assistance information” for access categories 1-2, our understanding is that this information would need to be provided by system information broadcast in the RRC layer. The Access category selection assistance information consists of the following:

1. Information whether the access category 1 is applicable to:

1)	a UE with access classes 11 configured in UICC;

2)	a UE with access classes 12 configured in UICC;

3)	a UE with access classes 13 configured in UICC;

4)	a UE with access classes 14 configured in UICC;

5)	a UE with access classes 15 configured in UICC; or

6)	any combination of these; and

2. Information whether the access category 2 is applicable to:

1)	a UE configured for delay tolerant service and registered in UE’s HPLMN or UE's EHPLMN;

2)	a UE configured for delay tolerant service and registered in UEs' most preferred VPLMN;

3)	a UE configured for delay tolerant service and registered in a PLMN other than UE’s HPLMN, other than UE's EHPLMN and other than UEs' most preferred VPLMN; or

4)	any combination of these.

Access category selection assistance information for access categories 1 and 2 is provided to the UE by system information broadcast in the RRC layer.

## Functional division between AS and NAS

Based on the above analysis of the stage-1 requirements on access control, it is quite clear that:

- Since several layers (NAS layers, MMTEL and possibly also RRC [5]) may identify an access attempt and determine the access category, the common denominator would be the RRC layer

- As previously agreed, we think that the RRC layer should perform the actual barring check, when requested so by upper layers (or possibly by the RRC layer itself for any RRC-initiated access attempts)

In LTE, an access category is associated with a timer, that is started upon a barred access attempt using that category, which blocks further access attempts and when the timer expires the UE map perform a new barring check for an access attempt using the same category. A corresponding timer would also be benefical for the unified access control. Since several layers may use the same access category, it is natural that the timer is managed by the same layer where the actual barring check is performed. The common denominator is the RRC layer where the access barring information is available and the barring check can be performed, and RRC should therefore also manage the timer. We propose:

The RRC layer performs the barring check and also manages the timer associated with each access category

Barring check is done by RRC based on request from NAS, against the barring timer (if already running) and the access barring information provided by RAN.

In the figure below we attempt to illustrate the information flow between the layers, to perform access barring check when an access attempt is about to start:

Figure 1: AS-NAS interface for barring check when an access attempt is about to start

It should be noted that the above information flow is used for access barring check when an access attempt was identified. Access attempts (such as establishing an MMTEL voice call) may be triggered at any time, also when already in RRC_CONNECTED. Therefore the information flow for performing access barring check for an identified access attempt is separate from the actual request to setup a NAS signalling connection.

When NAS requests the RRC layer to setup an an RRC connection used by a NAS signalling connection, RRC layer should assume that the barring checks has already been performed.  We propose:

At the moment when NAS requests setup of NAS signalling connection, RRC can assume that all the barring checks were already done and passed.

Above we identified the need of Access category selection assistance information for access categories 1-2. This information is provided by RAN to the UE using system information broadcast in the RRC layer. However, the information is also needed by NAS in order to determine the access category for an access attempt. Thefore we need another information flow for the passing of this information from AS to NAS:

Figure 2: AS-NAS interface for providing access category selection assistance information

We propose:

The RRC layer provides the access category selection assistance information to NAS.

In the LS from CT1 [7], it is also noted the need for this type of assistance information. From the CT1 LS:

“Question 9: Will the NR RRC layer provide the part of the barring control information related to determination of access category 1 and access category 2 (as indicated in NOTE 2 and NOTE 3 of Table 6.22.2-1 of TS 22.261) to the layers(s) in charge of access category decision?”

We propose to answer “YES” this this question. A draft reply LS to CT1 is provided in [6].

The AS-NAS interface proposed in this contribution needs to be further developed together with CT1.

In an associated contribution [5], we propose to apply access control at least some RRC-initiated signalling procedures, such as RRC Resume triggered by RAN area update. For these particular procedures, the access attempt and access categories would need to be identified determined by the RRC layer. We propose:

For the UE-initiated signalling procedures triggered by the RRC layer subject to access control, the RRC layer shall identify an access attempt and determine the access category for that attempt.

# Conclusion

In section 2 we made the following observations:

Observation 1	Access control is applied in all RRC states (RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED) at the time of initiating a new access attempt (e.g. new session request).

Observation 2	The stage-1 requirements for the unified access control expect that one access attempt is assigned one access category only.

Observation 3	A given access attempt will be detected by the layer having the information about the classification of the access attempt (e.g. 5GMM, 5GSM and MMTEL).

Observation 4	The access category is determined by the layer/entity having the information about the classification of the access attempt which identified the access attempt.

Observation 5	A given access category may be used for access attempts of more than one layer: e.g. SMS, which may be SMS over IP or SMS over NAS – which are different layers.

Observation 6	In order to enable determination of applicability of access category 1, “Access category selection assistance information” is needed to indicate whether the access category 1 is applicable to a UE and for each of the access classes 11-15 configured in the UICC.

Observation 7	In order to enable determination of applicability of access category 2, “Access category selection assistance information” is needed to indicate whether the access category 2 is applicable to a UE configured for delay tolerant service and registered in UE’s HPLMN/EHPLMN, the most preferred VPLMN, and/or other PLMNs.

Based on the discussion in section 2 we propose the following:

Proposal 1	Access category selection assistance information for access categories 1 and 2 is provided to the UE by system information broadcast in the RRC layer.

Proposal 2	The RRC layer performs the barring check and also manages the timer associated with each access category

Proposal 3	Barring check is done by RRC based on request from NAS, against the barring timer (if already running) and the access barring information provided by RAN.

Proposal 4	At the moment when NAS requests setup of NAS signalling connection, RRC can assume that all the barring checks were already done and passed.

Proposal 5	The RRC layer provides the access category selection assistance information to NAS.

Proposal 6	For the UE-initiated signalling procedures triggered by the RRC layer subject to access control, the RRC layer shall identify an access attempt and determine the access category for that attempt.

# References

R2-1710478, Access control for NR, Ericsson, RAN2 WG2#99bis, Prague, Czech Republic, 9th – 13th October 2017

R2-1710062 / S1-173552, Reply LS on unified Access Control for 5G NR, 3GPP SA1

S1-173548, Clarification on access control requirement, NTT DOCOMO, Ericsson, 3GPP TSG-SA WG1 Meeting #79, Guilin, China, 21 - 25 August 2017

3GPP TS 22.261, Service requirements for the 5G system, v15.2.0, 2017-09

R2-1712532, Access Control for RRC-initiated Access Attempts, Ericsson, RAN2 WG2#100, Reno, Nevada, USA, Nov 27th – Dec 1st 2017

R2-1712534, [DRAFT] Reply LS on requirements on unified access control for 5GS, Ericsson, RAN2 WG2#100, Reno, Nevada, USA, Nov 27th – Dec 1st 2017

R2-1712105 / C1-174626, LS on requirements on unified access control for 5GS, 3GPP CT1, RAN2 WG2#100, Reno, Nevada, USA, Nov 27th – Dec 1st 2017