3GPP TSG-RAN WG2 #100	R2-1712550

Reno, Nevada, USA, 27 November-1 December 2017

Source: 	Huawei, HiSilicon

Title: 	ASN.1 structure for L3 filter configuration

Agenda Item:	10.4.1.4.2

Document for:	Discussion

# Introduction

In email discussion [99bis#22], several companies expressed an interest in supporting different filter coefficients for different measurement objects ([2], Q7 and section 3.1 point 3).  This would mean that multiple indexed instances of QuantityConfig are provided inside MeasConfig, and each MeasObjectNR includes an index to the appropriate QuantityConfig.  This document provides a text proposal for capturing such a structure in the ASN.1.

Our consideration is that considering the range of frequencies in NR, at least for low vs. high frequencies (e.g., RAN4’s FR1/FR2 ranges) it is likely that different coefficients would be needed; for instance, these frequencies would be expected to have significant differences in channel coherence time as mentioned in the email discussion.  However, we normally avoid explicit frequency dependencies in RAN2 specifications, so it would be out of our usual practice to have hardcoded “LF coefficients” and “HF coefficients” (it would also be awkward to specify clearly).  Further, we do not see a major impact to UE implementation complexity to support flexible coefficients, so the flexibility appears as a worthwhile tradeoff.

This proposal introduces a new IE quantityConfigList into the MeasConfig with no delta signalling, i.e. the list is provided in full every time.  We anticipate that the list would normally be relatively short so this creates no great burden.  As an alternative, especially if the list is anticipated to be larger, it would be possible to introduce using the same “ToRemove/ToAddMod” pattern as the other lists in MeasConfig.

# Text proposal

MeasConfig information element

-- ASN1START

-- TAG-MEAS-CONFIG-START

MeasConfig ::=							SEQUENCE {

-- Measurement objects

measObjectToRemoveList					MeasObjectToRemoveList											OPTIONAL,

measObjectToAddModList					MeasObjectToAddModList											OPTIONAL,

-- Reporting configurations

reportConfigToRemoveList				ReportConfigToRemoveList										OPTIONAL,

reportConfigToAddModList				ReportConfigToAddModList										OPTIONAL,

-- Measurement identities

measIdToRemoveList						MeasIdToRemoveList												OPTIONAL,

measIdToAddModList						MeasIdToAddModList												OPTIONAL,

-- Quantity configurations

quantityConfigList						QuantityConfigList												OPTIONAL,

-- Other parameters

-- s-Measure config

s-MeasureConfig							CHOICE {

ssb-rsrp								RSRP-Range,

csi-rsrp								RSRP-Range

} 																										OPTIONAL,

-- Placehold for measGapConfig

measGapConfig							MeasGapConfig													OPTIONAL

}

MeasObjectToRemoveList ::=				SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=					SEQUENCE (SIZE (1..maxMeasId)) OF MeasId

ReportConfigToRemoveList ::=			SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

QuantityConfigList ::=					SEQUENCE (SIZE (1..maxQuantityConfigId)) OF QuantityConfig

-- TAG-MEAS-CONFIG-STOP

-- ASN1STOP

MeasObjectNR information element

-- ASN1START

-- TAG-MEAS-OBJECT-NR-START

MeasObjectNR ::=							SEQUENCE {

carrierFreq									ARFCN-ValueNR,

--RS configuration (e.g. SMTC window, CSI-RS resource, etc.)

referenceSignalConfig						ReferenceSignalConfig														OPTIONAL,

--Consolidation of L1 measurements per RS index

absoluteThresholdCellQuality			SEQUENCE {

absThreshSS-BlocksConsolidation			ThresholdNR																	OPTIONAL,

absThreshCSI-RS-Consolidation			ThresholdNR																	OPTIONAL

}																														OPTIONAL,

-- Filter coefficients applicable to this measurement object

quantityConfigIndex						INTEGER (1..maxQuantityConfigId),

--Config for cell measurement derivation

maxBeamsCellQuality						SEQUENCE {

nroSS-BlocksToAverage					INTEGER (1..maxNroSS-BlocksToAverage)										OPTIONAL,

nroCSI-RS-ResourcesToAverage			INTEGER (1..maxNroCSI-RS-ResourcesToAverage)								OPTIONAL

}

OPTIONAL,

--Frequency-specific offsets (only for events A3, A6)

offsetFreq									Q-OffsetRangeList,

-- Cell list

cellsToRemoveList							CellIndexList																OPTIONAL,

cellsToAddModList							CellsToAddModList															OPTIONAL,

-- Black list

blackCellsToRemoveList						CellIndexList																OPTIONAL,

blackCellsToAddModList						BlackCellsToAddModList														OPTIONAL,

-- White list

whiteCellsToRemoveList						CellIndexList																OPTIONAL,

whiteCellsToAddModList						WhiteCellsToAddModList														OPTIONAL

}

-- TODO: To be updated based on email discussion[99#31][NR] Additional information for SSB and CSI-RS config (Ericsson)

ReferenceSignalConfig::=     			 SEQUENCE {

ssb-MeasurementTimingConfiguration		SSB-MeasurementTimingConfiguration										OPTIONAL,

ssbPresence								CHOICE {

present									SEQUENCE {

frequencyOffset							TYPE_FFS!

-- FFS: How to give offset and timing if there are multiple SSBs on a carrier, or coding details details of the offset

},

notPresent								SEQUENCE {

-- FFS: How to inform the UE where else to find the SSB. FFS whether to indicate here a carrier or a cell ID or multiple cell IDs

},

}

-- CSI-RS resources to be used for for CSI-RS based RRM measurements

csi-rs-ResourceConfig-Mobility			SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM))	OF CSI-RS-ResourceConfig-Mobility	OPTIONAL -- Need N

-- Indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell:

useServingCellTimingForSync				BOOLEAN

}

-- A measurement timing configuration

SSB-MeasurementTimingConfiguration ::= 	SEQUENCE {

-- Primary measurement timing configuration. Applicable for intra- and inter-frequency measurements.

smtc1									SEQUENCE {

-- Periodicity and offset of the measurement window in which to receive SS/PBCH blocks.

-- Periodicity and offset are given in number of subframes.

-- (see 38.213, section REF):

periodicityAndOffset					CHOICE {

sf5										INTEGER (0..4),

sf10									INTEGER (0..9),

sf20									INTEGER (0..19),

sf40									INTEGER (0..39),

sf80									INTEGER (0..79),

sf160									INTEGER (0..159)

},

-- Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes

-- (see 38.213, section 4.1)

-- FFS: RAN1 discusses additional allowed durations:

duration								ENUMERATED { sf1, sf5 }

},

-- Secondary measurement timing confguration for explicitly signalled PCIs. It uses the offset and duration from smtc1.

-- It is supported only for intra-frequency measurements in RRC CONNECTED.

smtc2 									SEQUENCE {

-- PCIs that are known to follow this SMTC.

pci-List								SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysicalCellId		OPTIONAL

-- Periodicity for the given PCIs. Timing offset and Duration as provided in smtc1.

periodicty							TYPE_FFS!

}																										OPTIONAL -- Cond IntraFreqConnected

}

CSI-RS-ResourceConfig-Mobility ::= 		SEQUENCE {

csi-rs-ResourceId-RRM					CSI-RS-ResourceId-RRM,

cellId									PhysicalCellId,

-- subcarrier spacing of CSI-RS. It can take the same values available also for the data channels and for SSB

subcarrierSpacing						SubcarrierSpacing,

-- Contains periodicity and slot offset for periodic/semi-persistent CSI-RS (see 38.211, section x.x.x.x)

-- FFS: Whether ms80 and ms160 are supported

slotConfig					CHOICE {

ms5										INTEGER (0..4),

ms10									INTEGER (0..9),

ms20									INTEGER (0..19),

ms40									INTEGER (0..39),

ms80									INTEGER (0..79),

ms160									INTEGER (0..159)

},

-- Number of ports for CSI-RS (see 38.211, section x.x.x.x)

nrofAntennaPorts						ENUMERATED{X,...},

-- Resource Element mapping pattern for CSI-RS (see 38.211, section x.x.x.x)

resourceElementMappingPattern			TYPE_FFS!,

-- TODO: ADD DESCRIPTION (see 38.211, section x.x.x.x)

csi-rs-TransmissionBW					ENUMERATED {x,y,z, ...},

-- TODO: ADD DESCRIPTION (see 38.215, section x.x.x.x)

csi-rs-MeasurementBW					ENUMERATED {x,y,z, ...},

-- TODO: ADD DESCRIPTION (see 38.211, section x.x.x.x)

sequenceGenerationConfig				TYPE_FFS!

}

CSI-RS-ResourceId-RRM ::= 				INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

Q-OffsetRangeList ::=					SEQUENCE {

rsrpOffsetSSB							Q-OffsetRange				DEFAULT dB0,

rsrqOffsetSSB							Q-OffsetRange				DEFAULT dB0,

sinrOffsetSSB							Q-OffsetRange				DEFAULT dB0,

rsrpOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0,

rsrqOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0,

sinrOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0

}

CellsToAddModList ::=					SEQUENCE (SIZE (1..maxCellMeas)) OF CellsToAddMod

CellsToAddMod ::=						SEQUENCE {

cellIndex								INTEGER (1..maxCellMeas),

physCellId								PhysCellId,

cellIndividualOffset					Q-OffsetRangeList

}

BlackCellsToAddModList ::=				SEQUENCE (SIZE (1..maxCellMeas)) OF BlackCellsToAddMod

BlackCellsToAddMod ::=					SEQUENCE {

cellIndex								INTEGER (1..maxCellMeas),

physCellIdRange							PhysCellIdRange

}

WhiteCellsToAddModList ::=				SEQUENCE (SIZE (1..maxCellMeas)) OF WhiteCellsToAddMod

WhiteCellsToAddMod ::=					SEQUENCE {

cellIndex								INTEGER (1..maxCellMeas),

physCellIdRange							PhysCellIdRange

}

-- TAG-MEAS-OBJECT-NR-STOP

-- ASN1STOP

Proposal 1: Adopt the ASN.1 structure shown to describe filter coefficients per measurement object.

# Conclusion

Proposal 1: Adopt the ASN.1 structure shown to describe filter coefficients per measurement object.

# References

[1] R2-1703386, Cell quality derivation from multiple beam quality, Huawei/HiSilicon, RAN2#97bis

[2] R2-1704874, Measurement configuration for Idle RS, Huawei/HiSilicon, RAN2#98