3GPP TSG-RAN WG2 Meeting #100 	R2-1712695

# Reno, Nevada, USA, 27 November – 1 December 2017

Agenda item:	10.2.9

Source:		Intel Corporation

Title:		Support of measurements on legacy RATs for EN-DC

Document for:	Discussion and Decision

# Introduction

RAN2 specifications support inter-rat measurements on legacy RATs.  However, RAN4 agreements do not allow this for UEs configured for NSA operation.  This document discusses the topic in more detail

# Discussion

RAN2 LTE specifications support inter-RAT measurements of legacy technologies, UTRA, GERAN, CDMA2000 HRPD or CDMA2000 1xRTT frequencies while UE is connected in LTE.

However, as per RAN4 agreements captured below:

When UE is configured with EN-DC, measurements of legacy technologies (UTRA, GERAN, CDMA2000 HRPD or CDMA2000 1xRTT) is not supported.  It implies that Inter-RAT connected mode mobility related features such as connected mode CSFB, SRVCC or inter-RAT HO to legacy RATs cannot use legacy RAT measurements.

Observation #1: Based on RAN4 agreement, inter-RAT connected mode mobility related features such as CSFB, SRVCC or inter-RAT HO to legacy RATs cannot use legacy RAT measurements when EN-DC

Proposal #1: It is proposed to capture in LTE RRC specification that when EN-DC is configured, legacy RAT (UTRA, GERAN, CDMA2000 HRPD or CDMA2000 1xRTT) measurements are not supported in this release.

A corresponding text proposal to TS 36.331 is shown below.

Text proposal

## 5.5	Measurements

### 5.5.1	Introduction

The UE reports measurement information in accordance with the measurement configuration as provided by E-UTRAN. E-UTRAN provides the measurement configuration applicable for a UE in RRC_CONNECTED by means of dedicated signalling, i.e. using the RRCConnectionReconfiguration or RRCConnectionResume message.

The UE can be requested to perform the following types of measurements:

-	Intra-frequency measurements: measurements at the downlink carrier frequency(ies) of the serving cell(s).

-	Inter-frequency measurements: measurements at frequencies that differ from any of the downlink carrier frequency(ies) of the serving cell(s).

-	Inter-RAT measurements of UTRA frequencies.

-	Inter-RAT measurements of GERAN frequencies.

-	Inter-RAT measurements of CDMA2000 HRPD or CDMA2000 1xRTT or WLAN frequencies.

-	CBR measurements.

When EN-DC is configured, inter-RAT measurements of UTRA, GERAN, CDMA2000 HRPD or CDMA2000 1xRTT frequencies are not supported in this release of the specifications.

The measurement configuration includes the following parameters:

1.	Measurement objects: The objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object is a single E-UTRA carrier frequency. Associated with this carrier frequency, E-UTRAN can configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not considered in event evaluation or measurement reporting.

-	For inter-RAT UTRA measurements a measurement object is a set of cells on a single UTRA carrier frequency.

-	For inter-RAT GERAN measurements a measurement object is a set of GERAN carrier frequencies.

-	For inter-RAT CDMA2000 measurements a measurement object is a set of cells on a single (HRPD or 1xRTT) carrier frequency.

-	For inter-RAT WLAN measurements a measurement object is a set of WLAN identifiers and optionally a set of WLAN frequencies.

-	For CBR measurements a measurement object is a set of transmission resource pools for V2X sidelink communication.

NOTE 1:	Some measurements using the above mentioned measurement objects, only concern a single cell, e.g. measurements used to report neighbouring cell system information, PCell UE Rx-Tx time difference, or a pair of cells, e.g. SSTD measurements between the PCell and the PSCell.