3GPP TSG-RAN WG2 #100	R2-1712847

Reno, USA, 27th November – 1st December 2017

Agenda Item: 	10.2.7

Source:	Apple

Title:	Capability design for single UL transmission

Document for:		Discussion

# 1	Introduction

In RAN2 email discussion ([99bis#15][NR] Capability of signalling for 1 tx (Nokia)), there are two main issues:

- Issue 1: capability design is band combination or channel combination level;

- Issue 2: capability signaling approach in detail.

This contribution gives our view on the two issues of the capability design for single UL transmission.

# 2    Discussion

## 2.1 Issue 1: band combination or channel combination level?

The principle of single UL capability design was approved in RAN#77 [1], which includes

- RAN2: UE capability indicates that the UE does not allow 2 simultaneous UL transmission for the RAN4 specified channel allocations in a given band combination. If the network chooses to operate the UE in a way that is not consistent with this capability indication, then the UE behavior is not specified and the UE might not meet the performance criteria.

- RAN4: Define in RAN4 specs for which band combinations and channel allocations (to the extent specified by RAN4) within that band combination the UE is allowed to indicate that it does not support 2 simultaneous UL tx (this is not implying the granularity of the signalling).

Observation 1: According to principle agreed in RAN plenary, the single uplink capability should be provided in band combination level.

About RAN4 progress, according to LS from RAN4 in [2] and the agreed way forward in [3], RAN4 informs RAN2 the single uplink capability shall be per band combination, and RAN4 would specify formula/description to define which channel combinations within difficult band combinations are easy channel combinations. In easy channel combinations UE shall support simultaneous 2UL.

Observation 2: According to RAN4 solution, the single uplink capability is in band combination level, and RAN4 would specify the formula to identify easy channel combinations within difficult band combinations.

According to the current LTE/NR UE capability structure, in RF parameter level the band/band combination is the minimum granularity visible in RRC signalling, and there is no channel level capability.

Observation 3: According to the capability structure, only band/band combination level capability is visible in RRC.

Based on the previous RAN4 agreements and above observations, it’s straightforward to design per band combination capability signaling for single UL transmission.

Proposal 1: The UE capability signalling for single uplink transmission should be designed per band combination.

UE should check whether the configured channel combination is easy or not for serving cell set change cases, and enable/disable single uplink transmission mode based on both the check result and UE capability.

Two examples are shown below to explain how the UE capability signaling works, with 1-bit per band combination indication and the formula to be specified in RAN4.

Example 1: UE1 does not support simultaneous transmission in difficult BC

According to the capability in Table-1, the single uplink capability is only applicable on the difficult BCs, i.e. BC#1. UE1 does not support simultaneous uplink transmission in case of Channel combination#1 of BC#1.

Table-1. Example of single uplink capability for UE 1

| BC# | BC Difficult or not (Specified in RAN4) | Applicability of single uplink capability | Value of single uplink capability | Channel combos difficult or not (Using formula in RAN4) |
| --- | --- | --- | --- | --- |
| 1/ L1+N1 | Difficult | Yes | 1 (Simutanueous UL transmission is not supported) | Channel combination#1 within BC#1 is difficult channel combination |
| 2/ L1+N2 | Not difficult | N/A |  |  |

- UE capability report (Table-1)

LL1

NW

UE1

2. NW decides to configure serving cells including channel combo#1 of BC#1, and enables single uplink tx mode.

- RRC reconfiguration:

Configured serving cell set includes channel combination#1 of BC#1

LL1

4. UE1 identifies channel combo#1 is non-easy channel combo according to the formula, and enables single uplink tx mode.

- RRC reconfiguration complete

LL1

Figure-1. Example of single uplink mode procedure

- Step 1: UE1 reports its single uplink capability per band combination, indicating simultaneous UL transmission on BC#1 is not supported;

- Step 2: Based on network deployment, network decides to configure the serving cell set including channel combination#1 to UE1, identifies this is a difficult channel combination according to the RAN4 formula, and enables the single uplink transmission mode for the UE1;

- Step 3: Network configures the EN-DC with the serving cell set to UE1 via RRC reconfiguration message, including channel combo#1 of BC#1;

- Step 4: UE1 identifies channel combo#1 is a difficult channel combo according to the RAN4 formula, and enables single uplink transmission mode;

- Step 5: UE1 sends RRC reconfiguration complete message to network. Single UL transmission mode is enabled on both UE1 and network sides.

Example 2: Advanced UE2 supports simultaneous transmission in difficult BC

According to the capability in Table-2, UE2 supports simultaneous uplink transmission in case of Channel combination#1 of BC#1.

Table-2. Example of single uplink capability for UE 2

| BC# | BC Difficult or not (Specified in RAN4 band combos) | Applicability of single uplink capability | Value of single uplink capability | Channel combos difficult or not (Using formular in RAN4) |
| --- | --- | --- | --- | --- |
| 1/ L1+N1 | Difficult | Yes | 0 (Simutanueous UL transmission is supported) | Channel combination#1 within BC#1 is difficult channel combination |
| 2/ L1+N2 | Not difficult | N/A |  |  |

NW

UE2

2. NW decides to configure serving cells including channel combo#1 of BC#1, and no need to enable single uplink tx mode.

- UE capability report (Table-2)

LL1

4. UE2 support simutaneousl UL tx, without the need to identify if channel combo#1 is an easy channel combo or not.

- RRC reconfiguration:

Configured serving cell set includes channel combination#1 of BC#1

LL1

- RRC reconfiguration complete

LL1

Figure-2. Example of single uplink mode procedure

- Step 1: UE2 reports its single uplink capability per band combination, indicating simultaneous UL transmission on BC#1 is supported (even BC#1 is a difficult BC);

- Step 2: Based on network deployment, network decides to configure the serving cell set (i.e. channel combination#1) to UE2, can configure UE2 with simultaneous UL transmission, without the need to identify channel comb#1 is easy or not;

- Step 3: Network configures the EN-DC with the serving cell set to UE2 via RRC reconfiguration message, including channel combo#1 of BC#1;

- Step 4: UE2 follows the configuration and shall support simultaneous UL transmission without the need to identify channel combo#1 is easy or not;

- Step 5: UE2 sends RRC reconfiguration complete message to network.

## 2.2 Issue 2: Single uplink capability signalling approach

Two approaches have been proposed for the capability signaling details:

- Approach 1: bitmap format, which is outside the band combination;

A new RAN4 table is required to include all difficult band combinations. The index of the table is used for the location in the bitmap. Due to that the difficult band combination number would be increased rapidly in the future with the market growing up, to avoid the further extension complexity, the bitmap length has to be long enough.

Figure-3. Bitmap approach

- Approach 2: 1-bit capability introduced per band combination.

1-bit single uplink capability is introduced for each band combination, UE is allowed to indicate this capability only for the difficult band combinations specified in RAN4. This approach can also work well with the current NR bandcombiantion fallback mechansim. With the fallback mechanism, we can assume the 1-bit capability is also applicable to all BC sub-set. For example, in case of UE supports the super-set BC (L1+L2+N1), then the 1-bit capability is applicable for all fallback cases, i.e. (L1+L2+N1), (L1+N1) and (L2+N1).

Figure-4. Per band combination approach

Detailed analysis for the two approaches is shown in Table-3.

Table-3. Comparision of two approaches

|  | Approach 1: Bitmap | Approach 2: 1-bit per band combination |
| --- | --- | --- |
| Forward & backward compatibility | Restriction on the bitmap length. The difficult band combination number would be increasing with the market growping up. May also have potential backward compatibility issue if RAN4 changes band combination definition in the future. | No Problem. It’s easier to extend the capability when new band combinations are introduced. |
| Flexibility | Further extension cannot be avoided. It would introduce extra spec complexity and further clarification related discussion. | No problem. No futher change is expected. |
| RAN4 impact | Additional restriction to RAN4 spec. New RAN4 table is required for all difficult band combinations. | No additional restriction to RAN4 spec. |
| Overhead | The bitmap length would be longer than the supported BC, in order to avoid further extension. | 1 bit for each BC. The additional overhead is much smaller than the existing BC level parameters, e.g. EARFCN, and can be ignored. |

According to Table-3, it is obvious that per band combination approach has better performance in terms of forward/backward compatibility, flexibility and RAN4 impact. For overhead, bitmap approach has some uncertainty, which depends on the size of the bitmap. For 1-bit per band combination approach, it does not introduce much additional overhead, comparing to the total overhead of band combination signallings.

Proposal 2: 1-bit per band combination approach is adopted to indicate that the UE does not support simultaneous UL transmission.

- Proposal 2.1. Capability is applicable for all subsets of the BC which is difficult.

- Proposal 2.2. The difficult BC definition would be defined in RAN4 spec.

# 3    Conclusion

According to the anlaysis in section 2, it is proposed that

Proposal 1: The UE capability signalling for single uplink transmission should be designed per band combination.

Proposal 2: 1-bit per band combination approach is adopted to indicate that the UE does not support simultaneous UL transmission.

- Proposal 2.1. Capability is applicable for all subsets of the BC which is difficult.

- Proposal 2.2. The difficult BC definition would be defined in RAN4 spec.

# 4    References

- RP-172100	LS on single Tx switched UL	RAN #77			Sep., 2017

- R4-1711610 LS on single Tx switched UL	RAN4 #84bis	Oct., 2017

- R4-1711618 WF on single Tx switched UL	RAN4 #84bis	Oct., 2017