3GPP TSG-RAN WG2 #100	R2-1712850

Reno, USA, 27th November – 1st December 2017

Agenda Item: 	10.2.7

Source:	Apple

Title:	LTE impact of single UL transmission

Document for:		Discussion

# 1	Introduction

For single UL transmission, according to RAN1 progress (as below), the single uplink transmission in EN-DC is based on network scheduling.

- When the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), but the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers - UE can be configured with - Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell - For scheduling/HARQ timing of LTE FDD carrier, DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell is applied - UE is allowed to transmit NR UL signals at least in the subframe(s) where LTE UL transmission is not allowed according to the DL-reference UL/DL configuration - FFS whether or not a UE-specific subframe offset for the DL-reference UL/DL configuration can be configured considering system resource utilization and potential spec impact - Case 2: Release 15 LTE-FDD HARQ timing - No impact on LTE RAN1 specifications - Note: it doesn’t necessarily imply that UE has to support both cases

From UE point of view, in NR side there is no impact; and in LTE side, there are two cases, with and without DL-reference UL/DL configuration. In case of no DL-reference UL/DL configuration, there is no UE impact, but with DL-reference UL/DL configuration, there would be some impact in UE LTE side. This contribution gives our understanding on the LTE impact.

# 2	Discussion

The impact in LTE side is due to the DL-reference UL/DL configuration introduction, and it includes the following aspects.

- Capability for case 1 timing

Since it is unnecessary to introduce capability for case 2 (i.e. FDD timing), one-bit capability should be introduced to indicate whether UE support case 1. In case 1, the change is only about the HARQ/scheduling timing, hence, the capability should be baseband level capability, which is UE specific.

Proposal 1: 1-bit capability should be introduced in LTE side to indicate whether UE supports the case 1 in LTE side of single uplink mode.

Following is the corresponding TP for TS 36.331.

UE-EUTRA-Capability information element UE-EUTRA-Capability-v15x0-IEs ::= SEQUENCE { phyLayerParameters-v15 PhyLayerParameters-v15 OPTIONAL, irat-ParametersNR-r15 IRAT-ParametersNR-r15 OPTIONAL, basebandParameters-r15 BasebandParameters-r15 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } PhyLayerParameters-v15 ::= SEQUENCE { dlReferenceConfig-r15 ENUMERATED {supported} OPTIONAL } UE-EUTRA-Capability field descriptions; FDD/ TDD diff. dlReferenceConfig Indicates whether the UE supports the HARQ/scheduling timing according to reference UL/DL configuration on LTE FDD PCell in single uplink transmission mode.; -.

- RRC configuration

In case 1, DL-reference UL/DL configuration should be configured to UE for the HARQ/scheduling timing in LTE side. Hence, the UL/DL configuration should be introduced in RRC singaling for LTE PCell.

Proposal 2: TDD-Config-v15 should be added in RRCConnectionReconfiguration of LTE FDD PCell, which only includes subframeAssignment.

Following is the corresponding TP for 36.331.

TDD-Config-v15 ::= SEQUENCE { subframeAssignment ENUMERATED { sa0, sa1, sa2, sa3, sa4, sa5, sa6}, } RRCConnectionReconfiguration-v15x0-IEs ::= SEQUENCE { -- eNote May consider some grouping of EN-DC related fields, also to facilitate release endc-Config-r15 SEQUENCE { scg-ConfigNR-r15 OCTET STRING OPTIONAL, -- Need ON sk-Counter INTEGER (0.. 65535) OPTIONAL, -- Need ON rb-Config1-r15 OCTET STRING OPTIONAL, -- Need ON rb-Config2-r15 OCTET STRING OPTIONAL, -- Need ON -- eNote Update according to final TP from eMail [99b#17] tdd-Config-v15 TDD-Config-v15 OPTIONAL -- Cond SUT -- FFS Signalling details of TDM pattern for Single Tx UE } OPTIONAL -- Need ON nonCriticalExtension SEQUENCE {} OPTIONAL } Conditional presence; Explanation. SUT; If UE works in single uplink transmission mode in EN-DC, this field is optional present for FDD, Need OR..

- MAC impact

In case 1, the dl-reference UL/DL configuration is introduced for HARQ/scheduling timing in LTE side. Although the scheduling/HARQ timing and usage is defined in RAN1, due to HARQ RTT timer is calculated based on the timing, HARQ RTT timer should be updated to reflect this case.

Proposal 3: HARQ RTT Timer should be updated and reflect the case 1 which is based on the HARQ/scheduling timing according to dl-reference UL/DL configuration (i.e.tdd-Config-v15).

Following is the corresponding TP for TS 36.321.

## 7.7 HARQ RTT Timers For each serving cell, in case of FDD configuration and in case of Frame Structure Type 3 configuration on the serving cell which carries the HARQ feedback for this serving cell the HARQ RTT Timer is set to 8 subframes. For each serving cell, in case of TDD configuration and tdd-Config-v15 configured on the serving cell which carries the HARQ feedback for this serving cell the HARQ RTT Timer is set to k + 4 subframes, where k is the interval between the downlink transmission and the transmission of associated HARQ feedback, as indicated in subclauses 10.1 and 10.2 of [2], and for an RN configured with rn-SubframeConfig [8] and not suspended, as indicated in Table 7.5.1-1 of [11]. For BL UEs and UEs in enhanced coverage, HARQ RTT Timer corresponds to 7 + N where N is the used PUCCH repetition factor, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted. In case of TDD, HARQ RTT Timer corresponds to 3 + k + N, where k is the interval between the last repetition of downlink transmission and the first repetition of the transmission of associated HARQ feedback, and N is the used PUCCH repetition factor, where only valid UL subframes are counted as indicated in subclauses 10.1 and 10.2 of [2]. For NB-IoT the HARQ RTT Timer is set to k+3+N+deltaPDCCH subframes, where k is the interval between the last subframe of the downlink transmission and the first subframe of the associated HARQ feedback transmission and N is the transmission duration in subframes of the associated HARQ feedback, and deltaPDCCH is the interval from the last subframe of the associated HARQ feedback transmission plus 3 subframes to the first subframe of the next PDCCH occasion. Except for NB-IoT, UL HARQ RTT Timer length is set to 4 subframes for FDD and Frame Structure Type 3, and set to kULHARQRTT subframes for TDD, where kULHARQRTT equals to the kPHICH value indicated in Table 9.1.2-1 of [2]. For NB-IoT, the UL HARQ RTT timer length is set to 4+deltaPDCCH subframes, where deltaPDCCH is the interval from the last subframe of the PUSCH transmission plus 4 subframes to the first subframe of the next PDCCH

# 3	Conclusion

According to the anlaysis in section 2, it is proposed that

Proposal 1: 1-bit capability should be introduced in LTE side to indicate whether UE supports the case 1 in LTE side of single uplink mode.

Proposal 2: TDD-Config-v15 should be added in RRCConnectionReconfiguration of LTE FDD PCell, which only includes subframeAssignment.

Proposal 3: HARQ RTT Timer should be updated and reflect the case 1 which is based on the HARQ/scheduling timing according to dl-reference UL/DL configuration (i.e.tdd-Config-v15).