3GPP TSG-RAN WG2 #100	Tdoc R2-1712917

Reno, Nevada, USA, 27th October – 1st December 2017

Agenda Item:	10.2.2.3

Source:	Ericsson

Title:	Number of Supported DRBs in NR

Document for:	Discussion, Decision

# Introduction

Last RAN2 meeting discussed about the number of DRBs supported by EN-DC and NR. It was concluded in the discussions that the number of DRBs in NR will be in the range of 16 to 31 but the final number was not concluded. This contribution discusses about the number of DRBs supported by NR.

# Discussion

In LTE, the number of DRBs has been 8 since the initial release [1]. Until now, the number of DRBs has been constant as there is currently discussion ongoing in LTE to increase the number of DRBs to 11 or 15. In LTE, the increase of number of DRBs has impact on supported LCID space [2] as the LCID space is currently 5-bits, allowing 32 values.

LTE-Rel-15 may increase the number of DRBs to 15. This may require LCID extension.

The 5GC has new QoS framework that supports Network Slicing in RAN level. The number Network Slices supported by UEs is currently limited to 8. The slicing may be implemented in 5GC and in RAN by allocating one PDU session for each slice and by instantiating UPF and SMF entities in the 5GC. RAN2 has agreed previously that QFI from one PDU session may not be mapped into DRB of another PDU session. Additionally, when considering that RAN maps each QFI into a DRB based on the QoS profile associated with the QoS Flow and signalled via N2 (gNB – AMF), it may be observed that the number of DRBs has impact on network slicing implementation.

For example, when UE is configured with 8 slices, it is configured with 8 PDU sessions. Each PDU session has number of QFIs mapped into DRBs. As the DRBs are local for PDU session, the maximum number of DRBs per PDU session is between 1 and 4, depending on the agreed number of DRBs. When considering that the DRBs are dynamic resources rather than static, the number of DRBs per slice is between 1 and 25 per PDU session. This again, depending on the maximum number of DRBs RAN supports.

The number of DRBs supported by one PDU session may vary from 1 to 25 per PDU session depending on the maximum number of DRBs supported by UE.

Number of DRBs has impact on Network Slicing implementation.

Figure 1: Indirect mapping of IP Flows to QoS Flows to DRBs in UE

However, having multiple IP flows does not directly correspond to the number of Slices supported in the network. The IP flows are mapped into QoS Flows and the mapping profile is controlled by the network. Additionally, RAN maps QFIs into DRBs with SDAP configuration. This results that the number of IP Flows does not directly correspond to number of DRBs.

Because of indirect mapping of IP Flow to QoS Flow to DRB, number of IP Flows in application is not related to the number of DRBs required per PDU session.

One of the limiting factor for DRB configurations in LTE has been FGI capabilities, in which the FGI bit 20 together with the FGI bit 7 sets limit to the number of supported DRBs in 5AM and 3UM DRBs. The limits are upper limits, which means that the maximum number AM and UM UE can support simultaneously is 5AM and 3UM.  As indicated by the LS from SA2 [3][4], this limitation has created problems in advanced networks and this limitation is one of the reasons why the number of DRBs is re-considered in LTE. Therefore, such limitation should not be introduced in NR. The number of DRBs selected, should be supported by UEs regardless of the configuration.

UEs should not introduce limit on how DRBs should be configured, as in LTE. This may limit network operators on introducing advanced services.

UEs supports any combination of DRBs as long as the number of DRBs stays below maximum number of supported DRBs.

While the LS from SA2 introduced some use cases for the number of increased DRBs, there has not been a discussion on what the services would be to require large number of DRBs. RAN2 should discuss about realistic use cases associated with the number of DRBs

RAN2 is kindly requested to discuss about realistic use cases that require large number of DRBs for NR.

Introducing large number of DRBs has impact on the LCID space of the NR. Having 32 DRBs consumes half of the LCID space in the first release of NR. When taking into consideration the need for LCIDs, for example in MAC CE introduction, the LCID space may be consumed rather quickly. Currently, 9 LCIDs are reserved for UL and DL. Having 32 DRB IDs together with the 5 potential MAC CEs from RAN1 results that there are 18 “Reserved” LCID values left for future extensions. LTE Rel-11 had 16 DL and 14 UL values marked as “Reserved”. When considering that there are features in LTE, which are not included in the first release of NR, it is expected that the demand for the number of LCIDs in NR will not be less than historically for LTE [1].

LTE-Rel-11 had initially 16 DL and 14 UL LCID values marked as reserved. LCID values may potentially run out without extension in Rel-15. Corresponding value for NR is 18 when 32 DRBs are supported.

RAN2 establishes LCID space that has enough “Reserved” LCIDs to allow fixes and extensions in the future.

# Conclusion

In section 2 we made the following observations:

Observation 1	LTE-Rel-15 may increase the number of DRBs to 15. This may require LCID extension.

Observation 2	The number of DRBs supported by one PDU session may vary from 1 to 25 per PDU session depending on the maximum number of DRBs supported by UE.

Observation 3	Number of DRBs has impact on Network Slicing implementation.

Observation 4	Because of indirect mapping of IP Flow to QoS Flow to DRB, number of IP Flows in application is not related to the number of DRBs required per PDU session.

Observation 5	UEs should not introduce limit on how DRBs should be configured, as in LTE. This may limit network operators on introducing advanced services.

Observation 6	LTE-Rel-11 had initially 16 DL and 14 UL LCID values marked as reserved. LCID values may potentially run out without extension in Rel-15. Corresponding value for NR is 18 when 32 DRBs are supported.

Based on the discussion in section 2 we propose the following:

Proposal 1	UEs supports any combination of DRBs as long as the number of DRBs stays below maximum number of supported DRBs.

Proposal 2	RAN2 is kindly requested to discuss about realistic use cases that require large number of DRBs for NR.

Proposal 3	RAN2 establishes LCID space that has sufficient number of “Reserved” LCIDs for later use to allow future extendibility.

# References

R2-1712957. Number of DRBs in E-UTRA. Ericsson. Reno, Nevada, USA, 27th Nov– 1st Dec 2017

R2-1712956. LCID space extension. Ericsson. Reno, Nevada, USA, 27th Nov– 1st Dec 2017

R2-1710245. LS on the number of bearers. SA WG2 Meeting #122bis.

S2-175383. Discussion on LTE/EPS bearer limitations. Telstra. 21 - 25 August 2017, Sophia Antipolis, France