3GPP TSG-RAN WG2 #100	Tdoc R2-1712937

Reno, US, 27th Nov – 1st Dec 2017

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Remaining issues for SPS in NR

Document for:	Discussion, Decision

# Introduction

In RAN2-98AH, there are the following agreement regarding modelling of SPS /grant free transmission:

Agreements

=>	Modelling in the MAC for grant-free will be discussed after the difference between the two schemes is better understood pending RAN1 progress.  RAN2 will aim to have a unified MAC operation for common functionalities between grant-free and UL SPS with understanding that there can be differences after input from RAN1.

=>	RAN2 understands that to support UL SPS similar to LTE a mode of operation in which RRC configuration (with no initial PHY resources) with L1 activation/deactivation needs to be supported.  RAN2 will continue discussion on UL SPS, with LTE functionality.

=>	A common RRC signalling can be design to allow the configuration of different UL transmissions schemes.

In RAN1-89AH, there are the following agreements, two types of SPS/grant free scheme are supported in NR.

- Type 1: UL data transmission without grant is only based on RRC (re)configuration without any L1 signalling.

For this type, the radio resources are granted via RRC signaling for a UE. The parameters such as period, offset, MCS, HARQ configurations and power control configuration (if any) are all conveyed via RRC signaling.

- Type 2: UL data transmission without grant is based on both RRC configuration and L1 signalling to activation/deactivation for UL data transmission without grant.

For this type, some parameters such as period, offset for granted resource occurrence and some HARQ operation parameters are conveyed via RRC signaling while some other parameters such as MCS are conveyed via L1 signaling. L1 signaling also acts as activation and deactivation signal for SPS operation.

In RAN2-99bis, it was agreed that the UE stops using all configured downlink assignments and configured uplink grants using resources of this SCell/BWP when a SCell/BWP is deactivated. However, it was left open whether the grants are suspended or cleared.

In the email discussion [99bis#41] [NR UP/MAC] – Open issues on SPS and GF – Huawei, a number of open issues were discussed and are proposed for conclusion. From this discussion, the remaining item needing discussion are further investigated and discussed in this paper.

# Discussion

The RAN 1 agreements provide two schemes to configure semi-static grant, wherein Type 1 SPS/grant free scheme allocates semi-static grant via RRC signalling only and Type 2 SPS/grant free scheme allocates semi-static grant via RRC signalling plus L1 signal. However, from a UE perspective, the semi-static grant, no matter how the semi-static grant is allocated, activated or deactivated via either scheme, can be used for data transmission and receiving without difference. Hence, the functionality of two methods is similar.

In the proposed conclusion in the email discussion, general term(s) are suggested (question/proposal 31). However, from a MAC entity point of view, the use of SPS, along with considering a “configured grant” should be sufficient to cater for both in procedures and grant evaluations etc.

In MAC specification, the term SPS and “configured grant” is used.

In the following, we refer to type 1 scheme as RRC based SPS scheme and Type 2 scheme RRC plus L1 signal based SPS scheme. We also refer to the semi-static grant allocated via either scheme as configured grant.

For both RRC signal based and RRC plus L1 signal based SPS scheme, the essence is that the network grants a set of periodic resources for a UE. With these granted resources, the UE can transmit data whenever data comes. From this perspective, the two schemes for configured grant allocation should not be distinctly modelled to avoid considerable complexity. Considering the flexibility of RRC plus L1 signal based SPS scheme for link adaptation via L1 signal based activation and deactivation, it is preferred to model RRC plus L1 signal based SPS scheme as baseline framework, wherein optional parameters for RRC signal based SPS scheme such as time-frequency resources, MCS can be included in RRC signaling IE as an extension.

RRC signal based SPS scheme is modelled as a part of configurable options based on the framework of RRC plus L1 signal based SPS scheme.

For data retransmission, it was agreed that SPS C-RNTI is used. For RRC plus L1 signal SPS scheme, SPS C-RNTI is used to scrambling the L1 signal to activate/deactivate the configured grant. For both RRC based and RRC plus L1 signal SPS schemes, SPS C-RNTI can be used to schedule the retransmissions at transmission failure using configured grant. There is no benefit to define separate RNTIs for two schemes for the similar functionality. Also, t this point, there is no agreement on that simultaneous configured grants can be present at the same instant. Hence, we propose:

The same SPS C-RNTI is used for both RRC based and RRC plus L1 signal based SPS schemes.

In addition to grant release via explicit signalling, the following options are also being discussed for the configured grant handling when the corresponding carrier/BWP is deactivated.

- Option 1: configured grant is suspended when a carrier/BWP is deactivated and the configured grant can be reused when the carrier/BWP is reactivated;

- Option 2: configured grant is deactivated when a carrier/BWP is deactivated and the configured grant should be reallocated/reactivated when the carrier/BWP is reactivated;

For RRC plus L1 signal based SPS scheme, the delay and signalling cost to reactivate/reallocate a configured grant is minor via L1 signaling. However, for RRC based SPS scheme, the configured grant is allocated via RRC signalling, there is considerable delay for grant reconfiguration. To enable a UE to transmit data via configured grant as soon as an SCell/BWP is reactivated, Option 1 is proposed.

For RRC based SPS scheme, there is delay issue for configured grant reallocation/reactivation.

For RRC plus L1 signal based SPS scheme, configured grant suspending scheme is not necessary.

For RRC based SPS scheme, if the previous allocated configured grant in a SCell is automatically reactivated at the SCell reactivation, there are the following disadvantages:

  - The MCS, time-frequency radio resources, MCS and power control parameters cannot be adapted in accordance with the QoS requirement and radio condition change. If there is data rate change or radio condition change (path loss, interference, UE moving speed), automatically configured grant reactivation seems meaningless.

  - In addition, the resource allocation becomes more complex. If the gNB allocates configured grant in a SCell for a UE and the SCell is deactivated for the UE afterwards, there can be collision when the SCell/BWP is reactivated due to the gNB may reallocate the radio resource of the suspended configured grant to another UE for DL/UL data transmission. If the network reserves the radio resource of the suspended configured grant, it is impractical since the cost is not affordable to reserve the configured grant resource for a UE in a SCell even when the SCell is deactivated for the UE. If the network reallocates the radio resources to other UEs when the SCell is deactivated and withdraws the radio resource from the other UEs before reactivating the SCell for the UE, the delay cannot be actually reduced.

For either scheme, the transmission parameters cannot be adjusted with configured grant suspending and automatic reactivation.

For RRC plus L1 signal based SPS scheme, configured grant reallocation via L1 signal at reactivation at SCell/BWP reactivation ensures short delay and good link adaptation.

With configured grant suspending, there can be ambiguity at reallocating the resources corresponding to a suspended configured grant.

Based on the above analysis, it is not necessary to over-optimize the RRC based SPS scheme. Hence, we propose:

The allocated grant via either RRC based or RRC plus L1 signal based SPS scheme in a SCell is deactivated when the SCell is deactivated.

Automatic reactivation of a deactivated configured grant is not supported in NR.

In NR, a RRC_connected UE can be either in active or inactive state. For a UE in inactive state, the RRC context is kept in both UE and network; and the network/UE can reactivate the context (such as C-RNTI and DRB etc) when there comes a service for the UE. However, there is no benefit to still keep the allocated radio resources effective after the UE transfers from active state to inactive state. There are the following disadvantages to allow an inactive UE to access the allocated resources during the UE in active state:

- The traffic status of an inactive UE is not known at the network, which means possible waste of radio resources.

- The network loses control of the allocated resources:

  - For congestion control when there is high traffic load from inactive UEs sharing the resources

  - For resource reallocation: e.g. adjustment of granted resource size, or reallocation of the allocated resource for other purposes.

Considering the infeasibility mentioned above, we propose:

The configured grant is released when a UE enters to RRC_inactive state from RRC_connected state.

On question 14 and 15, for cases when the TA timer expires (pTAG/sTAG), it is clear that reconfiguration is anyway needed, i.e for SRS or PUCCH. Furthermore, the gNB should be able to keep UEs in synchronization in any case. As a result, the UE should clear all the resources and release a configuration.

The MAC entity shall release configured resource assignments for for all Serving Cells belonging to this TAG when TA timer associated with s/pTAG expires.

On question 17, and in cases where there is high priority data associated with the LCH using the configured grant, it can be assumed that, except for retransmissions the gNB is able to allocate grants that make it possible to transmit those packets. As a result, a dynamic grant addressed to SPS C-RNTI should override the configured grant for this transmission in case of overlap in time domain. This is similar to current in LTE.

A dynamic grant addressed to SPS C-RNTI override a configured grant transmission in case of overlap

# Conclusion

In section 2 we made the following observations:

Observation 1	For RRC based SPS scheme, there is delay issue for configured grant reallocation/reactivation.

Observation 2	For RRC plus L1 signal based SPS scheme, configured grant suspending scheme is not necessary.

Observation 3	For either scheme, the transmission parameters cannot be adjusted with configured grant suspending and automatic reactivation.

Observation 4	For RRC plus L1 signal based SPS scheme, configured grant reallocation via L1 signal at reactivation at SCell/BWP reactivation ensures short delay and good link adaptation.

Observation 5	With configured grant suspending, there can be ambiguity at reallocating the resources corresponding to a suspended configured grant.

Based on the discussion in section 2 we propose the following:

Proposal 1	In MAC specification, the term SPS and “configured grant” is used.

Proposal 2	RRC signal based SPS scheme is modelled as a part of configurable options based on the framework of RRC plus L1 signal based SPS scheme.

Proposal 3	The same SPS C-RNTI is used for both RRC based and RRC plus L1 signal based SPS schemes.

Proposal 4	The allocated grant via either RRC based or RRC plus L1 signal based SPS scheme in a SCell is deactivated when the SCell is deactivated.

Proposal 5	Automatic reactivation of a deactivated configured grant is not supported in NR.

Proposal 6	The configured grant is released when a UE enters to RRC_inactive state from RRC_connected state.

Proposal 7	The MAC entity shall release configured resource assignments for for all Serving Cells belonging to this TAG when TA timer associated with s/pTAG expires.

Proposal 8	A dynamic grant addressed to SPS C-RNTI override a configured grant transmission in case of overlap