3GPP TSG-RAN WG2 Meeting #100	R2-1713330

Reno, Nevada, 27 Nov - 1 Dec  2017

Agenda item:	9.4.4.1

Source:	Huawei

Title:	TP outcome of Email discussion [99bis#31][LTE/UAV] –  Capture handover simulation results with observations

Document for:	Discussion and Decision

Introduction

This contribution provides TR36.777 TP reflecting simulation results and observations from all the participating companies [1].

TP for TR36.777

/************************ Start of Text Proposal **************************/

- C.3 Mobility Simulation results

- C.3.1 Simulation results for UMa

- C.3.1.1 Introduction

All the results provided by companies are shown in the same figure for convenient comparison, and different figures are classified by the speed. At the same time the calibration results averaged over the results from all the participating companies are presented in the following figures for trend analysis.

- C.3.1.2 Handover rate

The simulation results from all participating companies are collected in Table 1 and illustrated in Figure 1.

Table C.3.1.2: Handover rate simulation data.

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Handover rate [HOs/UE/sec] | 3km/h | Nokia | 0.005232 | 0.001722 | 0.001875 | 0.004017 |  |
|  |  | Ericsson | 0.017208 | 0.005219 | 0.000551 | 0.000013 |  |
|  |  | Qualcomm | 0.187135 | 0.114129 | 0.114058 | 0.132070 |  |
|  |  | Huawei | 0.034800 | 0.018700 | 0.024300 | 0.065100 |  |
|  |  | ZTE | 0.001852 | 0.007700 | 0.012093 | 0.018543 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Nokia | 0.052900 | 0.016924 | 0.019693 | 0.057998 |  |
|  |  | Ericsson | 0.107378 | 0.057519 | 0.019132 | 0.001339 |  |
|  |  | Qualcomm | 0.188772 | 0.162526 | 0.185357 | 0.179368 |  |
|  |  | DCM | 0.094900 | 0.152300 | 0.085100 | 0.063500 |  |
|  |  | Huawei | 0.058500 | 0.110500 | 0.122800 | 0.087300 |  |
|  |  | ZTE | 0.031194 | 0.084321 | 0.142473 | 0.220468 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Nokia | 0.120974 | 0.034248 | 0.038232 | 0.113847 |  |
|  |  | Ericsson | 0.159379 | 0.082477 | 0.044263 | 0.016254 |  |
|  |  | Qualcomm | 0.192982 | 0.213240 | 0.268655 | 0.231368 |  |
|  |  | DCM | 0.146400 | 0.189200 | 0.131200 | 0.139300 |  |
|  |  | Huawei | 0.097100 | 0.191800 | 0.232500 | 0.181600 |  |
|  |  | ZTE | 0.068152 | 0.174528 | 0.304022 | 0.475238 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Nokia | 0.490931 | 0.092112 | 0.100394 | 0.287274 |  |
|  |  | Ericsson | 0.245963 | 0.091826 | 0.076752 | 0.060755 |  |
|  |  | Qualcomm | 0.235228 | 0.319743 | 0.407322 | 0.370152 |  |
|  |  | DCM | 0.237000 | 0.296200 | 0.228800 | 0.220400 |  |
|  |  | Huawei | 0.228700 | 0.385900 | 0.457200 | 0.485600 |  |
|  |  | ZTE | 0.216999 | 0.479752 | 0.486608 | 0.702849 |  |

Figure C.3.1.2 Handover rate simulation data

From the simulation results, the following initial findings are observed:

1.	From the simulation results from Nokia, Ericsson and Qualcomm(3km/h and 30km/h), it can be observed that the handover rate of terrestrial UEs is higher than that of aerial UEs. And as height increases the handover rate firstly decreases, then increases slightly within a small range.

2.	From the simulation results from DCM, Huawei, ZTE and Qualcomm(60km/h and 160km/h), it can be observed that the handover rate of terrestrial UEs is lower than that of aerial UEs. And as height increases the handover rate firstly increases obviously, then decreases slightly.

- C.3.1.3 HOF rate

Table C.3.1.3: HOF rate simulation data.

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ericsson | 41.25% | 43.94% | 73.46% | 40.00% |  |
| HO failure rate [%] | 3km/h | Qualcomm | 14.63% | 0.78% | 1.07% | 0.70% |  |
|  |  | Huawei | 6.37% | 60.54% | 88.47% | 89.12% |  |
|  |  | ZTE | 0.00% | 45.16% | 77.29% | 80.68% |  |
|  |  | Average | 15.56% | 37.61% | 60.07% | 52.62% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Ericsson | 33.37% | 40.11% | 60.85% | 84.21% |  |
|  |  | Qualcomm | 14.67% | 7.27% | 11.38% | 3.47% |  |
|  |  | DCM | 12.97% | 14.54% | 6.33% | 7.05% |  |
|  |  | Huawei | 16.83% | 72.07% | 89.00% | 89.28% |  |
|  |  | ZTE | 3.61% | 57.89% | 82.88% | 85.41% |  |
|  |  | Average | 16.29% | 38.38% | 50.09% | 53.88% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Ericsson | 33.25% | 48.09% | 53.85% | 82.62% |  |
|  |  | Qualcomm | 14.15% | 12.94% | 20.40% | 8.40% |  |
|  |  | DCM | 21.67% | 17.55% | 14.06% | 12.01% |  |
|  |  | Huawei | 27.15% | 78.17% | 91.11% | 92.01% |  |
|  |  | ZTE | 6.99% | 63.72% | 87.66% | 88.77% |  |
|  |  | Average | 20.64% | 44.09% | 53.41% | 56.76% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Ericsson | 33.24% | 60.94% | 59.51% | 74.20% |  |
|  |  | Qualcomm | 19.82% | 23.37% | 30.51% | 31.38% |  |
|  |  | DCM | 36.80% | 24.76% | 32.79% | 48.90% |  |
|  |  | Huawei | 45.03% | 85.62% | 93.28% | 97.13% |  |
|  |  | ZTE | 28.03% | 75.98% | 93.09% | 97.01% |  |
|  |  | Average | 32.59% | 54.13% | 61.84% | 69.72% |  |

Figure C.3.1.3 HOF rate simulation data

From the simulation results, the following initial findings are observed:

1.	Majority of the companies observed higher HOF rate for aerial UE in most cases. And the higher the speed or the height of aerial UE is, the higher HOF rate can be observed.

2.	The average HOF rate of the all the companies’ results is higher for the aerial UE than that for the terrestrial UE.

- C.3.1.4 RLF rate

Table C.3.1.4: RLF rate simulation data.

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Radio Link Failure (RLF) rate [RLF/UE/sec] | 3km/h | Nokia | 0.000480 | 0.001461 | 0.001502 | 0.000460 |  |
|  |  | Ericsson | 0.042740 | 0.041962 | 0.273787 | 0.449335 |  |
|  |  | Qualcomm | 0.001006 | 0.000234 | 0.000678 | 0.000608 |  |
|  |  | Huawei | 0.000526 | 0.023400 | 0.050500 | 0.128700 |  |
|  |  | ZTE | 0.000000 | 0.002401 | 0.010165 | 0.015101 |  |
|  |  | Average | 0.008951 | 0.013892 | 0.067326 | 0.118841 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Nokia | 0.004758 | 0.008422 | 0.009663 | 0.007100 |  |
|  |  | Ericsson | 0.074277 | 0.108172 | 0.198402 | 0.460071 |  |
|  |  | Qualcomm | 0.001029 | 0.004304 | 0.011696 | 0.005123 |  |
|  |  | Huawei | 0.000735 | 0.033000 | 0.065700 | 0.171700 |  |
|  |  | ZTE | 0.000562 | 0.025746 | 0.083155 | 0.126949 |  |
|  |  | Average | 0.016272 | 0.035929 | 0.073723 | 0.154189 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Nokia | 0.013473 | 0.016643 | 0.017751 | 0.014434 |  |
|  |  | Ericsson | 0.091698 | 0.154025 | 0.195801 | 0.453388 |  |
|  |  | Qualcomm | 0.002877 | 0.011649 | 0.02676 | 0.01586 |  |
|  |  | Huawei | 0.000800 | 0.029700 | 0.056200 | 0.151300 |  |
|  |  | ZTE | 0.002381 | 0.057655 | 0.163161 | 0.254138 |  |
|  |  | Average | 0.022246 | 0.053934 | 0.091935 | 0.177824 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Nokia | 0.081088 | 0.043263 | 0.044738 | 0.033587 |  |
|  |  | Ericsson | 0.120239 | 0.246539 | 0.229423 | 0.195604 |  |
|  |  | Qualcomm | 0.021708 | 0.04117 | 0.062409 | 0.074339 |  |
|  |  | Huawei | 0.000603 | 0.021400 | 0.039300 | 0.089000 |  |
|  |  | ZTE | 0.030417 | 0.182879 | 0.493908 | 0.628773 |  |
|  |  | Average | 0.050811 | 0.107050 | 0.173956 | 0.204261 |  |

Figure C.3.1.4 RLF rate simulation data

From the simulation results, the following initial findings are observed:

1.	Majority of the companies observed higher RLF rate for aerial UE in most cases. And the higher the speed of aerial UE is, the higher RLF rate can be observed.

2.	The average RLF rate of the all the companies’ results becomes higher for the aerial UE as the height increases.

- C.3.1.5 Time in handoff

Table C.3.1.5: Time in handoff simulation data.

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time in handoff [%] | 3km/h | Ericsson | 0.17% | 0.03% | 0.00% | 0.00% |  |
|  |  | Qualcomm | 1.60% | 0.48% | 0.53% | 0.59% |  |
|  |  | Huawei | 0.17% | 0.24% | 0.41% | 1.21% |  |
|  |  | ZTE | 0.02% | 0.04% | 0.03% | 0.04% |  |
|  |  | Average | 0.49% | 0.20% | 0.24% | 0.46% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Ericsson | 1.68% | 1.16% | 0.23% | 0.01% |  |
|  |  | Qualcomm | 1.61% | 1.48% | 2.46% | 1.23% |  |
|  |  | Huawei | 0.38% | 1.62% | 2.11% | 1.50% |  |
|  |  | ZTE | 0.27% | 0.43% | 0.48% | 0.66% |  |
|  |  | Average | 0.99% | 1.17% | 1.32% | 0.85% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Ericsson | 2.76% | 1.78% | 0.64% | 0.07% |  |
|  |  | Qualcomm | 1.76% | 2.88% | 5.60% | 2.62% |  |
|  |  | Huawei | 0.78% | 2.99% | 4.07% | 3.20% |  |
|  |  | ZTE | 0.58% | 0.83% | 1.00% | 1.41% |  |
|  |  | Average | 1.47% | 2.12% | 2.83% | 1.82% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Ericsson | 4.50% | 1.84% | 1.19% | 0.33% |  |
|  |  | Qualcomm | 3.83% | 7.50% | 11.93% | 11.97% |  |
|  |  | Huawei | 2.44% | 6.43% | 8.14% | 8.92% |  |
|  |  | ZTE | 1.56% | 1.94% | 2.92% | 2.98% |  |
|  |  | Average | 3.08% | 4.43% | 6.05% | 6.05% |  |

Figure C.3.1.5 Time in handoff simulation data

From the simulation results, the following initial findings are observed:

1.	Majority of the companies observed larger Time in handoff for aerial UE at least in moderate and high speed cases. For low speed cases, e.g. 3km/h, contrary trends from two companies are observed.

2.	The average Time in handoff of the all the companies’ results is higher for aerial UE than that of terrestrial UE in case of moderate and high speed.

- C.3.1.6 Time in Qout

Table C.3.1.6: Time in Qout simulation data.

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time in Qout [%] | 3km/h | Nokia | 0.10% | 33.22% | 33.18% | 49.10% |  |
|  |  | Qualcomm | 1.15% | 0.06% | 0.11% | 0.07% |  |
|  |  | Huawei | 0.28% | 3.36% | 6.83% | 17.69% |  |
|  |  | ZTE | 0.00% | 0.16% | 0.93% | 1.45% |  |
|  |  | Average | 0.38% | 9.20% | 10.26% | 17.08% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Nokia | 0.98% | 73.72% | 68.81% | 52.32% |  |
|  |  | Qualcomm | 1.16% | 0.94% | 1.85% | 0.59% |  |
|  |  | Huawei | 0.45% | 7.59% | 12.71% | 22.34% |  |
|  |  | ZTE | 0.01% | 0.88% | 5.54% | 9.44% |  |
|  |  | Average | 0.65% | 20.78% | 22.23% | 21.17% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Nokia | 2.78% | 77.31% | 73.23% | 52.56% |  |
|  |  | Qualcomm | 1.32% | 2.32% | 4.85% | 1.86% |  |
|  |  | Huawei | 0.86% | 10.57% | 16.62% | 25.72% |  |
|  |  | ZTE | 0.03% | 1.70% | 8.28% | 16.45% |  |
|  |  | Average | 1.25% | 22.97% | 25.74% | 24.15% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Nokia | 16.86% | 78.63% | 74.82% | 54.26% |  |
|  |  | Qualcomm | 3.39% | 7.05% | 11.65% | 11.24% |  |
|  |  | Huawei | 2.83% | 17.63% | 25.48% | 34.40% |  |
|  |  | ZTE | 0.36% | 4.18% | 18.23% | 31.00% |  |
|  |  | Average | 5.86% | 26.87% | 32.55% | 32.72% |  |

Figure C.3.1.6 Time in Qout simulation data

From the simulation results, the following initial findings are observed:

1.	Majority of the companies observed larger Time in Qout for aerial UE in all speed cases.

2.	The average Time in Qout of the all the companies’ results increases firstly, then decreases slightly as the height becomes higher.

- C.3.1.7 Ping-Pong rate

Table C.3.1.7: Ping-Pong rate simulation data.

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ping pong rate [%] | 3km/h | Nokia | 9.07% | 3.10% | 8.90% | 11.30% |  |
|  |  | Ericsson | 10.42% | 6.41% | 0.22% | 0.00% |  |
|  |  | Qualcomm | 41.83% | 23.04% | 22.73% | 22.95% |  |
|  |  | Huawei | 14.81% | 4.76% | 5.88% | 1.92% |  |
|  |  | Average | 19.03% | 9.33% | 9.43% | 9.04% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Nokia | 9.19% | 2.56% | 4.74% | 11.61% |  |
|  |  | Ericsson | 16.31% | 13.93% | 6.80% | 0.00% |  |
|  |  | Qualcomm | 40.29% | 18.46% | 14.87% | 13.20% |  |
|  |  | DCM | 15.28% | 7.89% | 0.01% | 0.00% |  |
|  |  | Huawei | 2.86% | 10.12% | 18.43% | 10.77% |  |
|  |  | Average | 16.79% | 10.59% | 8.97% | 7.12% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Nokia | 14.80% | 2.42% | 4.99% | 12.30% |  |
|  |  | Ericsson | 13.84% | 9.77% | 10.01% | 0.10% |  |
|  |  | Qualcomm | 35.39% | 17.21% | 16.65% | 8.20% |  |
|  |  | DCM | 15.78% | 6.35% | 1.76% | 0.00% |  |
|  |  | Huawei | 4.16% | 10.31% | 10.62% | 9.37% |  |
|  |  | Average | 16.79% | 9.21% | 8.81% | 5.99% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Nokia | 28.10% | 2.92% | 5.11% | 10.81% |  |
|  |  | Ericsson | 8.70% | 3.00% | 5.23% | 1.64% |  |
|  |  | Qualcomm | 21.55% | 17.00% | 21.82% | 10.63% |  |
|  |  | DCM | 13.25% | 7.07% | 5.89% | 0.04% |  |
|  |  | Huawei | 5.51% | 8.04% | 9.20% | 6.45% |  |
|  |  | Average | 15.42% | 7.61% | 9.45% | 5.91% |  |

Figure C.3.1.7 Ping-Pong rate simulation data

From the simulation results, the following initial findings are observed:

1.	Majority of the companies observed lower Ping-Pong rate for aerial UE in high speed cases.

2.	The average Ping-Pong rate of the all the companies’ results decreases slightly as the height becomes higher.

- C3.2 Simulation results for RMa

- C.3.2.1 Introduction

For RMa scenario only two companies provide simulation results, and the results are gathered and compiled into the same figure for every KPI.

- C.3.2.2 Handover rate

Table C.3.2.2: Handover rate simulation data

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Handover rate [HOs/UE/sec] | 3km/h | Nokia | 0.002176 | 0.002482 | 0.002676 | 0.000747 |  |
|  |  | Qualcomm | 0.075322 | 0.134409 | 0.104070 | 0.148468 |  |
|  |  | ZTE | 0.000742 | 0.002255 | 0.001990 | 0.004745 |  |
|  |  | Average | 0.026080 | 0.046382 | 0.036245 | 0.051320 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Nokia | 0.021061 | 0.028081 | 0.018318 | 0.006994 |  |
|  |  | Qualcomm | 0.079018 | 0.140912 | 0.121825 | 0.157520 |  |
|  |  | ZTE | 0.009880 | 0.018607 | 0.019290 | 0.017806 |  |
|  |  | Average | 0.036653 | 0.062533 | 0.053144 | 0.060773 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Nokia | 0.041395 | 0.061748 | 0.036844 | 0.014168 |  |
|  |  | Qualcomm | 0.086503 | 0.146854 | 0.147673 | 0.171860 |  |
|  |  | ZTE | 0.021206 | 0.039253 | 0.040596 | 0.039855 |  |
|  |  | Average | 0.031300 | 0.050501 | 0.038720 | 0.027011 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Nokia | 0.127528 | 0.191405 | 0.099513 | 0.039660 |  |
|  |  | Qualcomm | 0.096772 | 0.174901 | 0.197988 | 0.224491 |  |
|  |  | ZTE | 0.057453 | 0.099521 | 0.104998 | 0.167279 |  |
|  |  | Average | 0.093918 | 0.155275 | 0.134166 | 0.143810 |  |

Figure C.3.2.2 Handover rate simulation data

From the simulation results, the following initial findings are observed:

1.	Majority of the companies observed higher handover rate for aerial UE in most cases. And the higher the speed or the height of aerial UE is, the higher handover rate can be observed.

2.	The average handover rate of the all the companies’ results is higher for the aerial UE than that for the terrestrial UE.

- C.3.2.3 HOF rate

Table C.3.2.3: HOF rate simulation data

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HO failure rate [%] | 3km/h | Qualcomm | 6.17% | 1.10% | 3.39% | 0.65% |  |
|  |  | ZTE | 10.00% | 72.22% | 81.82% | 96.15% |  |
|  |  | Average | 8.08% | 36.66% | 42.60% | 48.40% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Qualcomm | 7.63% | 2.18% | 7.49% | 1.31% |  |
|  |  | ZTE | 2.26% | 59.38% | 77.22% | 81.19% |  |
|  |  | Average | 4.95% | 30.78% | 42.36% | 41.25% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Qualcomm | 7.13% | 2.25% | 10.83% | 2.46% |  |
|  |  | ZTE | 1.06% | 58.65% | 81.90% | 83.08% |  |
|  |  | Average | 4.09% | 30.45% | 46.36% | 42.77% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Qualcomm | 8.67% | 6.09% | 17.07% | 9.31% |  |
|  |  | ZTE | 7.64% | 56.71% | 86.06% | 95.69% |  |
|  |  | Average | 8.16% | 31.40% | 51.56% | 52.50% |  |

Figure C.3.2.3 HOF rate simulation data

From the simulation results, the following initial findings are observed:

1.	From the simulation results of ZTE, the HOF rate of terrestrial UE is lower than that of aerial UE, and the HOF rate keeps increasing as the height grows.

2.	From the simulation results of Qualcomm, for speed lower than 30km/h, the HOF rate of terrestrial UE is higher than that of aerial UE. For speed higher than 60km/h, the HOF rate of aerial UE at 100m height reaches the top, obviously higher than that of terrestrial UE. At other heights the HOF rate is lower than that of terrestrial UE.

- C.3.2.4 RLF rate

Table C.3.2.4: RLF rate simulation data

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Radio Link Failure (RLF) rate [RLF/UE/sec] | 3km/h | Nokia | 0.000140 | 0.000734 | 0.002236 | 0.000627 |  |
|  |  | Qualcomm | 0.000164 | 0.000234 | 0.000515 | 0.000211 |  |
|  |  | ZTE | 0.000037 | 0.001962 | 0.002985 | 0.009247 |  |
|  |  | Average | 0.000114 | 0.000977 | 0.001912 | 0.003362 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Nokia | 0.001435 | 0.007588 | 0.009062 | 0.003490 |  |
|  |  | Qualcomm | 0.000585 | 0.000515 | 0.002713 | 0.000561 |  |
|  |  | ZTE | 0.000112 | 0.007453 | 0.017651 | 0.052711 |  |
|  |  | Average | 0.000710 | 0.005185 | 0.009809 | 0.018921 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Nokia | 0.002756 | 0.016590 | 0.016757 | 0.006653 |  |
|  |  | Qualcomm | 0.000912 | 0.001123 | 0.006690 | 0.002035 |  |
|  |  | ZTE | 0.000112 | 0.014329 | 0.034414 | 0.091974 |  |
|  |  | Average | 0.001260 | 0.010681 | 0.019287 | 0.033554 |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Nokia | 0.012573 | 0.050450 | 0.044912 | 0.017231 |  |
|  |  | Qualcomm | 0.001895 | 0.006269 | 0.014901 | 0.006643 |  |
|  |  | ZTE | 0.002196 | 0.033254 | 0.095453 | 0.196799 |  |
|  |  | Average | 0.005554 | 0.029991 | 0.051755 | 0.073558 |  |

Figure C.3.2.4 RLF rate simulation data

From the simulation results, the following initial findings are observed:

1.	Majority of the companies observed higher RLF rate for aerial UE in most cases. And the higher the speed of aerial UE is, the higher RLF rate can be observed.

2.	The average RLF rate of the all the companies’ results becomes higher for the aerial UE, then decreases obviously as the height increases.

- C.3.2.5 Time in handoff

Table C.3.2.5: Time in handoff simulation data

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time in handoff [%] | 3km/h | Qualcomm | 0.42% | 0.59% | 0.53% | 0.63% |  |
|  |  | ZTE | 0.01% | 0.01% | 0.00% | 0.00% |  |
|  |  | Average | 0.22% | 0.30% | 0.27% | 0.32% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Qualcomm | 0.51% | 0.68% | 0.95% | 0.71% |  |
|  |  | ZTE | 0.09% | 0.09% | 0.06% | 0.04% |  |
|  |  | Average | 0.30% | 0.38% | 0.51% | 0.38% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Qualcomm | 0.58% | 0.79% | 1.66% | 0.95% |  |
|  |  | ZTE | 0.19% | 0.20% | 0.13% | 0.11% |  |
|  |  | Average | 0.38% | 0.50% | 0.90% | 0.53% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Qualcomm | 0.72% | 1.50% | 3.23% | 2.36% |  |
|  |  | ZTE | 0.49% | 0.54% | 0.37% | 0.51% |  |
|  |  | Average | 0.60% | 1.02% | 1.80% | 1.44% |  |

Figure C.3.2.5 Time in handoff simulation data

From the simulation results, the following initial findings are observed:

1.	The Time in handoff of terrestrial UE is lower than that of aerial UE, and as height increases the Time in handoff becomes larger first and then decreases slightly.

- C.3.2.6 Time in Qout

Table C.3.2.6: Time in Qout simulation data

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time in Qout [%] | 3km/h | Nokia | 0.03% | 3.32% | 53.56% | 34.29% |  |
|  |  | Qualcomm | 0.18% | 0.10% | 0.21% | 0.07% |  |
|  |  | ZTE | 0.00% | 0.19% | 0.30% | 0.90% |  |
|  |  | Average | 0.07% | 1.20% | 18.02% | 11.75% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Nokia | 0.29% | 7.57% | 76.01% | 64.14% |  |
|  |  | Qualcomm | 0.26% | 0.18% | 0.62% | 0.13% |  |
|  |  | ZTE | 0.00% | 0.45% | 1.65% | 5.31% |  |
|  |  | Average | 0.19% | 2.73% | 26.09% | 23.19% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Nokia | 0.57% | 8.48% | 77.72% | 74.58% |  |
|  |  | Qualcomm | 0.32% | 0.26% | 1.26% | 0.33% |  |
|  |  | ZTE | 0.00% | 0.68% | 2.88% | 9.12% |  |
|  |  | Average | 0.29% | 3.14% | 27.29% | 28.01% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Nokia | 2.66% | 13.80% | 79.59% | 80.98% |  |
|  |  | Qualcomm | 0.47% | 0.94% | 2.88% | 1.61% |  |
|  |  | ZTE | 0.02% | 1.04% | 7.27% | 18.79% |  |
|  |  | Average | 1.05% | 5.26% | 29.91% | 33.79% |  |

Figure C.3.2.6 Time in Qout simulation data

From the simulation results, the following initial findings are observed:

1.	The time in Qout of aerial UE is higher than that of terrestrial UE, but the values are quite small in Qualcomm’s results and very large in Nokia’s results.

- C.3.2.7 Ping-Pong rate

Table C.3.2.7: Ping-Pong rate simulation data

| Handover metrics | UE Speed [km/h] | UE Speed [km/h] Company | 0m | 50m | 100m | 300m | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ping pong rate [%] | 3km/h | Nokia | 6.44% | 5.38% | 2.50% | 0.00% |  |
|  |  | Qualcomm | 35.64% | 26.36% | 25.15% | 25.10% |  |
|  |  | Average | 21.04% | 15.87% | 13.83% | 12.55% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 30km/h | Nokia | 6.82% | 8.01% | 2.70% | 0.29% |  |
|  |  | Qualcomm | 35.55% | 24.99% | 22.58% | 23.22% |  |
|  |  | Average | 21.18% | 16.50% | 12.64% | 11.75% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 60km/h | Nokia | 7.00% | 11.74% | 3.23% | 0.75% |  |
|  |  | Qualcomm | 34.18% | 23.34% | 22.30% | 19.62% |  |
|  |  | Average | 20.59% | 17.54% | 12.76% | 10.19% |  |
|  |  | UE Speed [km/h] Company | 0m | 50m | 100m | 300m |  |
|  | 160km/h | Nokia | 15.00% | 19.29% | 4.27% | 2.61% |  |
|  |  | Qualcomm | 26.82% | 19.13% | 19.33% | 17.96% |  |
|  |  | Average | 20.91% | 19.21% | 11.80% | 10.29% |  |

Figure C.3.2.7 Ping-Pong rate simulation data

From the simulation results, the following initial findings are observed:

1.	Different trends of Ping-Pong rate are observed. In Nokia’s results the Ping-Pong rate reach the top at 50m height and decreases as height increases continually. In Qualcomm’s results the Ping-Pong rate of terrestrial UE is higher than that of aerial UE, and as height increases the Ping-Pong rate continue to lower down too.

- C3.3 Conclusions on mobility performance for Drones

Based on the simulation results from all companies, we have the following observations:

1. UMa scenario:

| KPI | Observations |
| --- | --- |
| Handover rate | 1. From the simulation results from Nokia, Ericsson and Qualcomm (3km/h and 30km/h), it can be observed that the handover rate of terrestrial UEs is higher than that of aerial UEs. And as height increases the handover rate firstly decreases, then increases slightly within a small range. 2. From the simulation results from DCM, Huawei, ZTE and Qualcomm (60km/h and 160km/h), it can be observed that the handover rate of terrestrial UEs is lower than that of aerial UEs. And as height increases the handover rate firstly increases obviously, then decreases slightly. |
| HOF rate | 1. Majority of the companies observed higher HOF rate for aerial UE in most cases. And the higher the speed or the height of aerial UE is, the higher HOF rate can be observed. 2. The average HOF rate of the all the companies’ results is higher for the aerial UE than that for the terrestrial UE. |
| RLF rate | 1. Majority of the companies observed higher RLF rate for aerial UE in most cases. And the higher the speed of aerial UE is, the higher RLF rate can be observed. 2. The average RLF rate of the all the companies’ results becomes higher for the aerial UE as the height increases. |
| Time in handoff | 1. Majority of the companies observed larger Time in handoff for aerial UE at least in moderate and high speed cases. For low speed cases, e.g. 3km/h, contrary trends from two companies are observed. 2. The average Time in handoff of the all the companies’ results is higher for aerial UE than that of terrestrial UE in case of moderate and high speed. |
| Time in Qout | 1. Majority of the companies observed larger Time in Qout for aerial UE in all speed cases. 2. The average Time in Qout of the all the companies’ results increases firstly, then decreases slightly as the height becomes higher. |
| Ping-Pong rate | 1. Majority of the companies observed lower Ping-Pong rate for aerial UE in high speed cases. 2. The average Ping-Pong rate of the all the companies’ results decreases slightly as the height becomes higher. |

2. RMa scenario:

| KPI | Observations |
| --- | --- |
| Handover rate | 1. Majority of the companies observed higher handover rate for aerial UE in most cases. And the higher the speed or the height of aerial UE is, the higher handover rate can be observed. 2. The average handover rate of the all the companies’ results is higher for the aerial UE than that for the terrestrial UE. |
| HOF rate | 1. From the simulation results of ZTE, the HOF rate of terrestrial UE is lower than that of aerial UE, and the HOF rate keeps increasing as the height grows. 2. From the simulation results of Qualcomm, for speed lower than 30km/h, the HOF rate of terrestrial UE is higher than that of aerial UE. For speed higher than 60km/h, the HOF rate of aerial UE at 100m height reaches the top, obviously higher than that of terrestrial UE. At other heights the HOF rate is lower than that of terrestrial UE. |
| RLF rate | 1. Majority of the companies observed higher RLF rate for aerial UE in most cases. And the higher the speed of aerial UE is, the higher RLF rate can be observed. 2. The average RLF rate of the all the companies’ results become higher for the aerial UE, then decreases obviously as the height increases. |
| Time in handoff | 1. The Time in handoff of terrestrial UE is lower than that of aerial UE, and as height increases the Time in handoff becomes larger first and then decreases slightly. |
| Time in Qout | 1. The time in Qout of aerial UE is higher than that of terrestrial UE, but the values are quite small in Qualcomm’s results and very large in Nokia’s results. |
| Ping-Pong rate | 1. Different trends of Ping-Pong rate are observed. In Nokia’s results the Ping-Pong rate reach the top at 50m height and decreases as height increases continually. In Qualcomm’s results the Ping-Pong rate of terrestrial UE is higher than that of aerial UE, and as height increases the Ping-Pong rate continues to lower down too. |

Based on the study, the conclusion was reached that further studies should be carried for mobility issues of the drones.

/************************ End of Text Proposal **************************/

- References

- R2-1713329 Summary of email discussion [99bis#31][LTE/UAV]  Capture handover simulation results with observations, Huawei, HiSilicon