3GPP TSG-RAN2 Meeting #100	R2-1713339

# Reno, Nevada, 27 November- 1 December 2017

Agenda item:	19.5

Source: 	Huawei, HiSilicon

Title: 	MAC Structure and Operation for URLLC

Document for:	Discussion and Decision

Introduction

In last RAN2 #99bis meeting, the following agreements on the packet duplication were reached.

Agreements:

1	PDCP data duplication for LTE shall assume NR PDCP data duplication as baseline.

2	RAN2 works on PDCP data duplication for both CA and DC.

3a	At least UM bearers are supported for PDCP duplication via CA.

4	PDCP enables reordering and duplication detection when PDCP duplication is configured.

6	MAC CE is used for activation and deactivation of PDCP duplication for each RB configured with duplication.

7	For CA case, LCP applies configured LCH to carriers/cells restriction for LCHs of a duplication RB and the restriction is lifted when duplication is deactivated as agreed in NR.

8	PDCP duplication is configured by RRC. The configuration also indicates whether the duplication is immediately started, which is the same as NR.

9	LCH to carriers/cells restriction is configured for CA duplication.

In this contribution, we present our views on MAC optimization for packet duplication and link selection to support URLLC.

Discussion

Data duplication scheme based on CA operation will be discussed in the following section..With carrier aggregation, the single data stream needs at some point to be duplicated at MAC layer, after the scheduling handling and multiplexing, and delivered via multiple component carriers. And three alternatives for the logical channels mapping to HARQ entities/ HARQ processes of component carriers will be elaborated in the following section.

  - MAC Structure/ HARQ Model for Data Duplication in CA mode

MAC Structure/ HARQ Model for Data Duplication in CA mode

- Alternative 1: One HARQ entity  per component carrier

- Alternative 2: One HARQ entity for the entire/multiple aggregated component carriers

  - 2-1: One HARQ process per component carrier

  - 2-2: One HARQ process for the entire/multiple aggregated component carriers, with different redundancy version

Figure 2: MAC structure of Alternative 1 for UL

Figure 3: MAC structure of Alternative 2-1 and 2-2 for UL

Alternative 1 is straight-forward structure and is also well matched to per-component-carrier processing as in LTE. The duplicated data transmits by individual HARQ process of individual HARQ Entity. For example, one URLLC TB could be generated after the scheduling handling and multiplexing, and delivered n TB copies to n HARQ entities of n CCs. These HARQ entities of n CCs will transmit the input TB via their HARQ process separately. Due to the transmission /reception over different HARQ entities/HARQ processes, the soft combination cannot be executed.

Alternative 2-1 shares several properties with alternative ‎1. The difference is the duplicated data transmits by individual HARQ process of a common HARQ Entity. In this alternative, packet duplication across multiple redundant links managed by a common HARQ entity is used to target the URLLC requirement, with optimum resource usage, e.g. pre-emption eMBB resource for URLLC. For example, one URLLC TB could be generated after the scheduling handling and multiplexing, and delivered 4 TB copies to 4 HARQ processes belong to one HARQ entity of 4 CCs. The shared HARQ entity of 4 CCs will transmit the input TB via their HARQ processes of 4 CCs separately. Due to the transmission /reception over different HARQ processes of a shared HARQ entity, the soft combination can be executed with optimization, e.g. correct RV NO. Configuration, allowing the soft HARQ combination over different HARQ processes.

Fig Alternative 2-1

Alternative 2-2 provides more compact data process by sharing a common HARQ process for all involved component carrier transmitting same data packet. For example, one URLLC TB could be generated after the scheduling handling and multiplexing, and delivered 4 TB copies to one shared HARQ process belong to one HARQ entity of 4 CCs. The shared HARQ entity of 4 CCs will transmit the input TB via one shared HARQ process of 4 CCs with different RV number separately. Due to the transmission /reception over one shared HARQ process, the soft combination can be naturally executed. And in the alternative 2-2, several component carries jointly transmits a same data packet in a HARQ process so that only one L1/L2 grant is needed to schedule the transmission and only one HARQ feedback signal is sent from the receiver side. In this alternative, the channel coding and diversity gain could be increased due to the different channel/interference conditions of different component carriers. Moreover, this achieves efficient HARQ operation due to a single HARQ is performed across the involved component carriers. Hence, it can reduce the HARQ handling latency compared to above two approaches. And the method is similar as TTI bundling in LTE, just a kind of bundling in frequency domain.

Observation 1: Alternative 2-2 enables efficient HARQ operation due to a single HARQ is performed across the involved component carriers. Hence, it can reduce the HARQ handling latency and naturally support HARQ soft combination across component carriers compared to above two approaches. And the method is similar as TTI bundling in LTE, just a kind of bundling in frequency domain.

Observation 2：RAN1 need to be involved for the study of the MAC structure which allows one HARQ process configured across multiple aggregated component carriers.

Fig Alternative 2-2

The two schemes are compared in Table 1.

Table 1. MAC Structure Alternatives

| Aspect | Alternative 1 | Alternative 2-1 | Alternative 2-2 |
| --- | --- | --- | --- |
| Number of HARQ aspect | The same number of HARQ entities and HARQ processes as LTE | Smaller number of HARQ entities than that of LTE and same number of HARQ processes as LTE | Smaller number of HARQ entities and processes than that of LTE |
| Frequency diversity and coding gain | Same as LTE | Same as LTE | Higher frequency diversity and coding gain |
| Link adaptation performance | Same: separate MCS across multiple component carriers | Same: separate MCS across multiple component carriers | Same: separate MCS across multiple component carriers |
| HARQ performance | Due to the transmission /reception over different HARQ entities/HARQ processes, the soft combination cannot be executed. | Due to the transmission /reception over different HARQ processes of a shared HARQ entity, the soft combination can be executed with optimization, e.g. correct RV NO. Configuration, allowing the soft HARQ combination over different HARQ processes. | Due to the transmission /reception over one shared HARQ process, the soft combination can be naturally executed. |
| Latency | Same as LTE | Same as LTE | Smaller than that of LTE via one HARQ processing |
| Reliability | Increased via duplication | Increased via duplication | Increased via duplication |

Proposal 1: It is proposed that the MAC structure allows one HARQ process configured across multiple aggregated component carriers, if it is desirable, with different redundancy version during data duplication operation for URLLC services.

Conclusion

By analysing the detailed delivery procedures of other SI, we made the following proposals in HRLLC:

Observation 1: Alternative 2-2 enables efficient HARQ operation due to a single HARQ is performed across the involved component carriers. Hence, it can reduce the HARQ handling latency and naturally support HARQ soft combination across component carriers compared to above two approaches. And the method is similar as TTI bundling in LTE, just a kind of bundling in frequency domain.

Observation2：RAN1 need to be involved for the study of the MAC structure which allows one HARQ process configured across multiple aggregated component carriers.

Proposal 1: It is proposed that the MAC structure allows one HARQ process configured across multiple aggregated component carriers, if it is desirable, with different redundancy version during data duplication operation for URLLC services. However, RAN1 is required to be involved in the study.